3GPP TSG-RAN WG4 Meeting # 113	R4-2419114

# Orlando, US, 18th – 22nd November, 2024

Source: 	China Telecom

Title: 	Further study on SRS IL imbalance

Agenda Item:	7.1.1.3.4

Document for:	Discussion

Introduction

In RAN4#112-bis, companies still submitted bulks of proposals with diverse solutions for SRS IL imbalance issue. But we have made some progress anyway and one candidate solution from Intel was captured in WF [1] as below.

The candidate solution can also cover our proposals so in this paper we reiterate our view and provide further analysis.

Discussion

- 2.1 General clarification

Since there are a few of companies still object to solve SRS IL imbalance issue in this WI, we keep the necessity analysis section which is the resubmission of R4-2412434, and our view about this part is unchanged.

With the introduce of 6Rx UE the power imbalance between SRS ports are more non-negligible and the value of ∆TRxSRS will be higher. For 8Rx handheld UE, the IL relaxation in TS 38.101-1 could reach 10.3dB at some scenarios and it is too large in performance evaluation.

Proposal1: 6Rx SRS IL is non-negligible that shall be solved in Rel-19 in this WI.

Due to the diversity of UEs implement the compensation mechanism at UE side is unpredictable much less the limited power headroom. On the one hand, the 3GPP specification only address the upper boundary of SRS IL issue. On the other hand, the relaxation in PCMAX_L,f formula are ∆TC,c, ΔTIB,c, and ∆TRxSRS and each of them contributes to the MOP so the IL imbalance is difficult to acquire for UE, which most likely reflect directly to the NW.

The actually power needed for SRS transmission can be assume as:

PSRS,total = PSRS,b,f,c(I,qs,l) + PIL

Where PSRS,b,f,c(I,qs,l) can be calculated according to TS 38.213.

If PSRS,total ≤ PCMAX_H,f,c, which means the power headroom is enough for UE to compensate the SRS IL (i.e. if PSRS,total ≤ PCMAX,f,c, UE can simply increase PSRS,total to compensate IL and do not have to change PCMAX,f,c; if PSRS,total ≥ PCMAX,f,c, UE also have chance to compensate IL through increasing PCMAX,f,c since the upper bound will not be exceeded.);

If PSRS,total ≥ PCMAX_H,f,c, UE is impossible to compensate the total SRS IL, but can still compensate partial of it by extending output power to PCMAX_H,f,c.

Therefore, UE self-compensation could be a simple and effective solution in some cases. For most of the scenarios, the power headroom could be enough to perform SRS IL compensation and it could be a baseline approach.

Proposal2: UE self-compensation could be a baseline approach.

- 2.3 Potential solutions

- Signaling aspects

First of all, self-compensation is an optional feature in Rel-19, so UE should report whether supporting SRS IL self-compensation or not and activated by configuration.

Proposal3: UE should report whether supporting SRS IL self-compensation or not and activated by configuration.

- Assistance information

For assistance information, no matter power headroom or MOP or IL value reporting, the purpose is assisting NW to know the output power condition when doing compensation. The pre-compensation maybe not fully compensated due to limited power headroom, and this information could help NW to perform post-compensation to correct the PMI. So the simplest way is UE reports whether SRS insertion loss has been fully compensated when do self-compensation.

Proposal4: For assistance information the simplest way is UE reports whether SRS insertion loss has been fully compensated when do self-compensation.

The signaling above is just a 1bit signal which is not contain numerical power information for post-compensation calculating and this is also why we always propose to report SRS power headroom and insertion loss value.

For SRS power headroom reporting, there are already defined in legacy Spec. in TS 38.321&38.213, so we can only care about the granularity and triggering for SRS IL imbalance issue. But it can only be reported dynamically with more timely but more signaling overhead. The insertion loss value supports static and dynamic reporting that UE can report ∆TRxSRS at the very beginning or the real-time value per resource, and it’s more precise and intuitive for NW to utilize since directly reflecting the imbalance situation. But as a new feature, we need much time to study the Spec. impact and the real execution is more complex.

The advantages and disadvantages can be summarized as below.

|  | Ad | Dis |
| --- | --- | --- |
| power headroom reporting | Spec. friendly and less modified; More timely | More signaling overhead; |
| insertion loss value reporting | Designed flexibly; More precise and intuitive | More complex; Far to reach agreement |

Observation1:

|  | Ad | Dis |
| --- | --- | --- |
| power headroom reporting | Spec. friendly and less modified; More timely | More signaling overhead; |
| insertion loss value reporting | Designed flexibly; More precise and intuitive | More complex; Far to reach agreement |

Conclusion

Observation1:

|  | Ad | Dis |
| --- | --- | --- |
| power headroom reporting | Spec. friendly and less modified; More timely | More signaling overhead; |
| insertion loss value reporting | Designed flexibly; More precise and intuitive | More complex; Far to reach agreement |

Proposal1: 6Rx SRS IL is non-negligible that shall be solved in Rel-19 in this WI.

Proposal2: UE self-compensation could be a baseline approach.

Proposal3: UE should report whether supporting SRS IL self-compensation or not and activated by configuration.

Proposal4: For assistance information the simplest way is UE reports whether SRS insertion loss has been fully compensated when do self-compensation.

References

- R4-2417209, WF on for 6Rx UE requirements, RAN4#112-bis, AT&T