3GPP TSG-RAN WG4 Meeting #113	R4-2419149

Orlando, US, 18 – 22 November, 2024

Title: 	Discussion on RRM impacts for SBFD operation

Source: 	Huawei, HiSilicon

Agenda item:	7.20.3.2

Document for:	Discussion

Introduction

RRM requirements for SBFD are discussed in RAN4#112-bis, and the outcomes are captured in WF [1]. Based on the discussion, following issues needs to be further discussed for impacts of SBFD operation.

- SSB based measurement

- CSI-RS based measurement

- Scheduling restriction

- MG and BWP switch

In this paper, we will provide our views on impacts of SBFD operation on RRM requirements.

Discussion

SSB based measurement

Issue 1-2-1: Requirements for SSB based measurement - Agreements (from online session) - Down-select one of the two options: - Option 1: - As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission. - This can be revisited if it proves to be conflicting with further RAN1 agreements. - Option 2: - For inter-frequency measurement without gap, UL transmission is prioritized. - Follow option 1 in other cases.

We support option 1.

In existing requirements, all SSB based neighbour cell measurement, including inter-frequency measurement without gap, are prioritized over UL. We do not see clear motivation to have a different rule for inter-frequency measurement without gap.

Inter-frequency measurement without gap is depending on certain NW deployment (inter-frequency SSB within UE active BWP) or certain UE capability (NFG or NCSG), while most of the measurements are intra-frequency measurement without gap and inter-frequency measurement with gap. Even some inter-frequency carriers are measured without gap, their SMTC may overlap with other measurements, and NW can still not schedule UL transmission on the overlapping SBFD slots since NW does not know which exact carrier UE is measuring, so in real deployment not much UL resources can be saved by this special rule. It is also inconsistent that only the UL transmission on SBFD symbols are prioritized while UL transmission on normal UL symbols are still de-prioritized.

On the other hand, making inter-frequency measurement without gap an exception as in option 2 will increase both UE and spec complexity, i.e. UE needs to check whether there is UL transmission before performing a certain type of SSB measurement (all other SSB measurement are performed unconditionally). From spec perspective, RAN4 may need to discuss whether both dynamically scheduled UL and semi-statically configured UL are prioritized over measurement, and what is the impact to the measurement requirements.

Proposal 1: As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission. This can be revisited if it proves to be conflicting with further RAN1 agreements.

CSI-RS based measurement

Issue 1-2-2: Requirements for CSI-RS based measurement - Agreements (from online session) - RAN4 to discuss whether and how L1 CSI-RS measurement requirements would be impacted by the following SBFD operations. - CSI-RS colliding with dynamically scheduled UL transmission - CSI-RS puncturing in frequency domain in SBFD symbols - CSI-RS distributed over SBFD and non-SBFD symbols - Align the understanding whether above SBFD operations are applicable for CSI-RS for RLM, BFR and L1-RSRP, or they are only applicable for CSI-RS for CSI. - RAN4 to discuss whether and how L3 CSI-RS measurement requirements would be impacted by SBFD operations, and further RAN1/2 conclusions can be considered.

CSI-RS colliding with dynamically scheduled UL transmission

In our view, CSI-RS dropping when colliding with dynamic UL is applicable to CSI-RS for L1-RSRP, and also CSI-RS for RLM/BFR. Following is the RAN1#116-bis agreement regarding collision handling between UL and DL on SBFD symbols, and it can be seen that it is generic for all semi-statically configured DL reception, and there is no exception for CSI-RS for particular usage.

Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UE does not receive DL channel/signal. - The above does not imply link direction indication is supported - FFS on dynamically scheduled UL transmission with repetition

When dynamic UL transmission is scheduled to be colliding with the CSI-RS resource, the measurement occasion will be dropped and the measurement period should be extended accordingly. RAN4 can further discuss whether and how to define the extension, and in our view, we can re-use the approach as in NR-U where the measurement occasion may get unavailable due to dynamic LBT outcome.

CSI-RS puncturing in frequency domain in SBFD symbols

In our view, CSI-RS puncturing outside DL usable PRBs is applicable to CSI-RS for L1-RSRP, and also CSI-RS for RLM/BFR. The fundamental limitation is that gNB will not be able to transmit anything in the UL subband, so all CSI-RS resources will be punctured in frequency domain.

All CSI-RS measurement requirements apply based on a condition that CSI-RS BW is larger than a threshold. For example, CSI-RS based RLM requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 24 RBs. It is clear that CSI-RS REs mapped outside DL usable PRBs should not be counted in the transmitted BW, and some clarification may be needed on the transmitted BW.

In last meeting, companies listed 3 cases with frequency domain puncturing:

- Case 1: residual CSI-RS BW within DL subband#1 is larger than threshold, residual CSI-RS BW within DL subband #2 may be larger or smaller than threshold

- Case 2: residual CSI-RS BW within each DL subband is smaller than threshold, but residual CSI-RS BW across 2 DL subbands is larger than threshold

- Case 3: residual CSI-RS BW across 2 DL subbands is smaller than threshold

In our view, the existing requirements could apply for Case 1. For Case 3, no requirement would apply. For Case 2, more discussion may be needed. It is clear that existing requirement could not apply because the measurement performance with 48 RBs across 2 subbands would be worse than with consecutive 48 RBs. The question is whether it is a meaningful scenario to define requirements.

CSI-RS distributed over SBFD and non-SBFD symbols

In our view, when a CSI-RS resource has occasions in both SBFD and non-SBFD symbols, restriction to one symbol type is applicable to CSI-RS for L1-RSRP, and FFS for CSI-RS for RLM/BFR. For CSI reporting, which also includes L1-RSRP reporting, RAN1#118-bis made the following agreement. As RLM/BFR is not part of the CSI reporting framework, we understand it is still unclear.

Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

With restriction of occasions to one symbol type, UE is only supposed to measure those occasions on the selected symbol type (either SBFD symbol or non-SBFD symbol). The measurement period requirements would depend on the time duration to collect enough samples on the resource occasions on the selected symbol type rather than the time duration to collect enough samples on all resource occasions (defined by sample number multiplied by the resource periodicity).

Summary

Above we discussed whether each of the SBFD operation is applicable to CSI-RS for L1-RSRP and CSI-RS for RLM/BFR. Our view is listed in Table 1.

Table 1: Applicability of SBFD operation to CSI-RS resource of different usage

|  | Dropped when colliding with dynamic UL | Punctured outside DL usable PRBs | Restricted to one symbol type with occasions on both symbol types |
| --- | --- | --- | --- |
| CSI-RS for CSI | yes | yes | yes |
| CSI-RS for L1-RSRP or L1-SINR | yes | yes | yes |
| CSI-RS for RLM/BFD/CBD | yes | yes | FFS |

We also analysed the impact of each SBFD operation on CSI-RS measurement requirements. Combined with Table 1, we have the following proposals for L1-RSRP and RLM/BFR requirements.

Proposal 2: For CSI-RS based L1-RSRP measurement,

- Measurement period is defined based on time duration to collect enough resource occasions on the selected symbol type

- If the selected symbol type is SBFD symbol, measurement period is extended to account for the dropped occasions due to collision with dynamic UL

- Measurement requirements apply when residual CSI-RS BW within at least one DL subband is larger than 48 RB, FFS for the case when residual CSI-RS BW across two DL subband is larger than 48 RB

Proposal 3: For CSI-RS based RLM/BFR measurement,

- FFS whether measurement period is defined based on time duration to collect enough resource occasions on the selected symbol type or on all symbol types

- If resource occasions on SBFD symbol are used, measurement period is extended to account for the dropped occasions due to collision with dynamic UL

- Measurement requirements apply when residual CSI-RS BW within at least one DL subband is larger than 24 RB, FFS for the case when residual CSI-RS BW across two DL subband is larger than 24 RB

For L3 CSI-RS measurement, as there is no enhancement from RAN1 or RAN2, we understand the UE measurement behavior would be same as in legacy, and in SBFD is used in cells transmitting CSI-RS for mobility, it should be made transparent to the measuring UE. In other words, the measuring UE will perform the measurement based on measurement configuration as in legacy, as if SBFD is not used in any cell transmitting CSI-RS for mobility.

For the requirements, we understand existing requirements would apply under the condition that all CSI-RS resources are available according to the configuration, e.g. no CSI-RS RE is dropped due to SBFD operation in the transmitting cell, as compared to configured CSI-RS BW.

Proposal 4: For CSI-RS based L3 measurement, existing requirements apply provided that all CSI-RS resources are available according to the configuration.

Scheduling restriction

One issue in scheduling restriction is whether to update the existing requirements to take DL/UL transition time into account. For example, RAN4 has defined scheduling restriction for TDD for L3 measurement but not for L1 measurement. The reason is that there is always guard period for the DL-to-UL transition, and UL transmission on UL symbols would never overlap with SSB reception for the serving cell in a reasonable configuration. However, there is no guard period between SBFD symbol and SSB symbol. UL transmission on SBFD symbol immediately after SSB symbol may collide with SSB reception due to TA.

Based on discussions in last meeting, we understand such a collision is already addressed by the following RAN1#116 agreement. Therefore, we suggest RAN4 not to update the existing scheduling restriction requirements to account for DL/UL transition time.

Agreement For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

Proposal 5: RAN4 not to update existing scheduling restriction requirements to account for DL/UL transition time.

Another issue raised up by some companies last meeting is the case when SSB or CSI-RS for neighbor cell is not within DL subband of UE’s serving cell. In our view, this means the measurement is inter-frequency. For CSI-RS, it would be measured in gaps. For SSB, it may be measured outside gaps if UE supports R16 inter-frequency measurement without gap and the SSB is within UE active BWP. In this case, UE would measure SSB outside DL subband, and the question is whether UE can receive data within the DL subband at the same time. We are open to discuss this issue.

- On one hand, UE should be able to receive SSB and DL data at same time within active BWP, e.g. if it is a normal DL symbol.

- On the other hand, UE is not expected to receive outside DL subband on SBFD symbol (we understand this is for serving cell DL).

Proposal 6: RAN4 to discuss scheduling restriction when SSB for L3 measurement is not completely contained in active DL subbands on SBFD symbols/slots.

MG and BWP switch

We do not see impacts of SBFD operation to BWP or CBW switch, as those procedures do not rely on measurement. However, some clarification may be needed because the requirements for DL (and UL) BWP switch are currently defined based on DL (and UL) transmission only on DL (and UL) slot. DL (and UL) transmission on SBFD symbols needs to be accounted.

For DCI-based BWP switch, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay + Y which starts from the beginning of DL slot n. Where, …

We do not see impacts of SBFD operation to the MG related requirements, but one clarification may be needed for the UL transmission immediately after MG. Currently it is only defined for UL slot, while with SBFD operation, the UL transmission can be on SBFD symbol.

It is up to UE implementation whether or not the UE is able to conduct transmission in the following slot(s), - when MGTA is not applied, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after measurement gap - when MGTA is applied and the SCS of the UL carrier is other than 15kHz, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after measurement gap - when MGTA is applied and the SCS of the UL carrier is 15kHz, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after the slot partially overlapped with measurement gap where UL slot denotes that all the symbols in the slot are uplink symbols, and L=1 if for the UL transmission is less than the length of one slot; L=2 otherwise.

Proposal 7: RAN4 to update the term “UL slot” to “UL slot and SBFD slot” in MG and BWP switch requirements when SBFD operation is enabled.

Conclusions

In this paper we provided our views on impacts of SBFD operation on RRM requirements.

Proposal 1: As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission. This can be revisited if it proves to be conflicting with further RAN1 agreements.

Proposal 2: For CSI-RS based L1-RSRP measurement,

- Measurement period is defined based on time duration to collect enough resource occasions on the selected symbol type

- If the selected symbol type is SBFD symbol, measurement period is extended to account for the dropped occasions due to collision with dynamic UL

- Measurement requirements apply when residual CSI-RS BW within at least one DL subband is larger than 48 RB, FFS for the case when residual CSI-RS BW across two DL subband is larger than 48 RB

Proposal 3: For CSI-RS based RLM/BFR measurement,

- FFS whether measurement period is defined based on time duration to collect enough resource occasions on the selected symbol type or on all symbol types

- If resource occasions on SBFD symbol are used, measurement period is extended to account for the dropped occasions due to collision with dynamic UL

- Measurement requirements apply when residual CSI-RS BW within at least one DL subband is larger than 24 RB, FFS for the case when residual CSI-RS BW across two DL subband is larger than 24 RB

Proposal 4: For CSI-RS based L3 measurement, existing requirements apply provided that all CSI-RS resources are available according to the configuratio

Proposal 5: RAN4 not to update existing scheduling restriction requirements to account for DL/UL transition time.

Proposal 6: RAN4 to discuss scheduling restriction when SSB for L3 measurement is not completely contained in active DL subbands on SBFD symbols/slots.

Proposal 7: RAN4 to update the term “UL slot” to “UL slot and SBFD slot” in MG and BWP switch requirements when SBFD operation is enabled.

Reference

- R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon