3GPP TSG-RAN WG4 Meeting #113	R4-2419150

Orlando, US, 18 – 22 November, 2024

Title: 	Discussion on RRM requirements for RedCap UE in NTN

Source: 	Huawei, HiSilicon

Agenda item:	7.26.4.1

Document for:	Discussion

Introduction

RRM requirements for RedCap UE in NTN are discussed in RAN4#112-bis, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to further discuss the following issues.

- RRM requirements

- Impact of 1Rx

- Impact of HD-FDD

- Other

In this paper, we will provide our views on RRM requirements for RedCap UE in NTN.

Discussion

RRM requirements

Issue 2-2-1: Further discussion on cell reselection requirement for (e)RedCap UE with FR1-NTN Online agreement <Agreement>: - The existing cell reselection requirements in RRC_IDLE/RRC_INACTIVE state defined for Rel-17 and Rel-18 (e)NTN can be the baseline for (e)RedCap 2Rx UE with FR1-NTN bands. - Both time-based and location-based cell reselection measurement criteria will be defined. - For both earth fixed and earth moving scenario - Both NTN-NTN and NTN-TN (from NTN to TN and from TN to NTN) cell reselection requirements will be defined. - FFS for the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

We support to define requirements for cell reselection requirement for inter-RAT NR NTN to E-UTRA TN measurement. The requirements have been defined for normal UE in NTN, and we do not see clear reason not to define them for RedCap UE in NTN, especially considering for RedCap UE in TN there are also requirements for inter-RAT E-UTRA measurement. On the other hand, we understand this is not aligned with the agreement from RAN4#112, so we are also fine not to define requirements if there is no consensus to revert the agreement in RAN4

Proposal 1: For (e)RedCap UE with FR1-NTN, define cell reselection requirement for inter-RAT NR NTN to E-UTRA TN measurement.

Issue 2-2-3: The Requirements to be further determined Online agreement <Agreement>: - Define NR Rel-17 and Rel-18 Conditional Handover requirements. - Include location-based CHO and time-based CHO. - Support earth moving cell scenario - Define RACH-less Handover requirements. - Define SDT requirements (including CG-SDT and RA-SDT) in RRC_INACTIVE state. - FFS TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN. - FFS: MDT requirements in RRC_IDLE and RRC_INACTIVE state.

For SDT requirements, we do not support to define requirements related to TA validation. RSRP change is used reflect the between UE and BS, and when the change in distance is larger than a threshold, TA will be considered as invalid. This works for TN, but for NTN the change in distance is already compensated by UE in UE specific TA, so RSRP change is not a good metric for TA validation. It is also noted that for PUR in IoT NTN, requirements related to RSRP change based TA validation are not defined.

We do not support to define requirements for MDT. We do not have MDT requirement for RedCap UE in TN, and there is no strong reason why RedCap UE performing MDT is important in NTN.

Proposal 2: For (e)RedCap UE with FR1-NTN,

- Do not define requirements related to TA validation in SDT

- Do not define requirements for MDT

Issue 2-3-1: RedCap R17 relaxed measurement Online agreement <Agreement>: - FFS: R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for normal UE are applicable for (e)RedCap UE with FR1-NTN bands. - FFS: RAN4 to define Rel-17 RedCap relaxed measurement requirements for cell reselection for (e)RedCap UE with FR1-NTN bands, and the corresponding requirements defined for RedCap UE can be the baseline. - If defined, the requirements apply provided that target cell is GSO

We support to define relaxed measurement requirements for cell reselection for RedCap UE in NTN. We can understand the concern about if the RSRP based condition for measurement relaxation is reasonable in NTN, but the is not an issue specific for RedCap UE. There is no clear reason why relaxed measurement should be supported for normal UE but not RedCap UE.

Also, we support to base the requirements on TN R17 relaxation, as we do not see why it could be used for RedCap UE in TN but not in NTN.

Proposal 3: For (e)RedCap UE with FR1-NTN, define cell reselection requirement for relaxed measurement based on TN R17 relaxation. The requirements apply provided that target cell is GSO.

Issue 2-3-2: eDRX configuration Online agreement <Agreement>: - For GSO: - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN. - For GSO, All DRX and eDRX cycle can be supported, and the DRX and eDRX related requirements for RedCap UE in TN are re-used. - Further discuss for LEO.

We support not to define eDRX requirements for LEO.

For moving cell, it is clear that eDRX is not applicable because even 2.56s DRX cycle was not considered in the requirements in R17. For fixed cell, we do not have strong view, but the Tdetect resulted from 5.12s eDRX cycle is already close to 2min, and it is not small compared to the typical dwell time of 5min.

Proposal 4: For (e)RedCap UE with FR1-NTN, do not define cell reselection requirement with eDRX for LEO scenario.

Issue 2-3-3: Multiple SMTCs <Way Forward>: - Proposal 1 (HW): RAN4 to discuss support of multiple SMTCs for (e)Redcap UE with FR1-NTN bands. - Option 1: make it optional for RedCap UE to support 2 SMTC per MO. - Option 2: change the requirements so that measurement for different SMTCs are not parallel. - Proposal 2 (ZTE): Multiple SMTCs shall be reused for RedCap UEs in NTN scenario. - Proposal 2a (Ericsson): RedCap for NTN supports (at the least) 2 SMTC. - Proposal 3 (OPPO): RAN4 to not consider the support of multiple SMTCs for (e)RedCap UE with FR1-NTN bands. - Proposal 4 (QC): NTN RRM requirements defined for simultaneous measurements on multiple satellites are not applicable for RedCap NTN UE.

NTN is mandatorily required to support 2 SMTCs per MO, and supporting more is an optional capability. When the two SMTCs do not overlap there is no scaling in measurement period meaning UE has to support parallel measurement for the two SMTCs. This may be a bit challenging for RedCap UE with reduced complexity. In our view, multiple-SMTC should still be supported for RedCap UE in NTN from requirement perspective, and there are two ways to address the complexity issue:

- Option 1: make it optional for RedCap UE to support 2 SMTC per MO,

- Option 2: change the requirements so that measurement for different SMTCs are not parallel.

We suggest RAN4 to discuss how to support multiple SMTCs for (e)Redcap UE with FR1-NTN bands.

Proposal 5: RAN4 to discuss support of multiple SMTCs for (e)Redcap UE with FR1-NTN bands.

- Option 1: make it optional for RedCap UE to support 2 SMTC per MO

- Option 2: change the requirements so that measurement for different SMTCs are not parallel

Issue 2-3-4: Concurrent gap <Way Forward>: - Proposal 1 (CMCC, HW): Consider concurrent gap for Redcap over NTN. - Proposal 1a (HW): Concurrent gaps are supported for (e)Redcap UE with FR1-NTN bands, and existing requirements for normal UE in NTN are re-used as baseline. - Proposal 2 (CATT, QC): NTN RRM requirements defined for concurrent measurement gaps are not applicable for RedCap NTN UE. - Proposal 3 (Ericsson): RedCap for NTN supports (at the least) 1 MG.

In our view, concurrent gap in NTN is an optional UE feature, and the requirements for normal UE will most likely be re-used for RedCap UE, so there is no motivation to preclude it. It should be noted that concurrent gap in NTN is mainly motivated by multiple SMTCs per carrier.

Proposal 6: Concurrent gaps are supported for (e)Redcap UE with FR1-NTN bands, and existing requirements for normal UE in NTN are re-used as baseline.

Issue 2-3-5: The number of searchers and CSSF <Way Forward>: - Proposal 1 (CMCC, CATT): Single searcher assumption and the CSSF defined for (e)RedCap UEs in TN will be reused for (e)RedCap UE with FR1-NTN.

We support to assume single searcher for defining CSSF for RedCap UE in NTN, as it is unreasonable to require a RedCap UE to support 2 searchers just because it supports NTN.

Proposal 7: Single searcher assumption and the CSSF defined for (e)RedCap UEs in TN will be reused for (e)RedCap UE with FR1-NTN.

Issue 2-3-6: FH in PRS measurement for NW verified location requirements <Way Forward>: - Proposal 1 (HW): RAN4 to discuss whether to support FH in the requirements for NW verified location for (e)RedCap UE with FR1-NTN bands. - Proposal 2 (Ericsson, OPPO): Not support FH in PRS measurement for RedCap for NTN.

We support not to consider FH in the requirements for NW verified location for (e)RedCap UE with FR1-NTN bands. FH in PRS measurement aims at improving measurement accuracy where the measurement is not restricted by the UE RF BW (20MHz) but can reach cell BW (100MHz). However, the cell BW in FR1-NTN bands are limited (~30MHz), so the gain from FH may be limited.

Proposal 8: Do not to consider FH in the requirements for NW verified location for (e)RedCap UE with FR1-NTN.

Issue 2-3-7: NCD-SSB <Way Forward>: - Option 1: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause on this aspect (i.e, the requirements are only applicable to CD-SSB). - Option 2: Use the terminology of CD-SSB explicitly for (e)RedCap with FR1-NTN requirements. - Note: Other options are not precluded.

We support option 1 for defining requirements. In existing TN requirements, the term SSB is used and a clarification like “The SSB and SMTC in this section applies for both CD-SSB and NCD-SSB if it is not additional specified” is added. It is reasonable to follow the same approach with an updated clarification statement like in option 1. The problem with option 2 is that there will be many changes required across all the SSB related requirements, thus it is very error prone and make spec maintenance very difficult.

Proposal 9: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause that the requirements are only applicable to CD-SSB.

Impact of 1Rx

Issue 3-2-1: RRC_IDLE/RRC_INACTIVE mode mobility for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN requirements for normal UEs. - For 1Rx (e)RedCap UEs with FR1-NTN bands, firstly to discuss the following proposals one by one: - Proposal 1: RAN4 to consider the offset for the reletive cell-specific RSRP thresholds for 1Rx (e)RedCap UEs with FR1-NTN bands, including Qrxlevmin and Qqualmin: - -1 dB offset can be reused. - Proposal 2: RAN4 to reduce the number of NR inter-frequency carriers that UE shall at least be capable of monitoring for 1Rx (e)RedCap UEs with FR1-NTN bands. - 6 NR inter-frequency carriers can be reused. - Proposal 3: The paremeters of Nserv_RedCap, Tdetect,NR_Intra/Inter_RedCap, Tmeasure,NR_Intra/Inter_RedCap and Tevaluate,NR_Intra/Inter_RedCap defined for cell re-selection requirements will be not distinguished between 1Rx and 2 Rx (e)Redcap UE, and the requirements will be the same. - Proposal 4: For the measurements of intra- and inter-frequency NR cells, - +1 dB offset on the threshold of absThreshSS-BlocksConsolidation can be reused as a baseline for 1Rx(e)RedCap UEs with FR1-NTN bands. - For the cases of rangeToBestCell is not configured and rangeToBestCell is configured, +1 dB offset can be reused for better cell ranked for 1Rx (e)RedCap UEs with FR1-NTN bands. - Proposal 5: In RRC_IDLE / RRC_INACTIVE state, +1 dB offset can be reused for a margin in FR1 for reselections when UE evaluating whether a newly detectable inter-frequency cell meets the reselection criteria for 1Rx (e)RedCap with FR1-NTN: - For 1 Rx RedCap by a margin of at least - 6 dB in FR1 for reselections based on ranking or - 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or - 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities. - Further discuss SDT requirements. - MDT requirements are related to issue 2-2-3, RAN4 to discuss them based on the conclusions on issue 2-2-3 later.

We support P1-P5 for 1RX in the WF as they clearly capture all the impacts of 1RX in TN, and we do not see any issue to re-use them for RedCap in NTN.

Another feature that needs discussion is enhanced cell reselection measurement. Based on agreement for Issue 2-2-1, we understand it will be supported for 2RX UE by re-using the existing requirements for normal UE in NTN. For 1RX UE, however, RAN4 needs to discuss new requirements if enhanced cell reselection measurement is to be supported for (e)Redcap UE with FR1-NTN bands. We are open to discuss whether to support it or not for 1RX UE.

Proposal 10: Adopt P1-P5 in the WF for cell reselection requirements for 1RX (e)RedCap UE with FR1-NTN bands. RAN4 to discuss whether to support enhanced cell reselection measurement for 1RX (e)Redcap UE with FR1-NTN bands.

Issue 3-2-2: NR HO requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For NR RACH-based HO requirements, - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RACH-based HO requirements for normal UEs. - For 1Rx (e)RedCap UEs with FR1-NTN bands, RACH-based handover delays should be extended compared to those defined for 2Rx (e)RedCap UEs. - Define the relaxation on Tsearch for unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused. - Relax by 1 sample: Tsearch = 2*Trs for intra-frequency HO - Relax by 2 samples: Tsearch = 5* Trs for inter-frequency HO - Further discuss the requirements for RACH-less HO, Time/location-based CHO with/without L3 measurement and Satellite switching with re-sync.

We support the WF as they clearly capture the impacts of 1RX in TN, and we do not see any issue to re-use them for RedCap in NTN. In addition, it is straightforward to use the same value for RACH-less HO, CHO and satellite switch with re-sync.

Proposal 11: Adopt the WF for HO requirements for 1RX (e)RedCap UE with FR1-NTN bands, and apply the WF also for RACH-less HO, CHO and satellite switch with re-sync.

Issue 3-2-3: RRC Re-establishment requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RRC re-establishment delay requirements for normal UEs. - For 1Rx (e)RedCap UEs with FR1-NTN bands, UE RRC re-establishment delay (TUE_re-establish_delay) should be extended compared to those defined for 2Rx (e)RedCap UEs. - Define the relaxation on Tidentify for known and unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused. - Relax the Tidentify_intra_NR and Tidentify_inter_NR, i by 1 more samples for FR1 known and unknown intra/inter-frequency cell.

Issue 3-2-4: RRC Connection Release with redirection requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RRC Connection Release with redirection requirements for normal UEs. - For 1Rx (e)RedCap UEs with FR1-NTN bands, the delay of RRC connection release with redirection to NR (Tconnection_release_redirect_NR) should be extended compared to those defined for 2Rx (e)RedCap UEs. - Define the relaxation on Tidentify-NR for FR1, and the same relaxation for 1 Rx RedCap UE in TN network can be reused. - Relax the Tidentify-NR by 1 more samples for FR1.

Issue 3-2-5: RA requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy random access requirements for normal UEs. - For 1Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to consider the offset for the following relative cell-specific RSRP thresholds for random access requirements - +1 dB offset can be reused as a baseline for rsrp-ThresholdSSB, msgA-RSRP-ThresholdSSB and msgA-RSRP-Threshold. - -1 dB offset can be reused as a baseline for rsrp-ThresholdMsg3.

Issue 3-3-1: Timing requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - The existing timing requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands. - UE transmit timing - UE timer accuracy - Timing Advance - The timing requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

Issue 3-4-1: RLM/BFD/CBD requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RLM/BFD/CBD requirements for normal UEs. - For 1Rx (e)RedCap UEs with FR1-NTN bands, - For SSB based and CSI-RS based RLM/BFD requirements, some of hypothetical PDCCH parameters including aggregation level (CCE) and bandwidth (PRBs) are defined as follows: - Aggregation level (CCE) of 16. - Bandwidth (PRBs) of 48. - For SSB based and CSI-RS based RLM/BFD requirements, SCS=60kHz is not applicable for (e)RedCap UEs with FR1-NTN. - The SSB and CSI-RS based RLM OOS and BFD evaluation periods should be extended for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - For RLM evaluation periods, - RAN4 not to extend the TEvaluate_in_SSB/CSI-RS for FR1. - RAN4 to extend the TEvaluate_out_SSB/CSI-RS by factor 2 for FR1. - For BFD evaluation periods, - RAN4 to extend the TEvaluate_BFD_SSB/CSI-RS by factor 2 for FR1.

Issue 3-4-2: Other requirements in signaling characteristics for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - The existing following requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands: - Active BWP switch delay, - active TCI state switch delay - UE specific CBW change requirements - Pathloss reference signal switching delay - The above requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

Issue 3-5-1: Measurement requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN measurement requirements for normal UEs. - For 1Rx (e)RedCap UEs with FR1-NTN bands, to discuss the following proposals one by one: - The time period for PSS/SSS detection defined for intra-/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter. - Not to extend the lower boundary. - The time period for SSB index identification (PBCH decoding) defined in intra-/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter. - Extend the lower boundary from ‘120 ms’ to ‘160ms’. - The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Extend the lower boundary from ‘200 ms’ to ‘400ms’ - Keep the same number of samples for T SSB_measurement_period_inter. - The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same. - The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

Issue 3-5-2: Network verified UE location for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, the measurement period and measurement accuracy requirements for Network verified UE location defined in NTN can be reused. - For 1Rx (e)RedCap UEs with FR1-NTN bands, the measurement accuracy requirements defined for 1Rx RedCap positioning can be referred, and the same relaxation for accuracy should be reused.

Issue 3-6-1: Measurement accuracy relaxation for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the measurement accuracy requirements defined in NTN. - For 1Rx (e)RedCap UEs with FR1-NTN bands, support to relax the measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs. - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused. - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB. - L1-RSRP: relax by 3dB. - RAN4 to reuse the conditions defined in NTN for a corresponding Band, and the values for SSB_RP and SSB Ês/Iot are the same for 1Rx (e)RedCap UEs and 2Rx (e)RedCap UEs with FR1-NTN bands.

We support the WF for the above issues, as they clearly capture all the impacts of 1RX in TN, and we do not see any issue to re-use them for RedCap in NTN.

Proposal 12: Adopt the WF for following requirements for 1RX (e)RedCap UE with FR1-NTN bands

- RRC Re-establishment requirements

- RRC Connection Release with redirection requirements

- RA requirements

- Timing requirements

- RLM/BFD/CBD requirements

- Other requirements in signaling characteristics

- Measurement requirements

- Network verified UE location delay and accuracy requirements

- Measurement accuracy requirements.

Impact of HD-FDD

Issue 4-1-1: General HD-FDD related impact <Way Forward>: - Proposal 1 (Xiaomi, CATT, QC, ZTE, CMCC): - RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reaching further conclusions. - Proposal 1a (CATT): - To following the TN, at least the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - Minimum requirement for L1 indication for RLM and LR - MAC-CE/DCI/RRC based uplink spatial relation switch delay - Scheduling availability of UE performing intra/inter measurements - Scheduling availability of UE during L1-RSRP measurement - Proposal 1b (QC): RAN4 to wait for further RAN1 process on the enhancements for HD RedCap and eRedCap UEs which may potentially impact the following aspects: - Additional latencies, Scheduling/Measurement restrictions, etc. due to DL/UL collisions.

P1a lists all the impacted requirements due to HD-FDD support in TN, and we do not see any issue to re-use them for RedCap in NTN. As to the ongoing discussion in RAN1 related to TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB, we understand it may results at collision handling rule for cases that were considered as error case in R17, but so far we do not see the impact to re-using the TN HD-FDD requirements. We are also open to other views.

Proposal 13: Adopt the TN requirements as baseline for the requirements listed in P1a for HD-FDD (e)RedCap UE with FR1-NTN bands.

Issue 4-2-1: The impact on NR Handover due to HD-FDD <Way Forward>: - Proposal 1 (vivo): RAN4 to reuse NR SAN FR1 – NR SAN FR1 Handover requirements for Red-cap NTN and additionally add the following applicability rule for HD-FDD case as current HO requirements for Redcap specified, - For RedCap NTN UE with HD-FDD, the handover requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU.

We believe this issue is already covered by Issue 4-1-1 and does not require separate discussion.

Issue 4-2-2: The impact on NR Satellite switching with re-synchronization due to HD-FDD <Way Forward>: - Proposal 1 (vivo): For requirements on NR Satellite switching with re-synchronization for Red-cap NTN, RAN4 to reuse the existing Satellite switching with re-synchronization requirements for NTN with adding the following applicability rules to adapt HD-FDD case: - For RedCap NTN UE with HD-FDD, the Satellite switching with re-synchronization requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU.

We support P1. Given the similarity between the requirements for normal HO and other mobility procedures, it is reasonable to define the same condition for HD-FDD UE for CHO and satellite switch with re-sync.

Proposal 13: For HD-FDD (e)RedCap UE with FR1-NTN bands, the CHO and satellite switching with re-sync requirements are met provided that

- SSB is available at the UE once every SMTC period during Tsearch

- One SSB is available during T∆

- One SSB is available during TIU.

Other

Issue 5-1-1: Specification structure <Way Forward>: - Proposal 1 (CATT, HW): - Use new sections and section number with new suffix X in the specification to capture requirements for (e)RedCap UE with FR1-NTN bands. - Utilize the reference method for the requirements that can be reused. - Proposal 2 (Nokia): - Adopt RedCap subclauses under NTN sections (under Suffix C).

We support option 1 to define requirements for (e)RedCap UE with FR1-NTN bands. In our view, this is a reasonable way to make the spec clear while avoid too much duplication.

Proposal 14: Use new sections and section number with new suffix X in the specification to capture requirements for (e)RedCap UE with FR1-NTN bands. Utilize the reference method for the requirements that can be reused.

Issue 5-2-1: How to declare the capabilities for (e)RedCap UE with FR1-NTN bands? <Way Forward>: - Proposal 1 (Nokia): - RAN4 to discuss whether a new capability is needed to declare RedCap support of NTN or whether this support is implicit from declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) AND declare support of NTN (nonTerrestrialNetwork-r17)

We understand this issue is in RAN2 scope as the related capabilities are not specifically related to RRM requirements but rather all operation.

Conclusions

In this paper we provided provide our views on RRM requirements for RedCap UE in NTN.

Proposal 1: For (e)RedCap UE with FR1-NTN, define cell reselection requirement for inter-RAT NR NTN to E-UTRA TN measurement.

Proposal 2: For (e)RedCap UE with FR1-NTN,

- Do not define requirements related to TA validation in SDT

- Do not define requirements for MDT

Proposal 3: For (e)RedCap UE with FR1-NTN, define cell reselection requirement for relaxed measurement based on TN R17 relaxation. The requirements apply provided that target cell is GSO.

Proposal 4: For (e)RedCap UE with FR1-NTN, do not define cell reselection requirement with eDRX for LEO scenario.

Proposal 5: RAN4 to discuss support of multiple SMTCs for (e)Redcap UE with FR1-NTN bands.

- Option 1: make it optional for RedCap UE to support 2 SMTC per MO

- Option 2: change the requirements so that measurement for different SMTCs are not parallel

Proposal 6: Concurrent gaps are supported for (e)Redcap UE with FR1-NTN bands, and existing requirements for normal UE in NTN are re-used as baseline.

Proposal 7: Single searcher assumption and the CSSF defined for (e)RedCap UEs in TN will be reused for (e)RedCap UE with FR1-NTN.

Proposal 8: Do not to consider FH in the requirements for NW verified location for (e)RedCap UE with FR1-NTN.

Proposal 9: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause that the requirements are only applicable to CD-SSB.

Proposal 10: Adopt P1-P5 in the WF for cell reselection requirements for 1RX (e)RedCap UE with FR1-NTN bands. RAN4 to discuss whether to support enhanced cell reselection measurement for 1RX (e)Redcap UE with FR1-NTN bands.

Proposal 11: Adopt the WF for HO requirements for 1RX (e)RedCap UE with FR1-NTN bands, and apply the WF also for RACH-less HO, CHO and satellite switch with re-sync.

Proposal 12: Adopt the WF for following requirements for 1RX (e)RedCap UE with FR1-NTN bands

- RRC Re-establishment requirements

- RRC Connection Release with redirection requirements

- RA requirements

- Timing requirements

- RLM/BFD/CBD requirements

- Other requirements in signaling characteristics

- Measurement requirements

- Network verified UE location delay and accuracy requirements

- Measurement accuracy requirements.

Proposal 13: Adopt the TN requirements as baseline for the requirements listed in P1a for HD-FDD (e)RedCap UE with FR1-NTN bands.

Proposal 13: For HD-FDD (e)RedCap UE with FR1-NTN bands, the CHO and satellite switching with re-sync requirements are met provided that

- SSB is available at the UE once every SMTC period during Tsearch

- One SSB is available during T∆

- One SSB is available during TIU.

Proposal 14: Use new sections and section number with new suffix X in the specification to capture requirements for (e)RedCap UE with FR1-NTN bands. Utilize the reference method for the requirements that can be reused.

Reference

- R4-2416865, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)