3GPP TSG-RAN WG4 Meeting #113					R4-2419162

18th Nov, 2024 – 22th Nov, 2024

Source: 	Qualcomm Incorporated

Title: 	Views on 8Rx UE demodulation and CSI requirements with inter-cell and intra-cell inter-user interference

Agenda item:	 7.16.2

Document for:	Discussion

# 1.Introduction

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices:

  - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

    - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline

  - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver

    - Reuse the existing interference model for 2/4Rx UE as a baseline

In Rel-18 topic NR_ENDC_ RF_FR1_enh2 WI, 8Rx performance requirements for SU-MIMO (single user) were discussed. In Rel-19, 8Rx performance requirements with inter-cell, and intra-cell inter-user interference to be defined for below objectives [1].

### In [2], RAN4 agreed on some of the scope related issues, and encouraged companies to run simulations with R17 interference settings. In this paper, we share some of our observations based on simulation results and share our proposals

# 2. PDSCH Demodulation Requirements intra-user inter-cell interference

## 2.1 Rank choice

### From our initial simulation results, the gap in SNR (@ 70% of maximum throughput) required between baseline and simplified receivers is proportional to the rank of serving cell as expected due to better interference rejection achievable with higher number of Rx antennas.

Simulation settings:

- TDD, 30KHz SCS, 40MHz BW

- TDA30-10, ULA

- Rel-17 interference settings with rank1 or rank2

- Antenna configuration: Rank1: 2 x 8; Rank2: 2 x 8 ; Rank4: 4 x 8

Table 2.1 Gap between baseline and simplified receiver

| Scenario | MCS | Baseline receiver gains w.r.t simplified receiver (dB) |
| --- | --- | --- |
| Rank 1 | 19 | 0.3 |
| Rank 2 | 17 | 1.8 |
| Rank 2 | 19 | 1.7 |
| Rank 4 | 13 | 3.5 |

Observation 1: Performance gap between the baseline and simplified receivers widens as rank increases.

Observation 2: For Rank1 with MCS19 (64 QAM), the performance gap between receiver types is too small.

Observation 3: For Rank2, the gap between receivers is substantial and marginally increases between MCS17 and MCS19.

Observation 4: For Rank2, signal power is greater than interference power (INR) by around 1-2 dB for both MCS 17 and MCS 19. Similar SINR range is chosen for Rel-17 4Rx and 2Rx requirements too.

Observation 5: For Rank4, MCS 13 (16 QAM) requires signal power higher than INR by around 3dB.

Requirements with Rank1

Rank1 does not meaningfully differentiate baseline and simplified 8Rx receivers for MCS 17 or MCS 19. To reduce number of requirements, and since Rank1 is already verified with 4Rx, separate 8Rx PDSCH requirements are unnecessary in our view. Rank1 can be used for CQI requirements.

Requirements with Rank2

Rank2 requirements are not specified with 4Rx and only Rank1 requirements are considered in Rel-17. Therefore, to demonstrate capability to reject higher rank interference, and support higher rank, Rank2 shall be considered for demodulation requirements.

From simulation results, it is also noticed that reasonable SINR (around 2dB) can support Rank2 with higher achievable throughputs than Rank1. To further reduce SINR regime, MCS 13 is an alternative.

Requirements with Rank4

To further demonstrate the true 8Rx (baseline) utility and performance gap with a simplified receiver, Rank4 for serving cell, and up to Rank2 for interference cell (with Rel-17 interference model) is a suitable scenario. From simulation results, MCS13 is a good fit to show higher performance gap between the two kinds of receivers.

Field scenarios

Although concerns were earlier expressed on the probability of Rank4 in cell-edge, the Rank4 residency data with 8Rx will be higher than that with 4Rx. Moreover, with rich scattering Rank4 with 8Rx has a higher probability than full rank (Rank4) with 4Rx. Moreover, Rank4 MCS13 can achieve higher throughput than with 2 layers and MCS17 at this SINR regime.

Proposal1: RAN4 to not have Rank1 requirements for inter-cell interference PDSCH demodulation performance requirements.

Proposal 2: RAN4 shall have Rank2 requirements with MCS 17 (Table1) and TDLA 30-10 ULA channel for inter-cell interference demodulation requirements.

Proposal 3: RAN4 shall have Rank4 requirements with MCS 13 and TDLA 30-10 ULA channel inter-cell interference demodulation requirements.

Proposal 4: RAN4 to use Rel-17 interference models for all demodulation requirements.

# 3. PDSCH Demodulation Requirements intra-cell inter-user interference

In [2], RAN4 agreed on few scoping details and agreed to further discuss based on simulation results. Rel-17 interference models in terms of number of interferers, power levels etc. are agreed to be used for simulation studies.

## 3.1 MMSE-IRC (Interference rejection)

The MMSE-IRC fundamentally stays the same for both inter-cell interference and intra-cell inter-user interference cases. The key aspect of interference + noise estimation and then whitening is common for both scenarios. Therefore, our view is the scenarios can be similar in defining requirements for intra-cell inter-user (MU-MIMO) too.

3.2 Rank choice

Rank1

Rank1 and Rank2 scenarios were specified in Rel-17. Extending the rationale in section 2, to keep number of requirements limited, Rank1 scenario can be skipped is our view. Rank2 scenario shall be demonstrate superior 8Rx performance vs 4Rx receiver and shall be considered for demodulation requirements.

Rank4

Rank4 for both serving and interfering cell requires TDD-OCC DMRS (MAXLEN = 2) support which is an optional feature. This implies all the UEs which don’t support this capability are not going to be verified with the requirement. Moreover, such DMRS scheme has a higher DMRS overhead and is not a preferred option for network operators in our view. Furthermore, such DMRS scheme is not considered in RAN4 scenarios other than 8L, 8Rx requirement. R18-enhanced DMRS is a more suitable option to further consider Rank4 case to balance DMRS overhead. Considering above two aspects, we are of the view to not define Rank4 requirements. We are working on simulations and will update during meeting once complete.

Proposal 5: RAN4 to consider Rank2 with MCS 13 and TDLA 30-10 for MU-MIMO demodulation requirements.

Proposal 6: RAN4 to not define MU-MIMO requirements for Rank1 and Rank4.

# 4. CQI requirements

For CQI requirements, two aspects need to be considered based on specs. defined so far. In rel-17, CQI requirements for inter-cell interference case were defined with Rank1 and TDLA30-10 channel. While in Rel-18, 8Rx CQI requirements are specified only for static channel.

With static channel, and AWGN propagation from interferer, MMSE is sufficient and MMSE-IRC  performance shall not be better than MMSE. Therefore TDLA30-10 ULA channel and other Rel-17 settings can be extended to 8Rx inter-cell interference CQI requirements.

Proposal 7: RAN4 to use Rank1 with Rel-17 settings for 8Rx CQI requirements

# 5. Release independence

Since interference requirements are being discussed in Rel-19 feature, release independence shall be considered from Rel-18 for

Proposal 5: Rel-19 8Rx performance requirements shall be independent from Rel-18.

# 6. Conclusions

Following is the summary of the proposals.

Proposal1: RAN4 to not have Rank1 requirements for inter-cell interference PDSCH demodulation performance requirements.

Proposal 2: RAN4 shall have Rank2 requirements with MCS 17 (Table1) and TDLA 30-10 ULA channel for inter-cell interference PDSCH demodulation requirements.

Proposal 3: RAN4 shall have Rank4 requirements with MCS 13 and TDLA 30-10 ULA channel for inter-cell interference demodulation requirements.

Proposal 4: RAN4 to use Rel-17 interference models for all demodulation requirements.

Proposal 5: RAN4 to consider Rank2 with MCS 13 and TDLA 30-10 for MU-MIMO demodulation requirements.

Proposal 6: RAN4 to not define MU-MIMO requirements with Rank1 and Rank4.

Proposal 7: RAN4 to use Rank1 with Rel-17 settings for 8Rx CQI requirements

Proposal 8: Rel-19 8Rx performance requirements shall be independent from Rel-18.

# References

[1] RP-241297, “Revised WID on NR demodulation performance: Phase 5”, 3GPP TSG RAN meeting, #104, June 17– June 20, 2024, Shanghai, China.

[2] R4-2416555, “Way Forward for [112bis][320] NR demodulation performance: Phase 5”, 3GPP TSG RAN meeting, #112bis, Oct14– Oct 18, 2024, Hefei, China.