3GPP TSG-RAN WG4 Meeting #113	R4-2419282

Orlando, USA 18th – 22nd November 2024

Source: 	Nokia

Title: 					NTN 3 MHz A-MPR simulations

Agenda Item: 			7.8.3.2

Document for:	Discussion

# 1	Introduction

We present A-MPR simulation results for band n256 and NS_24. We also discuss the simulation methodology for spectrum emissions measured in 2 ms time windows at NS_03N, NS_04N and NS_05N.

# 2	Simulation assumptions and scenario

### 2.1	n256

- Operating band n256 (UL 1980 MHz – 2010 MHz)

- Power class PC3

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc, CIM5 -70 dBc

- Only additional spurious emission mask from NS_24 is simulated. Other gating factors (ACLR, SEM, etc.) are excluded because they are handled by MPR.

Table 6.5.3.3.13-1: Additional requirements for "NS_24"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |
| 2010 ≤ f ≤ 2025 | -50 | 1 MHz |
| NOTE 1: This requirement applies at a frequency offset equal or larger than 5 MHz from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band. |  |  |

# 3	Simulation results and discussion for NS_24

In this section we present examples of the simulation results. A complete set of A-MPR triangles at Fc = 2003.5 MHz is presented in appendix.

Due to the note 1 in Table 6.5.3.3.13-1 we only consider channels with at least 5 MHz offset from the upper edge of the operating band. A-MPR is not needed when Fc is 2000 MHz or below. In Figure 1 we present the A-MPR triangles with QPSK modulation at Fc = 2003.5 MHz.

Observation 1: A-MPR is needed for n256 NS_24 and 3 MHz CBW in the range 2000 MHz < Fc ≤ 2003.5 MHz.

Figure 1: The simulated A-MPR for NS_24 with QPSK modulation at Fc=2003.5 MHz.

# 4	Simulation methodology for time-limited spectrum analysis

The additional spurious emission requirements for NS_03N, NS_04N, and NS_05N include a time limitation for spectral analysis. In some or all the mask segments, the emission power averaged over any 2 ms time window must not exceed the specified limit.

Here we study the effect of this time limitation on A-MPR simulations, using NS_03N as an example. The additional spurious emission requirements for NS_03N (TS 38.101-5, Table 6.5.3.3.3-1) are shown below. The 2 ms averaging window is very short for the 700 Hz measurement bandwidth (MBW). An FFT with a 700 Hz frequency bin spacing would have a duration of 1.4 ms, close to the 2 ms averaging window. Together, the narrow MBW and short averaging duration result in noisy measurement results which will often significantly exceed their expected value. The measurement noise is caused by the transmitted data and thus can only be mitigated by averaging a sufficient number of spectrum measurements. This is possible to some extent by calculating the FFTs from heavily overlapping time windows.

Table 6.5.3.3.3-1: Additional out-of-band requirements for "NS_03N"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |  |
| 1559 ≤ f < 1605 | -50 | 700 Hz | Discreet emissions averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -50 + 60/5 (f-1605) | 700 Hz |  |
| 1559 ≤ f < 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

To study the effect of the 2 ms averaging window for spectral analysis, we ran a simulation for NS_03N (n254) to obtain statistics using the set-up shown in Fig. 2.

Figure 2: Simulation set-up

Fig. 3 presents the cumulative probability density function (CDF) of the 2 ms average power measured with 700 Hz MBW at 1605 MHz – 700 Hz / 2. The CDF has been collected by simulating 1000 subframes. The 3 MHz channel is located at the lower edge of n254 UL, and the signal is OFDM/QPSK/15 kHz/full allocation at 0 dB back-off. The signal consists of only PUSCH and DM-RS symbols.

The maximum 2 ms average is approximately 6.5 dB higher than the long-time average. Approximately 40 % of 2 ms power averages exceed the long-time average. Notice that the 6.5 dB difference does not apply to the entire spectrum because the statistics depend on the measurement center frequency.

Observation 2: The 2 ms time limitation may not be ignored in spectral analysis. A sufficient number of 2 ms averaging windows is needed to make simulation results reliable.

Figure 3: Cumulative probability density function of 2 ms power average 
measured at 1605 MHz – 700 Hz / 2 for OFDM/QPSK/full allocation

# 5	Conclusions

We have simulated the A-MPR for n256 NS_24. We have made the following observation based on our simulation results.

Observation 1: A-MPR is needed for n256 NS_24 and 3 MHz CBW in the range 2000 MHz < Fc ≤ 2003.5 MHz.

We also studied the effect of the 2 ms averaging window for NS_03N, NS_05N, NS_05N.

Observation 2: The 2 ms time limitation may not be ignored in spectral analysis. A sufficient number of 2 ms averaging windows is needed to make simulation results reliable.

# 6	Appendix: A-MPR triangles

### 6.1	NS_24