3GPP TSG RAN WG4 Meeting #113																		R4-2419284

Orlando, USA, 18 Nov. 2024 – 22 Nov. 2024

Agenda Item:	5.25.1

Source: 	Ericsson

Title:	Discussions on the remaining open issues for MIMO Enh in Rel-18

Document for:	Discussion

# Introduction

In this contribution, we provide our views on the remaining open issues of NR MIMO evolution for downlink and uplink.

# Discussion

## UE behaviour when TTD exceeds MTTD

In last meeting TTD becoming larger than MTTD was due to gradual timing adjustment was discussed. However, we think MTTD depends on MRTD and TTD becomes larger than MTTD when RTD becomes larger than MRTD. We think TTD becoming larger than MTTD might not occur due to gradual timing adjustment. In our understanding, ideally the purpose of gradual timing adjustment is to bring the UE transmit timing error within allowed range so that it won’t cause the interference at the gNB. If UE doesn’t do the gradual timing adjustment, as TTD is equal to or more than MTTD, timing deviation in UL transmit timing might cause interference at the gNB though the UE stops doing gradual timing adjustment. We think stopping gradual timing adjustment will not solve the real issue.

We also understand that UE behaviour when RTD is greater than MRTD and TTD greater than MTTD was not specified in NR from Rel-15. Having said that, we also understand that following behaviour exist in 36.133.

TS 36.133 7.9.2 Inter band CA "A UE configured with two sTAGs may stop transmitting on the SCell if after timing adjusting due to received TA command the uplink transmission timing difference between SCell in one sTAG and SCell in other sTAG exceeds the maximum value the UE can handle as specified above."

However, similar mechanism or behaviour do not exist in NR from Rel-15. If companies think it is an issue, it should be discussed as part of general maintenance.

- How to handle TTD becoming larger than MTTD may be general RRM issue not only concerning Rel-18 MIMO. However, no solution or behaviour specified from Rel-15.

# Summary and Conclusion

In this contribution we have analysed extension of unified TCI framework RRM requirements to M-TRP and made following proposals.

- How to handle TTD becoming larger than MTTD may be general RRM issue not only concerning Rel-18 MIMO. However, no solution or behaviour specified from Rel-15.

# References

- R4-2414102