3GPP TSG-RAN WG4 Meeting # 113 	    R4-2419363

Orlando, US, 18  – 22 November 2024

Agenda Item:	7.18.2

Source: 	Ericsson

Title:	On requirements for AI/ML based mobility

Document for:	Approval

Introduction

RAN4#112bis was the first meeting where discussions on impact on RAN4 requirements for AI/ML based mobility took place. The agreements reached and the items identified FFS are captured in the WF document [1]. In this contribution FFS items and the impact of different prediction types for L3-based mobility on RAN4 requirements are further discussed.

Discussion

Cell level measurement prediction

In the last meeting, RAN4 discussed the prioritization of scenarios and sub-use cases for cell level RRM measurement prediction. On scenarios it was agreed to align RAN4 prioritization with the prioritization of scenarios in RAN2 and the following was concluded [1].

Agreement (RAN4#112bis):

RAN4 to start the discussion for scenario 2, 3, 4. Further clarification can be discussed for the scenarios.

| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| --- | --- | --- | --- | --- |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

RAN4 further discussed prioritization of sub-use cases and concluded by agreeing to follow the latest definition of the sub-use cases in RAN2.

Agreement (RAN4#112bis):

- Scenario 2, 3 and 4, the 3 cases can be discussed:

  - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

  - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

  - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

  - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed.

It is our observation that the definition of sub-use cases for cell level RRM measurement prediction has not changed in RAN2. The text highlighted in the RAN4 agreement above therefore has no relevance to the definitions of sub-use cases for cell level RRM measurement prediction and should be removed. The removal of the text is necessary to also maintain clarity in RAN4 agreements.

Proposal 1: Update agreement from RAN4#112bis on sub-use cases for cell level measurement prediction as:

- Scenario 2, 3 and 4, the 3 sub-cases can be discussed:

  - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

  - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

  - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

  - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed.

In the last RAN2 meeting, RAN2#127bis, simulations results submitted by participating companies for cell level prediction were discussed.  The discussions in RAN2 concluded by making the following observations for intra-frequency intra-cell temporal domain prediction scenarios [2].

From Chair’s note on temporal prediction simulation results (RAN2#127bis)

- For intra-frequency temporal domain, higher UE speeds result in larger prediction errors

- Initially, increasing the OW length can enhance prediction accuracy in the temporal domain case A, especially when the OW is relatively short. However, once the OW exceeds a certain threshold, further increases do not yield significant benefits. Conversely, for PW, longer durations correlate with decreased prediction accuracy.  RAN2 will not define the actual threshold and fast fading assumption.

- Majority of companies observe that among sub cases 1, 2, and 3, at least with shorter prediction window sub case 2 demonstrates the highest prediction accuracy.

In our view RAN4 should acknowledge the observations established by RAN2 based on the simulation results submitted by companies for intra-frequency intra-cell temporal prediction scenarios and prioritize RAN4 requirements discussion for the sub-use case 2 for the intra-frequency intra-cell temporal prediction scenarios agreed in RAN4#112bis.

Proposal 2: For intra-frequency intra-cell temporal prediction scenarios (scenario 2 and 4 agreed in RAN4#112bis), RAN4 to prioritize evaluation of impact of sub-use case 2 on RAN4 requirements.

Beam level measurement prediction

RAN4#112bis discussed the performance metric/KPIs to be considered by RAN4 for AI/ML based mobility requirement. The discussion concluded by reaching to the agreement below, where L1 beam-level prediction at point A1 and L3 beam-level prediction at point E in Figure 1 were agreed FFS [1].

Agreement from RAN4#112bis:

- RAN4 to first study the impacts on requirements for the following case:

  - L3 cell level RSRP (Point C) prediction

  - FFS L1 beam-level (Point A1)

  - FFS L3 beam-level (Point E)

Figure 1.  Measurement model as defined in TS 38.300 [3].

Agreement RAN2#127:

- Companies can consider to do L3 filtered beam level results for any of this cases.  L3 filtered beam level prediction cases are lower priority.

  - Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results.

  - Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results.

  - Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results.

- If companies do L3 filtered beam level prediction simulations, they should focus on FR2-to-FR2 intra-frequency temporal domain prediction case A.

The FFS items in the agreement above are relevant for beam level RRM measurement prediction sub-use cases that were agreed to be de-prioritized by RAN2 in RAN2#127 meeting. It is therefore our proposal to not consider or at least deprioritize studying impact on RAN4 requirement due to L3 beam-level measurement prediction at point E. Since the measurement reported by UE is the L3 cell level measurement, in our view it is also ok to not consider or deprioritize studying impact on requirement due to L1 beam-level prediction at point A1 by RAN4. It is however our observation that any mistake or error made at point A1 can migrate to the L3 cell level measurement/prediction done at point C and may have an impact on the accuracy of the measured/predicted L3 cell level measurement to be reported by the UE to the network. The impact of such error may need to be evaluated to better understand the impact of error in measurement/prediction of L1 beam-level measurement at point A1 on the accuracy of L3 cell level measurement/prediction at point C. This may also be relevant for sub-case 1 of cell-level measurement prediction. This can be one of the inputs from RAN4 to RAN2 that can be considered by RAN2 as one of the generalization aspects in their evaluations.

Observation 1:  The impact of error in prediction of L1 beam-level measurement predicted at point A1 on L3 cell level measurement predicted at point C may need to be evaluated.

Proposal 3: RAN4 to not consider studying impact on RAN4 requirement due to L3 beam-level measurement prediction at point E to align with RAN2 prioritization.

Impact of temporal prediction

For temporal prediction two cases, as elaborated in Figure 2 and Figure 3, are being considered by RAN2.

Temporal prediction case-A:

In temporal prediction case A, UE performs measurement within the observation window (OW) and uses these measurements to perform prediction of measurements valid for the prediction window (PW). An OW consists of one or more measurement occasions and within a PW there can be one or more prediction occasions. A high-level overview of OW, measurement occasions within OW, PW and prediction occasions within PW is provided in Figure 2. It is worth noting here that the prediction occasions or PW always starts after the completion of measurement occasions or OW. From the core requirement point of view, the measurement delay for temporal prediction case-A should correspond to the duration of start of first measurement occasion until the last measurement occasion before the prediction starts or in other words the core requirement should correspond to observation window plus some processing time.

Figure 2.  Elaboration of temporal prediction case-A.

Temporal prediction case-B:

Like the temporal prediction case-A, an OW consists of one or more measurement occasions and within a PW there can be one or more prediction occasions in temporal prediction case-B. The UE is expected to perform measurements in measurement occasions and predict the measurements valid for the prediction occasions inside the prediction window. Unlike temporal prediction case-A, within the OW the UE can skip performing measurements on some of the measurement occasions in temporal prediction case-B. A high-level overview of OW, measurement occasions within OW, skipped measurement occasions within OW, PW and prediction occasions within PW is provided in Figure 3. Measurement delay for temporal prediction case-B is also impacted by the duration of OW and processing time before the prediction can start.

Figure 3. Elaboration of temporal prediction case-B.

In both cases for temporal prediction, the duration of PW will have an impact on the duration of OW or the duration of OW will have an impact on the accuracy of measurements predicted by the UE to be valid for the measurement instances inside the PW. For example, to meet certain accuracy requirements for the predicted measurements, OW window of certain length may be required. The relationship between the OW length and the PW length to understand the impact of PW length on the OW length and the impact of the OW length on the accuracy of the predicted measurements for the instances inside the PW need to be studied.

Proposal 4: Measurement delay for temporal prediction cases is impacted by the duration of the observation window (OW).

Proposal 5: Study the relationship between the OW length and the PW length to understand the impact of the OW length on the accuracy of the predicted measurements for the instances inside the PW and the impact of PW length on the OW length.

Impact of frequency prediction

Frequency domain prediction is another type of prediction considered by RAN2 for AI/ML based mobility. Frequency domain prediction refers to the predictions of measurements of cells in one frequency based on the measurements performed by the UE on cells in another frequency. As agreed in the last meeting, the scope of inter-frequency prediction as of now is limited to carriers in the same FR. A high-level elaboration of RRM measurement prediction is exemplified in Figure 4. In this example, UE performs measurement on cell 1, cell 2, and cell 3. The measurements for cell 4, cell 5, and cell 6 are predicted by the UE based on the measurements performed on cell 1, cell 2, and cell 3. The measurement delay in this case should at least correspond to the time taken to perform measurements on cell 1, cell 2, and cell 3.

Proposal 6: RAN4 to evaluate impact on core requirement for frequency domain prediction by at least considering the time taken by the UE to perform measurements on the measured cells.

Figure 4.  RRM measurement prediction in frequency domain.

Impact of event prediction

RAN2 is considering direct approach and indirect approach for event prediction evaluations. In direct approach UE is expected to perform event prediction in temporal domain. In in-direct approach the UE makes use of the RRM measurement prediction in temporal domain for the event prediction. In this regard, accuracy of temporal prediction of cell level measurements will have a direct impact on the accuracy of in-direct event prediction. It is therefore our proposal to start evaluation of impact on requirement for event prediction for in-direct prediction use-case in RAN4.

Proposal 7:  RAN4 to prioritize evaluation of impact on requirement for event prediction for in-direct prediction use-case over direct prediction use case.

Summary

In this contribution issues related to cell/beam level measurement prediction and impact of different predictions on RAN4 requirements are discussed. The observations and proposals below summarize the discussion presented in this paper.

Proposal 1: Update agreement from RAN4#112bis on sub-use cases for cell level measurement prediction as:

- Scenario 2, 3 and 4, the 3 sub-cases can be discussed:

  - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

  - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

  - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

  - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed.

Proposal 2: For intra-frequency intra-cell temporal prediction scenarios (scenario 2 and 4 agreed in RAN4#112bis), RAN4 to prioritize evaluation of impact of sub-use case 2 on RAN4 requirements.

Observation 1:  The impact of error in prediction of L1 beam-level measurement predicted at point A1 on L3 cell level measurement predicted at point C may need to be evaluated.

Proposal 3: RAN4 to not consider studying impact on RAN4 requirement due to L3 beam-level measurement prediction at point E to align with RAN2 prioritization.

Proposal 4: Measurement delay for temporal prediction cases is impacted by the duration of the observation window (OW).

Proposal 5: Study the relationship between the OW length and the PW length to understand the impact of the OW length on the accuracy of the predicted measurements for the instances inside the PW and the impact of PW length on the OW length.

Proposal 6: RAN4 to evaluate impact on core requirement for frequency domain prediction by at least considering the time taken by the UE to perform measurements on the measured cells.

Proposal 7:  RAN4 to prioritize evaluation of impact on requirement for event prediction for in-direct prediction use-case over direct prediction use case.

References

- R4-2416855, WF on AI/ML Mobility, Moderator (Nokia).

- Chair’s note from RAN2#127bis, RAN2 Chair (InterDigital).

- TS38.300, Stage 2: NR and NG-RAN Overall Description, 3GPP.