3GPP TSG-RAN WG4 Meeting #113                                                                   R4-2419399

Orlando, US, 18th – 22nd November, 2024

Source: 	MediaTek Inc.

Title: 	TP on general considerations for the UE antenna for 14800-15350MHz

Agenda Item:	7.2.4.2

Document for:	Approval

1	Introduction

In RAN#103, a new SI [1] was agreed upon to study the IMT parameters for the frequency ranges of 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 15350 MHz. Whether FR1-like and FR2-like approaches for the UE RF architecture for 14800 to 15350 MHz are used has been discussed . Therefore, in this document we present a text proposal to capture general considerations on the UE antenna design trade-offs.

2	Text proposal

-------------------------------------------- TP BEGIN --------------------------------------------

6.5	Antenna characteristics

6.5.1	BS antenna characteristics

6.5.1.1		Antenna model

6.5.1.2	Antenna parameters

6.5.2	UE antenna characteristics

The request from ITU WP5D calls for a study on the IMT parameters for radio and antenna at 15 GHz. A critical system implementation question that we must address is whether to choose an FR1-like omni-directional antenna system or an FR2-like array antenna system in UE, given their distinct frequency positions. An FR2-like array antenna system incorporates analog beamforming into the array antenna to increase the coherent EIRP gain, thus helping to reduce the path loss between the UE and the base station. In an FR2-like system, the array antenna, LNAs, PAs, beamformers, and PMIC are integrated into a single package, known as an antenna in package (AiP), which results in high costs and imposes several limitations on system realization. Owing to the AiP's small form factor, the choice of technology for the active circuits is confined to silicon-on-insulator (SOI) or other high-integration technologies, which limits the use of high-efficiency technologies such as Gallium Arsenide (GaAs). Another significant constraint in an FR2-like system is that 3GPP conformance testing must be conducted using an OTA chamber, which adds complexity and uncertainty to product testing. The interconnects in the AiP should support multiple signal routings, including bias, controls, clocks, and signals, making this interface less cost-effective.

Digital beamforming

The advantage of an FR2-like analog beamforming system lies in its coherent (EIRP) gain, resulting in a higher SINR. This can also be achieved in an FR1-like system with multiple antennas, which can attain coherent EIRP gain through the strategic placement of co-directional and co-located antennas when coherent Tx is implemented. Each antenna in an FR1-like system is linked to its own digital chain, allowing for easy support of coherent Tx. This coherent Tx gain offers two major advantages: it can reduce the PA power requirement for each antenna element, thus enhancing the overall power consumption of the UE at 15GHz, and it can help increase the coverage distance between the base station and the UE, which is especially important for co-located FR1 and 15GHz base stations.

On the Rx side, this FR1-like multiple antenna system with digital beamforming provides the feasibility for different use cases. When UE is at the cell edge with weak signal strength from its serving cell, signals received from all antennas can be rotated by proper phases (i.e., beamforming weights), forming a certain Rx beam to maximize the receive signal power. The UE operation is similar to that in FR2. When UE is at cell center, instead of forming a beam to maximize the receive power, all antennas can be used jointly to achieve spatial multiplexing gain (e.g., signals from 4Rx antennas as the input of a 4x4 MIMO demapper). Therefore, UE performance can be optimized, depending on the target scenario.

Moreover, with digital beamforming, the raw received signals from individual Rx antennas are available. This allows UE to apply different beamforming weights to maximize signals with different angle of arrival (AoA). It provides the benefit to allow UE to simultaneously receive signals from more than 1 AoAs. E.g., UE can receive the PDSCH from its serving cell in one AoA, while searching for a neighboring cell from another AoA. This helps to avoid the scheduling restriction as defined in TS 38.133 spec.

-------------------------------------------- TP END --------------------------------------------

3 References

- RP-240787, “Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson.