3GPP TSG-RAN WG4 Meeting #113															R4-2419417

Orlando, USA, 18th – 22nd November, 2024

Agenda item:			7.2.4.2

Source:	Samsung

Title:	TP on FR1-like UE antenna performance for 14800 to 15350 MHz frequency range

Document for:	Approval

1	Introduction

In RAN4#112-bis, it was agreed to use FR1-like omnidirectional with 0 dBi for UE antenna assumption. This document is a text proposal for UE antenna characteristics in TR 38.922 based on the agreement.

2	Text Proposal

-------------------------------------------- TP BEGIN --------------------------------------------

## 6.5	Antenna characteristics

### 6.5.1	BS antenna characteristics

#### 6.5.1.1		Antenna model

#### 6.5.1.2	Antenna parameters

### 6.5.2	UE antenna characteristics

#### 6.5.2.x	FR1-like (metal frame) antennas

At the 15 GHz band, the higher number of antennas than currently used for FR1 could be implemented taking into account the smaller antenna size or correlation/isolation. However, increasing the number of antennas would not always guarantee the better performance given the more loss at higher frequency ranges. For example, additional antennas will result in the increased coupling among antennas, leading to a degradation in MIMO performance. Also, it would be not easy to optimize all the antenna feeding lines to ensure the better performance due to the higher insertion loss per unit length than FR1.

In order to figure out the impact with the FR1-like UE antennas, a simulation to estimate anticipated antenna gain with two general metal frame Tx antennas was conducted without beam steering assumptions. The antenna placement for the evaluation is captured in Figure X.

Figure [X]: Exemplary placement of three metal frame antennas within a mobile device.

With those assumptions, which are now general FR1-like UE antennas on the market, it is shown that the multi-Tx approach is not always better than single-Tx in terms of antenna gains as expected. Table [X] and Figure [Y] show peak antenna gains of each Tx scenario for cases of single-beam and combined-beam, which are not about the best beam based on the beam steering assumption.

Table [X]: Estimated antenna gains for one-Tx and two-Tx

|  | Ant1 | Ant2 | Ant3 |
| --- | --- | --- | --- |
| Gain | 3.9 | 4.8 | 2.7 |
|  | Ant1 + Ant2 | Ant1 + Ant 3 | Ant2 + Ant3 |
| Combined gain | 3.8 | 2.6 | 3.9 |
| Gain drop | -1.0 | -1.3 | -0.9 |

Figure [Y]: Estimated antenna gains for one-Tx and two-Tx

As we can see from Table [X] and Figure [Y], from the antenna gain perspective at least, the peak gain of combined two Tx antennas does not show better performance, but rather drops compared to the single beam case. This would be coming from two different radiation patterns as depicted in Figure [Z].

Figure [Z]: Simulated radiation patterns for one-Tx and two-Tx cases

-------------------------------------------- TP END --------------------------------------------