3GPP TSG-WG4 Meeting #113	R4-2419419

# Orlando, US, 18th - 22nd November, 2024

Source: 	NEC

Title: 	TP to TR 38.922: OBUE limits for AAS BS in 7125 to 8400 MHz frequency range

Agenda Item:	7.2.3	   Corrections for 7125 to 8400 MHz frequency range

[FS_NR_IMT_4400_7125_14800MHz]

Document for:	approval

Introduction

This contribution tries to correct the error in OBUE limits for AAS BS in 7125 to 8400 MHz frequency range.

Discussion

Wide Area BS operating band unwanted emission limits for AAS BS are shown in table 5.2.1.2-2.

The limits are shown as the basic limits. However, the values look too big. The values shown in the table shall be ones after scaling applied. Therefore, they need to be corrected by reducing 9 dB for each value.

The base text is copied from R4-2417103 which was agreed in the previous meeting RAN4#112bis. The corrections provided by this contribution are only in table 5.2.1.2-2.

- TP to TR 38.922

<Start of TP>

5.2	BS parameters

5.2.1	Transmitter characteristics

5.2.1.1	Power dynamic range

There is no power control in downlink and fixed power per resource block is assumed during the study phase. Hence 0 dB power dynamic range was agreed for the LS reply.

5.2.1.2	Spectral mask

Both Category A and B spectral mask are specified. Since the frequency range 7125 to 8400 MHz is just adjacent to existing NR band n104, it is proposed that existing spectral mask (Category B) for band n104 in TS 38.104 subclause 6.6.4 for conducted requirements and subclause 9.7.4.2 for radiated requirements are applicable for the range. And Category A limits for Wide Area BS are specified in Table 5.2.1.2-1 for non-AAS BS and in Table 5.2.1.2-2 for AAS BS.

Table 5.2.1.2-1: Wide Area BS operating band unwanted emission limits for non-AAS BS (Category A)

|  | -7dBm-750f_offsetMHz-0.05 |  |  |  |
| --- | --- | --- | --- | --- |

| Frequency offset of measurement filter ‑3dB point from the carrier frequency, Δf | Basic limits | Measurement Bandwidth |
| --- | --- | --- |
| 0 MHz  f < 20MHz | -7dBm-720f_offsetMHz-0.05 | 100 kHz |
| 20 MHz  f < min(40 MHz, fmax) | -14 dBm | 100 kHz |
| 40 MHz  f  fmax | -13 dBm | 1 MHz |
| NOTE: fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 40 MHz outside the downlink operating band. |  |  |

Table 5.2.1.2-2: Wide Area BS operating band unwanted emission limits for AAS BS (Category A)

| Frequency offset of measurement filter ‑3dB point from the carrier frequency, Δf | Basic limits | Measurement Bandwidth |
| --- | --- | --- |
| 0 MHz  f < 50MHz | 2-7 dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz  f < min(100 MHz, fmax) | -14 dBm | 100 kHz |
| 100 MHz  f  fmax | -13 dBm | 1 MHz |
| NOTE: fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |