3GPP TSG-RAN WG4 Meeting #113	R4-2419626

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	(NR_NTN_Solutions) On clarification of GEO and GSO definition

Agenda item:	4.2.2

Document for:	Discussion

Introduction

This paper aims at discussing the terminology adopted in the current specification for NTN in TS 38.101-5 (Rel-17 and Rel-18).

There are several instances in the specification where GEO terminology is used whereas in others GSO terminology is used. It is necessary to clarify the intent behind the terminology and align the wording for a clearer behavior. Currently, TS 38101-5 where the definition of GSO is not aligned with other specifications related to the NTN work item.

Discussion

In TS 38.101-5, GSO satellites are defined as “Geoestationary satellite orbit”, while GEO is defined as “Geostationary Earth Orbit”.

The current version of the abbreviation was introduced in RAN4 #109, by CR R4-2319859. And the main justification was that GSO, in ITU terms represents a “Geostationary orbit”, and 3GPP should align its terminology with that adopted by ITU, such that confusion can be avoided.

The justification has its merits, as some RF requirements were designed with pointers for the ITU specification. However, the change of terminology is not simply an editorial change, it impacts the specification and change the applicability of the rules designed for this specification.

Geostationary satellites (or geosynchronous equatorial satellites) are a subset of geosynchronous satellites and restricting the subset of satellties in this work item would eliminate significant market applications while discarding some of the work during this work item.

In the Rel-18 Work Item Description (RP-240779) the expression GSO is clearly directed to mean “Geosyncrhonous” and this has impacted several other RAN specifications. Observe from table 1 below:

Table 1. Definition of GSO per 3GPP specification

| GSO: Geosynchronous Orbit | GSO: Geostationary orbit |
| --- | --- |
| 36.331 36.102 36.133 36.300/38.300 36.521-3 36.521-4 38.306 | 38.101-5 |

Table 2. Definition of GSO in WID for NTN_enh

### 4.1 Objective of SI or Core part WI or Testing part WI The work item aims at specifying enhancements for NG-RAN based NTN (Non-Terrestrial Networks) according to the following assumptions with implicit compatibility to support HAPS (High Altitude Platform Station) and ATG (Air To Ground) scenarios: - GSO (Geo Synchronous Orbit) and NGSO (Non Geo Synchronous Orbit). NGSO includes Low Earth Orbit (LEO) and Medium Earth Orbit (MEO). - Earth fixed tracking area. Earth fixed & Earth moving cells for NGSO - FDD mode - UEs with GNSS (Global Navigation Satellite Systems) capabilities - Both “VSAT” (Very Small Aperture Terminal) devices with directive antenna (including fixed and moving platform mounted devices) and commercial handset terminals (e.g. Power class 3) are supported in FR1 - Only “VSAT” devices with directive antenna (including fixed and moving platform mounted devices) are supported in above 10 GHz bands. Note 1: In Rel-17 WID, “VSAT” device with external antenna on moving platform is equivalent to a device that operate on platforms in motion, and this is referred to as ESIM (Earth Station In Motion). Note 2: The Rel-17 NTN architecture is assumed. This WI is adding functionality in REL-18 to enhance features that were introduced in REL-15/16/17:

Maintaining the current definition will cause some attrition between the capabilities defined by 38.306, which is supposed to indicate the UE supports Geosynchronous satellite and a specification that only covers for Geostationary satellites.

Besides, the current definition is not harmonious with the same specification, as in the same meeting, RAN4 #109, Annex A.4.2 has been updated to indicate that doppler values for GSO satellite might reach values up to 0.93 ppm, which unequivocally indicate the  spirit of the specification was addressing geosynchronous satellites.

Therefore we make the following set of observations:

GSO is defined as geosynchronous satellite in other specifications, including other RAN4 specifications and the specification of UE capabilities (TS 38.306).

The “Geostationary” definition is inconsistent with the doppler values used inside TS 38.101-5, for the test cases, in Annex 4

The NTN work item uses GSO as Geosynchronous.

It is more likely that NTN deployments in the market are deployed in Geosyncrhonous orbits than in Geoestationary orbits.

- However, some companies have raised the issue that currently there is a lack of harmony between RAN4 specification and the provided definition in ITU, where GSO is sometimes defined as “Geostationary Orbit” . This concern arose from the fact that in the ITU definitions page the following is found [7]:

1.188 geosynchronous satellite: An earth satellite whose period of revolution is equal to the period of rotation of the Earth about its axis. 1.189 geostationary satellite: A geosynchronous satellite whose circular and direct orbit lies in the plane of the Earth’s equator and which thus remains fixed relative to the Earth; by extension, a geosynchronous satellite which remains approximately fixed relative to the Earth. (WRC-03) 1.190 geostationary-satellite orbit: The orbit of a geosynchronous satellite whose circular and direct orbit lies in the plane of the Earth's equator

And with this definition, the ITU later defines that GSO represents “Geostationary-satellite orbit”. However, more in-depth explanation is provided on ITU Articles 9 (Procedure for effecting coordination with or obtaining agreement of other administrations) [8] and Article 11 (Notification and recording of frequency assignments)[8]:

A.9.6A For the purpose of this Article, a geostationary satellite is a geosynchronous satellite with an orbit the inclination of which is less than or equal to 15°. (WRC-03) A.11.4A For the purpose of this Article, a geostationary satellite is a geosynchronous satellite with an orbit the inclination of which is less than or equal to 15°. (WRC-03)

This clarification provided by ITU reaffirms that GSO is used with the intend to cover for satellites that are geosynchronous (with an upper limitation of a satellite orbit inclination of 15 degrees). For the most part such satellite orbital inclination is sufficient to cover for the applicability of GSO as intended by the body of companies that provided effort to specify NTN features in Rel-17 and Rel-18.

ITU provides a clarification that for their normative work, a geostationary satellite is a geosynchronous satellite with an orbit the inclination of which is less than or equal to 15°. (WRC-03)

As a compromise for companies concerned with ITU alignment we propose the following:

Define in TS 38.101-5 that GSO represents a geostationary satellite with an orbit the inclination of which is less than or equal to 15°.

Conclusion

This paper discussed the alignment of the terminology for satellite orbits in RAN4 specification TS 38.101-5. The discussion encompass GSO and NGSO definitions, and the following observations and proposals were made:

Observation 1: GSO is defined as geosynchronous satellite in other specifications, including other RAN4 specifications and the specification of UE capabilities (TS 38.306).

Observation 2: The “Geostationary” definition is inconsistent with the doppler values used inside TS 38.101-5, for the test cases, in Annex 4

Observation 3: The NTN work item uses GSO as Geosynchronous.

Observation 4: It is more likely that NTN deployments in the market are deployed in Geosyncrhonous orbits than in Geoestationary orbits.

However, some companies have raised the issue that currently there is a lack of harmony between RAN4 specification and the provided definition in ITU, where GSO is sometimes defined as “Geostationary Orbit” . This concern arose from the fact that in the ITU definitions page the following is found [7]:

Observation 5: ITU provides a clarification that for their normative work, a geostationary satellite is a geosynchronous satellite with an orbit the inclination of which is less than or equal to 15°. (WRC-03)

Proposal 1: Define in TS 38.101-5 that GSO represents a geostationary satellite with an orbit the inclination of which is less than or equal to 15°.

- References

- TR 38.821. Solutions for NR to support non-terrestrial networks (NTN). V16.2.0. 3GPP.

- TS 38.300. R; NR and NG-RAN Overall description; Stage-2 V18.1.0. 3GPP.

- TS 38.331. NR; Radio Resource Control (RRC); Protocol specification. V18.1.0. 3GPP.

- TR 38.133. NR; Requirements for support of radio resource management V18.5.0. 3GPP.

- R4-2303687, Updated Rel-17 RAN4 UE feature list for NR, CMCC, Athens, Greece, February 27 – March 3, 2022

- R4-2316967, WF on NTN RAN task, MediaTek inc, Xiamen, China, 9th – 13th October 2023

- ITU-R, “Article 1 – Terms and Definitions”

- ITU-R, “Article 9 -Procedure for effecting coordination with or obtaining agreement of other administrations” (WRC-15)

- ITU-R, “Article 11 – Notification and Recording of Frequency Assignments” (WRC-15)