3GPP TSG-RAN4 Meeting #113	R4-2419643

# Orlando, US, 18th – 22nd November, 2024

Title:	Discussion on NR-NTN HPUE Tx requirements

Source:	Samsung

Agenda item:	7.8.2.2.1

Document for:	Approval

1	Introduction

In the RAN4#112bis meeting, the WF R4-2417122 [1] has agreed on the starting points for NR-NTN HPUE Tx requirements, including the ACLR, SEM and MOP tolerance.

In this paper, we will further discuss the Tx requirements based on the collected co-ex study results.

2	Tx requirements

In [1], the co-ex study has preliminary observation as follows:

Issue 1-8: NR-NTN ACIR/ACLR Agreement: - Consider following observations can be used as a starting point for RF requirements analysis/studies: - Candidate ACLR value for NR-NTN PC2/PC1.5/PC1, based on initial co-ex study results received in #112bis meeting, is 30 dBc - The current TN requirements are more stringent than the observations from co-existence results. Overlapping geographic coverage between NTN and TN is discussion in RAN4 in other WIs. - FFS for final ACLR requirements

And in one of our separate tdoc submitted to this meeting, our observation from the collected co-ex study results is that the 30 dBc as ACLR can be sufficient for PC 2/1.5/1 for NR-NTN considering its interference to the TN network.

Observation 1: From collected co-ex study results, 20dBc ACLR seems sufficient for PC2 with baseline isolation distance as 1.5km. This observation may need update with newly submitted co-ex results in this #113 meeting.

Observation 2: From collected co-ex study results, 30dBc ACLR seems sufficient for PC2 even without isolation distance. This observation may need update with newly submitted co-ex results in this #113 meeting.

Proposal 1: For PC2, co-ex result supports ACLR to be further relax to 20dBc if baseline isolation distance of 1.5km is considered.

And considering the concerns raised in last meeting addressed in WF[1] was about whether the derived ACLR

Issue 2-5: ACLR Agreement: - Co-existence study results and conclusion is an important factor for deriving out-band emission requirements - Check the derived ACLR for A-MPR and additional spurious emission against regulatory information.

Observation 3: Given the WF is to cross check the A-MPR and additional spurious emission requirements with the potential ACLR concluded from co-ex studies, it can be helpful to check the candidate ACLR from 20 ~ 30 dBc for PC2.

And the MPR, AMPR requirements are listed in the open issues pending for the decisions from co-ex study conclusion.

Open issues Issue 2-6: MPR - Option 1: RAN4 to consider applicable regulatory requirements in ECC/CEPT in MPR and A-MPR evaluations. (Qualcomm) - Option 2: Reusing NR-NTN PC3 MPR values (both single Tx and Tx diversity) for PC2 MPR as starting point (Mediatek, Samsung) - Option 3: MPR for NTN HPUE should be further evaluated according to the agreed ACLR value, PA model and other limiting factor for MPR evaluation. (ZTE) Way forward: - Further discuss with ACLR candidate values, following agreement and above options. Issue 3-3-2: MPR - Agreement: - FFS the MPR requirements. Following potential factors to be further discussed: - ACLR from co-existence study - Current TN MPR requirements - MPR simulation assumptions - Other factors not precluded if identified.. Issue 2-7: A-MPR - Option 1: RAN4 to consider applicable regulatory requirements in ECC/CEPT in MPR and A-MPR evaluations. (Qualcomm) - Option 2: To reduce number of power back-off simulations for the FR1 satellite bands, we suggest not considering scenarios/combinations that are not expected for the satellite bands: large RB allocations, CP-OFDM, higher-order modulations. (Apple) - Option 3: Regarding the A-MPR requirements, once HPUE e.g. PC2/PC1.5or PC1 are supported in any specific bands, then the A-MPR requirement should be revisited again to identify the appropriate the A-MPR requirements. (ZTE) Way forward: - Further discuss with following agreement and above options. Issue 3-3-3: A-MPR - Agreement - FFS A-MPR requirements. Following potential factors to be further discussed: - Simulation reduction by not considering large RB allocations, CP-OFDM, higher-order modulations - A-MPR simulation assumption including ACLR from co-existence study - Applicable bands - Regional requirements for A-MPR - Other factors.

Even though the A-MPR, MPR simulation assumptions include ACLR, which was usually an output from co-ex studies, the previous WF had provided 30 dBc as a temporary observations and can be used to conduct simulations in parallel together with the co-ex study discussion.

Observation 4: Given the ACLR was agreed to also consider the A-MPR, additional spurious emission impacts, to avoid two issues become chicken-egg dead-end, we suggest to do the A-MPR and MPR simulation with ACLR assumed as 30 dBc or lower values in parallel with the co-ex conclusion discussions.

3	SAR for handheld

Given the SAR issue seems to be identical for both NR-NTN and IoT NTN handheld devices, we provide our SAR discussions in another paper in IoT-NTN Tx requirement agenda.

4	Conclusion

For the NTN HPUE Tx requirement, we propose the following.

Observation 1: From collected co-ex study results, 20dBc ACLR seems sufficient for PC2 with baseline isolation distance as 1.5km. This observation may need update with newly submitted co-ex results in this #113 meeting.

Observation 2: From collected co-ex study results, 30dBc ACLR seems sufficient for PC2 even without isolation distance. This observation may need update with newly submitted co-ex results in this #113 meeting.

Proposal 1: For PC2, co-ex result supports ACLR to be further relax to 20dBc if baseline isolation distance of 1.5km is considered.

Observation 3: Given the WF is to cross check the A-MPR and additional spurious emission requirements with the potential ACLR concluded from co-ex studies, it can be helpful to check the candidate ACLR from 20 ~ 30 dBc for PC2.

Observation 4: Given the ACLR was agreed to also consider the A-MPR, additional spurious emission impacts, to avoid two issues become chicken-egg dead-end, we suggest to do the A-MPR and MPR simulation with ACLR assumed as 30 dBc or lower values in parallel with the co-ex conclusion discussions.

5	Reference

[1] R4-2417122, WF on co-existence study and HPUE for NR-NTN, Samsung