3GPP TSG-RAN WG4 Meeting #113	R4-2419881

# Orlando, US, 18 - 22 Nov, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0690 | rev | 1 | Current version: | 16.18.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_perf_enh-Perf) CR on UE CA demodulation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_perf_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The PDCCH configuration is wrong for some UE CA demodulation requirements since there is no enough resource to use AL8, e.g. for 30kHz SCS and 5MHz bandwidth and 120kHz SCS and 50MHz bandwidth. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify PDCCH configuration for some UE CA demodulation requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be inconsist between specification and RAN4 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A, 7.2A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <START OF THE CHANGE 1>

## 5.2A	PDSCH demodulation requirements for CA

The parameters specified in Table 5.2-1 for PDSCH single carrier tests are reused for PDSCH CA tests unless otherwise stated.

Table 5.2A-1: Common test parameters for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | FDD: 12TDD: Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| PDCCH configuration | Number of PDCCH candidates and aggregation levels |  | 30kHz SCS and 5MHz bandwidth: 1/AL1 others: Table 5.2-1 |
| Number of HARQ Processes |  |  | As defined in Table 5.2A-2 |
| TDD UL-DL pattern |  |  | 15kHz SCS: FR1.15-1 30kHz SCS: FR1.30-1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table 5.2A-3 |
| PUCCH format for HARQ-ACK feedback |  |  | PUCCH format 1 for cases where the number of ACK/NACK to be transmitted on single PUCCH is 2 or less. PUCCH format 3 for cases where the number of ACK/NACK to be transmitted on single PUCCH is more than 2. |

Table 5.2A-2: Test parameters for number of HARQ processes

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 8 |
|  | TDD PCell | For CC with Rank 2: 10 For CC with Rank 8: 8 | 8 |
| FDD 15 kHz + TDD 15 kHz CA | FDD PCell | 4 | 4 |
|  | TDD PCell | 8 | 8 |
| TDD 15 kHz + TDD 30 kHz CA | 15kHz PCell | 8 | 12 |
|  | 30kHz PCell | 8 | 8 |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | 4 | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 8 | N/A |

Table 5.2A-3: Test parameters for K1 values

| The number of slots between PDSCH and corresponding HARQ-ACK information |  | CCs with the same duplex mode and SCS with Pcell | CCs with different duplex mode and/or SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | {2} | {2} |
|  | TDD PCell | For CC with Rank 2: {8,7,6,5,5,4,3,11} For CC with Rank 8: {8,7,6,5,5,4,3} | {7,5,4,11,9} |
| FDD 15 kHz + TDD 15 kHz CA | FDD PCell | {2} | {2} |
|  | TDD PCell | {4,3,2,6} | {4,3,2,6,5} |
| TDD 15 kHz + TDD 30 kHz CA | 15kHz PCell | {4,3,2,6} | {4,4,3,3,2,2,6,6} |
|  | 30kHz PCell | {8,7,6,5,5,4,3,2} | {7,5,4,11} |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | {2} | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | For CC with Rank 2: {8,7,6,5,5,4,3,2} For CC with Rank 8: {8,7,6,5,5,4,3} | N/A |

# <END OF THE CHANGE 1>

# <START OF THE CHANGE 2>

#### 7.2A.2.1	Minimum requirements

For CA with different numbers of DL component carriers, the requirements are defined in Table 7.2A.2.1-3 based on the single carrier requirements for different bandwidth specified in Table 7.2A.2.1-2, with the parameters in Table 7.2A.2.1-1 and the downlink physical channel setup according to Annex C.5.1. The performance requirements specified in this sub-cluase do not apply for UE single carrier test.

Table 7.2A.2.1-1: Test parameters for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| PDCCH configuration | Number of PDCCH candidates and aggregation levels |  | 120kHz SCS and 50MHz bandwidth: 1/AL2 others: Table 7.2-1 |
| Number of HARQ Processes |  |  | 8 |
| TDD UL-DL pattern |  |  | 120kHz SCS: FR2.120-1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Annex A.1.3 |

Table 7.2A.2.1-2: Single carrier performance for TDD 120 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 50 | R.PDSCH.5-9.1 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 10.4 |
| 100 | R.PDSCH.5-9.2 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 10.2 |
| 200 | R.PDSCH.5-9.3 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 10.3 |
| 400 | R.PDSCH.5-9.4 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 10.3 |

Table 7.2A.2.1-3: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | TDD 120 kHz + TDD 120 kHz | As defined in Table 7.2A.2.1-2 |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 7.1.1.5. |  |  |

# <END OF THE CHANGE 2>