3GPP TSG-RAN WG4 Meeting #113                                                                R4-2419901

# Orlando, USA, 18th – 22nd Nov, 2024

Agenda item:			7.9.6

Source:	Moderator (Eutelsat Group)

Title:	Way Forward for [112][313] NR_NTN_Ku_Band_General

Document for:	Approval

# Background

The General requirements of the Rel-19 NR NTN Ku band work item had been discussed in RAN4#113. This WF captures the result of the discussion in the meeting.

# WF for NTN Ku band General and work plan

### Sub-topic 1-1: Defining bands by regulatory authority

Agree the following band definition

Table X: Band definitions for Ku band

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| nX(Note 1) | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| nY(Note 2) | 14000 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |
| NOTE 1: This band is applicable in the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. |  |  |  |

Companies to further evaluate by RAN4#114 pros and cons of defining one global band, potentially for both numerologies.

# WF for NTN Ku band Coexistence study based on ITU regulations

### It was not possible to agree all VSAT antenna co-existence parameters at RAN4 #113. Companies are encouraged to provide their preferred set of VSAT antenna parameters for the next meeting. Final decision to be made in RAN4 #114.

### Sub-topic 2-1: VSAT antenna pattern

Agree for VSAT antenna model, to progress the co-ex study with the VSAT equipped with parabolic antenna first. And to further discuss the phased array (flat-panel) antenna modeling in parallel.

Agree to use 38.921 unless there is a need to use sub array then 38.803

The following parameters are agreed:

Table 2-1 VSAT phased array antenna characteristics

| Ku band VSAT phased array antenna |  |  |
| --- | --- | --- |
| 1 | Ku band VSAT phased array antenna Characteristics |  |
| 1.1 | Antenna pattern | Table 2-2 and 2-3 (TR 38.921) |
| 1.2 | Element gain (dBi) (Note 2) | 0 or 3.5 ? |
| 1.3 | Y axis / Z axis element 3 dB beam width of single element (degree) | 90° for Y axis 90° for Z axis |
| 1.4 | Y axis / Z axis element front-to-back ratio (dB) | 30 dB |
| 1.5 | Antenna polarization | Circular（RHCP or LHCP） |
| 1.6 | Antenna array configuration (Y axis × Z axis)(Note 4) | 45x45 elements |
| 1.7 | Number of supported polarizations, P | 1 |
| 1.8 | Y axis / Z axis radiating element spacing | 0.5 lambda |
| 1.9 | Array Ohmic loss (dB) (Note 2) | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | [4.44 dBm] TBA |
| 1.11 | maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees |
| 1.12 | vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees |
| 1.13 | normal direction | Toward X+ axis |

Note 1:	Void

Note 2:	The element gain in row 1.2 includes the loss given in row 1.9.

Note 3:	The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element).

Note 4:	All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane.

Note 5: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration  (Y axis × Z axis).

Agree 3.5 dB element gain

### Sub-topic 2-2: SAN noise figure

Agree 3 dB

### Sub-topic 2-3: SAN G/T

Agree 30.9 dB/K and 10.9 dB/K based on 3 dB NF

### Sub-topic 2-4: VSAT Max Tx antenna gain

Agree first study is worst case (GSO) of 100 cm aperture (41.3dBi), 88.2 dBm EIRP, other options not precluded e.g. different size, EIRP and antenna type.

The antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization.

### Sub-topic 2-5: VSAT noise figure

Agreed at previous meeting (see also THALES R4-2419724) to have 2.5 dB NF and 6 dB NF (as we did for Ka-band).

### Sub-topic 2-8: SAN antenna pattern

Agreed at previous meeting to use section 6.4.1 from TR 38.811 which corresponds to the same proposal. See also the SAN corresponding antenna beamwidth computed based on this assumption.

### Sub-topic 2-9: TN BS/UE antenna patterns for 11 GHz

Agree to use TN BS and UE antenna and beam forming pattern modelling in TS 38.921 for 11GHz carrier frequency for Ku band coexistence study

### Sub-topic 2-10: TN BS antenna pattern for 14 GHz

[Moderator proposal] Agree to use TN BS antenna and beam forming pattern modelling in TS 38.922 for 14GHz carrier frequency for Ku band coexistence study. For TN UE, the isotropic antenna model will be used as mentioned in TS 38.922.

### Sub-topic 2-17: VSAT max Tx power

[Moderator proposal] Based on sub-topic 2-4, agreement is 50 W for GSO and 10 W for NGSO.

Companies are asked to review the open issues in sub-topics 2-6, 2-7, 2-11, 2-12, 2-13, 2-14, 2-15 and 2-16 to try to reach agreement at this meeting.

# WF for NTN Ku band System parameters

### Sub-topic 3-9: FR1 FR2 down selection

Overall observation is that both FR1 numerology and FR2 numerology are feasible.

There are use cases that favour either FR1 numerology or FR2 numerology

Some of the disadvantages of numerology combinations can be mitigated with implementation choices

WF on numerology:

Option 1: Propose to define two sets of bands, one for FR1 numerology and the other for FR2 numerology.

Option 2: Add 15 kHz and 30 kHz SCS to FR2-NTN

Option 3: Down select to FR1 numerology or FR2 numerology as per the WID

Option 1 has broad support

Option 2 requires further study by companies during this week.

- The FR1-NTN sync raster would need to be added to FR2-NTN.

- All FR2-NTN SCS would become optional to accommodate different use cases.

- Half as many bands would be required.

- Is RAN1 input not required?

- Any other unintended consequences?

Option 3 would be very difficult to agree

Based on the company comments, RAN4 agreemwnt is for option 1.

RAN4 will further discuss SCS options.

Company comments

[Intelsat] Preference is Option 1 with 15 kHz and 30 kHz SCS for FR1. Additionally, it is preferred to have 60 kHz SCS option for various services for FR1.

[MediaTek} On the numerology issue, MTK is fine to go with Option 1. Some down-selection on the SCS is fine, if the workload is the concern. In our view, Option 2 can be removed, because it is very confusing and it definitely requires additional works in RAN1 and RAN2.

[Lockheed Martin] Supports - Option 1: which Proposes to define two sets of bands, one for FR1 numerology and the other for FR2 numerology. As These bands would operate independently and offer flexibility !

[European Space Agency] ESA is supporting Option 1. If SCS priority might help the RAN4 workload, I am fine with 15 / 30 / 120.

[Eutelsat] Prefer option 1 with 15 & 30 kHz and 60 kHz as a possible option.

[Qualcomm] QC also support option 1. 1)	Regarding the feasibly of numerology, we don’t think RAN4 has the official agreements on those numbers. For example, in the UL timing analysis, there was no consensus that 31.6 m is only way or round trip path length uncertainty. We would not keep this wording in the WF but if companies think it is helpful, we can put it in the Annex and keep it for information. It would be preferable for us if you can remove the details from WF and capture them in the TR. 2)	Regarding the down-selection of SCS from FR1 and FR2, we slightly prefer to choose one from FR1 (e.g., 30kHz) and one from FR2 (e.g., 120khz) to control the workload.

[JSAT] Regarding FR1/FR2 down selection,, SKY Perfect JSAT supports Option 1: Propose to define two sets of bands, one for FR1 numerology and the other for FR2 numerology. FR1's small channel bandwidth is necessary not only for legacy GEO satellites but also for upcoming SDS/HTS GEO satellites. In particular, the transponder channel bandwidth of legacy GEO satellite, such as 27MHz, 36MHz or 72Mhz, makes it difficult to provide 5G NTN service without the smaller channels offered by FR1.

[Sharp] We support Option 1(Propose to define two sets of bands, one for FR1 numerology and the other for FR2 numerology) given the estimated work scope is less than other options.

[Thales] Supports option 1.

[Airbus] Supports option 1.

[SES] Supports option 1.

[ESA] Supports option 1.

[CHTTL] Supports option 1

# Annex: System parameters analysis for information

### Sub-topic 3-1: UL timing

FR1 numerology is feasible for NGSO.

FR1 numerology is feasible for GSO by adopting a new error model where the correlated part of Tp,san is discounted

Assuming a new error model where the correlated part of Tp,san is discounted:

FR2 numerology for Fixed and mobile VSAT served by NGSO and GSO with 60 kHz UL SCS is feasible

FR2 numerology for Fixed VSAT served by NGSO and GSO with 120 kHz UL SCS is feasible if the installation has a wide sky view down to 30 degrees, or a manual map-based installation.

FR2 numerology for Mobile VSAT served by NGSO and GSO with 120 kHz UL SCS is feasible for platform types that can achieve a 31.6 m one-way path length uncertainty

### Sub-topic 3-2: Phase noise (and residual frequency error impact)

FR2 numerology is feasible with regards to phase noise

FR1 numerology is feasible with regards to phase noise but may require a higher quality oscillator, particularly for 15 kHz SCS to avoid performance degradation.

### Sub-topic 3-3: Efficient channel utilization (allocation of RBs)

FR1 numerology is feasible with regards to efficient allocation of RBs to the required bandwidths

FR2 numerology is feasible with regards to efficient allocation of RBs to the required bandwidths but is less efficient than FR1 numerology for legacy narrower bandwidths but more efficient than FR1 for 250 MHz

### Sub-topic 3-4: Beam hopping granularity

Both FR1 numerology and FR2 numerology are feasible with regards to beam hopping granularity.

FR2 numerology provides more flexibility than FR1 numerology for systems employing analogue beamforming.

### Sub-topic 3-5: Link budget

Both FR1 numerology and FR2 numerology are feasible with regards to link budget.

### Sub-topic 3-6: Doppler

Both FR1 numerology and FR2 numerology are feasible with regards to Doppler.

### Sub-topic 3-7: Resilience to frequency Errors/ UE Doppler pre-compensation

Both FR1 numerology and FR2 numerology are feasible with regards to residual frequency error

FR1 numerology with 15 kHz SCS may experience reduced performance due to residual frequency error (a 1.4 kHz offset at 14 GHz)

### Sub-topic 3-8: Spectral efficiency & maximum throughput per channel configuration (currently CA not supported)

Both FR1 numerology and FR2 numerology are feasible with regards to spectral efficiency

Based on impairments related to residual frequency error, FR1 numerology with 15 kHz SCS may provide a lower maximum throughput than 30 Hz SCS or FR2 numerology.