3GPP TSG-RAN WG4 Meeting #113	R4-2420367

# Orlando, United States,18 – 22 Nov, 2024

Source: 	Spreadtrum, Ericsson, Huawei, Sony

Title:	TP to TR 38.769 on the RF Requirements for Ambient IoT device 1

Agenda Item:	7.21.3.2

Document for:	Approval

Introduction

The SID on solutions for Ambient IoT was revised in RAN#103 [1]. This contribution provides a text proposal for TR 38.769 on the RF requirements impact on the Ambient IoT device 1.

References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA, RAN #103, Mar. ,2024

- R4-2417088, WF on impacts of A-IoT on RF requirements, Moderator (Huawei), RAN4#112bis, Oct.,2024

Text Proposal to TR 38.769

<Start of Text Proposal>

- 6.7.4.1 Device 1

For device 1, Tx and Rx RF requirements are studied.

        - TX requirements

          - Transmitter power

For device 1, D2R backscattering is transmitted on a carrier wave provided externally, backscattering power and power backscattering loss are potentially needed to be defined to ensure the coverage range between reader and device. Maximum output power and other transmit output power requirements are potentially not needed.

The minimum backscattered modulated power can be specified for device 1. The backscatter power is the amount of power reflected back from the device when a successful command sequence is completed at the device at a specific sensitivity level.

The backscatter loss due to the modulation can be defined as scattering cross-section or modulation efficiency in equation (4). From equation (4), it can be observed that the modulation efficiency is defined as the ratio of the power radiated by the device to the power density incident on its antenna.

Assume the power density of the incident radio wave on the device antenna is S

S=Pt∙Gt4πr2 					(1)

Where the Pt is transmission power of a CW signal and Gt is the transmitter antenna gain, r is the distance between test equipment to the device.

The received power on device antenna is Pa

Pa=λ2∙Gd4π∙S 					(2)

Where the λ is the radio wavelength and Gd is the device antenna gain.

The power backscattered back in the direction of the transmitter is Pbackscatter:

Pbackscatter=K∙Pa∙Gd=4∙Ra2Za+Zc1,c22  ∙ λ2∙Gd24π∙ S		(3)

Where the Za=Ra+jXa  is complex antenna impedance and Zc1,c2=Rc1,c2+jXc1,c2 is complex chip load in different modulation status.

The backscatter loss or cross section of the device 1 can be defined as

σ=PbackscatterS=Ra2Za+Zc1,c22  ∙ λ2∙Gd2π 			(4)

The received backscattered signal at the test device:

Preceived=Pbackscatter∙Gr4πr2=Pt∙Gt4πr2 ∙Gr4πr2∙σ 				(5)

          - Output power dynamics

Output power dynamics includes minimum output power, transmit OFF power, transmit ON/OFF time mask and power control.

Power control is not designed for A-IoT device 1 D2R link, and the transmit power of device 1 comes from CW node, making Output power dynamics requirements unnecessary.

          - Transmitted signal quality

Transmitted signal quality includes frequency error, carrier leakage, in-band emission and EVM.

As D2R backscattering is transmitted on a carrier wave provided externally, the requirements for frequency error, carrier leakage and in-band emission are potentially unnecessary.

The waveform for D2R could be OOK or BPSK. OOK signal is not mapped by the legacy constellation. EVM may not be suitable for D2R OOK waveform, and whether EVM testing is feasible for BPSK waveform needs to be discussed.  Requirement to guarantee the modulation quality is potentially needed. How to define modulation quality requirements will be discussed in WI.

          - Output RF spectrum emissions

Output RF spectrum emissions includes occupied bandwidth, out of band emission (SEM, ACLR), spurious emissions and transmit intermodulation.

Occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. Occupied bandwidth requirement is potentially needed. The specific value is dependent on the physical layer design of D2R, which has not yet been finalized at this stage.

SEM and spurious emissions are related to co-existence study. Whether to define these requirements need to be discussed in WI. Based on the co-existence study, ACLR is not needed.

The transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna. However, device1 is low complexity and low cost, and has no active components at the transmitter. SEM, spurious emissions may cover this function and requirement. Transmit intermodulation requirement is unnecessary.

        - RX requirements

          - Reference sensitivity

Reference sensitivity is strongly related to the coverage of R2D.  Reference sensitivity is potentially needed.

          - Maximum input level

The received power level for device 1 is up to the Reader and the distance between Reader and device. No power control for device. Whether maximum input level to be defined needs to be discussed in WI.

          - Adjacent channel selectivity (ACS)

Device1 is RF-ED architecture with low complexity and cost, apart from the simple LPF and matching network, other components cannot suppress the interference. Hence, potentially, there is no need to define ACS.

          - Adjacent Subcarrier selectivity (ASCS)

Same as ACS, potentially, there is no need to define ASCS.

          - In-band blocking

Same as ACS, potentially, there is no need to define in-band blocking.

          - Out-of-band blocking

Same as ACS, potentially, there is no need to define in-band blocking.

          - Spurious emissions

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector. The received power comes from R2D signal and device 1 has no LNA. Hence, potentially, there is no need to define spurious emissions.

          - Spurious response

Same as ACS, potentially, there is no need to define spurious response.

          - Intermodulation Characteristic

Same as ACS, potentially, there is no need to define spurious response.

          - Interference rejection

The interference could be wide-band NR signal (e.g. in-band deployment) or CW signal (e.g. from unsynchronized reader/CW node) or D2R signal from other devices. Whether interference rejection to be defined needs to be discussed in WI.

<End of Text Proposal>