3GPP TSG-RAN WG4 Meeting #113	                                   R4-2420410

Orlando, US, 18th – 22nd November, 2024

Title:	LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx

Response to:

Release:	Rel-19

Work Item:	NR_CADC_SUL_R19

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

Name:                  Huiping Shan

E-mail Address:   shanhuiping@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 has discussed the UE RF requirements for 2UL/2DL inter-band FDD-FDD CA_n5-n8. It is agreed that 2UL/2DL CA_n5-n8 is supported using non-concurrent n5 DL and n8 UL.

Because n8 UL frequency range of CA_n5-n8 partially overlaps with n5 DL frequency range, there is no concurrent operation between n8 UL transmitting and n5 DL receiving, which is a special FDD-FDD inter-band CA case. Figure below conceptually illustrates the intended operation of this UL CA combination where the operation will be alternating between the two status under the RRC configuration (UL CA_n5-n8 and DL CA_n5-n8).

Usually for FDD-FDD interband CA, a band pair nA+nB can support simultaneous (nAUL, nBDL) and (nBUL, nADL) by default. For the specific band pair n5+n8, the capabilities for simultaneous (n5UL, n8DL) and (n8UL, n5DL) can be different, currently simultaneous (n5UL, n8DL) is supported but simultaneous (n8UL, n5DL) is not.

RAN4 kindly asks RAN2:

- To make necessary modifications in RAN2 specification and/or to add UE capability, if needed, to enable the described functionality.

- If there’s a capability defined, it’s mandatory for the corresponding band combinations when indicating UL CA support.

- Comparing to the TDD-TDD/TDD-FDD CA non-simultaneous Rx/Tx feature, it’s encouraged to double check whether there is an issue for this FDD-FDD CA case, and report back to RAN4 if those are found.

NOTE: This is intended only for special case when UL and DL from different FDD bands partially overlap. This is not intended for FDD bands which do not partially overlap.

NOTE: The described functionality is targeted also for cases when higher order CA configuration (e.g CA_n5A-n8A-nXXA) supports 2UL in n5 and n8.

Currently there is only one band combination, i.e., n5+n8 has such an issue, however, RAN4 appreciate if RAN2 may consider a generic future-proof solution in case more similar NR FDD-FDD inter-band CA band combinations where UL and DL from different FDD bands partially overlap are introduced in RAN4.

2. Actions:

To RAN WG2:

RAN4 respectively asks RAN2 to consider the above information. and to make necessary modifications into RAN2 specification and to define UE capability if necessary.

3. Date of Next TSG-RAN Meetings:

TSG RAN WG4 Meeting #114		Feb. 17 – Feb. 21, 2024	Athens, Greece

TSG RAN WG4 Meeting #114-bis		Apr. 17 – Apr. 21, 2024	China