3GPP TSG-RAN WG4 Meeting #113	R4-2417684

Orlando, US, 18-22 November 2024

Source:	Nokia

Title:	On remaining Rel-18 multi-Rx issues

Agenda item:	5.10.1

Document for:	Discussion

Introduction

In this contribution we bring forth some issues which need to be addressed in the test case for dual TCI state switching for m-DCI.

Discussion

It was agreed in the RAN4#111 that test case for dual TCI state switching for m-DCI would be defined using three probes. Of the three probes, RS1 and RS3 will be mapped to probe 1 while RS2 will be mapped to probe 2 and RS4 will be mapped to probe 3.

This means that initially during the test, the UE would receive from RS1 and RS2 and would then receive TCI state switch command to receive from RS3 and RS4. For the UE to receive from RS3 and RS4 it needs to be time and frequency synchronized with these RS. Since RS1 and RS3 are transmitted from the same probe, it means the UE doesn’t need to do a time and frequency synchronization with RS3 which would provide incorrect conformance test results.

Since three probes are to be used during the test case for TCI state switching delay in m-DCI, two RS (RS1 and RS3) will be transmitted from same probe.

Since the UE is already receiving from one of the RS transmitted by the probe before the TCI state switch, if it has to receive from another RS from the same probe, then the time/frequency synchronization which the UE needs to perform with the target RS cannot be tested accurately.

RAN4 agreement does not clarify if RS1 and RS3 will be transmitted with same power and at the same time from the same probe.

Test cannot verify accurately if the UE has performed time/frequency synchronization when it receives the command to switch to PDSCH QCL’d to RS3 if RS1 and RS3 are transmitted with same power and at the same time from the same probe.

To have meaningful test results in such situations, the test probe transmitting both RS1 and RS3 should emulate different radio conditions for RS1 and RS3. For example, RS1 should be transmitted at a different time/frequency and power as compared to RS3. This will ensure that the UE has done a time/frequency synchronization with the target RS before switching to the target TCI state when receiving a DCI command for a TCI state switch

When three probes are used to transmit four RS, the test probe transmitting more than one RS should emulate different DL transmit beams by transmitting different RS with different power and delay

When using the same probe for RS1 and RS3, the reference signals (RS1 and RS3) should be transmitted with a different power (5 dB difference) and with a time delay (difference of 25% of CP length).

Another issue with the test case is that the test switches UE’s PDSCH TCI states in T2. Which means that the UE during T1 also is receiving the PDSCHs. This does not come clear in the test setup.

Test case does not indicate that UE has PDSCH during T1.

We have proposed to fix the observation 5 in our companion CR.

Conclusion

The following observations and proposals were made in this paper:

Observation 1: Since three probes are to be used during the test case for TCI state switching delay in m-DCI, two RS (RS1 and RS3) will be transmitted from same probe.

Observation 2: RAN4 agreement does not clarify if RS1 and RS3 will be transmitted with same power and at the same time from the same probe.

Observation 3: Test cannot verify accurately if the UE has performed time/frequency synchronization when it receives the command to switch to PDSCH QCL’d to RS3 if RS1 and RS3 are transmitted with same power and at the same time from the same probe.

Observation 4: When three probes are used to transmit four RS, the test probe transmitting more than one RS should emulate different DL transmit beams by transmitting different RS with different power and delay

Proposal 1: When using the same probe for RS1 and RS3, the reference signals (RS1 and RS3) should be transmitted with a different power (5 dB difference) and with a time delay (difference of 25% of CP length).

Observation 5: Test case does not indicate that UE has PDSCH during T1.