3GPP TSG-RAN WG4 Meeting #113	R4-2417688

Orlando, US, 18-22 November 2024

Source:	Nokia

Title:	On Rel-19 CLTM requirements

Agenda item:	7.24.2.3

Document for:	Discussion

Introduction

The discussion about RRM requirements for conditional LTM started in the RAN4#112bis meeting. With the lack of RAN1/2 agreements in the last meeting, RAN4 was only able to make some high-level agreements about the scope of the objective. In this contribution we continue the discussion based on the agreements that were reached in the last RAN2 meeting.

Discussion

RAN2 has agreed the following about the Rel-19 CLTM design:

Agreements on C-LTM - Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions. - Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. - Source cell and each candidate cell provides its own execution condition for conditional LTM. - It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used. - RACH-less Conditional intra-CU LTM is supported. - RACH based conditional intra-CU LTM is supported. - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM. - Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. - For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach. - The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

Based on the listed agreements, RAN4 can already make some conclusions about the cell switch delay for conditional LTM. We discuss the status of the most relevant issues in the last meeting’s WF [1] based on the current design in the following.

Issue 1-1-4: Subsequent conditional LTM

RAN4 agreed to define cell switch delay requirements for subsequent CLTM unless any technical issue is found. It remained open whether the cell switch delay will be the same for initial and subsequent cell switch. In our view the current RAN2 agreements are not sufficient to make any conclusion about this aspect.

Wait for more RAN2 agreements regarding subsequent CLTM.

Issue 1-2-2: The starting point of conditional LTM delay

RAN2 has agreed that the source cell sends the conditional LTM configuration to the UE via RRCReconfiguration, which includes the LTM candidate configurations and the corresponding execution conditions. However, there is no agreement stating whether the condition evaluation begins at the time the UE receives the configuration. Therefore, we think RAN4 cannot make an agreement about the starting point yet.

Wait for more RAN2 agreements before agreeing on the starting point of the CLTM cell switch delay.

Issue 1-2-3: The ending point of conditional LTM delay

RAN2 has agreed to support both RACH-less and RACH-based CLTM and that the LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM. This in our understanding means that the ending point of Rel-18 cell switch delay can be reused for CLTM.

Reuse the ending point of Rel-18 LTM cell switch delay for RACH-less and RACH-based CLTM.

Issue 1-2-4: The components of conditional LTM delay

While the starting point of condition evaluation is still open, and while it has not yet been agreed whether early TCI state activation and early TA acquisition are mandatory or optional, we think it is difficult for RAN4 to agree on the exact components of CLTM. As discussed in the last meeting, parameters such as TEvent_DU and Tmeasure will be needed, but the necessity of other LTM components in the delay equation is to be discussed. We propose to wait for further RAN2 agreements on the starting point of condition evaluation and early DL and UL synchronization before making an agreement about the delay formula.

TEvent_DU and Tmeasure like components are likely needed in the CLTM cell switch delay, but the exact definition is pending more RAN2 agreements.

Wait for more RAN2 agreements before agreeing on the exact components of the CLTM cell switch delay.

Issue 1-2-9: Tinterrupt and Issue 1-2-11: The components of Tinterrupt

The components that are included in Tinterrupt depend obviously on the components that will be included in the CLTM cell switch delay. We do not think it is possible to agree on the detailed components before the design is more complete, but we would like to suggest that RAN4 discusses at least the need of including some of the Rel-18 components in the interruption, if included in the delay.

Such components are TLTM_RRC-processing, Tfirst-RS and TRS-proc. Independent of whether we will use these same components or somewhat edited components, we think it is reasonable to assume that these components do not need to be considered as part of the interruption. When the steps related to these components –ASN.1 decoding and fine DL synchronization – are done before the cell switch in LTM, the procedures do not cause an interruption. We proposed in Rel-18 that the components would not cause an interruption during the cell switch either, but at that time it was considered that since the network sends the cell switch command, the UE will not be scheduled after the receiving the cell switch command anymore. This is not the case in CLTM, where the cell switch decision happens at the UE side. Hence, we think ASN.1 decoding and DL synchronization components, if included in the cell switch delay, should not be part of Tinterrupt.

ASN.1 decoding and validity check is not part of Tinterrupt but may be part of DCLTM if not done before the starting point of the cell switch delay.

Fine DL synchronization (first SSB) is not part of Tinterrupt but may be part of DCLTM if not done before the starting point of the cell switch delay.

Issue 1-3-1: Early DL sync and Issue 1-3-3: Early TCI state activation

RAN2 has agreed to support early TCI state activation according for CLTM. It is unclear whether TCI state activation according to Rel-18 is to be supported directly as it is, or whether the activation procedure will be updated somehow, or whether there will be more than one option for early TCI state activation. If the Rel-18 TCI state activation is reused, RAN4 can reuse the Rel-18 early TCI state activation delay requirement in section 8.25 for CLTM. However, it is too early to make such conclusion so RAN4 should wait for more RAN2 input.

Wait for more RAN2 agreements before agreeing on early TCI state activation requirements.

Issue 1-3-2: Early UL sync

RAN2 has agreed that PDCCH ordered early TA acquisition and UE based TA estimation are supported for conditional LTM. If the PDCCH ordered RACH procedure follows the Rel-18 principles, RAN4 can reuse the Rel-18 requirements. If the procedure is changed somehow, RAN4 may need to define new requirements for CLTM specific PDCCH ordered RACH delay. We propose to wait for more RAN1/2 agreements before evaluating the need for new or edited requirements.

RAN4 to wait for more RAN1/2 agreements regarding PDCCH ordered RACH procedure for CLTM.

Regarding UE based TA estimation, RAN4 did not define requirements in Rel-18. Hence, RAN4 shall agree whether to not define those requirements for CLTM either, or to define requirements that apply for both LTM and CLTM.

RAN4 to discuss whether to define requirements for UE based TA estimation in LTM and CLTM or not.

Conclusion

The following observations and proposals were made in this paper:

Proposal 1: Wait for more RAN2 agreements regarding subsequent CLTM.

Proposal 2: Wait for more RAN2 agreements before agreeing on the starting point of the CLTM cell switch delay.

Proposal 3: Reuse the ending point of Rel-18 LTM cell switch delay for RACH-less and RACH-based CLTM.

Proposal 4: TEvent_DU and Tmeasure like components are likely needed in the CLTM cell switch delay, but the exact definition is pending more RAN2 agreements.

Proposal 5: Wait for more RAN2 agreements before agreeing on the exact components of the CLTM cell switch delay.

Proposal 6: ASN.1 decoding and validity check is not part of Tinterrupt but may be part of DCLTM if not done before the starting point of the cell switch delay.

Proposal 7: Fine DL synchronization (first SSB) is not part of Tinterrupt but may be part of DCLTM if not done before the starting point of the cell switch delay.

Proposal 8: Wait for more RAN2 agreements before agreeing on early TCI state activation requirements.

Proposal 9: RAN4 to wait for more RAN1/2 agreements regarding PDCCH ordered RACH procedure for CLTM.

Proposal 10: RAN4 to discuss whether to define requirements for UE based TA estimation in LTM and CLTM or not.