3GPP TSG-RAN WG4 Meeting #113	R4-2417689

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	On NR MIMO Phase 5 UE-initiated/event-driven beam management

Agenda item:	7.19.3.1

Document for:	Discussion

Introduction

In this document, we present Nokia’s views regarding the core requirements that RAN4 needs to discuss in relation to the UE-initiated beam management feature.

Discussion on reporting delay requirements

Regarding UE-initiated beam management (UEIBM) reporting delay requirements, the following was agreed and discussed in the last RAN4#112 bis meeting [1]:

Issue 1-1: How to define Event triggered measurement reporting delay? < Agreement> - Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air. - Ending point is the point when the UE transmits a [first PUCCH] triggered by the event. - “[first PUCCH]” can be updated based on further RAN1/2 agreement. Issue 1-3: Applicability of Event triggered measurement reporting delay requirements? < Agreement> - Applicable for all events agreed in RAN1/2 - Applicable to both Mode A and B - Note: applicable to the event/mode supported by UE

Hence, the starting and ending point of the delay as well as the applicability of the requirements to both modes A and B and all supported events is agreed. The details of the delay between the moment the reporting condition is met and the first PUCCH is sent remains open.

RAN1 has defined two types of triggering event determination for Event-2:

- Basic: UE initiates a beam report when L1-RSRP of the new beam becomes a threshold value better than the current beam.

- Optional (subject to UE capability): UE initiates a beam report if within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M.

Since a practical network deployment would have a mix of UEs, i.e., UEs only supporting the basic feature and UEs supporting the optional capability, RAN4 requirements should be defined for both the basic and the optional case. Additionally, as the start of the event triggered measurement reporting delay shall be when the event condition is met and the end point shall be when the UE sends the first PUCCH, there is no impact of the time window and M on the reporting delay. Hence, starting and ending point shall be the same for both the cases.

Requirements should be defined for both the basic functionality, i.e., without filtering, and the case with filtering (i.e., timer and counter M).

The definition of the starting and ending point of reporting delay shall be the same for the scenarios with filtering and without filtering.

Issue 1-2: How to define measurement period in Event triggered measurement reporting delay?

In order to define the measurement period in the event triggered measurement reporting delay, the existing L1-RSRP measurement period defined in Section 9.5 for CSI-RS and SSB based L1 measurements can be used as baseline.

Table 1

Table 2

Table 1 and Table 2 illustrate the various components which are taken into account while arriving at the legacy SSB based L1-RSRP measurement period in FR1 and FR2 respectively.  The existing requirements consider a reporting periodicity (Treport) which is not valid in the context of the UEI based reporting scenario and hence shall not be considered when defining the measurement period for UEI.

For the basic event-evaluation functionality (without time window), reuse existing L1-RSRP measurement period as a baseline while defining measurement reporting delay requirements for UE-initiated/event-driven beam management.

Reporting periodicity (Treport) shall not be considered for the event-triggered reporting delay.

Proposals 3 and 4 would apply for the basic scenario without configuration of the triggering window. For the time window option, as agreed in RAN1, an event condition is met when the number of Event-2 instances for at least one same new beam is greater than or equal to M within a time window UE initiated beam report occurs.

Agreement (RAN1 #117 Fukuoka) Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. Agreement (RAN1 #118bis Hefei) Regarding the triggering event determination for Event 2, the event instance(s) counting is per new beam. Further study candidate condition(s) of resetting the counting including whether resetting is needed.

There can be cases where the number of Event-2 instances within a time window exceeds M. This then implies that the length of the time window has a greater impact than the number of measurements samples (M) on the measurement period and hence needs to be considered.

An event condition is met when the number of Event-2 instances for at least one same new beam is greater than or equal to M within a time window.

The length of the measurement period will depend on the time window within which M or greater than M Event-2 instances of the same beam need to occur.

It is possible to have cases where the number of Event-2 instances within a time window exceed M.

L1-RSRP measurement period for UEIBM equals the time window where the number of event instances (≥ M) for one same beam have occurred to determine that the event triggering condition has been met.

All UEIBM events are associated to RSs that the UE must monitor/measure. Related to that, there are some issues that need to be taken into account when defining event-triggered L1 reporting delay requirements (see also Figure 1):

- We have at least two beams involved, and therefore at least two RSs that need to be monitored/measured by the UE for triggering the event;

- The periodicities of the two RSs can be different;

- Even if the two RSs have the same periodicity, that can be different from the periodicity of the first PUCCH channel;

- The actual time instant of the RSRP measurement of an SSB or CSI-RS is unique for all the UEs, and just need to meet the RSRP measurement period requirements defined in clause 9.5.4 of TS 38.133.

Therefore, in order to define a maximum delay for the UE transmission of the UL indication in the first PUCCH channel, we should consider as starting point the time instant a RS associated to either the current beam or a candidate new beam that actually triggered the UEIBM event is received by the UE.

The maximum delay requirement for the first PUCCH channel is determined in relation to the reception time instant of the RS that triggered the UEIBM event.

(a)

(b)

Figure 1. UEIBM Event-2 (assuming network configured 3 dB threshold between candidate new beam and current beam) triggered by (a) a candidate new beam RSRP measurement and (b) a current beam RSRP measurement.

Discussion on TCI switching requirements

As per the current RAN1 agreements on the UEI beam reporting, there seems to be no real impact on the TCI state switching delay requirements. Since it has been agreed in RAN1 to use the unified TCI state switching as a baseline, RAN4 can reuse the existing unified TCI state switching delay requirements for event triggered measurement reporting based TCI state switching as well.

This can be revisited based on further RAN1 progress in case required.

Existing unified TCI state switching delay requirements shall be reused for event triggered measurement reporting based TCI state switching

In case of a TCI state being considered as known or unknown, with reference to the RAN1 agreements in UEIBM so far, we see no impact on the existing conditions which determine if a TCI state can be considered as known or unknown.

Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM

Discussion on other measurement requirements

Issue 1-12: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management?

Currently, L1-RSRP has been agreed as the baseline quality metric for trigger events, although other quality metrics like L1-SINR are under discussion in RAN1. However, such measurements can be performed by the UE as done in legacy, so there is no need for RAN4 to define new measurement accuracy requirements specific for UEIBM.

Reuse legacy measurement accuracy requirements for measurement reports with UEIBM.

Issue 1-11: Whether to enhance beam sweeping in event triggered L1 beam reporting?

Clause 9.5.4 of TS 38.133 specifies measurement period requirements to compute L1-RSRP to be reported in the baseline periodic/semi-persistent/aperiodic reporting specified up to Rel-18. Such requirements are defined in both FR1 and FR2, and for SSBs in FR2 they read as follows

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2 Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P*N)*TSSB). DRX cycle ≤ 320ms; max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(1.5*M*P*N)*TDRX. Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting..

In FR2-1, the baseline value for the beam sweeping factor N is N=8. One of the main motivating factors for specifying the UE initiated beam management feature is to facilitate fast beam switching. However, if the beam sweeping factor is still assumed to be 8 while defining the RAN4 RRM requirements for UEIBM, then the resultant large measurement delay would not fulfil the one of objectives of the UEIBM feature which is to reduce latency. Rel-18 RRM requirements defined for multi-Rx support a smaller beam sweeping factor such as N= 2, 4, 6, subject to UE capability, thus resulting in shorter measurement delays. It is then logical to conclude that faster beam sweeping for L1-RSRP measurements during UEIBM should be enabled for UEs which have the faster beam sweeping capability.

Hence while defining requirements for event triggered L1 beam reporting, beam sweeping factor (N) should be based on UE capability.

Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting.

Issue 1-8: CSI-RS based L1-RSRP for inter-cell BM?

There are currently no baseline RAN4 requirements defined for inter-cell CSI-RS based L1-RSRP measurements for beam management. Hence, unless a clear use case is identified we propose not to introduce CSI-RS based L1-RSRP requirements for inter-cell beam management.

No baseline RAN4 requirements defined for CSI-RS based L1-RSRP measurements for inter-cell beam management.

RAN4 not to define requirements for CSI-RS based L1-RSRP measurements for inter-cell beam management.

Issue 1-13: Capabilities for Support of Event Triggering and Reporting Criteria? And Issue 1-14: Compatibility of capabilities with LTM

While we agree that RAN4 needs to define how many events the UE shall be able to monitor, we think this can be discussed towards the end of the work item. Same would apply for the coordination between MIMO and mobility events.

RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item.

Conclusion

In this paper, the following observations and proposals were made:

Proposal 1: Requirements should be defined for both the basic functionality, i.e., without filtering, and the case with filtering (i.e., timer and counter M).

Proposal 2: The definition of the starting and ending point of reporting delay shall be the same for the scenarios with filtering and without filtering.

Proposal 3: For the basic event-evaluation functionality (without time window), reuse existing L1-RSRP measurement period as a baseline while defining measurement reporting delay requirements for UE-initiated/event-driven beam management.

Proposal 4: Reporting periodicity (Treport) shall not be considered for the event-triggered reporting delay.

Observation 1: An event condition is met when the number of Event-2 instances for at least one same new beam is greater than or equal to M within a time window.

Observation 2: The length of the measurement period will depend on the time window within which M or greater than M Event-2 instances of the same beam need to occur.

Observation 3: It is possible to have cases where the number of Event-2 instances within a time window exceed M.

Proposal 5: L1-RSRP measurement period for UEIBM equals the time window where the number of event instances (≥ M) for one same beam have occurred to determine that the event triggering condition has been met.

Proposal 6: The maximum delay requirement for the first PUCCH channel is determined in relation to the reception time instant of the RS that triggered the UEIBM event.

Proposal 7: Existing unified TCI state switching delay requirements shall be reused for event triggered measurement reporting based TCI state switching

Proposal 8: Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM

Proposal 9: Reuse legacy measurement accuracy requirements for measurement reports with UEIBM.

Proposal 10: Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting.

Observation 4: No baseline RAN4 requirements defined for CSI-RS based L1-RSRP measurements for inter-cell beam management.

Proposal 11: RAN4 not to define requirements for CSI-RS based L1-RSRP measurements for inter-cell beam management.

Proposal 12: RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item.

- References

- R4-2416856, WF on RRM requirements for NR_MIMO_Ph5_Part1, RAN4#112bis, Hefei, Anhui, China, 14th ‒ 18th, October 2024.