3GPP TSG-RAN WG4 Meeting #113	R4-2418154

Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.21.4

Source:	Moderator (CMCC)

Title:	Topic summary for [113][132] FS_Ambient_IoT_solutions_part1

Document for:	Information

Introduction

This summary focuses on the R19 ambient IOT study item under agenda 7.21, 7.21.2.1, 7.21.2.2.  The summary in previous meetings are in R4-2405289, R4-2408945, R4-2412833 and R4-2415233. The way forward agreed in previous RAN4 meetings are R4-2406714, R4-2410567, R4-2414304 and R4-2417196.

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2418723 | Draft TP to TR38.769 on conclusion for A-IOT | CMCC |
| R4-2418801 | A-IoT general aspects (move to 133) | Huawei, HiSilicon |
| R4-2419477 | TP to TR 38.769: A-IoT general overview (move to 133) | Ericsson |
| R4-2418180 | TP to TR 38.769 on the impact from AIoT CW to NR | vivo |
| R4-2418724 | Evaluation methodology for ambient IOT co-existence | CMCC |
| R4-2418802 | A-IoT co-existence evaluation cases | Huawei, HiSilicon |
| R4-2419481 | Co-existence evaluation methodology and cases | Ericsson |
| R4-2419553 | TP to 38.769 on co-existence evaluation methodology | Qualcomm Incorporated |
| R4-2417831 | Further co-existence evaluation results for Ambient IoT in NR | CATT |
| R4-2417979 | Simulation analysis for Ambient IoT co-existence | Huawei, HiSilicon |
| R4-2417980 | TP to TR38.769: Co-existence simulation | Huawei, HiSilicon |
| R4-2417981 | Collection of simulation results for Ambient-IoT co-existence study | Huawei, HiSilicon |
| R4-2418034 | Coexistence evaluation on D1T1 deployment scenario | Sony |
| R4-2418181 | TP to TR 38.769 on D2T2 co-existence evaluation and analysis | vivo |
| R4-2418182 | Further evaluation on D2T2 co-existence | vivo |
| R4-2418403 | Discussion on co-existence evaluation results for Ambient IoT and NR | Spreadtrum Communications |
| R4-2418725 | Discussion on A-IoT co-existence evaluation | CMCC |
| R4-2418852 | Discussion on Ambient IoT Coexistence Evaluations | Qualcomm Incorporated |
| R4-2419199 | Discussion on Co-existence evaluations | ZTE Corporation, Sanechips |
| R4-2419366 | on co-existence evaluations | OPPO |
| R4-2419476 | Coexisting study simulation assumptions and initial results | Ericsson |
| R4-2419554 | TP to 38.769 on co-existence simulation assumptions | Qualcomm Incorporated |
| R4-2419649 | On considerations of frequency shifter | Samsung |

TP

| TDoc | Title | Source | Recommendation |
| --- | --- | --- | --- |
| R4-2418723 | Draft TP to TR38.769 on conclusion for A-IOT | CMCC | To be revised |
| R4-2419553 | TP to 38.769 on co-existence evaluation methodology | Qualcomm Incorporated | Updates on section 6.6.4, to be merged if agreed |
| R4-2417980 | TP to TR38.769: Co-existence simulation | Huawei, HiSilicon | Co-existence summary , to be revised |
| R4-2418181 | TP to TR 38.769 on D2T2 co-existence evaluation and analysis | vivo | Co-existence summary, to be merged to Huawei TP if agreed |
| R4-2418180 | TP to TR 38.769 on the impact from AIoT CW to NR | vivo | Updates on section 6.6.4, to be merged if agreed |
| R4-2418725 | Discussion on A-IoT co-existence evaluation | CMCC | Co-existence summary, to be merged to Huawei TP if agreed |
| R4-2419554 | TP to 38.769 on co-existence simulation assumptions | Qualcomm Incorporated | Updates on 6.6.3, to be merged if agreed |

Evaluation methodology and cases

- Topic 2-1: Performance metric and target SINR

Agreement in RAN4#110bis: - For NR system, use 5% throughput loss as performance metric as legacy. - For AIOT system, including reader, device, intermediate UE, further discuss the performance metric: - Option 1: [10%] BLER, [Rx power] - Option 2: SINR degradation - Other options are precluded Agreement in RAN4#111: - Use SINR for calibration purpose - FFS on performance metric for co-existence evaluation and requirements definition. Agreement in RAN4#112: For inter-system interference (between AIOT and NR): - If SINR degradation is smaller than or equal to [1]dB, it can be considered that inter-system interference is negligible. - If SINR degradation is lager than [1]dB, consider following criteria: - Outage percentage consider SINR level with [10%] BLER - FFS on the outage percentage - Note: For SINR degradation, SINR refers to the 5% and 50% CDF SINR For intra-system interference (between AIOT and AIOT): - Outage percentage consider SINR level with [10%] BLER - FFS on the outage percentage Agreement in RAN4#112bis: - Consider 3 SINR values for D2R and R2D respectively and derive the outage percentage value/range. - The 3 values can be selected according to following steps: - Select one set SNR/CNR values @ 10% BLER from companies based on RAN1 LLS (refer to latest RAN1 LLS summary) - Set the SNR/CNR range by removing the highest and lowest value from the set in step 1 - Pick 3 values from the range in step 2, e.g. highest, lowest, middle Note: R2D CNR needs to be transformed to SINR

Issue 2-1-1: SINR values @10% BLER for D2R

Proposal in RAN4#113:

|  | D1T1 D2R（coherent） | D2T2 D2R （coherent） |
| --- | --- | --- |
| min | -7.8 | -7.6 |
| max | 0.23 | -5 |
| average | -4.3 | -6 |

|  | D1T1 D2R （non-coherent） | D2T2 D2R （non-coherent） |
| --- | --- | --- |
| min | -3 | -9 |
| max | 13 | 2.1 |
| average | 5.5 | -3.2 |

Recommended WF:

Following last meeting agreement, moderator provides the target SINR values for D2R.

More discussion is needed on whether to use all the values above to derive the outage percentage for D2R

Issue 2-1-2: SINR values @10% BLER for R2D

|  | TDL-A device 1 CNR | R2D SNR for device 1 | TDL-A device 2a CNR | R2D SNR for device 2a |
| --- | --- | --- | --- | --- |
| min | 2.6 | -14.8 | 3.37 | -14.1 |
| max | 26.7 | 9.3 | 22.5 | 5.1 |
| average | 10.9 | -6.5 | 10.1 | -7.3 |

Recommended WF:

Following last meeting agreement, moderator provides the target CNR and transferred SINR values for R2D.

Further discuss whether the CNR to SNR conversion with a scaling factor

Option 1: no scaling factor

Option 2: Required SNR/CNR] - lin2dB([ED channel bandwidth] / [transmission bandwidth])

Option 3: others

Recommend confirming above CNR values @ 10% BLER to derive outage percentage, and discuss transferred SINR values under issue 2-2-1 seperately.

Issue 2-1-3: Outage percentage of AIOT system

Proposal in RAN4#113:

Proposal 1 (Ericsson, R4-2415453): Baseline outage percentile is 20% and needs improvement of coexisting simulation assumption to reduce the outage percentile.

Proposal 2 (CMCC, R4-2418725): If outage percentage is smaller than [5%], consider no outage or no performance impact for ambient IOT reader/device.

Recommended WF:

If outage percentage is smaller than [5%], consider no outage or no performance impact for ambient IOT reader/device.

- Topic 2-2: SINR definition

Issue 2-2-1: R2D SINR for device 1 and 2a

Agreement in RAN4#111: Assume no frequency selectivity for co-existence evaluation for calibration purposes for device 1 and 2a. Agreement in RAN4#112: - Alt-1: Assume no frequency selectivity - Alt-2: SINR; R2D with LPF. Interference from NR; Frequency selectivity: [7.5 dB]. Noise bandwidth; [180kHz]. SINRR2D_with_LPF=Wanted_SR2D_180kHzNoiseFloor 180kHz+(InterferenceNR10MHz/LPF_Selectivitylinear). - Alt-3: For R2D receiver with LPF. RF BW; 10MHz. BB BW; [180kHz]. SINRi (linear, not dB); Input SINR at the antenna connector with noise/interferencde BW=RF BW. SINRo (linear, not dB); Output SINR after BB LPF assuming downconverted noise/interference BW=BB BW. SINRo=SINRi28*SINRi+4*BWRFBWBB. - Alt-4: other LLS assumptions are not excluded Agreement in RAN4#113: - For R2D, SINR at antenna connector is used as the starting point, i.e interference and noise bandwidth is 10MHz.

Proposal 1 (Ericsson, R4-2419481): To factor in the noise increase in the SINR calculation based on the LLS results, the noise increase is listed in Table 1 for different PSD difference between A-IoT RB and NR RB. Such noise raise value can be added in the SINR calculated using the 180kHz noise bandwidth. With this, we suggest the procedure below for R2D SINR calculation:

Step 1: Calculate the SINR within the A-IoT transmission bandwidth (1 RB)

Step 2: Calculate the A-IoT signal power in linear scale and calculate the A-IoT signal PSD level within one RB (should be the same with A-IoT signal power as only one RB is transmitted in this case).

Step 3: Calculate the NR interference power P_NR in linear scale within the 10 MHz

Step 4: Calculate the PSD of NR signal within 1 RB:  P_NR /51

Step 5: Compare the PSD level difference in dB between PSD of NR signal in Step 4 and A-IoT signal power in Step 2.

Step 6: Look up for the noise raise value in dB in Table 1 referencing to the PSD difference in step 5

Step 7:  Calculate the new SINR value by subtracting the noise raise value in dB (Step 6) into the SINR in step 2

Step 8: Compare the SINR in step 7 to SNR threshold without the NR interference, if lower than SNR threshold, mark the device as outage.

Table 1: noise increase due to NR signal interferer for a noise bandwidth of 180kHz RF ED receiver

| Case#1 | PSD difference | SNR threshold | Noise raise factor (dB) |
| --- | --- | --- | --- |
| 1 | -22 | 17,8 | 0 |
| 2 | -21 | 18 | 0,2 |
| 3 | -20 | 18,15 | 0,35 |
| 4 | -19 | 18,3 | 0,5 |
| 5 | -18 | 18,5 | 0,7 |
| 6 | -17 | 18,8 | 1 |
| 7 | -16 | 19,2 | 1,4 |
| 8 | -15 | 19,5 | 1,7 |
| 9 | -14 | 20,1 | 2,3 |
| 10 | -13 | 21,2 | 3,4 |
| 11 | -12 | 22,8 | 5 |
| 12 | -11 | 26,7 | 8,9 |

Proposal 2 (Sony, R4-2418034): to obtain the SINR in R2D link correctly, the RF-ED receiver needs to be correctly modelled with consideration of the self-mixing effect.

Recommended WF:

More discussion is needed.

Issue 3-2-2: SINR definition for D2R

Agreement in RAN4#111: Do not consider CW interference for calibration purpose for D1T1-A2 and D2T2-A2 FFS on how to consider CW cancellation capability in formal simulation Agreement in RAN4#112: - SINR includes CW interference is used as the baseline reference for co-existence evaluation for CW reader. baseline SINR=received wanted signal powernoise + intra_system interferencewithin total receiver bandwidth+residual CW interference after cancellationin linear scale - SINR is calculated as total power ratio: SINR=received wanted signal powernoise + intra_system interference + inter_system interferencewithin total receiver bandwidth+residual CW interference after cancellationin linear scale Agreement in RAN4#112bis: - For -B and -A1, consider following baseline SINR: - Option 1 (last meeting agreement): baseline SINR for CW outside=received wanted signal powernoise + intrasysteminterferencewithin total receiver bandwidth+residual CW interference after cancellationin linear scale - Option 2 (new proposal): baseline SINR for CW outside=received wanted signal powernoise + intrasysteminterference+phase noisewithin total receiver bandwidth+residual CW interference after cancellationin linear scale Note: If option2 is used, companies needs to report phase noise modelling.

Proposal in RAN4#113：

Proposal 1 (Qualcomm, R4-2418852):

- Phase noise has non-negligible impact to the D2R SINR.

- RAN4 should specify phase noise requirements for the component which provides the phase noise to the system.

Recommended WF:

Recommend discussing in thread 133 on the potential phase noise requirements.

Evaluation parameters and results

- Topic 3-1: Adjacent RB Tx and Rx characteristics

Issue 3-1-1:  A-IOT reader and NR BS

Agreement in RAN4#112bis: In-band. Tx; Rx. NR UE/A-IOT Intermediate UE; For NR UE: - Baseline: Legacy UE IBE For intermediate UE: - Baseline: legacy UE IBE - Optional: NB-IOT ACLR; In-band A-IOT signal interference to NR is not considered when calculating SINR. Companies to check the interference caused by non-orthogonal A-IOT signal. NR BS; Option 1: 30dBc Option 2: 17dBc Option 3: 12dBc with 1 blank RB; In-band A-IOT signal interference to NR is not considered when calculating SINR. Companies to check the interference caused by non-orthogonal A-IOT signal. A-IOT BS; ACLR of legacy NB -IOT gNB (i.e. ACLR1:40dBc，ACLR2:50dBc); ACS of legacy gNB.

Proposals in RAN4#113:

Proposal-1 (Ericsson, R4-2419476): The inband leakage from iFFT modulation should be 12 dB considering averaging all leakage within the blanked RB.

Proposal 2 (Huawei, R4-2418802):

- the NR BS in-band ACLR range from 19 to 33dB with 0 to 2 guard RBs.

- Different NR BS in band ACLR values can be used for evaluate the number of guard RBs required to ensure coexistence performance.

Recommended WF:

Three options for NR BS Tx had already been captured as the simulation assumption. No further discussion is needed.

Issue 4-1-2: CW unwanted emissions

Agreement in RAN4#112: Discuss whether and how to model the CW unwanted emission.

Agreement:

The impact of CW node to adjacent NR carrier can be analysed and captured in TR.Note: no impact to co-existence simulation

Proposals in RAN4#113:

Proposal-1 (Ericsson, R4-2419476): CW unwanted emission can be modeled with 37 dBc for ACLR1 and 47 dBc for ACLR2

- Topic 3-2: Evaluation Conclusion

Moderator note:

Following table summarizes the observation of evaluation from companies’ contributions. Updates are needed according to agreements during RAN4#113 and more simulation input from companies.

| deployment scenario and topology |  | spectrum | Evaluation case No. | (Aggressor -> Victim) | Tentative Co-existence conclusion in RAN4#112bis | Proposals in RAN4#113 | Recommended WF in RAN4#113 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | D1T1-A2- legacy UE only outdoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |  | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | b | NR DL->reader | TBA | Proposal 1 (Huawei): SINR degradation is less than 1dB. Inter-system interference is negligible. Outage percentage for intra-system interference is 0.64%. Proposal 2 (Sony): the SINR degradation on both 50% and 5% in case b of deployment scenario 1-1 is negligible. Proposal 3 (Spreadtrum): There is no SINR degradation for inter-system. For intra-system, outage percentage is large than 5%. Proposal 4 (CMCC): SINR degradation is less than 1dB. Inter-system interference is negligible.No outage percentage is observed for min, max and average SNR. Proposal 5 (Ericsson): SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions. | [SINR degradation is less than 1dB. Inter-system interference is negligible.] [Outage percentage is TBA (depending on topic 2 conclusion)] |
|  |  |  | c | Reader->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |  | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage | Proposal 1 (Huawei): Outage percentage for intra-system interference is 0.2% Proposal 2 (spreadtrum): For intra-system, outage percentage is 0% when use min and average SINR. Outage percentage is 49% when use max SINR. Proposal 3 (CMCC): No outage percentage is observed for min, max and average SNR. | SINR degradation is less than 1dB. Inter-system interference is negligible. [Outage percentage is TBA (depending on topic 2 conclusion)] |
| t1-2 | D1T1-A2-legacy UE indoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |  | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | b | NR DL->reader | TBA | Proposal 1 (Huawei): SINR degradation is less than 1dB. Inter-system interference is negligible. Outage percentage for intra-system interference is 0.7% Proposal 2 (Sony): The SINR loss for NR DL as aggressor and reader as victim is less than 1 dB. Proposal 2 (Spreadtrum): there is no SINR degradation for inter-system. For intra-system, outage percentage is larger than 5%. Proposal 3 (CMCC): SINR degradation is less than 1dB. Inter-system interference is negligible. No outage percentage is observed for min, max and average SNR. Proposal 4 (Ericsson): SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions. | [SINR degradation is less than 1dB. Inter-system interference is negligible.] [Outage percentage is TBA (depending on topic 2 conclusion)] |
|  |  |  | c | Reader->NR DL | FFS: NR DL throughput degradation for both average throughput and cell edge throughput is observed. Note: TBD on the throughput degradation range FFS on the UE blocking issue. Interference coordination/mitigation solution can be considered. | Proposal 1 (Huawei): NR DL throughput degradation for cell edge throughput is observed, and NR DL throughput degradation for average throughput is less than 5%. Interference coordination/mitigation solution can be considered. Proposal 2 (Sony): the SINR degradation on NR DL due to the AIoT system are significant when the NR UE percentage is 100% indoor. Proposal 3 (Spreadtrum): - When 10% indoor UE, SINR degradation is less than 1dB. Throughput degradation is 2.65% for cell edge and 2% for average throughput. - When 100% indoor UE, SINR degradation is larger than 1dB.Throughput degradation is 82.9%. Proposal 4 (CMCC): when 10% NR UE indoor, about 10% throughput degradation for average throughput is observed, about 70% throughput degradation for cell edge throughput is observed | [Under the assumption of 10% NR UE indoor, NR DL throughput degradation is TBA] [Under the assumption of 100% NR UE indoor, significant NR DL throughput degradation is observed.] Interference coordination/mitigation solution can be considered. |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. | Proposal 1 (Huawei): Outage percentage for intra-system interference is 0.2% Proposal 2 (Spreadtrum): For intra-system interference, outage percentage is 0% when SINR is min value. Outage percentage is 0% when SINR is average value. Outage percentage is 48% when SINR is max value. Proposal 3 (CMCC): No outage percentage is observed for min, max and average SINR. | SINR degradation is less than 1dB. Inter-system interference is negligible. [Outage percentage is TBA (depending on topic 2 conclusion)] |
| 2-1 | D1T1-B-legacy UE only outdoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |  | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |
|  |  |  | f | NR UL->reader | TBA | Proposal 1 (Huawei): SINR degradation is less than 1dB. Inter-system interference is negligible. Outage percentage for intra-system interference is 0.26% Proposal 2 (Spreadtrum): For intra-system interference, outage percentage is larger than 39%. Proposal 3 (CMCC): SINR degradation is less than 1dB. Inter-system interference is negligible.No outage percentage is observed for min, max and average SNR. Proposal 4 (Ericsson): SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions. | [SINR degradation is less than 1dB. Inter-system interference is negligible.] [Outage percentage is TBA (depending on topic 2 conclusion)] |
|  |  |  | c | Reader->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed. |  | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed. |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. | Proposal 1(Huawei): Outage percentage for intra-system interference is 0.2% Proposal 2 (Spreadtrum): Outage percentage is larger than 49% when max SNR. Proposal 3 (CMCC): No outage percentage is observed for min, max and average SNR. | SINR degradation is less than 1dB. Inter-system interference is negligible. [Outage percentage is TBA (depending on topic 2 conclusion)] |
| 2-2 | D1T1-B-legacy UE indoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |  | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |
|  |  |  | f | NR UL->reader | TBA | Proposal 1(Huawei): SINR degradation is more than 1dB. Inter-system interference is Observed. Outage percentage for inter-system interference is 14.1% Proposal 2 (Sony): The SINR degradation on the AIoT system due to NR UL are significant when the NR UE percentage is 100% indoor. Proposal 2 (Spreadtrum): SINR degradation is larger than 1dB for inter-system. When 10% indoor UE, outage percentage is large than 62%; when 100% indoor UE, outage percentage is larger than 95%. Proposal 3 (CMCC): SINR degradation is 17dB for cell edge and 4dB for cell average. Outage percentage is: - 17% @ 0.23dB - 13% @-4.3dB - 10% @ -7.8dB Proposal 4 (Ericsson): SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions. | [SINR degradation is larger than 1dB.] [Outage percentage is TBA (depending on topic 2 conclusion)] |
|  |  |  | c | Reader->NR DL | FFS: NR DL throughput degradation for both average throughput and cell edge throughput is observed. Note: TBD on the throughput degradation range level FFS on the UE blocking issue. Interference coordination/mitigation solution can be considered. | Proposal 1 (Huawei): NR DL throughput degradation for cell edge throughput is observed, and NR DL throughput degradation for average throughput is less than 5%. Interference coordination/mitigation solution can be considered. Proposal 2 (Spreadtrum): when 10% indoor UE, SINR and throughput degradation can be negligible. When 100% indoor UE, SINR degradation is larger than 1dB and throughput degradation is 80.78% Proposal 3 (CMCC): about 30% throughput degradation for average throughput is observed, about 70% throughput degradation for cell edge throughput is observed | [Under the assumption of 10% NR UE indoor, NR DL throughput degradation is TBA] [Under the assumption of 100% NR UE indoor, significant NR DL throughput degradation is observed.] Interference coordination/mitigation solution can be considered. |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. | Proposal 1(Huawei): Outage percentage for intra-system interference is 0.2% Proposal 2 (Spreadtrum): outage percentage is 48% when use max SINR Proposal 3 (CMCC): No outage percentage is observed for min, max and average SINR. | SINR degradation is less than 1dB. Inter-system interference is negligible. [Outage percentage is TBA (depending on topic 2 conclusion)] |
| 3 (O) | D1T1-A2-legacy UE indoor | R2D: UL CW2D and D2R: DL | a | Device->NR DL | TBA | Proposal 1 (Qualcomm): Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed Proposal 2 (Ericsson): SINR degradation is less than 1dB. Inter-system interference is negligible. | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | b | NR DL->reader | TBA | Proposal 1 (Qualcomm): SINR degradation @5-ile is 2dB, 50-ile is 0.8dB with 140dB CW cancellation capability Proposal 2 (Ericsson): SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions. | [SINR degradation is larger than 1dB.] [Outage percentage is TBA (depending on topic 2 conclusion)] |
|  |  |  | g | Reader->NR UL | TBA | Proposal 1 (Qualcomm): 8.73% mean T-put loss, cell edge T-put is N/A due to outage Proposal 2 (Ericsson): SINR degradation is less than 1dB. Inter-system interference is negligible. | [NR UL throughput degradation is TBA] |
|  |  |  | h | NR UL->device | TBA | Proposal 1 (Qualcomm): 1.78dB for 5-ile SINR degradation, 1.22dB for 50-ile SINR degradation Proposal 2 (Ericsson): SINR degradation is less than 1dB. Inter-system interference is negligible. | [SINR degradation is TBA.] [Outage percentage is TBA (depending on topic 2 conclusion)] |
| 4 (O) | D1T1-B-legacy UE indoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | TBA | Proposal 1 (Ericsson): SINR degradation is less than 1dB. Inter-system interference is negligible. | TBA |
|  |  |  | f | NR UL->reader | TBA | Proposal 1 (Ericsson): SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions.. | TBA |
|  |  |  | g | Reader->NR UL | TBA | Proposal 1 (Ericsson): SINR degradation is less than 1dB. Inter-system interference is negligible. | TBA |
|  |  |  | h | NR UL->device | TBA | Proposal 1 (Ericsson): SINR degradation is less than 1dB. Inter-system interference is negligible. | TBA |
| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |  | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | f | NR UL->reader | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. | Proposal 1 (vivo, R4-2418182): The maximum outage percentage is 0% Proposal 2 (CMCC): No outage percentage is observed for min, max and average SINR Proposal 3 (OPPO): For average SNR for D2R both outage percentage is only 0.1% for coherent and non-coherent. Proposal 4 (Ericsson): SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions. | [SINR degradation is less than 1dB. Inter-system interference is negligible.] [Outage percentage is TBA (depending on topic 2 conclusion)] |
|  |  |  | g | Reader->NR UL | TBA | Proposal 1 (CMCC): about 6% throughput degradation is observed for average throughput, about 13% NR BS throughput degradation is observed for cell edge throughput. Proposal 2 (Ericsson): SINR degradation is less than 1dB. Inter-system interference is negligible. | [NR UL throughput degradation is TBA] |
|  |  |  | h | NR UL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. | Proposal 1 (vivo): The maximum outage percentage is less than 5% Proposal 2 (CMCC): For device 1, 4% outage percentage @ 9.3dB SNR (max), no outage percentage for average and min SNR. No outage percentage for device 2a Proposal 3 (OPPO): For average SNR for R2D the outage percentage is 0. Proposal 4 (Ericsson): SINR degradation is less than 1dB. Inter-system interference is negligible. | SINR degradation is less than 1dB. Inter-system interference is negligible. [Outage percentage is TBA (depending on topic 2 conclusion)] |
| 5-2 | D2T2-A2-legacy UE indoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |  | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | f | NR UL->reader | TBA | Proposal 1 (vivo): when 10% indoor UE, SINR degradation is larger than 1dB. The maximum outage percentage is less than 5% Proposal 2 (CMCC): SINR degradation is 20dB for cell edge and 4.8dB for average Outage percentage is: • 12% @ -5dB SNR, • 11% @-6dB • 10% @ -7.6dB SNR Proposal 3 (Ericsson): SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions. | [SINR degradation is larger than 1dB.] [Outage percentage is TBA (depending on topic 2 conclusion)] |
|  |  |  | g | Reader->NR UL | FFS: NR UL throughput degradation for both average throughput and cell edge throughput is observed. Note: TBD on the throughput degradation range Interference coordination/mitigation solution can be considered. | Proposal 1 (CMCC): about 6% throughput degradation is observed for average throughput, about 12% NR BS throughput degradation is observed for cell edge throughput. Proposal 2 (Ericsson): SINR degradation is less than 1dB. Inter-system interference is negligible. | [NR UL throughput degradation is TBA] |
|  |  |  | h | NR UL->device | TBA | Proposal 1 (vivo): when 10% indoor UE, SINR degradation is larger than 1dB. The maximum outage percentage is less than 10% Proposal 2 (CMCC): SINR degradation is 3dB for cell edge and 5dB for average. For device 1: - 11% @ 9.3dB SNR (max) - No outage percentage for min and average SNR For device 2a: - 5% @ 5.1 dB SNR (max) No outage percentage for min and average SNR Proposal 3 (Ericsson): SINR degradation is less than 1dB. Inter-system interference is negligible. | [SINR degradation is TBA.] [Outage percentage is TBA (depending on topic 2 conclusion)] |
| 6-1(O) | D2T2-B-legacy UE only outdoor | R2D: UL CW2D and D2R: DL | a | Device->NR DL | TBA | Proposal 1 (Ericsson): SINR degradation is less than 1dB. Inter-system interference is negligible. | TBA |
|  |  |  | b | NR DL->reader | TBA | Proposal 1 (vivo): SINR degradation is less than 1dB. Inter-system interference is negligible. The maximum outage percentage is 0% Proposal 2 (Ericsson): SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions. | [SINR degradation is TBA.] [Outage percentage is TBA (depending on topic 2 conclusion)] |
|  |  |  | g | Reader->NR UL | TBA | Proposal 1 (Ericsson): SINR degradation is less than 1dB. Inter-system interference is negligible. | TBA |
|  |  |  | h | NR UL->device | TBA | Proposal 1 (vivo): SINR degradation is less than 1dB. Inter-system interference is negligible. The maximum outage percentage is less than 5% Proposal 2 (Ericsson): SINR degradation is less than 1dB. Inter-system interference is negligible. | [SINR degradation is TBA.] [Outage percentage is TBA (depending on topic 2 conclusion)] |
| 6-2(O) | D2T2-B-legacy UE indoor | R2D: UL CW2D and D2R: DL | a | Device->NR DL | TBA | Proposal 1 (Ericsson): SINR degradation is less than 1dB. Inter-system interference is negligible. | TBA |
|  |  |  | b | NR DL->reader | TBA | Proposal 1 (vivo): when 10% indoor UE, SINR degradation is larger than 1dB. The maximum outage percentage is less than 5% Proposal 2 (Ericsson): SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions. | [SINR degradation is larger than 1dB.] [Outage percentage is TBA (depending on topic 2 conclusion)] |
|  |  |  | g | Reader->NR UL | TBA | Proposal 1 (Ericsson): SINR degradation is less than 1dB. Inter-system interference is negligible. | TBA |
|  |  |  | h | NR UL->device | TBA | Proposal 1 (vivo): when 10% indoor UE, SINR degradation is larger than 1dB. The maximum outage percentage is less than 5% Proposal 1 (Ericsson): SINR degradation is less than 1dB. Inter-system interference is negligible. | [SINR degradation is TBA.] [Outage percentage is TBA (depending on topic 2 conclusion)] |

- Topic 3-3: General remarks on coexistence findings

Proposal 1 (Sony, R4-2418034): RAN4 can conclude that some mechanisms to support a higher percentage of indoor UE need to be studied.

Proposal 2 (Sony, R4-2418034): RAN4 can conclude that mechanisms to coordinate multiple readers to be active simultaneously need to be studied and specified.

Proposal 3 (Spreadtrum, R4-2418403): Guard RB(s) between NR and A-IoT needs to be considered for co-existence between A-IoT and NR when 10% legacy UE is indoor for D1T1-B and 100% legacy UE is indoor for D1T1-A2.

Proposal 4 (CMCC, R4-2418725): Capture the following contents as the general remarks on coexistence findings.

- For the deployment scenarios with legacy NR UE indoor (scenario 1-2, 2-2, 5-2, 6-2), which NR throughput degradation is observed, some mechanisms based on gNB implementation can be considered. In existing deployed network to avoid/mitigate the interference from ambient IOT to NR UE. For example, for RRC idle mode, NR UEs can camp on a frequency different from ambient IOT system based on frequency priority configured by network; for RRC connected mode, gNB can handover NR UEs to a different cell (e.g. indoor gNB) based on existing measurement report triggering and event.

Proposal 5 (Qualcomm, R4-2418852): RAN4 consider decreasing the Tx power by power control for reader in D2T2 to solve the coexistence issue for reader interfering NR UL.

Proposal 6 (Samsung, R4-2419649): To clarify in the TR that the different types of frequency shifter were not discussed nor concluded from RAN4 perspective.

Proposal 7 (Huawei, R4-2418802): Guard RBs can be used to improve coexistence performance if necessary.

Proposal 8 (Ericsson, R4-2419476): The leakage from NR base station should be lowered by adding more guard RB, how to model this FFS.

Recommended WF:

Discuss whether and how to capture the above proposals in TR.