3GPP TSG RAN WG4 Meeting#113                                                               R4-2418286

Orlando, US, 18th - 22nd November 2024

Agenda item:			7.24.3

Source:	China Telecom

Title:	Topic summary for [113][228] NR_Mob_Ph4_Part2

Document for:	Information

Introduction

This summary includes the proposals from companies on RRM requirements for conditional LTM. The relevant objectives in the latest WID and RAN2’s conclusions are duplicated here for information:

Objectives on conditional Intra-CU LTM in WID - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

RAN2’s conclusions in last meeting 1. Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions. 2. Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. 3. Source cell and each candidate cell provides its own execution condition for conditional LTM. 4. It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used. 5. RACH-less Conditional intra-CU LTM is supported. 6. RACH based conditional intra-CU LTM is supported. 7. UE based TA measurement mechanism is supported for conditional intra-CU LTM. 8. PDCCH ordered early TA acquisition is supported for conditional LTM. 9. Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. 10. For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach. 11. The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

Recommended Topics to be treated online (in order of decreasing priority):

Issue 1-1-2: Measurement requirements on conditional LTM

Issue 1-2-1: Early UL Synchronization

Issue 1-2-2: Early TCI state activation

Issue 1-3-1: General aspects on Conditional LTM delay

Issue 1-3-3: The ending point of conditional LTM delay

Issue 1-4-2: General aspects on Subsequent Conditional LTM delay

Issue 1-3-2: The starting point of conditional LTM delay

Issue 1-4-1: General aspects on Subsequent Conditional LTM

Issue 1-4-3: The starting point of Subsequent conditional LTM delay

Issue 1-4-4: The ending point of Subsequent conditional LTM delay

Moderator’s recommendation is also provided under issue.

Topic #1: RRM discussion

Companies’ contributions summary

| TDoc | Source | Proposals |
| --- | --- | --- |
| R4-2418861 | China Telecom | Proposal 1: RAN4 needs to define requirements on cell switch delay, UE based TA measurement and PDCCH ordered early TA acquisition and early candidate TCI State activation/deactivation for conditional intra-CU LTM. Proposal 2: Discuss L1 measurement requirements on conditional LTM first and wait for the conclusion on the case where L3 measurement is used. Proposal 3: The starting point of initial conditional LTM and subsequent conditional LTM is different. And the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. Proposal 4: The requirements for conditional LTM can be defined in the clauses of Rel-18 LTM. Proposal 5: Conditional intra-CU LTM delay is the delay from the end of the last TTI containing the RRC command for conditional intra-CU LTM until the time the UE transmits the first UL message on the target cell. Proposal 6: DIntra-CLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process. Where: - TRRC is the RRC procedure delay. - TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional intra-CU LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM. - Tmeasure is defined from the end of TTCI-state activate until UE executes LTM to a target cell and interruption time starts. - TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined. - Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell. - TEarly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further. Further, the components of Tinterrupt could be defined as Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU - TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. - Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. - TRS-proc is the time for SSB processing. - TLTM-IU is the interruption uncertainty during LTM cell switch. Proposal 7：Revisit conditional intra-CU LTM delay when other WGs achieve further conclusions. Proposal 8: - Discuss the requirements on early candidate TCI State activation/deactivation and early UL-sync. - R18 LTM early TCI state activate and PDCCH ordered RACH could be baseline. Proposal 9: Cover both RACH-based and RACH-less conditional intra-CU LTM. |
| R4-2417944 | Xiaomi | Proposal 1: CHO requirements framework can be used as baseline for Conditional LTM. Proposal 2: The starting point of conditional LTM delay is from the end of the last TTI containing the RRC command implying conditional handover. Proposal 3: RAN4 to define the conditional LTM delay as DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution, where - TRRC is the RRC procedure delay, - TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM cell switch command until a condition exists at the measurement reference point which will trigger the conditional LTM cell switch, - Tmeasure can wait for more RAN2 progress, - Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmits the first UL message on the target cell. Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU , where the TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU can refer to clause 6.3.1.3, - TCLTM_execution is the UE execution preparation time for conditional LTM cell switch, including both RACH-based and RACH-less cases, and starts after UE realizes the condition of CLTM is met and identity of the target cell is determined. |
| R4-2419088 | vivo | Proposal 1: RAN4 to use CHO delay requirements framework as a baseline when defining conditional LTM delay requirements, the timeline of CLTM cell delay requirement is defined as DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM-interrupt + TCHO_execution Proposal 2: The starting point of conditional LTM delay is the time the UE receives a RRC message implying conditional LTM Proposal 3: The ending point of conditional LTM delay, respectively - for RACH-based, it is the time the UE acquires the first available PRACH occasion transmission on the target cell - for RACH-less based, it is the time the UE transmits the first uplink transmission on the target cell Proposal 4: RAN4 to wait for RAN2 conclusion on whether to support L3 based conditional LTM in FR1 Proposal 5: The definitions of TEvent_DU and Tmeasure for CLTM need further RAN2 inputs Proposal 6: For the interruption part of CLTM (TCLTM-interrupt), it highly depends on further discussion on procedure design in RAN2 group Observation 1: Conditional LTM implicitly supports ‘subsequent’ evaluation and execution on candidate cells without reconfiguration and re-initialization on the CLTM preparation from the network Proposal 7: For subsequent conditional LTM, the starting point of the delay (for both RACH-based and RACH-less based) is when UE transmits RRCReconfigurationcomplete message for the previous PCell. |
| R4-2417719 | CATT | Observation 1: RAN2 is also discussing L3 measurement based conditional LTM. Observation 2: It may be unreasonable for some UEs that support Rel-18 LTM to not support conditional LTM. Proposal 1: RAN4 to discuss this issue in parallel with RAN2. - It is better to align with R18 LTM for R19 conditional LTM to support L3 measurement based conditional LTM. - RAN4 needs to focus on the issues that need to be addressed if L3 measurement based conditional LTM is supported. Observation 3: RAN2 agreed to support both RACH-less and RACH based conditional intra-CU LTM in the last meeting. Proposal 2: Both RACH-less and RACH based conditional intra-CU LTM should be considered when defining RRM requirements. Proposal 3: For the ending point of conditional LTM delay, the legacy definition in R18 LTM can be reused. - For RACH-based conditional LTM, the ending point is the time when the UE transmits PRACH on the target cell. - For RACH-less conditional LTM, the ending point is the time when UE performs the first UL transmission on the indicated beam of the target cell. Proposal 4: The legacy requirments for L3 CHO can be a baseline, that is, DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution. Proposal 5: RAN4 need to wait for RAN1/RAN2 more progress and further study the LTM related features based on the above requirments. - For the interruption time in Rel-19 conditional LTM Cell Switch delay, the interruption time defined in Rel-18 LTM Cell Switch delay can be a baseline. Proposal 6: At least TRRC will not be considered in subsequent conditional LTM delay. Observation 4: RAN2 agreed to support UE based TA measurement mechanism for conditional intra-CU LTM in the last meeting. Observation 5: RAN2 agreed to support early TA acquisition for conditional intra-CU LTM in the last meeting. Proposal 7: RAN4 need to define RRM requirements for UE based TA measurement for conditional intra-CU LTM. Proposal 8: RAN4 to define PDCCH ordered RACH related requirements, and whether updates are needed compared to the Rel-18 should be discussed. - RAN4 should wait for more progress from other WGs. Observation 6: RAN2 agreed to support candidate TCI state activation/deactivation for conditional intra-CU LTM in the last meeting. Proposal 9: RAN4 to define early TCI state activation related requirements, and whether updates are needed compared to the Rel-18 should be discussed. - RAN4 should wait for more progress from other WGs. |
| R4-2418445 | CMCC | Proposal 1: it is proposed to discuss how to define cell switch delay requirements for conditional LTM and subsequent conditional LTM - Alt1: define seperate cell switch delay requirements for conditional LTM and subsequent conditional LTM - Alt2: define unified cell switch delay requirements to cover both conditional LTM and subsequent conditional LTM Proposal 2: with the assumption that it is agreed to define seperate cell switch delay requirements for conditional LTM and subsequent conditional LTM: - For conditional LTM, the starting point is the time when UE receives RRC command triggering conditional LTM. The ending point is the time the UE transmits the first UL message on the target cell. - For subsequent conditional LTM, the starting point is the time when UE transmits RRCReconfigurationcomplete message for the previous cell switch. The ending point is the time the UE transmits the first UL message on the target cell. Proposal 3: with the assumption that it is agreed to define seperate cell switch delay requirements for conditional LTM and subsequent conditional LTM: - For conditional LTM, cell switch delay is proposed as DLTM_Conditional = TRRC_delay + TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. - TRRC is the RRC procedure delay. - TEvent_DU is the delay uncertainty which is the time from when UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM. - Tmeasure is measurements time. - TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms. - TLTM-interrupt is the interruption time as specified in Rel-18. - For subsequent conditional LTM, cell switch delay is proposed as DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. - TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM. - Tmeasure is measurements time. - TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms. - TLTM-interrupt is the interruption time as specified in Rel-18. Proposal 4: with the assumption that it is agreed to define unified cell switch delay requirements to cover both conditional LTM and subsequent conditional LTM: - the starting point to cover both conditional LTM and subsequent conditional LTM is proposed as the time when UE transmits an RRCReconfigurationComplete message. - for conditional LTM, the transmitted RRCReconfigurationComplete message is for confirmation of the LTM configuration - for subsequent conditional LTM, the transmitted RRCReconfigurationComplete message is for the previous cell switch. - The ending point is the time the UE transmits the first UL message on the target cell. Proposal 5: with the assumption that it is agreed to define unified cell switch delay requirements to cover both conditional LTM and subsequent conditional LTM: - The cell switch delay to cover both conditionl LTM and subsequent LTM is proposed as DLTM_Conditional = DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. - TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationComplete message confirming the LTM configuration or the RRCReconfigurationComplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM - Tmeasure is measurements time. - TLTM-interrupt is the interruption time as specified in Rel-18. - TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined Proposal 6: since RAN2 is agreed that UE based TA measurement mechanism is supported for conditional intra-CU LTM, it is proposed to define related RAN4 requirements, e.g.timing requirements, for UE based TA adjustment. Proposal 7: it is proposed to cover both RACH-less conditional intra-CU LTM and RACH based conditional intra-CU LTM |
| R4-2418246 | Qualcomm Incorporated | Proposal 1: RAN4 should wait for further progress in other working groups and postpone detailed discussions on whether and how to define RRM requirements for subsequent conditional LTM and each component of C-LTM delay. Proposal 2: RAN4 to define RRM requirements for DL TODA-based UE autonomous TA estimation. FFS on the details and side conditions. |
| R4-2418588 | Apple | Proposal 1: in the agreed applicable scenario of “PCell switch to an LTM candidate cell that is a serving SCell in MCG”, clarify that if CLTM is triggered by L1 measurement (e.g. event LTM3 and LTM5) the scope is limited to active SCell. Proposal 2: no new measurement requirements are needed under this agenda. Proposal 3: since L1-RSRP measurement and LTM cell switch was decoupled in R18, L3 measurement shall also be considered in event configuration for conditional LTM. Proposal 4: assuming subsequent CLTM is to be supported by RAN2, RAN4 requirements for subsequent CLTM can be defined using CLTM requirements as baseline with the following updates: - The starting point of subsequent CLTM is the time UE transmits RRCReconfigurationcomplete message for the previous CLTM. - Note: RAN2 agreed that the LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM. - TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationcomplete message for the previous CLTM until a condition exists at the measurement reference point which will trigger the subsequent CLTM. Proposal 5: new requirements for CLTM can be defined in a new sub-section 6.3.1A. New requirements for subsequent CLTM can be defined in a new sub-section 6.3.1B. Proposal 6: CHO requirements framework can be used as baseline when discussing conditional LTM cell switch delay requirements. Proposal 7: The starting point is the end of the last TTI containing the RRC command for conditional LTM. Proposal 8: The ending point could be same as that defined in existing LTM cell switch delay requirements, i.e. time the UE transmits the first UL message on the target cell, which already cover both RACH based and RACH-less. Proposal 9: conditional LTM cell switch delay is defined as: DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution Where: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331. TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM cell switch command until a condition exists at the measurement reference point which will trigger the conditional LTM cell switch. Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes a LTM cell switch to a target cell and interruption time starts: - if SSB based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of SSB based L1-RSRP measurement as introduced in R18. - if CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of CSI-RS based L1-RSRP measurement which will be introduced in R19. - if L3 measurement is used in the event, Tmeasure is same as one measurement period of legacy L3 measurement. TCLTM_execution is the UE execution preparation time for conditional LTM cell switch, and starts after UE realizes the condition of CLTM is met and identity of the target cell is determined. TCLTM_execution can be up to [10ms]. TLTM-interrupt is the interruption time stated in clause 6.3.1.3, same as that defined in R18 LTM cell switch. Proposal 10: RAN4 requirements for CLTM shall cover both RACH-less and RACH-based cell switch. Proposal 11: only known cell needs to be covered in CLTM RRM requirements. |
| R4-2418113 | Samsung | Proposal 1: For ending point of conditional LTM delay, reuse the same as that in Rel-18 LTM cell switch delay: UE transmits the first UL message on the target cell. Proposal 2: Both RACH-based and RACH-less conditional LTM are supported. Observation 1: For start point of conditional LTM delay, there is no explicit MAC-CE command like Rel-18 NW based LTM. There is a RRC message includes the LTM candidate configurations and execution conditions. Proposal 4: RAN4 to discuss on start point of conditional LTM delay: - Same as L3 CHO. RRC message includes the LTM candidate configurations and execution conditions. - Similar as event-triggered L1 reporting. The CondEvent is met over the air. Wait for further RAN1/RAN2 progress. Proposal 5: The delay shall include the measurements time (Tmeasure) and the conditional execution preparation time (TCLTM_execution) and interruption time (Tinterrupt) at least. Others are waiting for further progress in other groups. |
| R4-2417892 | MediaTek Inc. | Proposal 1: Support performing T/F fine tracking (TΔ) if needed at first and then Tprocessing, to reduce the interruption time during conditional LTM. |
| R4-2418672 | Huawei, HiSilicon | Proposal 1: In FR1 it is feasible to support L3 measurement based C-LTM. The possible impacts are, - the L3 based condition evaluation period may be slower than L1 based - RAN2 may need some clarification on L3 beam specific LTM events/conditions. Proposal 2: The starting point of conditional LTM delay is the instance when the UE receives a RRC message implying conditional LTM. Proposal 3: For RACH based and RACH-less conditional LTM, the ending point of conditional LTM delay is the time instance the UE transmits the first UL message on the target cell. Proposal 4: TEvent_DU can wait for more progress in RAN2 as DL sync and UL sync are supported for conditional LTM. Proposal 5: If certain process can guarantee that UE first completes DL sync and UL sync and then performs execution condition evaluation, the delay for DL sync and UL sync can be contained in TEvent_DU. Proposal 6: In C-LTM delay, Tmeas is the L1 measurement period - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM. - If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to introduced in R19. - FFS on L3 measurement period depending on the conclusion of whether L3 based measurement is considered in R19 C-LTM Proposal 7: TCHO_execution can be reused in conditional LTM. Proposal 8: The term Tinterrupt may induce ambiguity when define conditional LTM delay, as interruptions may also happen in early PDCCH RACH procedure. Proposal 9: After TCHO_execution, the component TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU which are defined in R18 LTM cell switch delay can be reused. Proposal 10: C-LTM delay can be depicted as DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU Proposal 11: The starting point of the subsequent conditional LTM cell switch delay is the time UE transmits RRC Reconfigurationcomplete message for the previous PCell. Proposal 12: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay, as most components in conditional LTM delay can be reused for subsequent C-LTM. |
| R4-2417688 | Nokia | Proposal 1: Wait for more RAN2 agreements regarding subsequent CLTM. Proposal 2: Wait for more RAN2 agreements before agreeing on the starting point of the CLTM cell switch delay. Proposal 3: Reuse the ending point of Rel-18 LTM cell switch delay for RACH-less and RACH-based CLTM. Proposal 4: TEvent_DU and Tmeasure like components are likely needed in the CLTM cell switch delay, but the exact definition is pending more RAN2 agreements. Proposal 5: Wait for more RAN2 agreements before agreeing on the exact components of the CLTM cell switch delay. Proposal 6: ASN.1 decoding and validity check is not part of Tinterrupt but may be part of DCLTM if not done before the starting point of the cell switch delay. Proposal 7: Fine DL synchronization (first SSB) is not part of Tinterrupt but may be part of DCLTM if not done before the starting point of the cell switch delay. Proposal 8: Wait for more RAN2 agreements before agreeing on early TCI state activation requirements. Proposal 9: RAN4 to wait for more RAN1/2 agreements regarding PDCCH ordered RACH procedure for CLTM. Proposal 10: RAN4 to discuss whether to define requirements for UE based TA estimation in LTM and CLTM or not. |
| R4-2419447 | Ericsson | · Proposal 1: RAN4 specify subsequent C-LTM requirement. · Proposal 2: Detail of starting point of the subsequent C-LTM can wait for RAN2 further progress. · Proposal 3: RAN4 to specify requirement for both RACH based and RACH-less Conditional LTM. · Proposal 4: Rel-18 UE capability for early ASN.1 decoding or fast RRC processing shall be supported for Rel-19 Conditional LTM. · Proposal 5: Rel-19 conditional LTM 1st time cell switch delay starting point shall be upon UE receiving the RRC_Reconfiguration message. · Proposal 6: The starting point of the subsequent C-LTM shall wait for RAN2 further agreement. · Proposal 7: The ending point of the Rel-19 C-LTM both for the 1st time and the subsequent C-LTM shall be the time the UE transmits the first UL message on the target cell. · Proposal 8: RAN 4 requirement assumption for the early UL sync shall start with supporting both UE based TA measurement and PDCCH ordered early TA acquisition. · Proposal 9: RAN 4 requirement assumption for the early TCI activation shall based on the Rel-18 TCI activation framework. · Proposal 10: RAN4 to wait for RAN2 progress for defining cell switch delay requirements for conditional LTM. |
| R4-2418506 | ZTE Corporation, Sanechips | Proposal 1: for conditional LTM delay, CHO requirements framework can be used as baseline. Proposal 2: The starting point of conditional LTM delay is the end of the last TTI containing the RRC command for conditional LTM. Observation 1: Both RACH-less Conditional intra-CU LTM and RACH based conditional intra-CU LTM are supported in RAN2. Proposal 3: The ending point of Rel-18 LTM can be reused for conditional LTM delay. - For RACH-based conditiona LTM, the ending point is the first available PRACH occasion in the new cell. - For RACH-less conditiona LTM, the ending point is the first uplink transmission on the target cell. Proposal 4: use DCLTM = TRRC + TEvent_DU +Tmeasure + TCLTM_execution + TLTM-interrupt. as baseline and consider whether/how to capture the time for active the TCI state of candidate cell(s). TRRC is the RRC procedure delay defined in clause 12 in TS 38.331. TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes LTM cell switch to a target cell and interruption time starts. TCLTM_execution is the UE execution preparation time for conditional LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined. TLTM-interrupt is the interruption time as specified in Rel-18 LTM cell switch. Proposal 5: Early sync is supported for conditional LTM and the related requirements defined for LTM can be used as baseline, e.g., PDCCH ordered RACH delay requirements and early TCI state activation delay requirements. |

Open issues summary

General aspects on Conditional LTM

Issue 1-1-1: The applicable scenarios for conditional LTM

RAN4#112bis < Agreement > - Rel-19 CLTM requirements are applicable for NR SA scenarios defined in Rel-18 LTM - PCell switch to a neighboring LTM candidate cell - FR1 cell to FR1 cell - FR1 cell to FR2 cell - FR2 cell to FR2 cell - FR2 cell to FR1 cell - PCell switch to an LTM candidate cell that is a serving SCell in MCG - FR1 cell to FR1 cell - FR2 cell to FR2 cell - Further discuss the DC case if it will be included in the WID based on further RAN plenary discussion.

- Candidate solutions:

- Proposal 1: In the agreed applicable scenario of “PCell switch to an LTM candidate cell that is a serving SCell in MCG”, clarify that if CLTM is triggered by L1 measurement (e.g. event LTM3 and LTM5) the scope is limited to active SCell. (Apple)

- Recommended WF

  - CLTM is triggered by L1 measurement (e.g. event LTM3 and LTM5) the scope is limited to active SCell.

Issue 1-1-2: Measurement requirements on conditional LTM

RAN4#112bis < Agreement > - On whether L3 measurement based conditional LTM in FR1 can be supported - RAN4 can further discuss this issue in FR1 and make recommendation to RAN2 if RAN4 consensus can be reached. - As generic procedure for the objective led by RAN2, if any conclusion will be made in RAN2, RAN4 will follow the RAN2 conclusion unless any technical issue is identified. RAN2#127bis It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- Candidate solutions:

  - Proposal 1: Discuss L1 measurement requirements on conditional LTM first. (CTC)

  - Proposal 2: Wait for the conclusion on the case where L3 measurement is used. (CTC, vivo)

  - Proposal 3: Support L3 measurement based conditional LTM. (CATT, Apple, Huawei for FR1)

  - Proposal 3a: RAN4 needs to focus on the issues that need to be addressed if L3 measurement based conditional LTM is supported. (CATT)

  - Proposal 3b: (Huawei)

    - the L3 based condition evaluation period may be slower than L1 based.

    - RAN2 may need some clarification on L3 beam specific LTM events/conditions.

  - Proposal 4: No new measurement requirements are needed. (Apple)

- Recommended WF

  - Proposals 1, 2, 3 are related to whether support L3 measurement based conditional LTM.

  - Proposal 4 is related to new measurement requirements.

  - Discussion is needed.

Issue 1-1-3: Whether cover both RACH-based and RACH-less conditional LTM

- Candidate solutions:

- Proposal 1: Cover both RACH-based and RACH-less conditional intra-CU LTM. (CTC, Xiaomi, CATT, CMCC, Apple, Samsung, E///)

- Recommended WF

  - Cover both RACH-based and RACH-less conditional intra-CU LTM.

Issue 1-1-4: The specification structure of the requirements for conditional LTM

RAN4#112bis < Way Forward > FFS the following options: - Option 1: For specification structure, the requirements for conditional LTM can be defined in the clauses of Rel-18 LTM. - Other option is not precluded.

- Candidate solutions:

  - Proposal 1: The requirements for conditional LTM can be defined in the clauses of Rel-18 LTM. (CTC)

  - Proposal 1a: New requirements for CLTM can be defined in a new sub-section 6.3.1A. New requirements for subsequent CLTM can be defined in a new sub-section 6.3.1B. (Apple)

- Recommended WF

  - The requirements for conditional LTM can be defined in the clauses of Rel-18 LTM, FFS the exact sub-section.

Early synchronization and TCI state activation

Issue 1-2-1: Early UL Synchronization

RAN2#127bis - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM.

- Candidate solutions:

- Proposal 1: Define PDCCH ordered RACH related requirements for conditional intra-CU LTM. (CTC, CATT, E///, ZTE, Nokia)

  - Proposal 1a: R18 LTM PDCCH ordered RACH could be baseline. (CTC, CATT, ZTE)

  - Proposal 1b: Wait for more progress from other WGs. (CATT, Nokia)

- Proposal 2: Define RRM requirements for UE based TA measurement. (CTC, CATT, CMCC)

  - Nokia: Discuss whether to define requirements for UE based TA estimation in LTM and CLTM or not.

- Proposal 3: Define RRM requirements for DL TODA-based UE autonomous TA estimation. FFS on the details and side conditions. (QC)

- Recommended WF

  - Define PDCCH ordered RACH related requirements for conditional intra-CU LTM.

  - R18 LTM PDCCH ordered RACH could be baseline.

  - Define RRM requirements for UE based TA measurement.

Issue 1-2-2: Early TCI state activation

RAN2#127bis - Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- Candidate solutions:

- Proposal 1: Define the requirements on early candidate TCI State activation/deactivation. (CTC, CATT, E///, ZTE, Nokia)

  - Proposal 1a: R18 LTM early TCI state activate could be baseline. (CTC, CATT, E///, ZTE)

  - Proposal 1b: Wait for more progress from other WGs. (CATT, Nokia)

- Recommended WF

  - Define the requirements on early candidate TCI State activation/deactivation.

  - R18 LTM early TCI state activate could be baseline.

Conditional LTM delay

Issue 1-3-1: General aspects on Conditional LTM delay

- Candidate solutions:

- Proposal 1: CHO requirements framework can be used as baseline for Conditional LTM. (Xiaomi, CTC, vivo, CATT, Apple, ZTE)

- Proposal 2: Discuss how to define cell switch delay requirements for conditional LTM and subsequent conditional LTM. (CMCC)

- Alt1: define separate cell switch delay requirements for conditional LTM and subsequent conditional LTM

- Alt2: define unified cell switch delay requirements to cover both conditional LTM and subsequent conditional LTM

- Proposal 3: Wait for further progress in other working groups and postpone detailed discussions on whether and how to define RRM requirements for each component of C-LTM delay. (QC)

- Proposal 4: Support performing T/F fine tracking (TΔ) if needed at first and then Tprocessing, to reduce the interruption time during conditional LTM. (MTK)

- Proposal 5: Rel-18 UE capability for early ASN.1 decoding or fast RRC processing shall be supported for Rel-19 Conditional LTM. (E///)

- Recommended WF

  - CHO requirements framework can be used as baseline for Conditional LTM.

  - Proposals 2, 4, 5 need discussion separately.

Issue 1-3-2: The starting point of conditional LTM delay

- Candidate solutions:

- Proposal 1: The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM. (CTC, Xiaomi, Apple, Samsung, ZTE)

- Proposal 2: The starting point of conditional LTM delay is the time the UE receives an RRC message implying conditional LTM. (vivo, CMCC, Huawei)

- Proposal 3: The starting point is the time when UE transmits an RRCReconfigurationComplete message that is for confirmation of the LTM configuration. (CMCC, E///)

- Proposal 4: Similar as event-triggered L1 reporting. The CondEvent is met over the air. (Samsung)

- Proposal 5: Wait for more RAN2 agreements. (Nokia)

- Recommended WF

  - The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM.

Issue 1-3-3: The ending point of conditional LTM delay

- Candidate solutions:

- Proposal 1: The ending point of conditional LTM is the time the UE transmits the first UL message on the target cell. (CTC, CMCC, Apple, Samsung, Huawei, Nokia, E///)

- Proposal 1a: (vivo, CATT, ZTE)

  - for RACH-based, it is the time the UE acquires the first available PRACH occasion transmission on the target cell

  - for RACH-less based, it is the time the UE transmits the first uplink transmission on the target cell

- Recommended WF

  - The ending point of conditional LTM is the time the UE transmits the first UL message on the target cell.

Issue 1-3-4: The components of conditional LTM delay

- Candidate solutions:

- Proposal 1: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process. (CTC)

    - Proposal 1a: TEarly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further. (CTC)

- Proposal 2: DCLTM = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution. (Xiaomi, vivo, CATT, CMCC, Apple, ZTE)

- Proposal 3: DCLTM = TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution. (CMCC)

- Proposal 4: DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. (Huawei)

    - Proposal 4a: TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU which are defined in R18 LTM cell switch delay can be reused. (Huawei)

- Proposal 5: The delay shall include the measurements time (Tmeasure) and the conditional execution preparation time (TCLTM_execution) and interruption time (Tinterrupt) at least. Others are waiting for further progress in other groups. (Samsung)

- Proposal 6: TEvent_DU and Tmeasure like components are likely needed, exact components are needed to wait for more RAN2 agreements. (Nokia)

- Proposal 7: Wait for RAN2 progress. (E///)

- Recommended WF

  - Discussion is needed.

Issue 1-3-5: TRRC

- Candidate solutions:

- Proposal 1: TRRC is the RRC procedure delay. (CTC, Xiaomi, CMCC, Apple, ZTE)

- Recommended WF

  - Discussion is needed.

Issue 1-3-6: TEvent_DU

- Candidate solutions:

- Proposal 1: TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM. (CTC, Xiaomi, CMCC, Apple, ZTE)

- Proposal 2: Wait for more RAN2 progress. (vivo, Huawei)

    - Proposal 2a: If certain process can guarantee that UE first completes DL sync and UL sync and then performs execution condition evaluation, the delay for DL sync and UL sync can be contained in TEvent_DU.

- Proposal 3: Wait for more RAN2 agreements. (Nokia)

- Recommended WF

  - Discussion is needed.

Issue 1-3-7: Tmeasure

- Candidate solutions:

- Proposal 1: Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts. (CTC, CMCC, Apple, ZTE)

    - Proposal 1a: (Apple, Huawei)

  - If SSB based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of SSB based L1-RSRP measurement as introduced in R18.

  - If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of CSI-RS based L1-RSRP measurement which will be introduced in R19.

    - Proposal 1b: If L3 measurement is used in the event, Tmeasure is same as one measurement period of legacy L3 measurement. (Apple)

    - Proposal 1c: FFS on L3 measurement period depending on the conclusion of whether L3 based measurement is considered in R19 C-LTM. (Huawei)

- Proposal 3: Wait for more RAN2 progress. (Xiaomi, vivo, Nokia)

- Recommended WF

  - Discussion is needed.

Issue 1-3-8: TCLTM_execution / TLTM_execution

- Candidate solutions:

- Proposal 1: TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined. (CTC, Xiaomi, CMCC, Apple, Huawei, ZTE)

- Recommended WF

  - Discussion is needed.

Issue 1-3-9: Tinterrupt

- Candidate solutions:

- Proposal 1: Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell. (CTC, Xiaomi, CMCC, Apple, ZTE, CATT)

- Proposal 2: It highly depends on further discussion on procedure design in RAN2 group. (vivo)

- Proposal 3: The term Tinterrupt  may induce ambiguity when define conditional LTM delay, as interruptions may also happen in early PDCCH RACH procedure. (Huawei)

- Proposal 4: ASN.1 decoding and validity check and fine DL synchronization (first SSB) is not part of Tinterrupt but may be part of DCLTM if not done before the starting point of the cell switch delay. (Nokia)

- Recommended WF

  - Discussion is needed.

Issue 1-3-10: The components of Tinterrupt

- Candidate solutions:

  - Proposal 1: Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. (CTC, Xiaomi)

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

- Recommended WF

  - Discussion is needed.

Subsequent Conditional LTM

Issue 1-4-1: General aspects on Subsequent Conditional LTM

- Candidate solutions:

- Proposal 1: Conditional LTM implicitly supports ‘subsequent’ evaluation and execution on candidate cells without reconfiguration and re-initialization on the CLTM preparation from the network. (vivo)

- Recommended WF

  - Discussion is needed.

Issue 1-4-2: General aspects on Subsequent Conditional LTM delay

RAN4#112bis < Agreement> - RAN4 to define cell switch delay requirements for conditional LTM - RAN4 to define subsequent conditional LTM requirement unless there is any technical issue identified. - Wait for other WGs input to further discuss detailed RRM requirements.

- Candidate solutions:

- Proposal 1: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. (CTC, Apple, Huawei)

    - Proposal 1a: TRRC will not be considered in subsequent conditional LTM delay. (CATT, CTC)

    - Proposal 1b: TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationcomplete message for the previous CLTM until a condition exists at the measurement reference point which will trigger the subsequent CLTM. (Apple)

- Proposal 2: Wait for further progress in other working groups and postpone detailed discussions on whether and how to define RRM requirements for subsequent conditional LTM. (QC, Nokia)

- Recommended WF

  - Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

Issue 1-4-3: The starting point of Subsequent conditional LTM delay

- Candidate solutions:

- Proposal 1: The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM. (CTC, vivo, CMCC, Apple, Huawei)

- Proposal 2: Wait for RAN2 further progress. (E///)

- Recommended WF

  - The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM.

Issue 1-4-4: The ending point of Subsequent conditional LTM delay

- Candidate solutions

- Proposal 1: The ending point is the time the UE transmits the first UL message on the target cell. (CTC, CMCC, E///)

- Recommended WF

  - The ending point is the time the UE transmits the first UL message on the target cell.

Issue 1-4-5: The components of subsequent conditional LTM delay

- Candidate solutions:

- Proposal 1: DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. (CMCC)

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM.

- Tmeasure is measurements time.

- TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

- TLTM-interrupt is the interruption time as specified in Rel-18.

- Recommended WF

  - Discussion is needed.