3GPP TSG-RAN WG4 Meeting # 113											R4-2418287

Orlando, US, 18th – 22nd November, 2024

Agenda item:			6.25.3

Source:	Moderator (Nokia)

Title:	Topic summary for [113][229] NR_XR_Ph3

Document for:	Information

# Introduction

This summary includes the proposals and issues for discussion related to Rel-19 enhancements for extended reality for NR Phase 3, under thread [113][229] NR_XR_Ph3.

This summary is based on the contributions submitted to the agenda items:

- 7.25	XR for NR Phase 3	[NR_XR_Ph3]

- 7.25.1	General aspects	[NR_XR_Ph3-Core]

- 7.25.2	RRM core requirements	[NR_XR_Ph3-Core]

- 7.25.3	Moderator summary and conclusions	[NR_XR_Ph3-Core]

The issues for discussion are organized in topics which are as follows:

- Topic 1 Scenarios for measurement skipping

- Topic 2 Measurements with gaps

- Topic 3 XR skipping requirements impact

- Topic 4 Time offset for measurement skipping processing

- Topic 5 Need/feasibility of UE assistance information

- Topic 6 Additional signalling aspects

- Topic 7 Reply LS to RAN1

- Topic 8 UE capabilities

- Topic 9 RAN4 TU allocation for XR

The following issues are proposed to be prioritized for discussion

- Issue 1-1: Deployment scenarios

- Issue 1-2: Types of measurements to be considered

- Issue 1-3: L3 Measurements without gaps with scheduling restrictions

- Issue 1-4: L1 measurements with scheduling restrictions and procedures

- Issue 2-1: General approach for defining requirements for measurements with gaps

- Issue 3-1: How to capture impact of measurement skipping

- Issue 4-1: Time offset requirement for measurement gap skipping

- Issue 4-2: Principles for defining time offset

- Issue 5-1: General on UAI

- Issue 6-1: Semi-static schemes

- Issue 6-2: Skipping indication on frequency domain

- Issue 7-1: Response on RAN1 working assumption and time offset

The latest work item description can be found at:

- RP-241771, Revised WID: XR (eXtended Reality) for NR Phase 3, Nokia

# Topic #1: RRM requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2419326 | Nokia | General aspects for XR enhancements Reply LS to RAN1 Observation 1: RAN4 already defined requirements for features where dynamic adaptation of measurements were necessary, e.g. NR-U and Pre-configured gaps. Proposal 1: RAN4 to reply to RAN1 that Alt 1-1 is feasible from RAN4 perspective and RAN4 is working on the related requirements on timeline. It is noted we have included a proposal for the draft LS in the Annex A of this paper. TU allocation for RAN4 RRM work Observation 2: Considering on one hand the many aspects for RRM requirements to be discussed in RAN4 related to XR enhancements and the current duration of online discussion estimated to be 1.25 TU, thus well exceeding 0.5 TU, and the tight time frame for Rel-19, the current TU allocation of 0.5 for RRM is considered clearly insufficient. Proposal 2: RAN4 to request a higher TU allocation for the NR_XR_Ph3 work at RAN#106, in particular to increase TU allocation from 0.5 to 1.0 TU starting from RAN4 #114. |
| R4-2419707 | Ericsson | Draft LS response on UE assistance information <moderator> to be discussed as part of the UAI |
| R4-2417606 | InterDigital, Europe, Ltd. | Discussion on NR_XR_Ph3 RRM core requirements Observation 1: NR-DC, EN-DC and NE-DC deployments involve different schedulers that may lead to colliding DCI gap skipping orders. Proposal 1: Deprioritize discussions for NR-DC, EN-DC and NE-DC. Observation 2: Supporting IRAT gap skipping for 2G and 3G chipsets may affect their measurement timelines. Proposal 2: Support Option 2 for further discussions: - Option 2: Agree at least on the following and further discuss other scenario - Intra-frequency - Inter-Frequency - LTE inter-RAT - XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC Proposal 3: Support Option 2: Measurement skipping does not apply for measurements without gaps. Proposal 4: Support Option 2: Measurement skipping does not apply for L1 measurements and procedures Proposal 5: Support Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM. Proposal 6: Support Option 2: As a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS. Proposal 7: Support Option 2: - For the cases where measurement impact is needed - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended. - FFS how to extend. - Reuse existing measurement accuracy requirements. - For the cases where measurement impact is not needed - FFS the feasible conditions and necessary UE assistance information. Proposal 8: Support Option 2: Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period. Proposal 9: Support Option 4: assume subclause 5.3.1 of TS38.214 as a baseline for X offset. Proposal 10: If UAI mechanism is agreed, the rules for UAI gap skipping shall be left for UE implementation. Proposal 11: Support Option 4: Follow RAN1 working assumption and consider DCI only. Proposal 12: Respond to RAN1 LS: • RAN4 confirms that working assumption in Al. 1-1 is feasible • The work for the time offset delay X determination is ongoing and further updates will be sent to RAN1 in the future. |
| R4-2418298 | NTT DOCOMO, INC. | Discussions on RRM core requirements for XR Proposal 1: At least for 100MHz CBW band, support skipping of measurements for L3 measurements without gaps Proposal 2: At least for 100MHz CBW band, support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc Proposal 3: At least for 100MHz CBW band, support skipping of L1 measurements and procedures for LTM within and outside gap |
| R4-2418446 | CMCC | Discussion on XR (eXtended Reality) for NR Phase 3 Proposal 1: it is proposed that measurement skipping apply for measurements without gaps. Proposal 2: it is proposed that measurement skipping apply for L1 measurements and procedures. Observation 1: Since how UE perform measurement is up to UE implementation, network has no information about whether measurement is performed in a cetain measurement occasion. Observation 2: if the skipped measurement occasions are limited to the unused measurement occasions reported by UAI, the legacy measurement requirements can be applied, and no need to relax measurement requirements which may result in connection failure. Proposal 3: it is proposed to introduce UE assistance information related to measurement occasions, e.g. the number of needed measurement occasions for measurement or the number of measurement occasions which can be skipped. Proposal 4: it is proposed that - Apply legacy measurement delay and accuracy requirements for the following condition: - the skipped measurement occasion(s) indicated by network is the unused measurement occasion reported by UAI - Otherwise, extend measurement delay while keep legacy accuracy requirements unchanged Proposal 5: for RAN1 LS on solutions to enable transmission/reception in gaps/restrictions, it is proposed to confirm RAN1 working assumption about DCI indication. In addition to DCI solution, it is proposed to introduce semi-static schemes. Observation 3: In Rel-17 positioning, N2 is used as the time offset between UE determination of the presence of DL channels of higher priority than the DL PRS in the DL PRS processing window and the first symbol of the DL PRS processing window, to determine whether UE is required to receive DL channels and drop all PRS within the DL PRS processing window (N2 is about UE PUSCH preparation procedure time). Proposal 6: the minimum time offset(s) X between indication to skip and skipped measurement occasion is proposed as N2 symbols, which is to follow the same approach for Rel-17 positioning about the time offset between UE determination of the presence of DL channels and DL PRS processing window (N2 is about UE PUSCH preparation procedure time, which is defined in Clause 6.4, TS38.214). |
| R4-2418589 | Apple | Discussion of RRM impact of XR for NR Phase 3 Observation 1: Pre-MG deactivated delay is 6~8ms per RAN4 requirements. Proposal 1: as baseline, 7ms minimum time offset is needed between indication to skip and skipped measurement occasion. Observation 2: if only dynamic solution is supported, it could be challenging to enable it in DC scenario. Proposal 2: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported, especially for DC scenario. Proposal 3: the following types of measurement can be considered in this work item: • Intra-frequency • Inter-Frequency • LTE inter-RAT Observation 3: intra-frequency handover without gap is the most typical handover in real deployment, wherein UE is not configured with measurement gap. Observation 4: we already have mechanisms supported in standard to relax RRM measurement, RLM and BFD under certain conditions. At least it is feasible to schedule UE on RRM, RLM and BFD occasions which would cause scheduling restriction but UE is not actually performing measurement. Proposal 4: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RRM measurement outside measurement gap. Proposal 5: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD. Proposal 6: introduce an optional UE capability to support skipping of measurements without gaps. Proposal 7: priorities of each gap type: Cap types; First prio; Second prio; Don’t include. Type 1; √. Type 2; √. NCSG; √. MUSIM; √. Positioning; √. Rel 18 concurrent gap; √. PreConfigured; √. Proposal 8: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic. Proposal 9: max number of occasions cancelled during a measurement period shall be defined. Proposal 10: UE assistance information is helpful for NW to decide whether and how many measurement occasions can be cancelled, e.g. UE indicates NW a status whether it can tolerate measurement cancellation. |
| R4-2418657 | Huawei, HiSilicon | Discussion on RRM requirements for Rel-19 XR Proposal 1: Deprioritize FR2-2 scenario for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements Proposal 2: RAN4 to considering following type of measurement: o Intra-frequency o Inter-Frequency o LTE inter-RAT o And the requirements do not apply when UE is required to perform 2G and/or 3G measurement. Proposal 3: RAN4 to only consider Tx/Rx in occasions of L3 measurement. Proposal 4: Deprioritize Tx/Rx in L3 measurement without gap with scheduling restriction. Proposal 5: RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether to the feature can apply to this MG type. Observation 1: Even considering the minimum requirements in the spec without reducing the number of samples, there is already unused Gap occasion. Observation 2: If gNB take these unused gap occasion for XR service, there is no impact on mobility measurement performance, and has minor impacts on current UE implementations Proposal 6: To enable different UE implementations, RAN4 to introduce UAI with different measurement requirements, and it is up to UE to report one of / or both UAI: • UAI about the numbers and/or positions of MG can be skipped, when legacy measurement requirements can apply. • UAI about the numbers and/or positions of MG can be skipped, when measurement delay will be extended accordingly. Proposal 7: To enable different UE implementations and give flexibility from NW perspective to skip MG with different urgency: - For the case where there is no measurement impact, the conditions are as follows: - UE reports the UAI about the numbers and/or positions of MG can be skipped, when legacy measurement requirements can apply, and - The MGs indicated to be canceled by NW is within the MGs occasion indicated via UAI. - For case when the measurement delay is extended, the conditions are as follows: - UE reports the UAI about the numbers and/or positions of MG can be skipped, when legacy measurement requirements can apply, and - The MGs indicated to be canceled by NW is more than/ or different from the MGs occasion indicated via UAI. - Or - UAI about the numbers and/or positions of MG can be skipped, when measurement delay will be extended accordingly, and - The MGs indicated to be canceled by NW is within the MGs occasion indicated via UAI. Observation 3: RAN2 is also consider RRC-based approach and RAN2 is waiting for RAN4 evaluation. Proposal 8: RAN4 suggest RAN2 to introduce the semi-static approach (Alt.2) that NW indicate the indicate occasions where to skip gaps, and UE can have a stable measurement pattern/period. Whether UE supports both approaches can be up to UE capability discussion. Proposal 9: For the time offset for MG skipping, it should be at least 5 ms if both NR frequency layers and LTE inter-RAT is to be measurement within the MG. If only NR frequency layer is to be measurement within the MG, shorter time offset could be considered. E.g. 4 ms. |
| R4-2418759 | vivo | Further discussion on RRM impacts for XR and reply LS Proposal 1: Send an LS to RAN2 and RAN1 to ask if DC scenarios are supported for gap cancellation from their perspective. Proposal 2: All intra-frequency, inter-Frequency, LTE inter-RAT and UTRAN FDD measurements are supported for gap skipping in NR SA and carrier aggregation. Proposal 3: As baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS. Proposal 4: Measurement skipping does not apply for L3 measurements without gaps. Proposal 5: RAN4 does not consider enabling data transmission/reception on L1-RS symbols that need to be used for L1 measurements if there is scheduling restriction. Proposal 6: Only L1 measurements for LTM within gaps are supported for gap skipping. Proposal 7: Both of the two cases, i.e., there is impact to measurements and no impact to measurements due to gap skipping, are supported. Proposal 8: The patterns of gaps occasions not used for measurement within a time period should be provided as UE assistance information. Proposal 9: NW may indicate the percentage of measurement gaps not used for measurement during a time period for UE to provide information about gaps not used for measurement. Proposal 10: UE to indicate whether it can meet L3 measurements requirements when unused gaps reported by UE is used for data scheduling. Proposal 11: RAN4 to further study how UE assistance information/UE capability is reported when DRX is configured. Proposal 12: RAN4 to discuss the information about gaps not used for measurement are reported as UE capability or UE assistance information. Proposal 13: When only those gaps not used for measurement based on UE assistance information / UE capability are scheduled for data transmission/reception, the UE should meet existing requirements. Proposal 14: When there is impact to measurements due to gap skipping, RRM measurement requirements are defined by using NR-U like approach with carefully deciding the max number of gaps skipped during the measurement period. Proposal 15: RAN4 to study possible restrictions on dynamic indication by DCI, if it is confirmed, to achieve good trade-off between data deliver delay and mobility performance. Proposal 16: RAN4 is to inform RAN1 on the conclusion of whether L3 measurements outside gap and L1 measurements for restriction skipping are supported or not. Proposal 17: RAN4 agrees to introduce indication via MAC-CE command and/or RRC signaling for gap skipping and restriction skipping due to L3 measurements outside gap (if supported). Proposal 18: RAN4 agrees to use indication via DCI for restriction skipping due to L1 measurements (if supported). Proposal 19: It needs to be clarified that when per-FR gaps are configured, the indication of the particular gap to be skipped by DCI or MAC-CE/RRC is for which gap pattern(s). Proposal 20: Minimum time offset(s) X between indication to skip and skipped measurement occasion is 5ms. Proposal 21: For MAC-CE based and/or RRC based semi-static indication, minimum time offsets is defined as the minimum time offset(s) between the end of received MAC-CE command or RRC signaling and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped. Proposal 22: Minimum time offset(s) X between indication by MAC-CE command to skip and skipped measurement occasion is 3Nslotsubframe,μ+5ms/NR slot length. Proposal 23: Minimum time offset(s) X between indication by RRC signaling to skip and skipped measurement occasion is RRC processing delay defined in Clause 12 of TS 38.331. Observation 1: Scheduling restrictions are different in terms of conditions and restricted symbols for L3 measurements within gaps, L3 measurements outside gaps and L1 measurements. Observation 2: Type 1 MG and Type 2 MG, including per-UE gap and per-FR gap, and pre-configured MG can be considered in XR WI in Rel-19, where Type 2 MG needs further clarification. Observation 3: Depending on measurement configuration, e.g., gap, SMTC, MO etc., some of the gaps may not be used for measurements while measurement requirements being satisfied. Observation 4: It is up to UE implementation which gaps are not used for measurements under current measurement configuration. Observation 5: Significant gain on throughput is observed if UE can provide information about unused gaps for measurement without impact on mobility performance. Observation 6: Channel condition, e.g., RSRP, does not provide information about which SMTC occasions are not used for measurements. Moreover, additional information about gaps/SMTCs not used for measurement is still needed. Observation 7: UE mobility information does not help how gaps/SMTC occasions are used for measurement unless relaxed L3 measurement requirements are specified. Observation 8: It is not clear which restrictions should be skipped if there is overlapping of reference signals for L1 measurements and SMTC occasions for L3 measurements outside gap. Observation 9: There is UE complexity issue and low efficiency issue for dynamic indication via DCI is used for gap skipping and restriction skipping due to L3 measurements outside gaps. |
| R4-2418890 | Nokia | XR simulation results Results highlights - Semi static (Alt 3) approach achieved only 25% capacity gain in comparison with baseline using measurement gaps. - DCI-based approach (Alt 1-1) achieved from 50 to 75% capacity gain (from 4 user per cell to 6 or 7 XR users per cell) depending on the time offset between DCI and measurement gap. - The smaller the time offset between the DCI and start of the measurement gap is, the better the skipping performance is, with less skipping being necessary to achieve a good capacity gain. |
| R4-2419271 | ZTE Corporation, Sanechips | Discussion on RRM aspects of R19 XR Proposal 1: Reply RAN1 LS with confirmation of the scheduling DCI indication used for skipping indication. Proposal 2: Need to define time offset requirement between the dynamic gap/restriction skipping indication and XR traffic reception. The exact value of this time offset should be smaller than 5ms. Proposal 3: For L3 measurement without gap, not support measurement skipping. Proposal 4: For L1 measurement, not support measurement skipping. Proposal 5: For L1 measurement for mobility, apply same decision as normal L1 measurement. Proposal 6: Besides NR SA, the scenarios of NR-DC, EN-DC and NE-DC are applicable for measurement skipping. Proposal 7: XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC. Proposal 8: Regarding which type of MG to be consider to skip, we have the following suggestion: - First priority: Type 1 MG - Second priority: Pre-configured MG - Not consider: NCSG, Type 2 MG, MUSIM, Positioning, R18 concurrent MG Proposal 9: For the valid duration of the dynamic indication, since the gap/restriction indication field is included in the scheduling DCI, one-shot validity is preferred, i.e. only valid for the gap/restriction(s) overlapping with the XR Tx/Rx scheduled by this DCI. Proposal 10: For the gap/restriction skipping impact in frequency domain, - For the gap skipping, depend on whether the UE capable of per-FR gap - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission. - Otherwise, no need to distinguish different FR. - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped. Proposal 11: Both partial and fully overlapping between the gap/restriction and scheduled XR transmission both should be considered. Proposal 12: To guarantee the measurement performance when the MG/restriction skipping happens, one straightforward solution is to introduce scaling factor to make sure enough measurement samples. Besides the ratio between applicable chances and total chances as in legacy, some channel quality could be considered when determining the exact scaling factor. Proposal 13: No matter whether allow the UAI mechanism, it is up to NW to decide whether and which gap/restriction can be skipped. Proposal 14: As a semi-static mechanism, the UAI of measurement occasion indication is problematical under the assumption of dynamic gap/restriction indication. |
| R4-2419327 | Nokia | Discussion on XR ph3 RRM requirements Scenarios for measurement skipping - FR2-2 measurement delay requirements are 1.5x, 3x and 4.5x longer than FR2-1 measurements for 120 kHz, 480 kHz and 960 kHz SCS. - RAN4 to not apply measurement skipping for FR2-2 measurements. RAN4 to not consider dual connectivity deployment scenarios for XR enhancements. RAN4 to consider following RRM measurement types for XR enhancements - Intra-frequency - Inter-frequency - LTE inter-RAT - RRM requirements for XR enhancements do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC. Some measurements without gaps scenarios cause interruptions. Interruptions will cause scheduling problems for latency sensitive traffic. Requirements for measurements without gaps with interruptions using NeedForInterruptions-r18 do not limit interruption location in time. Measurements without gaps based on NeedForGaps-r16 do not have interruption requirements and restrictions. Measurement skipping does not apply for measurements without gaps. Measurement skipping does not apply for L1 measurements and procedures. Measurement skipping does not apply for L1 measurements and procedures for LTM. Measurements with gaps As a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS. Impact of measurement skipping In Rel-16, RAN4 defined requirements for NR_unlic, where RRM measurement delay is extended due to CCA/LBT failures without impact to measurement accuracy. As minimum requirements, cell detection, index reading, and measurement period is extended due to measurement skipping. - a. FFS how to extend - b. Reuse existing measurement accuracy requirements. RAN4 to define RRM requirements such that the network can configure which frequency layers are measured with higher priority. Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay. Limiting the maximum number of occasions that can be skipped will have an impact on the network deployment flexibility. No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration. Time offset for measurement skipping Once the UE has realized that it has received a RRM measurement skipping indication command, the UE only needs to continue normal operation without executing complex RF retuning and gap activation procedures. Shorter time offset between DCI skipping command to measurement gap enables better XR capacity for the same skipping ratio. Larger timing offset requires increased skipping ratio to provide the same user happiness which could result in mobility problems. The UE shall be capable of skipping a gap starting at least 1ms after the DCI with RRM measurement skipping indication is received. Need/feasibility of UAI The UE cannot determine SNR and number of samples needed for a cell that is not yet detected. There is no benefit in the UE indicating preference for measurement skipping that is not matching with the XR traffic. Do not define UAI related to measurement occasions. For UEs configured with search threshold (s-MeasureConfig), - the UE shall inform the network that it is stopping doing non-serving cell measurements because the s-MeasureConfig condition is no longer fulfilled. Additional skipping schemes The network has information on mobility performance of the UE as well as on the scheduled data traffic on DL and UL. Our simulations showed for the same skipping ratio only 25% capacity gain for semi-static scheme (Alt 3), whereas 50% to 75% capacity gain was achieved for dynamic schemes (Alt 1-1) Semi-static schemes are dependent on accurate XR traffic prediction, which may depend on a training stage at XR traffic start, and delay the start of using skipping for XR traffic. RAN1 already made down selection between DCI and RRC schemes. Follow RAN1 working assumption and consider DCI scheme only. CA scheduling restrictions RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs. If it is assumed that for cells within the applicable range of carriers, UE can share (parts of) receiver over different cells, measurement requirement impact due to skipping of gap/restriction should be accounted for in all those cells. While measurements are skipped on CC A, UE cannot perform concurrent gap-assisted measurements on CC B due to dependency on common receiver components for a range of carriers. RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated. |
| R4-2419515 | MediaTek inc. | Discussion on RRM requirements for XR Proposal 1: RAN4 should de-prioritize FR2-2 and NR-DC, EN-DC, NE-DC. Proposal 2: In order to maintain short time offset for gap cancellation, XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC. Proposal 3: In order to reduce the measurement delay, RAN4 shall specify that the UE operating with XR is not required to measure 2G/3G frequency layers. Proposal 4: RAN4 RRM shall consider that XR requirements are applicable to (i) intra-freq, (ii) inter-freq, and (iii) LTE inter-RAT. Proposal 5: RAN4 should keep the scheduling restriction impact on XR traffic FFS for now until there are sufficient progress in MG cancellation impact. Proposal 6: RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type. Proposal 7: RAN4 shall discuss whether the UE is required to perform positioning and MUSIM measurements while the UE is operating with XR data traffic. Proposal 8: RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic. Proposal 9: Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay. Proposal 10: RAN4 shall introduce a scaling factor to extend the total measurement delay by a factor of Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window. Proposal 11: If the XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC, then the time offset requirements for MG skipping can be 5ms. Proposal 12: RAN4 shall follow RAN1 agreement to deprioritise the UAI based solution. Proposal 13: RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period. Proposal 14: RAN4 to send reply to RAN1 with the following information: (i) XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC, (ii) time offset for gap skipping is equal to 5ms, and (iii) RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period. |
| R4-2419516 | MediaTek inc. | Reply LS on gaps/restrictions that are caused by RRM measurements |
| R4-2419706 | Ericsson | Draft LS response on UE gaps and restrictions that are caused by RRM measurements Proposal 1: FR2-2 is not considered in this WI. Observation 1: Only NR SA and NR-DC are explicitly supported for XR, according to WID. Proposal 2: EN-DC is not considered in this WI. Proposal 3: NE-DC is not considered in this WI. Proposal 4: At least the following NR measurement types with respect to carrier frequency should be considered in the requirements discussions for XR: o NR inter-frequency with MG, o NR intra-frequency with MG. Proposal 5: The following inter-RAT measurements should be considered in the requirements discussions for XR: o NR – E-UTRAN FDD, o NR – E-UTRAN TDD. Proposal 6: If EN-DC scenario is supported in RAN4 XR requirements, then for UE in EN-DC, the XR-related requirements apply when the UE is not configured with the following inter-RAT measurements (can only be configured by E-UTRAN PCell): o E-UTRAN – UTRAN FDD/TDD, o E-UTRAN – GSM, GSM, cdma2000 1 x RTT and HRPD, o FFS: E-UTRAN – NR. Proposal 7: DRX configuration needs to be considered in discussions on MG cancellation, e.g., because: - UE is not required to perform RRM measurements more frequently than once per DRX cycle, therefore depending on whether or not a UE is configured with DRX, the resource selection for XR may need to be performed differently, e.g., this may impact which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted when the selection is being decided. - UE can be configured with more than one DRX configurations, - Cancelling MGs under some DRX configurations may be less preferred compared to other DRX configurations. Proposal 8.1: RAN4 considers Type 1 MGs for MG cancelling in Rel-19 XR enhancements, as a baseline. Proposal 8.2: MGs associated with PRS (positioning) measurements shall not be cancelled. Proposal 8.3: MUSIM MGs shall not be cancelled. Proposal 8.4: FFS: Network controlled pre-configured MGs, at least until the core requirements impacts are settled for Type 1 MGs. Proposal 8.5: FFS: Concurrent MGs, at least until the core requirements impacts are settled for Type 1 MGs. Proposal 9: To avoid limiting the XR operation to good network radio conditions only and hereby limiting the XR service coverage, RRM measurement requirements are enhanced for UEs under XR operation, at least for UEs not equipped with an additional rx chain for XR. Proposal 10: RAN4 to discuss the following enhancement methods to the legacy RRM measurement period requirements to account for the UE XR operation: 1) pre-defined fixed extended RRM measurement period for UE under XR operation (e.g., based on a fixed scaling factor), 2) dynamically extendable RRM measurement period (e.g., similar to NR-U), depending on the number of actually cancelled MGs. Proposal 11: RAN4 to discuss MG configurations relevant for MG cancellation, e.g., - minimum MGL, - maximum MGL, - maximum MGRP (e.g., to avoid too long delays), etc. Proposal 12: RAN4 to discuss the impact on RRM samples and how they are combined with MG cancellation for XR, e.g., maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be discussed. Proposal 13: The measurement accuracy requirements are not impacted by MG cancellation. Proposal 14: RAN4 can confirm the RAN1 working assumption, with the understanding that the UE will expect XR data on the same carrier frequency, on which the corresponding DCI is received. Proposal 15: RAN4 will define the minimum time offset between the end of the received DCI and the MG indicated by the DCI. Proposal 16: The minimum time offset can be different for FR1 and FR2. Proposal 17: The minimum time offset between the end of the PDCCH symbol carrying DCI and the first subframe of the target measurement gap includes: o DCI processing time, o measurement gap timing advance (MGTA) configuration, o FFS any other delays. Proposal 18: FFS The minimum time offset accounts for UL timing adjustment for UL XR data within the target MG. Proposal 19: FFS The minimum time offset accounts for a guard period for DL-to-UL switching for UL XR data within the target MG. Proposal 20: RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI. Proposal 21: RAN4 to discuss whether the information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period can be defined in TS 38.133. Proposal 22: RAN4 will not specify any XR-related requirement, based on XR UAI. Proposal 23: The impact of DRX configuration on UAI, e.g., which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted at the UE and network side, should also be considered in UAI discussions. |
| R4-2419765 | Qualcomm Incorporated | RRM requirements for XR enhancements Proposal 1: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Send LS to RAN2 and RAN1. Proposal 2: Following DCI-based pre-configured MG activation/deactivation delay, specify the minimum offset X between the indication to skip and the measurement occasion to be skipped as 5 ms. Proposal 3: UE ignores any further DCI indication received after the first DCI bit ‘1’ indicating cancellation of a particular MG occasion. Proposal 4: Dynamic signaling to enable Tx/Rx doesn’t apply to scheduling restrictions caused by L1 measurements. Proposal 5: Dynamic signaling to enable Tx/Rx doesn’t apply to scheduling restrictions caused by L3 measurements without gaps. Proposal 6: RAN4 to consider extending the L3 requirements with MGs such as cell identification and measurements delays when one or more MGs are deactivated by the network. - For DCI based dynamic indication, follow the NR-U approach to extend the L3 delays by the number of MGs deactivated during the period. Proposal 7: When overlapping between gaps/restrictions that are caused by RRM measurements occurs, the dynamic indication to enable Tx/Rx in a particular gap/restriction that are caused by RRM measurements is applied after the overlapping is resolved. Proposal 8: RAN4 to consider introducing UAI to indicate one or more of the following aspects for MG deactivation - Maximum number of consecutive MGs that can be skipped - Maximum number of MGs that can be skipped within a duration - MG type that can be skipped |

## Open issues summary

### Topic 1 Scenarios for measurement skipping

#### Issue 1-1: Deployment scenarios

- Background information: In RAN4 #112 bis the following agreement regarding deployment scenarios was reached R4-2416864:

< Agreement>: - Frequency ranges - Agree on FR1 and FR2-1 - Scenarios - Start the discussion for - NR-SA - Carrier aggregation <Way forward>: - Further discussion needed for - FR2-2 - NR-DC, EN-DC, NE-DC

- Proposals related to frequency range

  - Option 1 (Huawei, Mediatek): Deprioritize FR2-2 scenario for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements

  - Option 2 (Nokia, Ericsson): FR2-2 is not considered in this WI.

- Proposals related to dual connectivity

  - Option 1 (Ericsson): EN-DC and NE-DC are not considered in this WI, based on WID.

  - Option 2 (Interdigital, Mediatek): Deprioritize discussions for NR-DC, EN-DC and NE-DC.

  - Option 3 (Nokia): RAN4 to not consider dual connectivity deployment scenarios for XR enhancements.

  - Option 4 (ZTE): NR-DC, EN-DC and NE-DC are applicable for measurement skipping.

  - Option 5 (vivo): Send an LS to RAN2 and RAN1 to ask if DC scenarios are supported for gap cancellation from their perspective.

- Recommended WF

  - For frequency range, please check if we can agree in option 1 or option 2.

  - For dual connectivity scenarios, discussion regarding how to consider each scenario is needed

#### Issue 1-2: Types of measurements to be considered

- Proposals regarding which types of measurements are considered for XR enhancements

  - Proposal 1 (Apple, Interdigital, Huawei, Nokia, Mediatek, Ericsson):

    - Intra-frequency

  - Proposal 2 (Apple, Interdigital, Huawei, Nokia, Mediatek, Ericsson):

    - Inter-frequency

  - Proposal 3 (Apple, Interdigital, Huawei, Nokia, Mediatek, Ericsson):

    - LTE inter-RAT (NR-E-UTRAN FDD/TDD)

  - Proposal 4 (Interdigital, Huawei, Nokia, Mediatek, ZTE):

    - XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC

  - Proposal 5 (vivo):

    - UTRAN FDD measurements are supported for gap skipping

  - Proposal 6 (Ericsson):

    - If EN-DC scenario is supported in RAN4 XR requirements, then for UE in EN-DC, the XR-related requirements apply when the UE is not configured with the following inter-RAT measurements (can only be configured by E-UTRAN PCell):

      - E-UTRAN – UTRAN FDD/TDD,

      - E-UTRAN – GSM, GSM, cdma2000 1 x RTT and HRPD,

      - FFS: E-UTRAN – NR.

- Recommended WF

  - Discussion needed

    - Proposals 1, 2 and 3 are agreeable for most companies

    - Discussion on Proposal 4 and Proposal 5 is needed regarding UTRAN and 2G measurements, also depends on the outcome of Issue 1-1

    - Proposal 6 needs discussion depending on the outcome of Issue 1-1

#### Issue 1-3: L3 Measurements without gaps with scheduling restrictions

- Proposals

  - Option 1: Skipping of measurements without gaps with scheduling restrictions are supported

    - Option 1a (CMCC, Apple): Support skipping of measurements without gaps

    - Option 1b (NTT DOCOMO): Support skipping of measurements without gaps for 100MHz CBW band

  - Option 2: Skipping of measurements without gaps with scheduling restrictions are deprioritized or not supported

    - Option 2a (Interdigital, vivo, ZTE, Nokia, Qualcomm, Ericsson): Measurement skipping does not apply for measurements without gaps

    - Option 2b (Huawei, Mediatek): Deprioritize Tx/Rx in L3 measurement without gap with scheduling restriction.

- Recommended WF

  - Please consider the options.

#### Issue 1-4: L1 measurements with scheduling restrictions and procedures

- Proposals

  - Option 1: Support skipping of L1 measurements and procedures.

    - Option 1a (CMCC): Support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc

    - Option 1b (Apple): it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD.

    - Option 1c (NTT DOCOMO): At least for 100MHz CBW band, support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc

  - Option 2: Not to support or deprioritize skipping of L1 measurements and procedures.

    - Option 2a (Interdigital, Huawe?, vivo, ZTE, Nokia, Qualcomm, Ericsson): Measurement skipping does not apply for L1 measurements and procedures

    - Option 2b (Mediatek): RAN4 should keep the scheduling restriction impact on XR traffic FFS for now until there are sufficient progress in MG cancellation impact.

- Recommended WF

  - Discussion needed

#### Issue 1-5: L1/L2 triggered mobility measurements and procedures

- Previous WF

<Way forward>: - Option 1: support skipping of L1 measurements and procedures for LTM within and outside gap - Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM

- Proposals

  - Option 1 (NTT DOCOMO): At least for 100MHz CBW band, support skipping of L1 measurements and procedures for LTM within and outside gap

  - Option 2 (Interdigital, Huawei, Nokia): measurement skipping do not apply for L1 measurements and procedures for LTM

  - Option 3 (vivo): Only L1 measurements for LTM within gaps are supported for gap skipping.

  - Option 4 (ZTE): For L1 measurement for mobility, apply same decision as normal L1 measurement.

- Recommended WF

  - Discussion is needed

#### Issue 1-6: LS to RAN1 on L3/L1 decisions

- Proposals

  - Proposal 1 (vivo): RAN4 is to inform RAN1 on the conclusion of whether L3 measurements outside gap and L1 measurements for restriction skipping are supported or not.

- Recommended WF

  - To be discussed after conclusion on Issues 1-3, 1-4, 1-5

### Topic 2 Measurements with gaps

#### Issue 2-1: General approach for defining requirements for measurements with gaps

- Proposals

  - Option 1 (Huawei, Mediatek): RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type.

  - Option 2 (Interdigital, vivo, Nokia, Ericsson): As a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS.

- Recommended WF

  - The selection of the measurement gap types is a topic that has been taking long discussion. Please consider if a combination of proposals 1 and 2 can be considered as a baseline.

#### Issue 2-2: Types of measurement gaps

- In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2:

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

- Company proposals on the priorities for RAN4 to work are summarized using based on the following table

| Gap types | First priority | Second priority | Don’t include |
| --- | --- | --- | --- |
| Type 1 | Apple, ZTE Ericsson |  |  |
| Type 2 | Apple | Ericsson FFS | ZTE |
| NCSG | Apple | Ericsson FFS | ZTE |
| MUSIM |  | Apple | ZTE MUSIM Ericsson |
| Positioning |  |  | Apple ZTE Ericsson |
| Rel 18 concurrent gap | Apple |  | ZTE |
| PreConfigured | Apple | ZTE |  |

- Recommended WF

  - The selection of the measurement gap types is a topic that has been taking long discussion. Please consider if agreement on Issue 2-1 is possible.

### Topic 3 XR skipping requirements impact

#### Issue 3-1: How to capture impact of measurement skipping

- Previous WF

<Way forward>: - Option 1: - As minimum requirements, cell detection, index reading, and measurement period is extended. - FFS how to extend - Reuse existing measurement accuracy requirements. - Option 2: - For the cases where measurement impact is needed - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended. - FFS how to extend. - Reuse existing measurement accuracy requirements. - For the cases where measurement impact is not needed - FFS the feasible conditions and necessary UE assistance information.

- Proposals related to measurement accuracy requirements

  - Option 1 (Nokia, Interdigital, Ericsson, CMCC): Reuse existing measurement accuracy requirements.

  - Option 2 (Mediatek): RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic.

- Proposals related to measurement delay requirements:

  - Option 1: Proposals without distinction of XR and legacy requirements

    - Option 1a (Nokia): As minimum requirements, cell detection, index reading, and measurement period is extended.

    - Option 1b (Apple): as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

    - Option 1c (Qualcomm): extend the L3 requirements with MGs such as cell identification and measurements delays when one or more MGs are deactivated by the network.

    - Option 1d (Ericsson): RRM measurement requirements are enhanced for UEs under XR operation, at least for UEs not equipped with an additional rx chain for XR.

  - Option 2: Proposals with distinction of conditions where measurement impact is not needed

    - Option 2a (Interdigital):

      - For the cases where measurement impact is needed

        - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended.

          - FFS how to extend.

      - For the cases where measurement impact is not needed

        - FFS the feasible conditions and necessary UE assistance information.

    - Option 2b (CMCC):

      - Apply legacy measurement delay and accuracy requirements for the following condition:

        - the skipped measurement occasion(s) indicated by network is the unused measurement occasion reported by UAI

      - Otherwise, extend measurement delay while keep legacy accuracy requirements unchanged

- Recommended WF

  - For measurement accuracy, please check if Option 1 is agreeable

  - For measurement delay discussion is needed.

#### Issue 3-2: Principles for measurement delay calculation

- Proposals

  - Option 1: Options based on scaling factor

    - Option 1a (Nokia, Mediatek): Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay.

    - Option 1b (Mediatek): Introduce a scaling factor to extend the total measurement delay by a factor of Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window.

    - Option 1c (ZTE): introduce scaling factor to make sure enough measurement samples. Besides the ratio between applicable chances and total chances as in legacy, some channel quality could be when determining the exact scaling factor.

    - Option 1d (Ericsson): pre-defined fixed extended RRM measurement period for UE under XR operation (e.g., based on a fixed scaling factor).

  - Option 2 (Interdigital, vivo, Qualcomm): Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period.

    - Option 2a (Ericsson): dynamically extendable RRM measurement period (e.g., similar to NR-U), depending on the number of actually cancelled MGs.

- Recommended WF

  - Discussion is needed

#### Issue 3-3: Conditions for measurement skipping

- Proposals

  - Option 1 (Apple): max number of occasions cancelled during a measurement period shall be defined.

  - Option 2 (Nokia): No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

  - Option 3 (Ericsson): discuss MG configurations relevant for MG cancellation, e.g.,

    - minimum MGL (e.g., MGL≥3 ms, due to low gain with MG skipping),

    - maximum MGL (e.g., MGL≤6 ms),

    - maximum MGRP (e.g., MGRP≤320 ms to avoid too long delays), etc.

  - Option 4 (Ericsson): RAN4 to discuss the impact on RRM samples and how they are combined during the UE XR operation, e.g., maximum acceptable time separation between two measurement occasions available for RRM measurements.

- Recommended WF

  - Discussion on the conditions for skipping is needed.

#### Issue 3-4: Impact on different frequency layers

- Proposals

  - Proposal 1 (Mediatek): In order to reduce the measurement delay, RAN4 shall specify that the UE operating with XR is not required to measure 2G/3G frequency layers.

  - Proposal 2 (Nokia): RAN4 to define requirements such that the network can configure which frequency layers are measured with higher priority

- Recommended WF

  - Discuss the proposals. Proposal 1 is related to issues 1-1.

#### Issue 3-5: DRX

- Proposal

  - Proposal 1 (Ericsson): DRX configuration needs to be considered in discussions on MG cancellation, e.g., because:

    - UE is not required to perform RRM measurements more frequently than once per DRX cycle, therefore depending on whether or not a UE is configured with DRX, the resource selection for XR may need to be performed differently, e.g., this may impact which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted when the selection is being decided.

    - UE can be configured with more than one DRX configurations,

    - Cancelling MGs under some DRX configurations may be less preferred compared to other DRX configurations.

- Recommended WF

  - Discuss the proposal

#### Issue 3-6: Measurement delay impact dependency on UAI

- Proposals

  - Proposal 1 (Huawei): To enable different UE implementations and give flexibility from NW perspective to skip MG with different urgency:

    - For the case where there is no measurement impact, the conditions are as follows:

      - UE reports the UAI about the numbers and/or positions of MG can be skipped, when legacy measurement requirements can apply, and

      - The MGs indicated to be canceled by NW is within the MGs occasion indicated via UAI.

    - For case when the measurement delay is extended, the conditions are as follows:

      - UE reports the UAI about the numbers and/or positions of MG can be skipped, when legacy measurement requirements can apply, and

      - The MGs indicated to be canceled by NW is more than/ or different from the MGs occasion indicated via UAI.

Or

      - UAI about the numbers and/or positions of MG can be skipped, when measurement delay will be extended accordingly, and

      - The MGs indicated to be canceled by NW is within the MGs occasion indicated via UAI.

  - Proposal 2 (vivo): When only those gaps not used for measurement based on UE assistance information / UE capability are scheduled for data transmission/reception, the UE should meet existing requirements.

  - Proposal 3 (CMCC):

- Apply legacy measurement delay and accuracy requirements for the following condition:

- the skipped measurement occasion(s) indicated by network is the unused measurement occasion reported by UAI

- Otherwise, extend measurement delay while keep legacy accuracy requirements unchanged

- Proposal 4 (Ericsson):

- RAN4 will not specify any XR-related requirement, based on XR UAI.

- Recommended WF

  - Discussion is needed

#### Issue 3-7: Concurrent gaps

- Proposal

  - Proposal 1 (Qualcomm): When overlapping between gaps/restrictions that are caused by RRM measurements occurs, the dynamic indication to enable Tx/Rx in a particular gap/restriction that are caused by RRM measurements is applied after the overlapping is resolved.

- Recommended WF

  - Discussion depends on the outcome of which gaps are considered.

### Topic 4 Time offset for measurement skipping processing

#### Issue 4-1: Time offset requirement for measurement gap skipping

- Proposals

  - Option 1 (Apple): 7 ms

  - Option 2 (Qualcomm, vivo): 5 ms

  - Option 3 (ZTE): Smaller than 5 ms

  - Option 4 (Nokia): 1ms

  - Option 5 (Interdigital): assume subclause 5.3.1 of TS38.214 as a baseline for X offset.

  - Option 6 (Mediatek): If the XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC, then the time offset requirements for MG skipping can be 5ms.

  - Option 7 (CMCC): N2 symbols, which is to follow the same approach for Rel-17 positioning about the time offset between UE determination of the presence of DL channels and DL PRS processing window  (N2 is about UE PUSCH preparation procedure time, which is defined in Clause 6.4, TS38.214).

  - Option 8 (Huawei):

    - 5 ms if both NR frequency layers and LTE inter-RAT is to be measurement within the MG.

    - If only NR frequency layer is to be measurement within the MG, shorter time offset could be considered. E.g. 4 ms.

- Recommended WF

  - Discussion needed

#### Issue 4-2: Principles for defining time offset

- Proposals

  - Proposal 1 (Ericsson): RAN4 will define  the minimum time offset between the end of the received DCI and the MG indicated by the DCI.

  - Proposal 2 (Ericsson): The minimum time offset can be different for FR1 and FR2.

  - Proposal 3 (Ericsson): The minimum time offset between the end of the PDCCH symbol carrying DCI and the first subframe of the target measurement gap includes:

    - DCI processing time,

    - measurement gap timing advance (MGTA) configuration,

    - FFS any other delays.

  - Proposal 4 (Ericsson): FFS: The minimum time offset accounts for UL timing adjustment for UL XR data within the target MG.

  - Proposal 5 (Ericsson): FFS: The minimum time offset accounts for a guard period for DL-to-UL switching for UL XR data within the target MG.

- Recommended WF

  - Discussion needed

#### Issue 4-3: Time offset for other skipping schemes

- Proposals

  - Proposal 1 (vivo):

    - For MAC-CE based and/or RRC based semi-static indication, minimum time offsets is defined as the minimum time offset(s) between the end of received MAC-CE command or RRC signaling and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped.

    - Minimum time offset(s) X between indication by MAC-CE command to skip and skipped measurement occasion is 3N_"slot" ^("subframe" ,μ)+5ms/NR slot length.

    - Minimum time offset(s) X between indication by RRC signaling to skip and skipped measurement occasion is RRC processing delay defined in Clause 12 of TS 38.331.

- Recommended WF

  - No need to discuss before reaching conclusion on additional skipping schemes

#### Issue 4-4: Reply LS to RAN1 (original in R1-2407561 (R4-2414910))

- Option 1: Draft LS response in R4-2419706 (Ericsson).

- Option 2: Draft LS response in R4-2419516 (Mediatek)

- Option 3: Draft LS response in R4-2419326 (Nokia)

- Option 4: Draft LS response in R4-2418759 (vivo)

### Topic 5 Need/feasibility of UE assistance information

Sub-topic description

RAN1 has sent an LS (R1-2405736) to RAN4, requesting RAN4 to decide whether to introduce any UE assistance information related to measurement occasions. This sub-topic includes issues relating to the need and feasibility of such UE assistance information.

Open issues and candidate options before meeting:

#### Issue 5-1: General on UAI

- Proposals related to measurement occasions

  - Option 1: Define UAI related to measurement occasions

    - Option 1a (CMCC, Huawei, Qualcomm): number of occasions which can be skipped

    - Option 1b (CMCC): number of measurement occasions that are needed for measurement

    - Option 1c (Apple): UE indicates NW a status whether it can tolerate measurement cancellation.

    - Option 1d (Huawei, vivo): patterns of gaps positions of MG that can be skipped

    - Option 1e (vivo): study how UE assistance information/UE capability is reported when DRX is configured.

    - Option 1f (Qualcomm): Maximum number of consecutive MGs that can be skipped

    - Option 1g (Qualcomm): MG type that can be skipped

  - Option 2:

    - Option 2a (Nokia): Do not define UAI related to measurement occasions.

    - Option 2b (Mediatek): RAN4 shall follow RAN1 agreement to deprioritise the UAI based solution

    - Option 2c (Ericsson): RAN4 will not specify any XR-related requirement, based on XR UAI.

      - Draft LS response in R4-2419707.

  - Option 3: The information about gaps not used for measurement are reported as UE capability.

  - Option 4 (Ericsson): The impact of DRX configuration on UAI, e.g., which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted at the UE and network side, should also be considered in UAI discussions.

- Proposals related to channel conditions

  - Option 1 (Nokia): For UEs configured with search threshold (s-MeasureConfig), the UE informs the network that it is stopping doing non-serving cell measurements because the s-MeasureConfig condition is no longer fulfilled.

- Recommended WF

  - Please discuss which option can be agreed

  - For conditions related to measurement occasions, consider if UAI is needed/feasible, and if it may be defined as UE capability as in Option 3.

#### Issue 5-2: Rules for UAI gap skipping

- Proposals

  - Proposal 1 (Interdigital): the rules for UAI gap skipping shall be left for UE implementation.

  - Proposal 2 (Ericsson): RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI.

  - Proposal 3 (ZTE): It is up to NW to decide whether and which gap/restriction can be skipped.

  - Proposal 4 (ZTE): As a semi-static mechanism, the UAI of measurement occasion indication is problematical under the assumption of dynamic gap/restriction skipping indication.

- Recommended WF

  - Discuss this issue based on the outcome of 5-1.

### Topic 6 Additional signalling aspects

#### Issue 6-1: Semi-static schemes

- Proposals

  - Option 1 (Interdigital, Nokia): Follow RAN1 working assumption and consider DCI only.

  - Option 2 (Apple, Huawei, Mediatek, Qualcomm, CMCC): also consider RRC/semi-static approach.

    - Option 2a (Apple): In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported, especially for DC scenario.

    - Option 2b (Huawei): RAN4 suggest RAN2 to introduce the semi-static approach (Alt.2) that NW indicate the indicate occasions where to skip gaps, and UE can have a stable measurement pattern/period. Whether UE supports both approaches can be up to UE capability discussion.

    - Option 2c (Mediatek): RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period.

    - Option 2d (Qualcomm): In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Send LS to RAN2 and RAN1.

    - Option 2e (CMCC):  In addition to DCI solution, it is proposed to introduce semi-static schemes.

  - Option 3 (vivo):

    - RAN4 agrees to introduce indication via MAC-CE command and/or RRC signaling for gap skipping and restriction skipping due to L3 measurements outside gap (if supported).

    - RAN4 agrees to use indication via DCI for restriction skipping due to L1 measurements (if supported).

- Recommended WF

  - Please discuss the proposals, considering the existing agreements from RAN1.

#### Issue 6-2: Skipping indication on frequency domain

- Proposals

  - Option 1 (ZTE): For the gap/restriction skipping impact in frequency domain,

    - For the gap skipping, depend on whether the UE capable of per-FR gap

      - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission.

      - Otherwise, no need to distinguish different FR.

    - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped.

  - Proposal 2 (Nokia):

    - RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs.

    - RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated.

  - Proposal 3 (vivo): It needs to be clarified that when per-FR gaps are configured, the indication of the particular gap to be skipped by DCI or MAC-CE/RRC is for which gap pattern(s).

- Recommended WF

  - This issue has not been discussed yet in RAN4. Companies views is needed.

#### Issue 6-3: Network indication

- Proposals

  - Proposal 1 (vivo):

    - NW may indicate the percentage of measurement gaps not used for measurement during a time period for UE to provide information about gaps not used for measurement.

- Recommended WF

  - Please discuss the proposal.

#### Issue 6-4: DCI indication details

- Proposals

  - Proposal 1 (ZTE): For the valid duration of the dynamic indication, since the gap/restriction indication field is included in the scheduling DCI, one-shot validity is preferred, i.e. only valid for the gap/restriction(s) overlapping with the XR Tx/Rx scheduled by this DCI.

  - Proposal 2 (Qualcomm): UE ignores any further DCI indication received after the first DCI bit ‘1’ indicating cancellation of a particular MG occasion.

- Recommended WF

  - For the proponents of Proposal 1 and 2, please clarify why this should be discussed in RAN4 and not in RAN1.

#### Issue 6-5: Partial measurement skipping

- Proposals

  - Proposal 1 (ZTE): Both partial and fully overlapping between the gap/restriction and scheduled XR transmission both should be considered.

- Recommended WF

  - Please discuss the proposal.

### Topic 7 Reply LS to RAN1

#### Issue 7-1: Response on RAN1 working assumption and time offset

- LS from RAN1:

  - R1-2407561	LS out	LS on gaps/restrictions that are caused by RRM measurements	RAN1, Nokia, RAN1#118

# 1 Overall description RAN1 discussed solutions to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. The following working assumption and agreement were taken: Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s).. # 2 Actions To RAN4 ACTION: RAN1 respectfully asks RAN4 to check the working assumption and inform RAN1 if there are concerns that RAN1 should address.

- Proposals

  - Option 1: Options confirming feasibility of Alt 1-1

    - Option 1a (Interdigital, Nokia): RAN4 confirms that working assumption in Al. 1-1 is feasible

      - The work for the time offset delay X determination is ongoing and further updates will be sent to RAN1 in the future.

    - Option 1b (ZTE): Reply RAN1 LS with confirmation of the scheduling DCI indication used for skipping indication.

    - Option 1c (Ericsson): RAN4 can confirm the RAN1 working assumption, with the understanding that the UE will expect XR data on the same carrier frequency, on which the corresponding DCI is received.

  - Option 2: proposals confirming RAN1 working assumption and requesting additional schemes to be considered

    - Option 2a (CMCC): confirm RAN1 working assumption about DCI indication. In addition to DCI solution, it is proposed to introduce semi-static schemes.

    - Option 2b (Mediatek): RAN4 to send reply to RAN1 with the following information: (i) XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC, (ii) time offset for gap skipping is equal to 5ms, and (iii) RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period.

- Recommended WF

  - Please discuss the options.

### Topic 8 UE capabilities

#### Issue 8-1: UE Capability for L3 Measurements without gaps with scheduling restrictions

- Proposals

  - Option 1 (Apple): introduce an optional UE capability to support skipping of measurements without gaps.

- Recommended WF

  - Depends on the outcome of issue 2-3

### Topic 9 RAN4 TU allocation for XR

#### Issue 9-1: Allocation for RRM

- Proposals

  - Proposal 1 (Nokia): RAN4 to request a higher TU allocation for the NR_XR_Ph3 work at RAN#106, in particular to increase TU allocation from 0.5 to 1.0 TU starting from RAN4 #114.

- Recommended WF

  - Discuss the proposal