| 3GPP TR 37.719-11-11 V0.2.0 (2024-11) |  |
| --- | --- |
| Technical Report --- |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; Rel-19 Dual Connectivity of EN-DC and NE-DC configurations consisting of 2 different bands downlink (DL) with 2 different bands uplink (UL) (1 LTE band and 1 NR band) (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Background	7

4.1	TR Maintenance	7

5	DC of 1 LTE band and 1 NR band:  General Part	8

5.1 	Co-existence analysis for DC	8

5.1.1	 Inter-band EN-DC/NE-DC within FR1	8

5.1.2	 Inter-band EN-DC/NE-DC including FR2	10

5.2 	Interference analysis for other system	10

5.3 	Spurious emission band UE co-existence for EN-DC/NE-DC	11

5.4 	Handling of Band specific requirements for inter-band EN-DC/NE-DC including FR2	11

6	DC of 1 LTE band and 1 NR band:  Specific Band Combination Part	12

6.1	Inter-band DC within FR1	12

6.1.1	DC_68_n1	12

6.1.2	DC_68_n3	14

6.1.3	DC_68_n7	16

6.1.4	DC_68_n8	17

6.1.5	DC_68_n20	19

6.1.6	DC_68_n38	22

6.1.7	DC_68_n40	24

6.1.8	DC_68_n41	26

6.1.9	DC_68_n77	28

6.1.10	DC_68_n78	30

6.1.11	DC_68_n79	33

6.2	Inter-band DC within FR2	35

6.2.1	DC_X_nY	35

6.3	Intra-band contiguous DC	36

6.3.1	DC_(n)X	36

6.4	Intra-band non-contiguous DC	37

6.4.1	DC_X_nX	37

Annex A: Additional guidelines	38

A.1	Band group definition	38

Annex B (informative):  Change history	39

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for Dual Connectivity of EN-DC and NE-DC configurations consisting of 2 different bands downlink (DL) with 2 different bands uplink (UL) (1 LTE band and 1 NR band) under Rel-19 time frame.

The purpose is to gather the relevant background information and studies in order to address Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) for the Rel-19 band combinations in Table 1-1, 1-2, 1-3 and 1-4, including EN-DC and NE-DC combinations. The actual requirements are added to the corresponding technical specifications.

Table 1-1: Release 19 EN-DC of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) within FR1

| DC combination | Uplink EN-DC configuration |
| --- | --- |
| DC_68A_n1A | DC_68A_n1A |
| DC_68A_n3A | DC_68A_n3A |
| DC_68A_n7A | DC_68A_n7A |
| DC_68A_n8A | DC_68A_n8A |
| DC_68A_n20A | DC_68A_n20A |
| DC_68A_n38A | DC_68A_n38A |
| DC_68A_n40A | DC_68A_n40A |
| DC_68A_n41A | DC_68A_n41A |
| DC_68A_n77A | DC_68A_n77A |
| DC_68A_n78A | DC_68A_n78A |
| DC_68A_n79A | DC_68A_n79A |

Table 1-2: Release 19 EN-DC of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) including FR2

| DC combination | Uplink EN-DC configuration |
| --- | --- |

Table 1-3: Release 19 NE-DC of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) within FR1

| DC combination | Uplink NE-DC configuration |
| --- | --- |

Table 1-4: Release 19 NE-DC of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) including FR2

| DC combination | Uplink NE-DC configuration |
| --- | --- |

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[3]	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

[4]	3GPP TS 36.101: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”

[5]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Background

The present document is a technical report for Dual Connectivity (DC) of 1 LTE band and 1 NR band under Rel-19 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

5	DC of 1 LTE band and 1 NR band: 
General Part

5.1 	Co-existence analysis for DC

5.1.1	 Inter-band EN-DC/NE-DC within FR1

For each inter-band EN-DC/NE-DC band combination, sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference, harmonic mixing, cross band isolation, and intermodulation interference from another band part of the same DC configuration.

For 2UL inter-band DC, the harmonic/harmonic mixing, cross-band isolation, and IMD products can be analysed by the templates shown in Table 5.1.1-1, Table 5.1.1-2 and Table 5.1.1-3.

Table 5.1.1-1 Band X and Band nY UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | X | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | flow |  |  |  |  |  |  |
| nY | flow | fhigh |  |  |  |  |  |  |
| DL1 |  |  | N/A |  |  |  |  | UL harmonic |
| DL23 |  |  |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 |  |  |  |  | N/A |  | N/A |  |
| DL4 |  |  |  | N/A | N/A | N/A | N/A |  |
| DL54 |  |  |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | Editor’s Note: The analysis conclusion for the UL harmonic/harmonic mixing shall be included here, even if no issues are identified. |  |  |  |  |  |
| UL/DL harmonics |  | nY | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | flow |  |  |  |  |  |  |
| X | flow | fhigh |  |  |  |  |  |  |
| DL1 |  |  | N/A |  |  |  |  | UL harmonic |
| DL23 |  |  |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 |  |  |  |  | N/A |  | N/A |  |
| DL4 |  |  |  | N/A | N/A | N/A | N/A |  |
| DL54 |  |  |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | Editor’s Note: The analysis conclusion for the UL harmonic/harmonic mixing shall be included here, even if no issues are identified. |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range. NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only applies for DL>3GHz. NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.1.1-2: Band X and Band nY cross-band isolation analysis

| Band | X |  | nY |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fxlow | fxhigh | fylow | fyhigh |
| fUL (MHz) |  |  |  |  |
| fDL (MHz) |  |  |  |  |
| UL CBW (MHz)2 | Minimum CBWx | Maximum CBWx | Minimum CBWy | Maximum CBWy |
| ACLR1 range | fxUL_low-maxULCBWx | fxUL_high+maxULCBWx | fyUL_low-maxULCBWy | fyUL_high+maxULCBWy |
| ACLR1 (MHz) |  |  |  |  |
| ACLR2 range | fxUL_low-2*maxULCBWx | fxUL_high+2*maxULCBWx | fyUL_low-2*maxULCBWy | fyUL_high+2*maxULCBWy |
| ACLR2 (MHz) |  |  |  |  |
| ACLR3 range | fxUL_low-3*maxULCBWx | fxUL_high+3*maxULCBWx | fyUL_low-3*maxULCBWy | fyUL_high+3*maxULCBWy |
| ACLR3 (MHz) |  |  |  |  |
| ACLR4 range | fxUL_low-4*maxULCBWx | fxUL_high+4*maxULCBWx | fyUL_low-4*maxULCBWy | fyUL_high+4*maxULCBWy |
| ACLR4 (MHz) |  |  |  |  |
| ACLR5 range1 | fxUL_low-5*maxULCBWx | fxUL_high+5*maxULCBWx | fyUL_low-5*maxULCBWy | fyUL_high+5*maxULCBWy |
| ACLR5 (MHz) |  |  |  |  |
| Analysis | Editor’s Note: The analysis conclusion for the cross-band isolation shall be included here, even if no issues are identified. |  | Editor’s Note: The analysis conclusion for the cross-band isolation shall be included here, even if no issues are identified. |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1-1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible. NOTE 2: The maximum UL channel bandwidth of the band (noted maxULCBW) is used to calculate the ACLR ranges. |  |  |  |  |

Table 5.1.1-3: Band X and Band nY UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | – |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | – |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

For UL 3CC intra-band plus inter-band DC, the intra-band UL CA IMD overlap with the other DL band and the 3CC UL IMD can be analysed by the templates shown in Table 5.1.1-4, and Table 5.1.1-5.

Table 5.1.1-4: Band X and CA nYC intra-band UL CA IMD overlap with the other DL band analysis.

| (All in MHz) | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| nY fUL |  |  | Order | fmin | fmax |
| X fDL |  |  |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | fmin | fmax |  |  |  |
| IMD3 (2-1) | fUL_low-Max2CCBW | fUL_high+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
| IMD5 (3-2) | fUL_low-2*Max2CCLBW | fUL_high+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  |  |  | Order | fmin | fmax |
| IMD7 (4-3) | fUL_low-3*Max2CCBW | fUL_high+3*Max2CCBW | IMD4 (3-1) | 2*fUL_low-Max2CCBW | 2*fUL_high+Max2CCBW |
| IMD9 (5-4) | fUL_low-4*Max2CCBW | fUL_high+4*Max2CCBW | IMD6 (4-2) | 2*fUL_low-2*Max2CCBW | 2*fUL_high+2*Max2CCBW |
| IMD11 (6-5) | fUL_low-5*Max2CCBW | fUL_high+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  |  |  | Order | fmin | fmax |
| IMD13 (7-6) | fUL_low-6*Max2CCBW | fUL_high+6*Max2CCBW | IMD5 (4-1) | 3*fUL_low-Max2CCBW | 3*fUL_high+Max2CCBW |
| Analysis | Editor’s Note: The analysis conclusion for the UL CA IMD shall be included here, even if no issues are identified. |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band UL CA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous UL CA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fUL_high-fUL_low) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or Sim Rx/Tx TDD bands <1GHz - IMD6 is considered for FDD or Sim Rx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band. |  |  |  |  |  |

Table 5.1.1-5: Band X and Band nYC 3CC UL IMD products

| Band / CA1 | X |  | CA_nYC |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low | fx_high | fy_low | fy_high |
| fUL (MHz) | – |  | – |  |
| fDL (MHz) | – |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | – |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | – |  | – |  |
| Analysis | Editor’s Note: The analysis conclusion for the 3UL CA IMD shall be included here, even if no issues are identified. |  |  |  |
| NOTE 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1-1, the analysis can be ignored. NOTE 2: For contiguous intra-band UL CA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Note that, for each DC band combination, the MSD table shall be provided for interference that requires sensitivity degradation for an Rx band according to the same manner as specified in section 7.3B in TS 38.101-3.

5.1.2	 Inter-band EN-DC/NE-DC including FR2

Based on the agreement in RAN4#95-e (R4-2008931), it is allowed to propose configurations for EN-DC including FR2 with one draft CR.

5.2 	Interference analysis for other system

When 2UL inter-band EN-DC/NE-DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems shown in Table 5.2-1.

Table 5.2-1: Harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Harmonic/IMD |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  |  |  |
| Galileo | 1559 | - | 1591 |  |  |  |
| GLONASS | 1591 | - | 1610 |  |  |  |
| GPS | 1563 | - | 1587 |  |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 |  | US/Europe |  |
|  | 2400 | - | 2494 |  | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 |  | US |  |
|  | 5150 | - | 5350 |  | Europe |  |
|  | 5470 | - | 5725 |  |  |  |
|  | 5150 | - | 5825 |  | Asia |  |

To mitigate the impact of interference to other system is implementation dependent, thus it is optional in section 6 for each DC band combination.

5.3 	Spurious emission band UE co-existence for EN-DC/NE-DC

Table 5.3-1 specifies the spurious emission requirements for the specified EN-DC configurations for coexistence with protected bands.

Table 5.3-1: Spurious emissions for uplink inter-band DC (two bands)

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_X_nY | E-UTRA Band xx, yy, zz |  |  |  |  |  |  |
|  | NR band aa, bb, cc |  |  |  |  |  |  |
|  | Frequency range |  |  |  |  |  |  |
| NOTE X: |  |  |  |  |  |  |  |

Note that as specified in the TS 38.101-3, the intersection of the requirements for the individual bands specified in clause 6.5.3.2 of [2] and clause 6.6.3.2 of [4] shall also apply for the specified uplink EN-DC configurations. Intersection of a requirement means that both UL constituent bands have the same protected band requirement specified and if one or both protected bands have note(s) associated those note(s) also apply.

Note that for the NE-DC configurations that have a corresponding specified EN-DC configuration, the requirements in the Table 6.5B.3.3.2-1 of TS 38.101-3 apply on each component carrier with all component carriers are active.

5.4 	Handling of Band specific requirements for inter-band EN-DC/NE-DC including FR2

Unless otherwise stated, for inter-band EN-DC/NE-DC configurations including FR2, there are no additional requirements of spurious emission band UE co-existence, MSD analysis, and ∆TIB and ∆RIB values.

- For spurious emission band UE co-existence, no requirements for FR2 NR bands to protect E-UTRA and FR1 NR bands are applied to the constituent FR2 NR bands. Spurious emission band UE co-existence requirements specified in TS 36.101 are applied to the constituent E-UTRA bands for the EN-DC/NE-DC configuration.

- For spurious emission band UE co-existence table, although the requirements are specified, it seems that they are not testable under the condition RAN4 originally expected. In addition, the impact of the harmonics and IMD issues between bands below 6GHz and bands above 24.25GHz inside UEs had been ignored. Hence at least the co-existence between bands below 6GHz and bands above 24.25GHz should not have any issues if the assumption that no impact of harmonic and IMD on co-existence between FR1 and FR2 is right.

- MSD for E-UTRA and FR2 NR bands of inter-band DC combinations is set to N/A.

- ΔTIB,c and ΔRIB,c for E-UTRA and FR2 NR bands of inter-band DC combinations are set to zero.

6	DC of 1 LTE band and 1 NR band: 
Specific Band Combination Part

6.1	Inter-band DC within FR1

6.1.1	DC_68_n1

6.1.1.1	Configuration for DC

Table 6.1.1.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_68A_n1A | DC_68A_n1A | No |

6.1.1.2	Maximum output power for DC

Table 6.1.1.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_68A_n1A | 23 | +2/-3 |

6.1.1.3	Spurious emission band UE co-existence for DC

According to TS 36.101, there is no requirement for B68 on specific UE protection in addition to a set of protected bands.  According to TS 38.101-1, band n1 has no regulatory emissions requirements so there is no need to include this combination in UE co-existence table.

6.1.1.4	Co-existence analysis for DC

Table 6.1.1.4-1 Band 68 and Band n1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | 68 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | flow | 698 | 1396 | 2094 | 2792 | 3490 |  |
| n1 | flow | fhigh | 728 | 1456 | 2184 | 2908 | 3606 |  |
| DL1 | 2110 | 2170 | N/A | - | D | - |  | UL harmonic |
| DL23 | 4220 | 4340 |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 |  |  | - | - | N/A | - | N/A |  |
| DL4 |  |  | - | N/A | N/A | N/A | N/A |  |
| DL54 |  |  | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | The 3rd harmonic of band 68 may fall into RX frequencies of band n1. |  |  |  |  |  |
| UL/DL harmonics |  | n1 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | flow | 1920 | 3840 | 5760 | 7680 | 9600 |  |
| 68 | flow | fhigh | 1980 | 3960 | 5940 | 7920 | 9900 |  |
| DL1 | 753 | 783 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1506 | 1566 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2259 | 2349 | - | - | N/A | - | N/A |  |
| DL4 | 3012 | 3132 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3765 | 3915 | - | D | N/A | N/A | N/A |  |
| Analysis |  |  | The 2nd harmonic mixing of band n1 may fall into 5th DL of band 68. |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range. NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only applies for DL>3GHz. NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 6.1.1.4-2: Band 68 and Band n1 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1192 – 1282 |  | 2618 – 2708 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 464 – 584 |  | 3112 – 3262 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3316 – 3436 |  | 4538 – 4688 |  |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 114 – 264 |  | 5032 – 5242 |  |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2384 – 2564 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4014 – 4164 |  | 6458 – 6668 |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5236 – 5416 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6952 – 7222 |  | 812 – 992 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4304 – 4544 |  | 1656 – 1866 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8378 – 8648 |  | 4712 – 4892 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7156 – 7396 |  | 5934 – 6144 |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.1.1.4-2, there is no IMD issue.

6.1.1.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_28_n1 as specified in TS 38.101-3.

Table 6.1.1.5-1: ΔTIB,c

| Inter-band DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n1 | 0.6 | 0.3 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

Table 6.1.1.5-2: ΔRIB,c

| Inter-band DC configuration | ΔRIB,c for NR band / E-UTRA band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n1 | 0.2 | - |
| NOTE 1: “-” denotes ΔRIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

6.1.1.6	Analysis of MSD requirements

The REFSENS requirement for DC_68_n1 due to UL harmonic are reused from DC_28_n1 and are shown in table 6.1.1.6-1:

Table 6.1.1.6-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 68 | n1 | 5 | 15 | 8 | 5 | 10.2 | NOTE 3 | UL3/DL1 direct-hit |

The coexistence study shows the possibility of 2nd harmonic mixing of band n1 may fall into 5th DL of band 68. However the impact of harmonic mixing is not considered severe therefor no MSD needs to be defined.

6.1.2	DC_68_n3

6.1.2.1	Configuration for DC

Table 6.1.2.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_68A_n3A | DC_68A_n3A | No |

6.1.2.2	Maximum output power for DC

Table 6.1.2.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_68A_n3A | 23 | +2/-3 |

6.1.2.3	Spurious emission band UE co-existence for DC

Table 6.1.2.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_68_n3 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

6.1.2.4	Co-existence analysis for DC

Table 6.1.2.4-1 Band 68 and Band n3 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | 68 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | flow | 698 | 1396 | 2094 | 2792 | 3490 |  |
| n3 | flow | fhigh | 728 | 1456 | 2184 | 2908 | 3606 |  |
| DL1 | 1805 | 1880 | N/A | - | - | - | - | UL harmonic |
| DL23 | 3620 | 3760 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 5427 | 5640 | - | - | N/A | - | N/A |  |
| DL4 | 7220 | 7520 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 9025 | 9400 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic impacts |  |  |  |  |  |
| UL/DL harmonics |  | n3 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | flow | 1710 | 3420 | 5130 | 6840 | 8550 |  |
| 68 | flow | fhigh | 1785 | 3570 | 5355 | 7140 | 8925 |  |
| DL1 | 753 | 783 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1506 | 1566 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2259 | 2349 | - | - | N/A | - | N/A |  |
| DL4 | 3012 | 3132 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3765 | 3915 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic mixing impact |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range. NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only applies for DL>3GHz. NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 6.1.2.4-2: Band 68 and Band n3 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 982 - 1076 |  | 2408 - 2513 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 254 - 389 |  | 2692 - 2872 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3106 - 3241 |  | 4118 - 4298 |  |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 309 - 474 |  | 4402 - 4657 |  |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1964 - 2174 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3804 - 3969 |  | 5828 6083 |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4816 - 5026 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6112 - 6442 |  | 1007 - 1202 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3674 - 3959 |  | 1236 - 1476 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7538 - 7868 |  | 4502 - 4697 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6526 - 6811 |  | 5514 - 5754 |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.1.2.4-2, there is no IMD issue for this combination.

6.1.2.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_28_n3 as specified in TS 38.101-3.

Table 6.1.2.5-1: ΔTIB,c

| Inter-band DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n3 | 0.3 | 0.3 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

Table 6.1.2.5-2: ΔRIB,c

| Inter-band DC configuration | ΔRIB,c for NR band / E-UTRA band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n3 | - | - |
| NOTE 1: “-” denotes ΔRIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

6.1.2.6	Analysis of MSD requirements

Based on the co-existence studies for DC_68_n3, no MSD requirements are needed for this combination.

6.1.3	DC_68_n7

6.1.3.1	Configuration for DC

Table 6.1.3.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_68A_n7A | DC_68A_n7A | No |

6.1.3.2	Maximum output power for DC

Table 6.1.3.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_68A_n7A | 23 | +2/-3 |

6.1.3.3	Spurious emission band UE co-existence for DC

According to TS 36.101, there is no requirement for B68 on specific UE protection in addition to a set of protected bands.  According to TS 38.101-1, band n7 has no regulatory emissions requirements so there is no need to include this combination in UE co-existence table.

6.1.3.4	Co-existence analysis for DC

Table 6.1.3.4-1 Band 68 and Band n7 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | 68 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | flow | 698 | 1396 | 2094 | 2792 | 3490 |  |
| n7 | flow | fhigh | 728 | 1456 | 2184 | 2908 | 3606 |  |
| DL1 | 2620 | 2690 | N/A | - | - | - | - | UL harmonic |
| DL23 | 5240 | 5380 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 7860 | 8070 | - | - | N/A | - | N/A |  |
| DL4 | 10480 | 10760 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 13100 | 13450 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic impacts |  |  |  |  |  |
| UL/DL harmonics |  | n7 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | flow | 2500 | 5000 | 7500 | 10000 | 12500 |  |
| 68 | flow | fhigh | 2570 | 5140 | 7710 | 10280 | 12850 |  |
| DL1 | 753 | 783 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1506 | 1566 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2259 | 2349 | - | - | N/A | - | N/A |  |
| DL4 | 3012 | 3132 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3765 | 3915 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic mixing impact |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range. NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only applies for DL>3GHz. NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 6.1.3.4-2: Band 68 and Band n7 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1772 - 1872 |  | 3198 - 3298 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1044 - 1174 |  | 4272 - 4442 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3896 - 4026 |  | 5698 - 5868 |  |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 316 - 476 |  | 6772 - 7012 |  |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3744 - 3544 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4594 - 4754 |  | 8198 - 8438 |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6396 - 6596 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9272 - 9582 |  | 222 - 412 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6044 - 6314 |  | 2816 - 3046 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10698 - 11008 |  | 5482 - 5292 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8896 - 9166 |  | 7094 - 7324 |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.1.3.4-2, there is no IMD issue for this combination.

6.1.3.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_28_n7 as specified in TS 38.101-3.

Table 6.1.3.5-1: ΔTIB,c

| Inter-band DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n7 | 0.3 | 0.3 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

Table 6.1.3.5-2: ΔRIB,c

| Inter-band DC configuration | ΔRIB,c for NR band / E-UTRA band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n7 | - | - |
| NOTE 1: “-” denotes ΔRIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

6.1.3.6	Analysis of MSD requirements

Based on the co-existence studies for DC_68_n7, no MSD requirements are needed for this combination.

6.1.4	DC_68_n8

6.1.4.1	Configuration for DC

Table 6.1.4.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_68A_n8A | DC_68A_n8A | No |

6.1.4.2	Maximum output power for DC

Table 6.1.4.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_68A_n8A | 23 | +2/-3 |

6.1.4.3	Spurious emission band UE co-existence for DC

Table 6.1.4.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_68_n8 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz |  |  |  |  |  |  |  |

6.1.4.4	Co-existence analysis for DC

Table 6.1.4.4-1 Band 68 and Band n8 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | 68 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | flow | 698 | 1396 | 2094 | 2792 | 3490 |  |
| n8 | flow | fhigh | 728 | 1456 | 2184 | 2908 | 3606 |  |
| DL1 | 925 | 960 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1850 | 1920 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2775 | 2880 | - | - | N/A | - | N/A |  |
| DL4 | 3700 | 3840 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 4625 | 4800 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic impacts |  |  |  |  |  |
| UL/DL harmonics |  | n8 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | flow | 880 | 1760 | 2640 | 3250 | 4400 |  |
| 68 | flow | fhigh | 915 | 1830 | 2745 | 3660 | 4575 |  |
| DL1 | 753 | 783 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1506 | 1566 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2259 | 2349 | - | - | N/A | - | N/A |  |
| DL4 | 3012 | 3132 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3765 | 3915 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic mixing impact |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range. NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only applies for DL>3GHz. NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 6.1.4.4-2: Band 68 and Band n8 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 152 - 217 |  | 1578 - 1643 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 481 - 576 |  | 1032 - 1132 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2276 - 2371 |  | 2458 - 2558 |  |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1179 - 1304 |  | 1912 - 2047 |  |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 304 - 434 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2974 - 3099 |  | 3338 - 3473 |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 3156 - 3286 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2792 - 2962 |  | 1877 - 2032 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1184 - 1349 |  | 264 - 424 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4218 - 4388 |  | 3672 - 3827 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4036 - 4201 |  | 3854 - 4014 |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.1.4.4-3, there is no IMD issue for this combination.

6.1.4.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_28_n8 as specified in TS 38.101-3.

Table 6.1.4.5-1: ΔTIB,c

| Inter-band DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n8 | 0.5 | 0.6 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

Table 6.1.4.5-2: ΔRIB,c

| Inter-band DC configuration | ΔRIB,c for NR band / E-UTRA band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n8 | 0.1 | 0.2 |
| NOTE 1: “-” denotes ΔRIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

6.1.4.6	Analysis of MSD requirements

Based on the co-existence studies for DC_68_n8, no MSD requirements are needed for this combination.

6.1.5	DC_68_n20

6.1.5.1	Configuration for DC

Table 6.1.5.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_68A_n20A | DC_68A_n20A | No |

6.1.5.2	Maximum output power for DC

Table 6.1.5.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_68A_n20A | 23 | +2/-3 |

6.1.5.3	Spurious emission band UE co-existence for DC

Table 6.1.5.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_68_n20 | Frequency range | 758 | - | 788 | -50 | 1 |  |

6.1.5.4	Co-existence analysis for DC

Table 6.1.5.4-1 Band 68 and Band n20 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | 68 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | flow | 698 | 1396 | 2094 | 2792 | 3490 |  |
| n20 | flow | fhigh | 728 | 1456 | 2184 | 2908 | 3606 |  |
| DL1 | 791 | 821 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1582 | 1642 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2373 | 2463 | - | - | N/A | - | N/A |  |
| DL4 | 3164 | 3284 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3955 | 4105 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic impacts |  |  |  |  |  |
| UL/DL harmonics |  | n20 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | flow | 832 | 1664 | 2496 | 3228 | 4160 |  |
| 68 | flow | fhigh | 862 | 1724 | 2586 | 3448 | 4310 |  |
| DL1 | 753 | 783 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1506 | 1566 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2259 | 2349 | - | - | N/A | - | N/A |  |
| DL4 | 3012 | 3132 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3765 | 3915 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic mixing impact |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range. NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only applies for DL>3GHz. NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 6.1.5.4-2: Band X and Band nY cross-band isolation analysis

| Band | 68 |  | n20 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fxlow | fxhigh | fylow | fyhigh |
| fUL (MHz) | 698 | 728 | 832 | 862 |
| fDL (MHz) | 753 | 783 | 791 | 821 |
| UL CBW (MHz)2 | Minimum CBWx | Maximum CBWx | Minimum CBWy | Maximum CBWy |
|  | 5 | 15 | 5 | 20 |
| ACLR1 range | fxUL_low-maxULCBWx | fxUL_high+maxULCBWx | fyUL_low-maxULCBWy | fyUL_high+maxULCBWy |
| ACLR1 (MHz) | 693 | 743 | 827 | 882 |
| ACLR2 range | fxUL_low-2*maxULCBWx | fxUL_high+2*maxULCBWx | fyUL_low-2*maxULCBWy | fyUL_high+2*maxULCBWy |
| ACLR2 (MHz) | 688 | 758 | 822 | 902 |
| ACLR3 range | fxUL_low-3*maxULCBWx | fxUL_high+3*maxULCBWx | fyUL_low-3*maxULCBWy | fyUL_high+3*maxULCBWy |
| ACLR3 (MHz) | 683 | 773 | 817 | 922 |
| ACLR4 range | fxUL_low-4*maxULCBWx | fxUL_high+4*maxULCBWx | fyUL_low-4*maxULCBWy | fyUL_high+4*maxULCBWy |
| ACLR4 (MHz) | 678 | 788 | 812 | 942 |
| ACLR5 range1 | fxUL_low-5*maxULCBWx | fxUL_high+5*maxULCBWx | fyUL_low-5*maxULCBWy | fyUL_high+5*maxULCBWy |
| ACLR5 (MHz) | 673 | 803 | 807 | 962 |
| Analysis | ACLR5 of UL band 68 can impact DL n20 |  | No cross-band isolation issue |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1-1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible. NOTE 2: The maximum UL channel bandwidth of the band (noted maxULCBW) is used to calculate the ACLR ranges. |  |  |  |  |

Table 6.1.5.4-3: Band 68 and Band n20 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 104 - 164 |  | 1530 - 1590 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 534 - 624 |  | 936 - 1026 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2228 - 2318 |  | 2362 - 2452 |  |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1232 - 1352 |  | 1768 - 1888 |  |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 208 - 328 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2926 - 3046 |  | 3194 - 3314 |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 3060 - 3180 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2600 - 2750 |  | 1930 - 2080 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1040 - 1190 |  | 370 - 520 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4026 - 4176 |  | 3624 - 3774 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3892 - 4042 |  | 3758 - 3908 |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.1.5.4-3, there is no IMD issue for this combination.

6.1.5.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_28_n20 as specified in TS 38.101-3.

Table 6.1.5.5-1: ΔTIB,c

| Inter-band DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n20 | 0.5 | 0.5 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

Table 6.1.5.5-2: ΔRIB,c

| Inter-band DC configuration | ΔRIB,c for NR band / E-UTRA band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n20 | - | - |
| NOTE 1: “-” denotes ΔRIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

6.1.5.6	Analysis of MSD requirements

Based on the co-existence studies for DC_68_n20, MSD requirements for cross band isolation. The MSD value for DC_28_n105 has been reused.

Table 6.1.5.6-1: Reference sensitivity exceptions (MSD) due to cross band isolation for EN-DC in NR FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| 68 | n20 | 720.5 | 15 | 15 | 25 (RBstart=50) | 793.5 | 5 | 5.7 | >ACLR2 |

6.1.6	DC_68_n38

6.1.6.1	Configuration for DC

Table 6.1.6.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_68A_n38A | DC_68A_n38A | No |

6.1.6.2	Maximum output power for DC

Table 6.1.6.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_68A_n38A | 23 | +2/-3 |

6.1.6.3	Spurious emission band UE co-existence for DC

Table 6.1.6.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_68_n38 | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 5, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 5, 22 |
| NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 24: This requirement is applicable for power class 3 UE for any channel bandwidths up to 20 MHz. For channel bandwidth within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when the carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when the carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For carriers overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 25: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

6.1.6.4	Co-existence analysis for DC

Table 6.1.6.4-1 Band 68 and Band n38 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | 68 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | flow | 698 | 1396 | 2094 | 2792 | 3490 |  |
| n38 | flow | fhigh | 728 | 1456 | 2184 | 2908 | 3606 |  |
| DL1 | 2570 | 2620 | N/A | - | - | - | - | UL harmonic |
| DL23 | 5140 | 5240 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 7710 | 7860 | - | - | N/A | - | N/A |  |
| DL4 | 10280 | 10480 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 12850 | 13100 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic impacts |  |  |  |  |  |
| UL/DL harmonics |  | n38 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | flow | 2570 | 5140 | 7710 | 10280 | 12850 |  |
| 68 | flow | fhigh | 2620 | 5240 | 7860 | 10480 | 13100 |  |
| DL1 | 753 | 783 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1506 | 1566 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2259 | 2349 | - | - | N/A | - | N/A |  |
| DL4 | 3012 | 3132 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3765 | 3915 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic mixing impact |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range. NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only applies for DL>3GHz. NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 6.1.6.4-2: Band 68 and Band n38 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1842 - 1922 |  | 3268 - 3348 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1114 - 1224 |  | 4412 - 4542 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3966 -4076 |  | 5838 - 5968 |  |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 386 - 526 |  | 6982 - 7162 |  |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3684 - 3844 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4664 - 4804 |  | 8408 - 8588 |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6536 - 6696 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9782 - 9552 |  | 172 - 342 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 10978 – 11208 |  | 5362 - 5532 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6254 - 6464 |  | 2956 - 3146 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9106 - 9316 |  | 7234 - 7424 |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.1.6.4-2, there is no IMD issue for this combination.

6.1.6.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_28_n38 as specified in TS 38.101-3.

Table 6.1.6.5-1: ΔTIB,c

| Inter-band DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n38 | 0.3 | 0.3 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

Table 6.1.6.5-2: ΔRIB,c

| Inter-band DC configuration | ΔRIB,c for NR band / E-UTRA band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n38 | - | - |
| NOTE 1: “-” denotes ΔRIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

6.1.6.6	Analysis of MSD requirements

Based on the co-existence studies for DC_68_n38, no MSD requirements are needed for this combination.

6.1.7	DC_68_n40

6.1.7.1	Configuration for DC

Table 6.1.7.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_68A_n40A | DC_68A_n40A | No |

6.1.7.2	Maximum output power for DC

Table 6.1.7.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_68A_n40A | 23 | +2/-3 |

6.1.7.3	Spurious emission band UE co-existence for DC

Table 6.1.7.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_68_n40 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz |  |  |  |  |  |  |  |

6.1.7.4	Co-existence analysis for DC

Table 6.1.7.4-1 Band 68 and Band n40 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | 68 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | flow | 698 | 1396 | 2094 | 2 792 | 3 490 |  |
| n40 | flow | fhigh | 728 | 1456 | 2184 | 2908 | 3 606 |  |
| DL1 | 2300 | 2400 | N/A | - | - | - | - | UL harmonic |
| DL23 | 4600 | 4800 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 6900 | 7200 | - | - | N/A | - | N/A |  |
| DL4 | 9200 | 9600 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 11500 | 12000 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic impacts |  |  |  |  |  |
| UL/DL harmonics |  | n40 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | flow | 2300 | 4600 | 6900 | 9200 | 11500 |  |
| 68 | flow | fhigh | 2400 | 4800 | 7200 | 9600 | 12000 |  |
| DL1 | 753 | 783 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1506 | 1566 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2259 | 2349 | D | - | N/A | - | N/A |  |
| DL4 | 3012 | 3132 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3765 | 3915 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | The 3rd harmonic mixing of band n40 may fall into RX frequencies of band 68. |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range. NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only applies for DL>3GHz. NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 6.1.7.4-2: Band 68 and Band n40 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1572 - 1702 |  | 2998 - 3128 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 844 - 1004 |  | 3872 - 4102 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3696 - 3856 |  | 5298 - 5528 |  |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 116 - 306 |  | 6172 - 6502 |  |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3144 - 3404 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4394 - 4584 |  | 7598 - 7928 |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5996 - 6256 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8472 - 8902 |  | 392 - 612 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5444 - 5804 |  | 2416 - 2706 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9898 - 10328 |  | 5092 - 5312 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8296 - 8656 |  | 6694 - 6984 |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.1.7.4-2, there is no IMD issue for this combination.

6.1.7.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_28_n40 as specified in TS 38.101-3.

Table 6.1.7.5-1: ΔTIB,c

| Inter-band DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n40 | 0.3 | 0.3 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

Table 6.1.7.5-2: ΔRIB,c

| Inter-band DC configuration | ΔRIB,c for NR band / E-UTRA band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n40 | - | - |
| NOTE 1: “-” denotes ΔRIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

6.1.7.6	Analysis of MSD requirements

The REFSENS requirements for DC_68_n40 due to UL harmonic are reused from DC_28_n40 and are shown in table 6.1.7.6-1:

Table 6.1.7.6-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n40 | 68 | 10 | 15 | 25 | 5 | 37.8 | NOTE 4 | UL1/DL3 |

6.1.8	DC_68_n41

6.1.8.1	Configuration for DC

Table 6.1.8.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_68A_n41A | DC_68A_n41A | No |

6.1.8.2	Maximum output power for DC

Table 6.1.8.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_68A_n41A | 23 | +2/-3 |

6.1.8.3	Spurious emission band UE co-existence for DC

Table 6.1.8.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_68_n41 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2530 | - | 2535 | -25 | 1 | 23 |
|  | Frequency range | 2505 | - | 2530 | -30 | 1 | 23 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 23: Applicable when contained within 2545 – 2575 MHz in Japan. Channel bandwidth shall be confined so that there is at least BWChannel separation between 2535 MHz and lower BWChannel edge in the current release. With this BWChannel placement the requirement is covered by general SEM and the spurious emission limits. |  |  |  |  |  |  |  |

6.1.8.4	Co-existence analysis for DC

Table 6.1.8.4-1 Band 68 and Band n41 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | 68 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | flow | 698 | 1396 | 2094 | 2792 | 3490 |  |
| n41 | flow | fhigh | 728 | 1456 | 2184 | 2908 | 3606 |  |
| DL1 | 2496 | 2690 | N/A | - | - | - | - | UL harmonic |
| DL23 | 4992 | 5380 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 7488 | 8070 | - | - | N/A | - | N/A |  |
| DL4 | 9984 | 10760 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 12480 | 13450 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic impacts |  |  |  |  |  |
| UL/DL harmonics |  | n41 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | flow | 2496 | 4992 | 7488 | 9984 | 12480 |  |
| 68 | flow | fhigh | 2690 | 5380 | 8070 | 10760 | 13450 |  |
| DL1 | 753 | 783 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1506 | 1566 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2259 | 2349 | - | - | N/A | - | N/A |  |
| DL4 | 3012 | 3132 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3765 | 3915 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic mixing impacts |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range. NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only applies for DL>3GHz. NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 6.1.8.4-2: Band 68 and Band n41 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1768 - 1992 |  | 3194 - 3418 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1040 -1294 |  | 4264 - 4682 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3892 - 4146 |  | 5690 - 6108 |  |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4590 - 4874 |  | 8186 - 8798 |  |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3536 -3984 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 312 - 596 |  | 6760 - 7372 |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6388 - 6836 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9256 - 10062 |  | 102 - 416 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 10682 - 11488 |  | 5288 - 5602 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6032 - 6674 |  | 2808 - 3286 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8884 - 9526 |  | 7086 - 7564 |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.1.8.4-2, there is no IMD issue for this combination.

6.1.8.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_28_n41 as specified in TS 38.101-3.

Table 6.1.8.5-1: ΔTIB,c

| Inter-band DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n41 | 0.3 | 0.3 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

Table 6.1.8.5-2: ΔRIB,c

| Inter-band DC configuration | ΔRIB,c for NR band / E-UTRA band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n41 | - | - |
| NOTE 1: “-” denotes ΔRIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

6.1.8.6	Analysis of MSD requirements

Based on analysis in section 6.1.8.4, there are no MSD requirements for this combination.

6.1.9	DC_68_n77

6.1.9.1	Configuration for DC

Table 6.1.9.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_68A_n77A | DC_68A_n77A | No |

6.1.9.2	Maximum output power for DC

Table 6.1.9.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_68A_n77A | 23 | +2/-3 |

6.1.9.3	Spurious emission band UE co-existence for DC

Table 6.1.9.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_68_n77 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz |  |  |  |  |  |  |  |

6.1.9.4	Co-existence analysis for DC

Table 6.1.9.4-1 Band 68 and Band n77 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | 68 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | flow | 698 | 1396 | 2094 | 2792 | 3490 |  |
| n77 | flow | fhigh | 728 | 1456 | 2184 | 2908 | 3606 |  |
| DL1 | 3300 | 4200 | N/A | - | - | - | D | UL harmonic |
| DL23 | 6600 | 8400 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 9900 | 12600 | - | - | N/A | - | N/A |  |
| DL4 | 13200 | 16800 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 16500 | 21000 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | The 5rd harmonic of band 68 may fall into RX frequencies of band n77. |  |  |  |  |  |
| UL/DL harmonics |  | n77 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | flow | 3300 | 6600 | 9900 | 13200 | 16500 |  |
| 68 | flow | fhigh | 4200 | 8400 | 12600 | 16800 | 21000 |  |
| DL1 | 753 | 783 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1506 | 1566 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2259 | 2349 | - | - | N/A | - | N/A |  |
| DL4 | 3012 | 3132 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3765 | 3915 | D | - | N/A | N/A | N/A |  |
| Analysis |  |  | 5th harmonic mixing due to UL1 of band n77 |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range. NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only applies for DL>3GHz. NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 6.1.9.4-2: Band 68 and Band n77 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2572 - 3502 |  | 3998 - 4928 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1844 - 2804 |  | 5872 - 7702 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4696 - 5656 |  | 7298 - 9128 |  |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1116 - 2106 |  | 9172 - 11902 |  |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 5144 - 7004 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5394 - 6384 |  | 10598 - 13328 |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7996 - 9856 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12472 - 16102 |  | 388 -1408 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8444 -11204 |  | 4416 - 6306 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13898 - 17528 |  | 6092 - 7112 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11296 - 14056 |  | 8694 - 10584 |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.1.9.4-2, the 5th order IMD products may fall in the own Rx of Band 68.

6.1.9.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_28_n77 as specified in TS 38.101-3.

Table 6.1.9.5-1: ΔTIB,c

| Inter-band DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n77 | 0.5 | 0.8 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

Table 6.1.9.5-2: ΔRIB,c

| Inter-band DC configuration | ΔRIB,c for NR band / E-UTRA band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n77 | 0.2 | 0.5 |
| NOTE 1: “-” denotes ΔRIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

6.1.9.6	Analysis of MSD requirements

The REFSENS requirement for DC_68_n77 due to UL harmonic and harmonic mixing and MSD requirement due to 5th order IMD are reused from DC_28_n77 and are shown in tables below:

Table 6.1.9.6-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 68 | n77 | 5 | 15 | 5 | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |

Table 6.1.9.6-2: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | 68 | 10 | 15 | 25 | 5 | 31 | NOTE 2 | UL1/DL5 |

Table 6.1.9.6-3: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_68A_n77A | 68 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |

6.1.10	DC_68_n78

6.1.10.1	Configuration for DC

Table 6.1.10.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_68A_n78A | DC_68A_n78A | No |

6.1.10.2	Maximum output power for DC

Table 6.1.10.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_68A_n78A | 23 | +2/-3 |

6.1.10.3	Spurious emission band UE co-existence for DC

Table 6.1.10.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_68_n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz |  |  |  |  |  |  |  |

6.1.10.4	Co-existence analysis for DC

Table 6.1.10.4-1 Band 68 and Band n78 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | 68 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | flow | 698 | 1396 | 2094 | 2792 | 3490 |  |
| n78 | flow | fhigh | 728 | 1456 | 2184 | 2908 | 3606 |  |
| DL1 | 3300 | 3800 | N/A | - | - | - | D | UL harmonic |
| DL23 | 6600 | 7600 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 9900 | 11400 | - | - | N/A | - | N/A |  |
| DL4 | 13200 | 15200 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 16500 | 19000 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | The 5rd harmonic of band 68 may fall into RX frequencies of band n78. |  |  |  |  |  |
| UL/DL harmonics |  | n78 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | flow | 3300 | 6600 | 9900 | 13200 | 16500 |  |
| 68 | flow | fhigh | 3800 | 7600 | 11400 | 15200 | 19000 |  |
| DL1 | 753 | 783 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1506 | 1566 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2259 | 2349 | - | - | N/A | - | N/A |  |
| DL4 | 3012 | 3132 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3765 | 3915 | D | - | N/A | N/A | N/A |  |
| Analysis |  |  | 5th harmonic mixing due to UL1 of band n78 |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range. NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only applies for DL>3GHz. NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 6.1.10.4-2 Band 68 and Band n78 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2572 - 3102 |  | 3998 - 4528 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1844 - 2404 |  | 5872 - 6902 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4696 - 5256 |  | 7298 - 8328 |  |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1116 - 1706 |  | 9172 - 10702 |  |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 5144 - 6204 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5394 - 5984 |  | 10598 - 12128 |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7996 - 9056 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12472 - 14502 |  | 388 - 1008 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 13898 - 15928 |  | 6092 6712 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8444 - 10004 |  | 4416 - 5506 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11296 - 12856 |  | 8694 - 9784 |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.1.10.4-2, the 5th order IMD products may fall in the own Rx of Band 68.

6.1.10.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_28_n78 as specified in TS 38.101-3.

Table 6.1.10.5-1: ΔTIB,c

| Inter-band DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n78 | 0.5 | 0.8 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

Table 6.1.10.5-2: ΔRIB,c

| Inter-band DC configuration | ΔRIB,c for NR band / E-UTRA band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n78 | 0.2 | 0.5 |
| NOTE 1: “-” denotes ΔRIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

6.1.10.6	Analysis of MSD requirements

The REFSENS requirement for DC_68_n78 due to UL harmonic and harmonic mixing and MSD requirement due to 5th order IMD are reused from DC_28_n78 and are shown in tables below:

Table 6.1.10.6-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 68 | n78 | 5 | 15 | 5 | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |

Table 6.1.10.6-2: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | 68 | 10 | 15 | 25 | 5 | 31 | NOTE 2 | UL1/DL5 |

Table 6.1.10.6-3: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_68A_n78A | 68 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 |
|  | n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |

6.1.11	DC_68_n79

6.1.11.1	Configuration for DC

Table 6.1.11.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_68A_n79A | DC_68A_n79A | No |

6.1.11.2	Maximum output power for DC

Table 6.1.11.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_68A_n79A | 23 | +2/-3 |

6.1.11.3	Spurious emission band UE co-existence for DC

Table 6.1.11.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_68_n79 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz |  |  |  |  |  |  |  |

6.1.11.4	Co-existence analysis for DC

Table 6.1.11.4-1 Band 68 and Band n79 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | 68 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | flow | 698 | 1396 | 2094 | 2792 | 3490 |  |
| n79 | flow | fhigh | 728 | 1456 | 2184 | 2908 | 3606 |  |
| DL1 | 4400 | 5000 | N/A | - | - | - | - | UL harmonic |
| DL23 | 8800 | 10000 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 13200 | 15000 | - | - | N/A | - | N/A |  |
| DL4 | 17600 | 20000 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 22000 | 25000 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic impact |  |  |  |  |  |
| UL/DL harmonics |  | n79 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | flow | 4400 | 8800 | 13200 | 17600 | 22000 |  |
| 68 | flow | fhigh | 5000 | 10000 | 15000 | 20000 | 25000 |  |
| DL1 | 753 | 783 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1506 | 1566 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2259 | 2349 | - | - | N/A | - | N/A |  |
| DL4 | 3012 | 3132 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3765 | 3915 | . | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic mixing impact |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range. NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only applies for DL>3GHz. NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 6.1.11.4-2: Band 68 and Band n79 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3672 - 4302 |  | 5098 - 5728 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2944 - 3604 |  | 8072 - 9302 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5796 - 6456 |  | 9498 - 10728 |  |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2216 - 2906 |  | 12472 - 14302 |  |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 7344 - 8604 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6494 - 7184 |  | 13898 - 15728 |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10196 - 11456 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16872 - 19302 |  | 1488 - 2208 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 18298 - 20728 |  | 7192 - 7912 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11744 - 13604 |  | 6616 - 7906 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14596 - 16456 |  | 10894 - 12184 |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.1.11.4-2 there is no IMD issue for this combination.

6.1.11.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_28_n79 as specified in TS 38.101-3.

Table 6.1.11.5-1: ΔTIB,c

| Inter-band DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n79 | - | - |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

Table 6.1.11.5-2: ΔRIB,c

| Inter-band DC configuration | ΔRIB,c for NR band / E-UTRA band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n79 | - | - |
| NOTE 1: “-” denotes ΔRIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

6.1.11.6	Analysis of MSD requirements

Based on the co-existence studies for DC_68_n3, no MSD requirements are needed for this combination.

6.2	Inter-band DC within FR2

< Editor’s note: Note that based on the agreement in RAN4#95-e (R4-2008931), it is allowed to propose configurations for EN-DC including FR2 with one draft CR. >

6.2.1	DC_X_nY

6.2.1.1	Configuration for DC

Table 6.2.1.1-1:  Inter-band EN-DC configurations including FR2 (two bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_XA_nYA | DC_XA_nYA |

6.2.1.2	Spurious emission band UE co-existence for DC

< Editor’s note: this section is optional. >

6.2.1.3	MSD analysis for DC

< Editor’s note: this section is optional. >

6.2.1.4	∆TIB and ∆RIB values

< Editor’s note: this section is optional. >

6.3	Intra-band contiguous DC

6.3.1	DC_(n)X

6.3.1.1	Channel bandwidths per operating band for DC

Table 6.3.1.1-1: Supported channel bandwidths per DC configuration

|  |  | E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | BCS |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths NR for carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_(n)XAA | DC_(n)XAA |  |  |  |  |  |
| NOTE X: |  |  |  |  |  |  |

6.3.1.2	Configuration for DC

Table 6.3.1.1-2:  Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_(n)XAA | DC_(n)XAA |  |
| NOTE X: |  |  |

< Editor’s note: Please use the same NOTE numbering as in TS 38.101-3.>

#### 6.3.1.3	Maximum output power for DC

Table 6.3.1.3-1: Maximum output power for EN-DC (continuous sub-blocks)

| EN-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_(n)XAA1 | 23 | +2/-3 |
| NOTE X: |  |  |

#### 6.3.1.4	Spurious emission band UE co-existence for DC

<Text will be added.>

6.3.1.5	Analysis of MSD requirements

<Text will be added.>

#### 6.3.1.6	MPR, A-MPR requirements

<Text will be added.>

6.4	Intra-band non-contiguous DC

6.4.1	DC_X_nX

#### 6.4.1.1	Channel bandwidths per operating band for DC

Table 6.4.1.1-1: Supported channel bandwidths per DC configuration

|  |  | E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | BCS |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths NR for carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_XA_nXA | DC_XA_nXA |  |  |  |  |  |
| NOTE X: |  |  |  |  |  |  |

< Editor’s note: Please use the same NOTE numbering as in TS 38.101-3.>

#### 6.4.1.2	Configuration for DC

Table 6.4.1.2-1:  Intra-band non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_XA_nXA | DC_XA_nXA |  |
| NOTE X: |  |  |

< Editor’s note: Please use the same NOTE numbering as in TS 38.101-3.>

#### 6.4.1.3	Maximum output power for DC

Table 6.4.1.3-1: Maximum output power for EN-DC (non-continuous sub-blocks)

| EN-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_XA_nXA | 23 | +2/-3 |
| NOTE X: |  |  |

< Editor’s note: Please use the same NOTE numbering as in TS 38.101-3.>

#### 6.4.1.4	Spurious emission band UE co-existence for DC

<Text will be added.>

6.4.1.5	Analysis of MSD requirements

<Text will be added.>

6.4.1.6	∆RIBNC values

<Text will be added.>

#### 6.4.1.7	MPR, A-MPR requirements

<Text will be added.>

Annex A:
Additional guidelines

A.1	Band group definition

For the following MSD issues detection, a criterion applies in terms of band group as defined in Table A.1-1.

-   For the cross-band isolation, even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if the UL aggressor band and DL aggressor band are part of the same or adjacent band group.

-   For the 3CC UL IMD products, if the two bands are not part of the same or adjacent band groups, the analysis can be ignored.

Table A.1-1: Band group definition for same or adjacent band-group criterion

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-a | FR1-b | FR1-c | FR1-d | FR1-e |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5150-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

Annex B (informative): 
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | RAN4-112 | R4-2412116 |  |  |  | Initial TR skeleton | 0.0.0 |
| 2024-08 | RAN4-112 | R4-2412157 |  |  |  | Some update in section 5.1 & 6.1 based on companies’ comment. | 0.1.0 |
| 2024-11 | RAN4-113 | R4-2418686 |  |  |  | included TPs/pCRs: DC_68A_n1A DC_68A_n3A DC_68A_n7A DC_68A_n8A DC_68A_n20A DC_68A_n38A DC_68A_n40A DC_68A_n41A DC_68A_n77A DC_68A_n78A DC_68A_n79A | 0.2.0 |