3GPP TSG-RAN WG4 Meeting #114                                                                                             R4-2501003

Athens, GR, Feb. 17 – Feb. 21, 2025

Source: 	CATT

Title: 	Discussion on RF requirements for NTN RedCap UE

Agenda item:	7.29.2.1

Document for:	Approval

- Introduction

In RAN4 #113 meeting, A WF [1] on NR NTN phase 3 UE RF requirements was agreed. However, there are still some remaining issues about NR NTN UE requirements need to be discussed in the next meeting.

In this contribution, we would like to share our views on NR NTN (e) RedCap UE requirements.

- Discussion

The agreement on NTN RedCap UE RF requirements was achieved during the previous meetings, as shown below:

## Issue 1-1: PC3 output power for (e)RedCap Agreement: - FFS whether and how a potential new scheme to increase output power is addressed in specifications - Alt 1:Further confirm whether rel-18 power boost is already applicable for NR NTN including (e)RedCap - If confirmed, no further work is needed. - Alt 2: Power increase by network controlled optional new signalling (e.g capability) - Alt 3: Change in power tolerances - E.g. Upper tolerance exception to UE in specific regions via table note. - FFS whether solution is common for HD-FDD and FD-FDD - If no solution agreed, PC3 tolerance will remain unchanged to +/- 2 dB. ## Issue 1-2: HD-FDD refsens for 2 Rx Agreement: Further discussion on potential improvement beyond TN HD-FDD 2 RX REFSENS level is limited to maximum 0.2 dB improvement based on average of inputs to RAN4. ## Issue 1-3: Simultaneous operation with GNSS Agreement: Simultaneous GNSS and NR-NTN operation for (e)Redcap cannot be guaranteed in some occasions for some NTN bands for some specific UE implementations. Send LS to RAN2 requesting common IDC solution for (e)RedCap NR NTN and non-RedCap NR NTN which further improves the existing IDC framework to be more suitable for NR NTN – GNSS co-existence. Final solution is up to RAN2. ## Issue 2-1: UL capacity enhancements Agreement: No RF impact on inter-slot OCC over 2 slots.

In the previous meeting, RAN4 has discussed how to increase the Tx power of NTN RedCap UE and several alternative approaches proposed, such as power boosting / new capability / new TT. For alt 1 Rel-18 power boosting, this seems reasonable as the power boosting is an existing feature that has already been introduced in Rel-18 and does not have an additional impact. Meanwhile, an appropriate increase in the Tx power of (e)RedCap HD-FDD NTN UE would be beneficial, so power boosting could be a potential solution.

Observation 1: The power boosting feature could be a potential solution for increasing (e)RedCap HD-FDD NTN UE Tx power.

For alt 3 increasing the upper test tolerance, since the test tolerance takes into account manufacturing tolerances, temperature and other factors that may have an effect on the transmitter power. A margin of 2 dB already introduces a large fluctuation in the transmitter power of the UE, and this margin is mainly defined for the conformance test procedure of the UE.

Meanwhile, the maximum transmission power of PC3 is already close to PC2 (after considering tolerance), which means that further relaxation may cause the NTN (e)RedCap UE to encounter the problems faced by PC2. For instance, additional SAR limit would require the introduction of additional signalling, and higher Tx power may cause some coexistence issues. Therefore, we believe that the test tolerance value does not need any additional modification.

Proposal 1: The 2 dB test tolerance for PC3 could be considered for non-RedCap HD-FDD NTN UE and (e)RedCap HD-FDD NTN UE.

Considering that the simultaneous GNSS operation has reached an agreement in previous meeting, we believed that NTN RedCap UE can correctly obtain the GNSS information and correctly perform pre-compensation now. Therefore, the frequency error requirements of NR RedCap UE could be reused for NTN RedCap UE.

Proposal 2: The NR UE frequency error requirements could be reused for NTN (e)RedCap UE.

- Conclusion

This contribution provides our views on NR NTN (e) RedCap UE requirements. The following proposals are concluded as follows:

Observation 1: The power boosting feature could be a potential solution for increasing (e)RedCap HD-FDD NTN UE Tx power.

Proposal 1: The 2 dB test tolerance for PC3 could be considered for non-RedCap HD-FDD NTN UE and (e)RedCap HD-FDD NTN UE.

Proposal 2: The NR UE frequency error requirements could be reused for NTN (e)RedCap UE.

- References

- R4-2419897, Way Forward for [113][310] NR_NTN_Ph3_UE_RF, Qualcomm, RAN4# 113.