3GPP TSG-RAN WG4 Meeting #114	R4-2501016

# Athens , GR, 17th – 21st Feb, 2025

Title: 	Discussion on enhancements for Rel-19 ATG RRM requirements

Source: 	CATT

Agenda item:	7.12.3

Document for:	Discussion

- Introduction

In last RAN4 meeting, RRM requirements for Rel-19 ATG enhancement were discussed. The conclusions for ATG RRM requirements for CA were captured in the approved WF [1] and the open issues can be found in the summary [2]. In this paper, we further discuss the potential RRM impact for Rel-19 ATG RRM requirements and give our views.

- Discussion

  - 2.1 Timing

- MRTD for ATG inter-band CA

In the last meeting, RAN4 discussed how to define the MRTD requirements for ATG FR1 inter-band CA, and there was still controversy about the value of the MRTD. The related WF is copied as below [1]:

Issue 1-2-1: MRTD for ATG inter-band CA FFS - Option 1: - For the MRTD except Case#4: 33µs. Further discuss for case 4. - Option 2: - For the MRTD except Case#4: 33µs. - If case 4 is not excluded based on RF discussion: 3µs. - Option 3: - For the MRTD for all cases: 33µs.

For inter-band CA, 33us was defined for MRTD requirements in FR1 CA [4], and it included TAE (3us) and the propagation delay (30us) in FR1. For ATG FR1 inter-band CA, since only co-location scenario is considered, so the propagation delay can be 0, so technically speaking 3us may be enough.

However, since the normal UE can handle 33us MRTD in FR1 CA, considering that ATG UE has stronger capabilities, it is reasonable for ATG UE to handle 33us MRTD in FR1 CA. In our view, if the type of ATG UE with an antenna array on the bands of PCC and SCC and one common Rx beam assumed (Case#4) is excluded based on RF discussion, MRTD can be 33us for all types of ATG UEs.

If Case#4 is not excluded based on RF discussion, it can be considered in RRM requirements, and the extension to 33us MRTD may be challenging for Case#4, and RAN4 can further discuss the value. Therefore, we can accept defining different values for different types of ATG UE.

Observation 1: ATG UE has stronger capabilities compared to normal UEs.

Proposal 1: If the type of ATG UE with an antenna array on the bands of PCC and SCC and one common Rx beam assumed is excluded based on RF discussion, MRTD can be 33us for all types of ATG UEs.

Proposal 2: If the type of ATG UE with an antenna array on the bands of PCC and SCC and one common Rx beam assumed (Case#4) is not excluded based on RF discussion, RAN4 to define different values for different types of ATG UE,

- For the MRTD except Case#4: 33µs.

- If case 4 is not excluded based on RF discussion, RAN4 can further discuss the value of MRTD.

  - 2.2 Signalling characteristics

- SCell Addition/Activation mechanism

In the last meeting, the candidate proposal of SCell Addition/Activation mechanism is copied as below [1]:

Issue 1-3-3: SCell Addition/Activation mechanism FFS - Proposal 1: Define the corresponding enhanced requirements with some conditions, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted, and it depends on UE to apply these requirements. - Proposal 2: Further consider distance triggered SCell addition and/or activation, which can be achieved through network configuration of SCell addition and/or activation condition - Proposal 3: Should not consider distance triggered SCell addition and/or activation; existing activation methods are sufficient

Considering that the specific scenario of R19 ATG such as co-located CA and LOS propagation conditions, the further optimization methods for SCell addition and activation can be considered, as the coverage areas of PCell and SCell are relatively fixed and SCell quality is related to the distance between UE and the center of cell. Omitting the measurements of SCell addition will achieve throughput gains.

However, we think that introducing distance triggered SCell addition and/or activation is complex and it will also involve RAN2’s work, from RAN4’s perspective, a better way is to define the corresponding enhanced requirements with some conditions, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted, and it depends on UE to apply these requirements.

Observation 2: The coverage areas of PCell and SCell are relatively fixed and SCell quality is related to the distance between UE and the center of cell, and omitting the measurements of SCell addition will achieve throughput gains.

Proposal 3: Considering that the specific scenario of R19 ATG such as co-located CA and LOS propagation conditions, the further optimization methods for SCell addition and activation can be considered

Observation 3: Introducing distance triggered SCell addition and/or activation is complex and it will also involve RAN2’s work.

Proposal 4: A better way is to define the corresponding enhanced requirements with some conditions, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted, and it depends on UE to apply these requirements.

  - 2.3 Measurement procedure

- Deactivated SCell measurement

In the last meeting, RAN4 discussed the requirement for seactivated SCell measurement and agreed that beam sweeping factor N1 is needed. Whether L3/L1 measurement sharing factor is needed was still FFS. The agreement is copied as follows [1]:

Issue 1-4-4: Deactivated SCell measurement Agreement: - beam sweeping factor N1 is needed. - FFS on L3/L1 measurement sharing factor

For L1/L3 measurement sharing factor, since RAN4 did not define L1 measurement requirements for deactivated SCell in current spec, at least for the ATG UE antenna types except for Case 4, we believe that there is no need to introduce L3/L1 measurement sharing factor Klayer1_measurement

For the ATG UE antenna type Case 4, since there is only one common Rx beam on the bands of PCC and SCC, even though UE doesn’t need to perform L1 measurement on the deactivated SCell, UE still need to perform L1 measurement on the PCell, so the L1/L3 measurement sharing factor is needed.

Proposal 5: At least for the ATG UE antenna types except for Case 4, L3/L1 measurement sharing factor is not needed.

Proposal 6: For the ATG UE antenna type Case 4, the L1/L3 measurement sharing factor is needed.

- Scheduling Availability/Restrictions of ATG UE with the antenna array performing measurements on FR1

Considering that CA has been introduced for R19 ATG, the scheduling restriction extension to intra-band/inter-band CA should be identified, and it is highly related to the following UE antenna assumptions.

Agreement: - Refine the agreement from last meeting as: - For intra-band contiguous CA, same antenna type should be applied on a band. - For inter-band CA, either different or same antenna type can be applied on different bands. - For inter-band CA, consider the following cases: - 1. ATG UE with omnidirectional antennas on the bands of PCC and SCC. - 2. ATG UE with an omnidirectional antenna on the band of PCC and an antenna array on the band of SCC. - 3. ATG UE with an antenna array on the band of PCC and an omnidirectional antenna on the band of SCC. - [4. ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam assumed.] - 5. ATG UE with an antenna array on the bands of PCC and SCC, and two simultaneous separate Rx beams assumed.

In the last meeting, RAN4 has agreed the Scheduling Availability/Restrictions of UE performing measurements on FR1 TDD band, the Scheduling Availability/Restrictions of ATG UE with the antenna array performing measurements on FR1 were still FFS. The candidate proposal is copied as follows [3]:

Issue 1-4-6: Scheduling Availability/Restrictions of ATG UE with the antenna array performing measurements on FR1 FFS - Proposal 1: For intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements, the scheduling restriction extension to intra-band/inter-band CA should be identified for R19 ATG - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols. - For inter-band CA, - For case 2, case 3 and case 5, the scheduling restrictions on receiving PDCCH/PDSCH/TRS/CSI-RS for CQI due to SCell/PCell will not apply to another serving cell in a different band. - If case 4 is considered, the scheduling restrictions on receiving PDCCH/PDSCH/TRS/CSI-RS for CQI due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

When ATG UE with the antenna array performing measurements on FR1, for intra-band contiguous CA, since RAN4 agreed that the same antenna type should be applied on a band, so the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols. For inter-band CA, for case 2 and case 3 where the UEs with independent antenna mode and case 5 where UE can form two separate Rx beams, the scheduling restrictions on receiving PDCCH/PDSCH/TRS/CSI-RS for CQI due to SCell/PCell will not apply to another serving cell in a different band. If case 4 is considered, ATG UE with the antenna array on both PCC and SCC will only form one Rx beam, the scheduling restrictions on receiving PDCCH/PDSCH/TRS/CSI-RS for CQI due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

Proposal 7: For intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements, the scheduling restriction extension to intra-band/inter-band CA should be identified for R19 ATG.

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

- For inter-band CA,

- For case 2, case 3 and case 5, the scheduling restrictions on receiving PDCCH/PDSCH/TRS/CSI-RS for CQI due to SCell/PCell will not apply to another serving cell in a different band.

- If case 4 is considered, the scheduling restrictions on receiving PDCCH/PDSCH/TRS/CSI-RS for CQI due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

- Scheduling Availability/Restrictions of ATG UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

In the last meeting, the candidate proposal of Scheduling Availability/Restrictions of ATG UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1 is copied as follows [1]:

Issue 1-4-7: Scheduling Availability/Restrictions of ATG UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1 FFS - Proposal 1: - For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols. - When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured. - Proposal 2: - For Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement following restrictions should be considered for inta-band carrier aggregation: - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols. - For Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for BFD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement following restrictions should be considered for inter-band carrier aggregation: - For ATG UE with omnidirectional antennas on the bands of PCC and SCC, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured. - For ATG UE with the antenna array on PCell and one or more omni-directional antenna on SCell, there are no scheduling restrictions on FR1 SCel configured in other bands than the bands in which PCell is configured. - For ATG UE with the omni-directional antenna on PCC and antenna array on SCC, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured. - For ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols. - For ATG UE with the antenna array on both PCC and SCC, and two Rx beams can be formed simultaneously, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement, when intra-band carrier aggregation in FR1 is configured, we support the views in option 1 and option 2, which are the same, that is, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

The controversial part is the scheduling availability when inter-band carrier aggregation within FR1 is configured. At least for the ATG UE antenna types except for Case 4, we think that there are no scheduling restrictions on FR1 service cells configured in bands other than PCell.

Proposal 8: For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement.

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

- When inter-band carrier aggregation within FR1 is configured, at least for the ATG UE antenna types except for Case 4, there are no scheduling restrictions on FR1 serving cell(s) configured in bands other than PCell.

- Conclusion

This document discussed the issues for R19 ATG RRM requirements and presented following observations and proposals:

Observation 1: ATG UE has stronger capabilities compared to normal UEs.

Proposal 1: If the type of ATG UE with an antenna array on the bands of PCC and SCC and one common Rx beam assumed is excluded based on RF discussion, MRTD can be 33us for all types of ATG UEs.

Proposal 2: If the type of ATG UE with an antenna array on the bands of PCC and SCC and one common Rx beam assumed (Case#4) is not excluded based on RF discussion, RAN4 to define different values for different types of ATG UE,

- For the MRTD except Case#4: 33µs.

- If case 4 is not excluded based on RF discussion, RAN4 can further discuss the value of MRTD.

Observation 2: The coverage areas of PCell and SCell are relatively fixed and SCell quality is related to the distance between UE and the center of cell, and omitting the measurements of SCell addition will achieve throughput gains.

Proposal 3: Considering that the specific scenario of R19 ATG such as co-located CA and LOS propagation conditions, the further optimization methods for SCell addition and activation can be considered

Observation 3: Introducing distance triggered SCell addition and/or activation is complex and it will also involve RAN2’s work.

Proposal 4: A better way is to define the corresponding enhanced requirements with some conditions, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted, and it depends on UE to apply these requirements.

Proposal 5: At least for the ATG UE antenna types except for Case 4, L3/L1 measurement sharing factor is not needed.

Proposal 6: For the ATG UE antenna type Case 4, the L1/L3 measurement sharing factor is needed.

Proposal 7: For intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements, the scheduling restriction extension to intra-band/inter-band CA should be identified for R19 ATG.

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

- For inter-band CA,

- For case 2, case 3 and case 5, the scheduling restrictions on receiving PDCCH/PDSCH/TRS/CSI-RS for CQI due to SCell/PCell will not apply to another serving cell in a different band.

- If case 4 is considered, the scheduling restrictions on receiving PDCCH/PDSCH/TRS/CSI-RS for CQI due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

Proposal 8: For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement.

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

- When inter-band carrier aggregation within FR1 is configured, at least for the ATG UE antenna types except for Case 4, there are no scheduling restrictions on FR1 serving cell(s) configured in bands other than PCell.

- Reference

[1]	R4-2420237, WF on RRM requirements for NR_ATG_enh, CMCC

[2]      R4-2418275, Topic summary for [113][217] NR_ATG_enh, CMCC