3GPP TSG-RAN WG4 Meeting #114				                                              R4-2501017

Athens , GR, 17th – 21st Feb, 2025

Title:	Discussion on RRM requirements for on-demand SSB SCell operation

Source:	CATT

Agenda Item:	7.25.2.1

Document for:	Discussion

- Introduction

In last RAN4 meeting, the RRM requirements for Rel-19 NES enhancement were discussed. The conclusions for OD-SSB SCell operation were captured in the approved WF [1] and the open issues can be found in the summary [2]. In this paper, we further discuss the potential RRM impact for on-demand SSB SCell operation and give our views.

- Discussion

Sub-topic 1-1: General On-demand SSB(OD-SSB) requirements

Direct SCell activation at SCell addition

Based on RAN1 agreements, the RRC based OD-SSB SCell operation is supported for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. We understand this case means the direct SCell activation at SCell addition and the OD-SSB indication is provided when UE receives SCell activation command which is scenario #2A. With this agreement, we think that RAN4 can discuss and define the SCell activation delay requirements for this case.

Proposal 1: RAN4 to define OD-SSB based SCell activation delay requirements for direct SCell activation at SCell addition for scenario #2A.

L1 measurement before SCell activation

In the last meeting, L1 measurement before SCell activation was not determined, and the WF is copied as below [1]:

Issue 1-1-2: L1 measurement before SCell activation - Proposals - Option 1: L1 measurements are not supported for deactivated SCell. - Option 2: Wait RAN1’s further clarification on the applicable scenario.

According to the current requirements in RAN4, L1 measurements requirement are not defined for deactivated SCell. Therefore, at least for L1 measurement before SCell activation, we support to align with current requirements and don’t introduce any new enhancement, that is, RAN4 not to define the RRM requirements for L1 measurement for deactivated SCell.

Proposal 2: RAN4 not to define the RRM requirements for L1 measurement for deactivated SCell.

Sub-topic 1-2: OD-SSB based deactivated SCell L3 measurement (Case 1)

For deactivated SCell L3 measurement for case 1, the following agreements on how to perform measurement after time window within one time OD-SSB activation/deactivation were reached in last meeting [1]:

RAN4#113: Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation? Agreement: - After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective. - The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism

In RAN1#119, RAN1 further discussed OD-SSB deactivation mechanism and agreed the two approaches. The related agreement about OD-SSB deactivation mechanism in RAN1 is copied as follows:

Agreement RAN1#119 For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer RAN1#118-bis: - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired

In our view, the timeline of procedure described in the sketch map for issue 1-2-3 in the WF just only a possible case [1]. According to RAN1’s agreement, UE may receive an explicit indication of deactivation for on-demand SSB before the end of the fast mode time window, or the number N of on-demand SSB bursts may be shorter than OD-SSB samples defined for fast mode time window, so we believe that once OD-SSB is deactivated, the UE will terminate the deactivated SCell measurement.

Observation 1: UE may receive an explicit indication of deactivation for on-demand SSB before the end of the fast mode time window, or the number N of on-demand SSB bursts may be shorter than OD-SSB samples defined for fast mode time window.

Therefore, for the procedure of OD-SSB based deactivated SCell measurement, the clarification on UE behavior can be added, that is, once it is determined that OD-SSB is deactivated based on the final conclusion of RAN1, the UE will stop the deactivated SCell measurement, regardless of whether it is within/outside the fast mode time window.

Proposal 3: When UE receive explicit indication of deactivation for on-demand SSB via MAC-CE or after the N on-demand SSB bursts indicated in the configuration have passed, the UE will be required to stop the deactivated SCell measurement, regardless of whether it is within/outside the fast mode time window.

Another issue for the procedure of OD-SSB based deactivated SCell measurement is about how to define the time window and whether the time window is configured or pre-defined. The agreement reached in the last meeting is as below [1]:

Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure Sub-issue 3&4: How to define the time window and whether the time window is configured or pre-defined? Agreement: The time window is based on the number of OD-SSB samples and UE provided one valid report. - The uncertainty time should also be considered, such as T_min, T_processing and T_uncertainy - UE should have enough number of OD-SSB samples to achieve the measurement requirement. - The number of OD-SSB is FFS. - FFS whether UE measurement reporting is needed

It was agreed that the time window is based on the number of OD-SSB samples and UE provided one valid report in the last meeting. The number of OD-SSB and whether UE measurement reporting is needed were still FFS [1].

For whether UE measurement reporting is needed, to guarantee the deactivated SCell measurement performance and make the best use of OD-SSB, we believe that the duration of time window at least should be able to accommodate one measurement period of deactivated SCell measurement which is based on OD-SSB periodicity. In our view, the ending point of the time window can be the time when the UE first finishes OD-SSB based deactivated SCell measurement after receiving the OD-SSB indication, and the UEs that firstly report the target OD-SSB based deactivated SCell measurement results will be allowed to enter slow mode in advance after the report is completed.

Observation 2: The duration of time window at least should be able to accommodate one measurement period of deactivated SCell measurement which is based on OD-SSB periodicity.

Proposal 4: For the deactivated SCell L3 measurement for case 1, RAN4 needs to clarify the concept of time window, that is, whether the time window is defined from BS perspective or the UE perspective.

- If the time window is defined from BS perspective, the length of time window will be fixed.

- If the time window is defined from UE perspective, the length of time window will change for different UEs.

Proposal 5: For the deactivated SCell L3 measurement for case 1, if the time window is defined from BS perspective,

- The ending point of time window can be the time when the UE first finishes OD-SSB based deactivated SCell measurement after receiving the OD-SSB indication.

- The UEs that firstly report the target OD-SSB based deactivated SCell measurement results will be allowed to enter slow mode in advance after the report is completed.

In addition, another issue is related to the time relation between multiple times OD-SSB activation/deactivation. The issue can be discussed from following two aspects:

- Whether to define the minimum interval(T2, stage 4) between multiple times OD-SSB activation(reconfiguration)?

- What is the UE behaviour when OD-SSB further activation/reconfiguration after T2?

For the first issue, from the network perspective, we prefer not to define the minimum interval(T2, stage 4) between multiple times OD-SSB activation(reconfiguration). Anyway, when to further activate SCell should depend on the network and we do not expect any restrictions on the behavior of network. So, RAN4 can focus on defining the different UE behaviour when OD-SSB is further activated/reconfigured according to different interval(T2, stage 4).

For the second issue, we first need to discuss a critical value of [X] seconds that can avoid cell identification and maintain the measured SCell as known. If a cell which has been detected based on OD-SSB before and the interval T2 is shorter than detectable time [X] seconds, the UE is able to maintain the measured SCell as known and doesn’t need to perform cell identification. If the interval T2 is longer than detectable time [X] seconds, the UE need to perform cell identification during the fast measurement window.

Proposal 6: RAN4 not to define the minimum interval(T2, stage 4) between multiple times OD-SSB activation(reconfiguration).

Proposal 7: When OD-SSB is further activated/reconfigured, RAN4 to focus on defining different UE behaviour based on a critical value of [X] seconds that can avoid cell identification and maintain the measured SCell as known.

Proposal 8: For UE behaviour during further activation/reconfiguration of OD-SSB,

- If a cell which has been detected based on OD-SSB before and the interval T2 is shorter than detectable time [X] seconds, the UE is able to maintain the measured SCell as known and doesn’t need to perform cell identification.

- If a cell which has been detected based on OD-SSB before and the interval T2 is longer than detectable time [X] seconds, the UE need to perform cell identification during the fast measurement window.

Sub-topic 1-3: OD-SSB based deactivated SCell L3 measurement (Case 2)

For Case #2 where always-on SSB is periodically transmitted on the cell, the UE behaviors before OD-SSB is activated and after OD-SSB is deactivated have been agreed, but the UE behavior when OD-SSB is activated were still FFS.

In RAN4#112bis, RAN4 have sent the LS to RAN1 to clarify the relation of always-on SSB and OD-SSB, and RAN1 has replied to LS in the last meeting, the specific content is as follows [3]:

Reply to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?): - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB. Reply to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?): - RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst. - RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB. Reply to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?): - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB. Reply to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?): - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

According to the above, since whether the time location, the frequency location for the case where always-on SSB is CD-SSB and the spatial relation between always-on SSB and OD-SSB can be different are not determined, so RAN4 need to further wait for RAN1 more progress on the possible relationship between always-on SSB and OD-SSB.

Proposal 9: RAN4 need to further wait for RAN1 more progress on the possible relationship between always-on SSB and OD-SSB.

In RAN4#112bis, RAN4 agreed to first discuss the deactivated SCell requirement for Scenario 1, that is, OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities, and the agreement is copied as follows:

Issue 1-3-1: Always-on SSB and OD-SSB time/frequency domain relation - Background From moderator’s understanding, the following scenarios for always-on SSB and OD-SSB relation are possible based on RAN1’s current agreements. - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different frequencies. Agreement: - In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement. - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies. - Send the above information to RAN1, further discuss the text including the questions to RAN1 in the LS. - RAN4 will inform RAN1 if any further agreement.

But for the same offset in agreed Scenario 1, we think it is not clear enough and the offset need to be further clarified. In RAN1#119 meeting, the agreement on configuration of OD-SSB from RAN1 is copied as follows:

Agreement - For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2. - For Case #1 (i.e., No always-on SSB on the cell), - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives. - Alt A: Same as for Case #1 - Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset)

According to the above, for Case 1, there are two parameters to configure time domain location of OD-SSB, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index. For Case 2, it is same as Case 1 or based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB. Therefore, ‘the same offset’ needed to be clarified in RAN4, such as SFN offset or the whole time domain offset including SFN and half frame.

Proposal 10: For the deactivated SCell L3 measurement for case 2, RAN4 need to further clarify the meaning of ‘the offset’ in scenario 1 based on RAN1’s discussion.

In the last RAN4 meeting, for the UE behavior when OD-SSB is activated, the following options for scenario 1 were proposed:

RAN4#113: Issue 1-3-1: Scenario 1 - OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered - Option 1: UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 1-1: The measurement periodicity is OD-SSB periodicity - Option 1-2: The measurement periodicity is based on combining OD-SSB and AO-SSB - Option 2: UE always follow legacy requirements based on measCycleSCell - Option 3: The same approach in Case 1 can be reused if the serving cell is unknown. UE follows legacy requirements based on measCycleSCell if the serving cell is already known.

In our understanding, the most basic intention is to maximize the NES gain and to use a unified approach as much as possible. For case 2, we support option 1 to use the same framework as case 1, that is, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode.

For the UE behaviour during the fast L3 measurement time window, in the agreed Scenario 1, since the transmission of AO-SSB will be a sub-set of the transmission of OD-SSB, that is,  the transmission of OD-SSB will cover AO-SSB, so we support Option 1-1 and think that the measurement periodicity is OD-SSB periodicity.

Proposal 11: For the deactivated SCell L3 measurement for scenario 1 in case 2, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode.

- The measurement periodicity in the time window is OD-SSB periodicity.

In addition, according to the responses from RAN1, RAN1 has been identified that the main use case also include the periodicity of on-demand SSB being equal to that of always-on SSB, so RAN4 also need to discuss another new Scenario 4 in addition to the three agreed upon scenarios, that is, OD-SSB and always-on SSB are with the same frequency, same spatial relation, same periodicity but different time offset. In this Scenario 4, OD-SSB and AO-SSB may be combined for measurement, and UE can use an equivalent new shorter SSB periodicity, as shown in Figure 1. Meanwhile, in some cases, OD-SSB and AO-SSB cannot be combined for measurement, as shown in Figure 2.

Observation 3: RAN1 has been identified that the main use case also includes the periodicity of on-demand SSB being equal to that of always-on SSB.

Proposal 12: RAN4 also need to discuss another new scenario introduced by RAN1, that is, OD-SSB and always-on SSB are with the same frequency, same spatial relation, same periodicity but different time offset.

Figure 1. The case where OD-SSB and AO-SSB can be combined for measurement

Figure 2. The case where OD-SSB and AO-SSB cannot be combined for measurement

According to the above, it can be observed that when OD-SSB and always-on SSB are with the different SFN and the same half frame index, and their SFNs satisfy the following relationship, UE can perform measurement based on an equivalent new shorter SSB periodicity.

SFNSSB1=SFNSSB2±TSSB1(2*10ms)

Where:

- SFNSSB  and SFNSSB2 are the SFNs of OD-SSB or AO-SSB.

- TSSB1 is the periodicity of OD-SSB or AO-SSB.

If the SFNs and half frame indices of OD-SSB and always-on SSB don’t satisfy the above conditions, UE cannot perform measurement based on an equivalent new shorter SSB periodicity, so we believe that UE can perform measurement based on the first-arrival SSB.

Proposal 13: For Case 2, when OD-SSB and always-on SSB are with the different SFN and the same half frame index, and their SFNs satisfy the following relationship, the measurement periodicity in the time window can be based on an equivalent new shorter SSB periodicity by combining OD-SSB and AO-SSB.

SFNSSB1=SFNSSB2±TSSB1(2*10ms)

Where:

- SFNSSB  and SFNSSB2 are the SFNs of OD-SSB or AO-SSB.

- TSSB1 is the periodicity of OD-SSB or AO-SSB.

Proposal 14: For Case 2, if the SFNs and half frame indices of OD-SSB and always-on SSB don’t satisfy the above conditions, UE cannot perform measurement based on an equivalent new shorter SSB periodicity, the measurement periodicity in the time window can be based on the first-arrival SSB.

In addition to the above options, at least within the fast measurement time window, if the agreed relationship of always-on SSB and OD-SSB in RAN1 are diverse and complex, we believe that the UE also can determine which SSB to use for deactivated SCell measurement based on the network indication.

Proposal 15: At least within the fast measurement time window, the UE also can determine which SSB to use for deactivated SCell measurement based on the network indication.

Meanwhile, we noticed that RAN2 is also discussing the L3 measurement framework and the SSB to be used in case 2. To avoid the duplicated discussion and possible misalignment between different groups, we think we can send the LS to RAN2 to inform our related agreements if reached.

3. L3 RRM Framework - Option1: Based on different measurement configuration when OD-SSB is transmitted - Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted 4. Whether always-on SSB and/or OD-SSB are measured in case 2? Proposal 1: For case #2, RAN2 to discuss which option is followed by UE for RRM measurements of SCell configured with on demand SSB. - Option1: UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB - Option2: UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements.

Proposal 16: Send LS to RAN2 to inform RAN4 agreements on the measurement framework, which includes UE measurement behavior on the deactivated SCell on the selected SSB when/before/after OD-SSB indication is received for Case 1 and Case 2.

Sub-topic 1-4: SCell activation part

The known cell condition  – measurement reporting

In RAN4#112 meeting, it was agreed that RAN4 focuses on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2A. And the known cell condition principle was also agreed as below in RAN4#112bis meeting and RAN4#113 meeting:

RAN4#112bis： Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded. RAN4#113： Issue 1-4-1: The known cell condition – measurement reporting Agreement: - In case 1, known/unknown condition should be defined based on only OD-SSB. - In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB. - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue. - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB. - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure.

For Case 2, RAN4 agreed that known/unknown condition should be defined based on either always-on SSB or OD-SSB, but RAN4 also need to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure.

In our view, since the measurement before SCell activation is to be used during SCell activation procedure, the SSB used in known condition and SCell activation procedure should be with the spatial relation, and RAN4 should discuss how to define such applicability into the specification.

Proposal 17: It should be clarified that the SSBs used in known cell condition and SCell activation procedure should be with the same spatial relation.

After defining the known cell condition, the existing requirements framework can be reused by clarifying the signaling processing time, TFirstSSB/TFirstSSB_MAX and the type of SSB to be used in the SCell activation procedure.

SCell activation requirement - uncertainty time

For the definition of TFirstSSB/TFirstSSB_MAX which is the time to the end of the first complete SSB burst after SCell activation command, it depends on the discussion on SSB type used in SCell activation. At least for the case where OD-SSB is used for SCell activation, it can be updated to the end of first actual transmitted SSB to reflect time uncertainty between OD-SSB indication and actual OD-SSB transmission.

Proposal 18: Update the definition of TFirstSSB and TFirstSSB_MAX to the time to the end of first complete actual transmitted SSB.

- At least for the case where OD-SSB is used for SCell activation, the time uncertainty between OD-SSB indication and actual OD-SSB transmission should be reflected.

Separate MAC CE processing time

For the signalling processing time, the main issue is for scenario #2A. For scenario #2A, the initial understanding is that the OD-SSB indication and SCell activation command are provided by a single MAC CE. But RAN2 has the following agreements in RAN2#127bis meeting for scenario #2A which means the two MAC CE signalling can be separate.

RAN2#127bis Agreements on OD-SSB SCell - Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

Although RAN2 believed that using NW multiplexing two MAC CEs in one single MAC PDU can achieve simultaneous transmission of two MAC CEs, UE still receives the two MAC CEs separately.

Observation 4: Although RAN2 believed that using NW multiplexing two MAC CEs in one single MAC PDU can achieve simultaneous transmission of two MAC CEs, UE still receives the two MAC CEs separately.

From RAN4 perspective, since there is no any limitation on the order and time gap between the MAC CE for OD-SSB indication and SCell activation, at least for the case where the MAC CE of OD-SSB indication is received after the MAC CE of SCell activation is received, RAN4 need to discuss whether such additional MAC CE would impact the SCell activation delay requirements and whether to define requirements applicability regarding the time gap between the MAC CEs for OD-SSB indication and SCell activation for scenario #2A.

In addition, RAN4 also can discuss whether any restriction or requirements applicability are needed for scenario #2A regarding the time gap between the MAC CE for OD-SSB indication and SCell activation. For example, when the transmission interval between two MAC CEs is less than N, the scenario can still be considered as scenario 2A, that is, the relevant time gap can be ignored in the SCell activation delay requirements, where the value of N needs to be discussed.

Proposal 19: At least for the case where the MAC CE of OD-SSB indication is received after the MAC CE of SCell activation is received, RAN4 to discuss the RRM impact of separate MAC CE for SCell activation and on-demand SSB indication.

- To discuss whether separate MAC CE processing time is needed in the activation delay requirements.

- To discuss whether the separate MAC CE processing time can be covered by the waiting time for the first available SSB.

- To discuss whether any requirements applicability are needed for scenario #2A regarding the time gap between the two MAC CEs.

- Conclusion

In this paper, we have some further discussions on on-demand SSB SCell operation, and the following proposals are given:

Proposal 1: RAN4 to define OD-SSB based SCell activation delay requirements for direct SCell activation at SCell addition for scenario #2A.

Proposal 2: RAN4 not to define the RRM requirements for L1 measurement for deactivated SCell.

Observation 1: UE may receive an explicit indication of deactivation for on-demand SSB before the end of the fast mode time window, or the number N of on-demand SSB bursts may be shorter than OD-SSB samples defined for fast mode time window.

Proposal 3: When UE receive explicit indication of deactivation for on-demand SSB via MAC-CE or after the N on-demand SSB bursts indicated in the configuration have passed, the UE will be required to stop the deactivated SCell measurement, regardless of whether it is within/outside the fast mode time window.

Observation 2: The duration of time window at least should be able to accommodate one measurement period of deactivated SCell measurement which is based on OD-SSB periodicity.

Proposal 4: For the deactivated SCell L3 measurement for case 1, RAN4 needs to clarify the concept of time window, that is, whether the time window is defined from BS perspective or the UE perspective.

- If the time window is defined from BS perspective, the length of time window will be fixed.

- If the time window is defined from UE perspective, the length of time window will change for different UEs.

Proposal 5: For the deactivated SCell L3 measurement for case 1, if the time window is defined from BS perspective,

- The ending point of time window can be the time when the UE first finishes OD-SSB based deactivated SCell measurement after receiving the OD-SSB indication.

- The UEs that firstly report the target OD-SSB based deactivated SCell measurement results will be allowed to enter slow mode in advance after the report is completed.

Proposal 6: RAN4 not to define the minimum interval(T2, stage 4) between multiple times OD-SSB activation(reconfiguration).

Proposal 7: When OD-SSB is further activated/reconfigured, RAN4 to focus on defining different UE behaviour based on a critical value of [X] seconds that can avoid cell identification and maintain the measured SCell as known.

Proposal 8: For UE behaviour during further activation/reconfiguration of OD-SSB,

- If a cell which has been detected based on OD-SSB before and the interval T2 is shorter than detectable time [X] seconds, the UE is able to maintain the measured SCell as known and doesn’t need to perform cell identification.

- If a cell which has been detected based on OD-SSB before and the interval T2 is longer than detectable time [X] seconds, the UE need to perform cell identification during the fast measurement window.

Proposal 9: RAN4 need to further wait for RAN1 more progress on the possible relationship between always-on SSB and OD-SSB.

Proposal 10: For the deactivated SCell L3 measurement for case 2, RAN4 need to further clarify the meaning of ‘the offset’ in scenario 1 based on RAN1’s discussion.

Proposal 11: For the deactivated SCell L3 measurement for scenario 1 in case 2, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode.

- The measurement periodicity in the time window is OD-SSB periodicity.

Observation 3: RAN1 has been identified that the main use case also includes the periodicity of on-demand SSB being equal to that of always-on SSB.

Proposal 12: RAN4 also need to discuss another new scenario introduced by RAN1, that is, OD-SSB and always-on SSB are with the same frequency, same spatial relation, same periodicity but different time offset.

Proposal 13: For Case 2, when OD-SSB and always-on SSB are with the different SFN and the same half frame index, and their SFNs satisfy the following relationship, the measurement periodicity in the time window can be based on an equivalent new shorter SSB periodicity by combining OD-SSB and AO-SSB.

SFNSSB1=SFNSSB2±TSSB1(2*10ms)

Where:

- SFNSSB  and SFNSSB2 are the SFNs of OD-SSB or AO-SSB.

- TSSB1 is the periodicity of OD-SSB or AO-SSB.

Proposal 14: For Case 2, if the SFNs and half frame indices of OD-SSB and always-on SSB don’t satisfy the above conditions, UE cannot perform measurement based on an equivalent new shorter SSB periodicity, the measurement periodicity in the time window can be based on the first-arrival SSB.

Proposal 15: At least within the fast measurement time window, the UE also can determine which SSB to use for deactivated SCell measurement based on the network indication.

Proposal 16: Send LS to RAN2 to inform RAN4 agreements on the measurement framework, which includes UE measurement behavior on the deactivated SCell on the selected SSB when/before/after OD-SSB indication is received for Case 1 and Case 2.

Proposal 17: It should be clarified that the SSBs used in known cell condition and SCell activation procedure should be with the same spatial relation.

Proposal 18: Update the definition of TFirstSSB and TFirstSSB_MAX to the time to the end of first complete actual transmitted SSB.

- At least for the case where OD-SSB is used for SCell activation, the time uncertainty between OD-SSB indication and actual OD-SSB transmission should be reflected.

Observation 4: Although RAN2 believed that using NW multiplexing two MAC CEs in one single MAC PDU can achieve simultaneous transmission of two MAC CEs, UE still receives the two MAC CEs separately.

Proposal 19: At least for the case where the MAC CE of OD-SSB indication is received after the MAC CE of SCell activation is received, RAN4 to discuss the RRM impact of separate MAC CE for SCell activation and on-demand SSB indication.

- To discuss whether separate MAC CE processing time is needed in the activation delay requirements.

- To discuss whether the separate MAC CE processing time can be covered by the waiting time for the first available SSB.

- To discuss whether any requirements applicability are needed for scenario #2A regarding the time gap between the two MAC CEs.

- Reference

- R4-2420098, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN4#113

- R4-2418282, Topic summary for [112bis][215] Netw_Energy_NR_enh_Part1, Ericsson, RAN4#113

- R1-2410917, Reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell, RAN1#119