3GPP TSG RAN WG4 Meeting #114		R4-2501073

Athens, Greece, Feb 17th – 21st , 2025

Title: 	Further discussion on other RF requirements for LP-WUS

Agenda Item:	7.26.2.3

Source: 	CATT

Document for:	Discussion

# 1 Introduction

RAN4#113 has further discussed other Rx requirements for LP-WUS, however, there are some issues still open [1]:

Issue 3-1-1: IBB, OBB requirements - Proposals - Option 1: the interference levels for IBB could be relaxed compared to the values defined for MR (at least for the LP-WUR type which targets ultra-low power consumption). OBB could be kept unchanged at least for LP-WUR type which targets moderate power consumption, FFS for ultra-low power consumption LP-WUR. - Option 2: In-band blocking requirement for legacy UE could be reused. the OBB requirement for legacy UE in TS 38.101-1 could be reused. WF: - Further discuss IBB and OBB requirements Issue 3-2-1: spurious response requirements - Proposals - Option 1: spurious response should be relaxed at least for the LP-WUR type which targets ultra-low power consumption. - Option 2: Reuse the spurious response requirements from TS 38.101-1 for LP-WUR. WF: - FFS whether spurious response can be relaxed Issue 3-2-2: Maximum input level requirements Agreements: - For maximum input power level, further discuss - Option 1: use legacy -25dBm - Option 2: scaled to -26.5dBm. - Option 3: same as MR, FFS test signal

In this contribution, we provide our views on these open issues.

# 2 Discussion

First of all, it is noteworthy that at receiver side selectivity, blocking, spurious response and maximum input level requirements are conventionally defined based on the principle of 95% maximum throughput, however, the principle does not apply to LP-WUS obviously since LP-WUS reception is more or less a detection issue. A feasible way to define these Rx requirements is to use MR as a reference.

Proposal 1: RAN4 to define Rx requirements for LP-WUS by using MR as a reference since the conventional principle does not apply.

In previous discussions, there were proposals to introduce different Rx requirements based on various power consumption targets, such as moderate or ultra-low levels. However, assuming similar coverage targets between MR and LR, the actual input level for LP-WUR is not influenced by the power consumption level target. Additionally, specifying and testing such requirements would present challenges. Therefore, we propose not to pursue this approach.

Proposal 2: RAN4 not to introduce different Rx requirements for LP-WUS based on various power consumption targets.

With considerations above, it is reasonable to reuse IBB, OBB and spurious response requirements of MR for LP-WUS.

Proposal 3: RAN4 to define the same IBB requirement for LP-WUS as MR.

Proposal 4: RAN4 to define the same OBB requirement for LP-WUS as MR.

Proposal 5: RAN4 to define the same spurious response requirement for LP-WUS as MR.

For maximum input level, it is slightly different. Since maximum input level of MR is -25dBm for NR carrier bandwidth 3MHz, 5MHz, 10MHz, 15MHz and 20MHz, the case with 3MHz channel bandwidth has the highest PSD, and in this case LP-WUS occupies 11 RBs out of 15 RBs in total, hence the maximum input level could be scaled down according to NR channel bandwidth 3MHz, i.e., 10*log10(NRB = 15/11) = 1.35dB, and if we take round to 0.5dB, then the maximum input level for LP-WUR should be -25-1.5 = -26.5dBm.

Proposal 6: RAN4 to define maximum input level as -26.5dBm for LP-WUS.

# 3 Conclusion

In this contribution, we have the following proposals on other RF requirements for LP-WUS:

Proposal 1: RAN4 to define Rx requirements for LP-WUS by using MR as a reference since the conventional principle does not apply.

Proposal 2: RAN4 not to introduce different Rx requirements for LP-WUS based on various power consumption targets.

Proposal 3: RAN4 to define the same IBB requirement for LP-WUS as MR.

Proposal 4: RAN4 to define the same OBB requirement for LP-WUS as MR.

Proposal 5: RAN4 to define the same spurious response requirement for LP-WUS as MR.

Proposal 6: RAN4 to define maximum input level as -26.5dBm for LP-WUS.

# Reference

[1] R4-2420528, “WF on UE RF requirements for LP-WUS”, vivo