3GPP TSG-RAN WG4 Meeting #114	R4-2501078

Athens, Greece, Feb. 17th – 21st, 2025

Source: 	Murata Manufacturing Co. Ltd

Title: 	MSD test point for DC_(n)3CA

Agenda item:	6.2.2	UE RF requirements for EN-DC and NE-DC of 2 DL with 2 UL (DC_R19_1BLTE_1BNR_2DL2UL)	[DC_R19_xBLTE_yBNR-Core]

Document for:	Discussion

- Introduction

MSD test points for DC_(n)3CA are discussed in this contribution.

- Discussion

2.1 Background Information

In RAN#106, DC_(n)3CA has been requested [1]. Table 1 shows bandwidth combinations for these DC combinations.

Table 1 Bandwidth combinations for DC_(n)3CA

| DC configuration | Uplink DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_(n)3CA | DC_(n)3AA | See CA_3C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 10, 20, 25, 30, 40 |  | 50 | 0 |
|  |  |  | 10, 20, 25, 30, 40 | See CA_3C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_(n)3CA | DC_3A_n3A | See CA_3C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 10, 20, 25, 30, 40 |  | 50 | 0 |
|  |  |  | 10, 20, 25, 30, 40 | See CA_3C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |

DC_(n)3CA is completely new DC combination configurations so RAN4 should study test points to relax REFSENS for LTE DL carriers. This contribution brings candidate MSD test points for DC_(n)3CA with 1UL configuration and discusses options for 2UL test points.

2.2 DC_(n)3CA with 1UL DC_(n)3AA and DC_3A_n3A configurations

For 1UL configuration, common test points can be specified for DC_(n)3CA since the NR UL carrier will be the same allocation regardless of the different Uplink DC configurations. As per [2], this contribution discusses possible configurations when NR UL carrier is configured to closest LTE DL carriers as described in Figure 1. To maximize the interference on LTE DL carriers by NR UL carrier, the NR UL carrier should be configured to be as wide as possible.

Figure 1: Landscape of DC_(n)3CA with 1UL configurations

Regarding LTE carrier bandwidth, the configuration table refers to CA_3C BCS0 in TS 36.101 Table 5.6A 1-1, as shown in Table 2.

Table 2: CA_3C BCS0, extracted from TS 36.101 Table 5.6A 1-1

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated | Bandwidth combination set |
|  | (NOTE 3) | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | bandwidth [MHz] |  |
| CA_3C | CA_3C | 5, 10, 15 | 20 |  |  |  | 40 | 0 |
|  |  | 20 | 5,10,15,20 |  |  |  |  |  |
| NOTE 3: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |  |  |  |  |  |

This notes that the minimum aggregated LTE DL carrier bandwidth is 25MHz, therefore, the maximum NR UL carrier bandwidth will also be limited to 25MHz. Accordingly, this contribution will focus on the following carrier bandwidth combinations; NR UL 25MHz + LTE DL1 5MHz + LTE DL2 20MHz and NR UL 25MHz + LTE DL1 20MHz + LTE DL2 5MHz.

Following the similar approach where Lcrb is adjusted to ensure a maximum impact of Tx emission, as outlined in our companion paper [3], this contribution proposes the following test points in Table 3.

Proposal1: Consider the test points for DC_(n)3CA with 1UL configuration in Table 3.

Table 3: Candidate test points for DC_(n)3CA with 1UL configuration

| EN-DC configuration | E-UTRA/NR band | FC (UL) | Channel bandwidth | UL | FC (DL) | MSD | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | allocation (LCRB) | (MHz) |  |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | [TBD] | FDD |
|  | 3 | N/A | 20 | N/A | 1845 | [TBD] |  |
|  | n3 | 1772.5 | 25 | 50 (RBstart = 83) | 1867.5 | N/A |  |
| DC_(n)3CA | 3 | N/A | 20 | N/A | 1840 | [TBD] |  |
|  | 3 | N/A | 5 | N/A | 1852.5 | [TBD] |  |
|  | n3 | 1772.5 | 25 | 50 (RBstart = 83) | 1867.5 | N/A |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | [TBD] |  |
|  | 3 | N/A | 20 | N/A | 1845 | [TBD] |  |
|  | n3 | 1772.5 | 25 | 6 (RBstart = 127) | 1867.5 | N/A |  |
| DC_(n)3CA | 3 | N/A | 20 | N/A | 1840 | [TBD] |  |
|  | 3 | N/A | 5 | N/A | 1852.5 | [TBD] |  |
|  | n3 | 1772.5 | 25 | 3 (RBstart = 130) | 1867.5 | N/A |  |

2.3 DC_(n)3CA with 2UL DC_(n)3AA or DC_3A_n3A configurations

If the 2UL configuration is also confirmed by the requester, the configurations will be illustrated as follows.

Figure 2: Landscape of DC_(n)3CA with 2UL configurations DC_3A-n3A(left) and DC_(n)3AA (right)

Regarding 2UL DC_(n)3AA, the existing MSD test points for DC_(n)3AA BCS0 allow only singled switched UL. The DC_(n)3AA BCS1 configuration is to be discussed as outlined in our companion paper [3]. For the case 2UL DC_3A_n3A, DC_3A_n3A itself includes both 1UL and 2UL test points, and the 2UL test points were introduced in Rel-16[4].

When confirming DC_(n)3CA 2UL configuration, these ongoing and existing studies can be taken into consideration.

Observation 1: Regarding 2UL configurations for DC_(n)3CA, RAN4 can refer to the relevant discussions and specifications.

Proposal 2: Confirm with the requester if 2UL configuration should be specified for DC_(n)3CA.

- Conclusion

Observation 1: Regarding 2UL configurations for DC_(n)3CA, RAN4 can refer to the relevant discussions and specifications.

Proposal 1: Consider the test points for DC_(n)3CA with 1UL configuration in Table 3.

Table 3: Candidate test points for DC_(n)3CA with 1UL configuration

| EN-DC configuration | E-UTRA/NR band | FC (UL) | Channel bandwidth | UL | FC (DL) | MSD | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | allocation (LCRB) | (MHz) |  |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | [TBD] | FDD |
|  | 3 | N/A | 20 | N/A | 1845 | [TBD] |  |
|  | n3 | 1772.5 | 25 | 50 (RBstart = 83) | 1867.5 | N/A |  |
| DC_(n)3CA | 3 | N/A | 20 | N/A | 1840 | [TBD] |  |
|  | 3 | N/A | 5 | N/A | 1852.5 | [TBD] |  |
|  | n3 | 1772.5 | 25 | 50 (RBstart = 83) | 1867.5 | N/A |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | [TBD] |  |
|  | 3 | N/A | 20 | N/A | 1845 | [TBD] |  |
|  | n3 | 1772.5 | 25 | 6 (RBstart = 127) | 1867.5 | N/A |  |
| DC_(n)3CA | 3 | N/A | 20 | N/A | 1840 | [TBD] |  |
|  | 3 | N/A | 5 | N/A | 1852.5 | [TBD] |  |
|  | n3 | 1772.5 | 25 | 3 (RBstart = 130) | 1867.5 | N/A |  |

Proposal 2: Confirm with the requester if 2UL configuration should be specified for DC_(n)3CA.

References:

[1] RP-243257, “Revised WID for DC_R19_xBLTE_yBNR”, Nokia, RP#106, Madrid, Spain

[2] R4-2417156, “WF on DC_(n)8AA test point and MSD test point clean-up“, Skyworks Solutions Inc., RAN4#112-bis, Hefei, China

[3] R4-2500820, “MSD test point for DC_(n)3AA BCS1”, Murata Manufacturing Co. Ltd, RAN4#114, Athens, Greece

[4] R4-1904974, “TP for TR 37.716-11-11: MSD requirements for DC_3_n3 intra-band non-contiguous EN-DC”, CHTTL, Skyworks, RAN4#90bis, Xian, China