# 3GPP TSG-RAN WG4 Meeting #114                              R4-2501081

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.22.3.1

Source: 	CMCC

Title: 			 Discussion on RRM requirements for event-driven beam management for NR MIMO Phase 5

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR MIMO Phase 5 was approved [1], the objectives are duplicated as following.

RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In last meeting, there was discussion on NR MIMO Phase 5, and a WF was agreed [2]. This contribution provides discussion on RRM requirements for event-driven beam management.

- Discussion

Issue 1-2: How to define measurement period (evaluation time / measurement time) in Event triggered measurement reporting delay? < Agreement> Agreement: For specifying the RAN4 delay requirement for event 2 - Measure both the current beam and new beam after the starting point, FFS the details. - Note: this does not imply to define the delay requirement based on the sum of the current beam measurement period and new beam measurement period, which is pending on further RAN1 discussion. - Start the RAN4 RRM discussion for FR2. - Send LS to RAN1 to ask whether FR1 is covered as well. FFS the exact wording in the LS.

According to the WID [2], one of the objectives is to specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management. Our interpretention about this objective is that even though the target is FR2, it does not preclude FR1. And according to our knowledge, both FR1 and FR2 are considered in RAN1.

In last meeting, it was agreed to start the RAN4 RRM discussion for FR2, and send LS to RAN1 to ask whether FR1 is covered as well. However, it is noticed that the LS was missing. In our view, the agreements about FR1 and FR2 are a package. In order not to delay the work of FR1, it is better to send LS to RAN1 ASAP. It is proposed to follow previous agreements and send LS to RAN1 in RAN4#114 meeting. The draft LS is provided in the Annex.

Proposal 1: for whether FR1 is covered for UE-initiated/event-driven beam management, it is proposed to follow previous agreements and send LS to RAN1 in RAN4#114 meeting. The draft LS is provided in the Annex.

For specifying the RAN4 delay requirement for event 2 (event 2: the L1-RSRP of the new beam becomes a threshold value better than the current beam), it was agreed to measure both the current beam and new beam after the starting point, FFS the details. One issues idenfitied is how to define the requirements if the RS period of current beam and RS period of new beam is different. This issue can be resolved based on RAN1 latest agreements. According to RAN1 LS [4], as duplicated as below, it can be seen that the periodicity of the current beam RS should be the same as that of the new beam RS(s). RAN4 requirements can be defined with this assumption.

RAN1 LS to RAN4 (R1-2410914) 1. Overall Description: In RAN1#119, regarding UE-initiated/event-driven beam management for Rel-19 NR MIMO Ph5, the following agreement on the evaluation periodicity for determining Event-2 instance was reached: Agreement Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC). - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s) - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS. - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS. - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s). - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}. - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s). Note: There is the same periodicity for the new beam RS(s). FFS: Down-selection among above Alt(s) for cross-CC beam reporting. 2. Actions: To: RAN4 ACTION: RAN1 respectfully asks RAN4 to take the above into account.

Proposal 2: for definition of delay requirements for Event 2, it is proposed to follow RAN1 agreements that the periodicity of the current beam RS is the same as that of the new beam RS(s).

Issue 1-4: Reporting delay requirements <Way forward > - Option 1: (Qualcomm) - Tgap + TL1-RSRP-measure; TL1-RSRP-measure in Issue 1-2; The measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on; In test cases, set Tgap = 0; - Option 2: (Nokia) - The maximum delay requirement for the first PUCCH channel is determined in relation to the reception time instant of the RS that triggered the UEIBM event. - Option 3: (Xiaomi) - If multiple beams will be measured, total measurement time will be maximum measurement period between multiple beams. - Option 4: (Samsung) - Need further RAN1 progress for RSs relationship between current beam and new beams. - Option 5: (CMCC) - The event triggered measurement reporting delay shall be less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index. The measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on - Option 6: (Apple) - The total reporting delay is Tfirst-RS-NB + TL1-RSRP-NB + TPUCCH, Add TPucch in reporting delay - Option 7: ( MediaTek) - The total reporting delay is TL1-RSRP_measure + Tfirst UL channel, TL1-RSRP_measure is base on SSB or CSI-RS - Event evaluation period for a SSB or CSI-RS is same as their respective measurement period - Option 8: (Ericsson) - Event triggered measurement reporting delay for SSB is TL1-RSRP_event triggered_measurement_Period_SSB (ms) + TTime to first UL channel and event triggered measurement delay for CSI-RS is TL1-RSRP_event triggered_measurement_Period_CSI-RS (ms) + TTime to first UL channel - RAN4 to discuss components and criteria for defining TTime to first UL channel.

It was agreed in last meeting that for UE-initiated/event-driven beam reporting, the starting point is to reuse the same definition in L3 event trigger report which is the event is met over the air. The ending point is the point when the UE transmits a first PUCCH triggered by the event.

For legacy L3 based event triggered measurement report, as duplicated as following, the measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. It can be seen that the definition of L1 based event triggered measurement reporting delay is similar as the definition for L3 based event triggered measurement reporting delay. And it is proposed to follow the approach for L3 measurement report delay to add clarification that this beam reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on

TS 38.133 9.2.4.3 Event Triggered Reporting Reported RSRP, RSRQ, and RS-SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). The UE shall not send any event triggered measurement reports as long as no reporting criteria is fulfilled. The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index or T identify intra without index defined in clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

As duplicated as above, the L3 event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify, within which UE shall be able to identify a new detectable cell. Since inter-cell measurement is considered, it can be considered to use Rel-18 LTM measurement delay as baseline to define measurement reporting delay for SSB bsed UE-initiated/event-driven beam reporting.

For Rel-18 LTM, the measurement period is defined as TL1-RSRP_Measurement_Period if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell. Otherwise, UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period + TSSB_time_index, with the assumption that the target cell is known. Similar as event triggered reporting for L3 measurement, it can be considered that the event triggered L1 measurement reporting delay, measured without filtering is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Perioa + TSSB_time_index. When filtering is used, an additional delay can be expected.

Proposal 3: for reporting delay for SSB based UE-initiated/event-driven beam reporting, it is proposed to take measurement period defind for Rel-18 LTM as baseline

- in detail, the reporting delay for SSB bsed UE-initiated/event-driven beam reporting is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index , which are specified for L1-RSRP measurement for Rel-18 LTM .

According to RAN1 agreements, on UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management. And in previous meeting, RAN4 also agreed that L1-RSRP is used as measurement metrics for beam report, and agreed to define CSI-RS based L1-RSRP measurement requirements. CSI-RS is periodic CSI-RS. For CSI-RS based beam report, legacy measurement delay requirements only apply to CSI-RS resources of the serving cell. While according to RAN1 design, measurement on beam from neighbour cell are also considered, it is necessary to discuss whether to reuse legacy L1 measurement delay requirements or to define new delay requirements to cover neighbour cell. And the details can be further discussed.

Proposal 4: for reporting delay for CSI-RS based UE-initiated/event-driven beam reporting, it is proposed to define L1 measurement reporting delay requirements, and FFS whether legacy CSI-RS based L1 measurement delay requirements can be reused.

Issue 1-11: Capabilities for Support of Event Triggering and Reporting Criteria? <Way forward > - Option 1: (Nokia) - RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item. - Option 2: (Apple) - Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added. - Option 3: (Ericsson) - Define UE capability for total number of events supported for UE-initiated beam management across single and multiple TRP. - Option 4: (MediaTek) - Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency.

For legacy event triggered L3 measurement reporting, the capabilities for support of event triggering reporting criteria are defined (section 9.1.4, TS38.133). Similarly, for Rel-19 event triggered L1 measurement reporting, it is necessary to define the capability. According to the discussion in last meeting, some companies mentioned that the number of events to be supported by UE shall be discussed in RAN1 first. We do not have strong view. But at least according to our knowledge, RAN1 have not discussed this. Considering that reporting criteria for L3 are specified in RAN4, it is preferred to define the reporting criteria for L1 in RAN4.

Proposal 5: it is proposed to define capabilities for support of UE-initiated/event-driven beam reporting.

Issue 1-12: Compatibility of capabilities with LTM <Way forward > - Option 1: (Nokia) - RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item. - Option 2: (Samsung, Apple) - Do not combine capabilities across different L1 measurements. - Option 3: (Ericsson) - RAN4 to wait for further progress in RAN1/2 for the UE capability discussions and compatibility between MIMO and mobility. - Option 4: (MediaTek) - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold. - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

Event triggered L1 measurement reporting are considered for both Rel-19 MIMO and Rel-19 Mobility. However, considering that Rel-19 MIMO and Rel-19 Mobility are two parallel on-going work item, it is not prefered to mix the discussion.

Proposal 6: it is not prefered to mix Rel-19 mobility and Rel-19 MIMO. The capability for supported number of events triggered L1 measurement reporting is defined seperately for Rel-19 mobility and Rel-19 MIMO.

Issue 1-13: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management? <Way forward > - Option 1: (Qualcomm, Nokia, Xiaomi, Apple, MediaTek) - The existing accuracy requirements for L1 measurements can be applied. Not to define new accuracy requirements. - Option 2: (Samsung) - Focus on core requirements firstly, FFS on accuracy requirements. - Option 3: (CMCC) - For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline. - For CSI-RS based beam report, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements taking neighbour cell measurement into account. - Option 4: (ZTE) - Discuss the accuracy for the two trigger-event solutions respectively. - For the basic trigger-event detection, tight the existing L1-RSRP accuracy requirement; - For the optional trigger-event detection, resue the existing L1-RSRP accuracy requirement.

Except reporting delay requirements, reporting accuracy requirements also need to be defind. Similar as L3 based measurement report, for UE-initiated/event-driven beam reporting, the reported L1 measurement contained in event triggered beam reports shall meet the accuracy requirements.

According to RAN1 agreements, on UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management. And in last meeting, RAN4 also agreed that L1-RSRP is used as measurement metrics for beam report, and agreed to define CSI-RS based L1-RSRP measurement requirements. CSI-RS is periodic CSI-RS.

For SSB based beam report, accuracy requirements defined for Rel-18 LTM can be considered. Rel-18 LTM specify intra-frequency and inter-frequency L1-RSRP measurement accuracy requirements. For both cases, absolute and relative accuracy are specified with legacy side condition SNR= -3dB. And  reuse legacy absolute and relative accuracy requirements for intra-frequency LTM L1-RSRP measurement. For inter-frequency L1-RSRP measurement accuracy, absolute and relative accuracy are the same, which equal to intra-frequency absolute accuracy. It is necessary to discuss whether LTM measurement accuracy can be reused for  event triggered L1 measurement reporting. In our view, the accuracy requirements for Rel-18 LTM can be used as baseline. We are also open to update accuracy requirements based on new scenarios/conditions if needed.

For CSI-RS based beam report, legacy accuracy requirements only apply to CSI-RS resources of the serving cell. While according to RAN1 design, measurement on beam from neighbour cell are also considered, it is necessary to define new accuracy requirements to cover neighbour cell.

Proposal 7: for UE-initiated/event-driven beam reporting, except reporting delay requirements, it is proposed to define reporting accuracy requirements.

- The reporting accuracy requirement is that the reported L1 measurement contained in event triggered beam reports shall meet the accuracy requirements

- For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline.

- For CSI-RS based beam reprt, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements taking neighbour cell measurement into account.

- Conclusion

This contribution provides discussion on RRM requirements for event-driven beam management for NR MIMO Phase 5. The proposals are:

Proposal 1: for whether FR1 is covered for UE-initiated/event-driven beam management, it is proposed to follow previous agreements and send LS to RAN1 in RAN4#114 meeting. The draft LS is provided in the Annex.

Proposal 2: for definition of delay requirements for Event 2, it is proposed to follow RAN1 agreements that the periodicity of the current beam RS is the same as that of the new beam RS(s).

Proposal 3: for reporting delay for SSB based UE-initiated/event-driven beam reporting, it is proposed to take measurement period defind for Rel-18 LTM as baseline

- in detail, the reporting delay for SSB bsed UE-initiated/event-driven beam reporting is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index , which are specified for L1-RSRP measurement for Rel-18 LTM .

Proposal 4: for reporting delay for CSI-RS based UE-initiated/event-driven beam reporting, it is proposed to define L1 measurement reporting delay requirements, and FFS whether legacy CSI-RS based L1 measurement delay requirements can be reused.

Proposal 5: it is proposed to define capabilities for support of UE-initiated/event-driven beam reporting.

Proposal 6: it is not prefered to mix Rel-19 mobility and Rel-19 MIMO. The capability for supported number of events triggered L1 measurement reporting is defined seperately for Rel-19 mobility and Rel-19 MIMO.

Proposal 7: for UE-initiated/event-driven beam reporting, except reporting delay requirements, it is proposed to define reporting accuracy requirements.

- The reporting accuracy requirement is that the reported L1 measurement contained in event triggered beam reports shall meet the accuracy requirements

- For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline.

- For CSI-RS based beam reprt, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements taking neighbour cell measurement into account.

- Reference

- RP-234007, New WID: NR MIMO Phase 5

- RP-242394, WID revision: NR MIMO Phase 5

- R4-2420096, WF on RRM requirements for NR_MIMO_Ph5_Part, Samsung

- R1-2410914, LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management

- R4-2416856, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung

- R4-2414035, WF on RRM requirements for NR_MIMO_Ph5, Samsung

- Annex: draft LS on UE-initiated/event-driven beam management

3GPP TSG RAN WG4 Meeting #114                                           R4-242xxxx

Athens, Greece, 17th – 21st February, 2025

Title:	LS on UE-initiated/event-driven beam management

Response to:

Release:	Rel-19

Work Item:	NR_MIMO_Ph5

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

Name:	Jingjing Chen

E-mail Address: chenjingjing@chinamobile.com

1. Overall Description:

RAN4 discussed RRM requirement of enhancement for UE-initiated/event-driven beam management based on following objective of Rel-19 MIMO WI.

Objective of NR MIMO Phase 5 (RP-242394) - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

Companies in RAN4 have different views on whether FR1 is in scope or not for UE-initiated/event-driven beam management. RAN4 seeks clarification from RAN1 perspective regarding whether FR1 is considered for UE-initiated/event-driven beam management.

2. Actions:

To: RAN WG1

ACTION: 	RAN4 respectfully asks RAN1 to provide clarification on whether FR1 is covered for UE-initiated/event-driven beam management.

3. Dates of next TSG RAN WG4 meetings

RAN4#114bis	 07 - 11 April , 2025		Wuhan, China

RAN4#115	 19 - 23 May, 2025		St Julian’s, Malta