# 3GPP TSG-RAN WG4 Meeting #114                            R4-2501105

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	5.12.1

Source: 	CMCC

Title: 			 (NR_MG_enh2-Core) Discussion on concurrent gaps with NCSG

Document for:	Discussion

- Introduction

In last meeting, there was discussion on concurrent gaps with NCSG, and a WF was approved [1]. This contribution provides discussion on this issue.

- Discussion

- Sub-topic 1-1: Rel-18 UE behavior for deactivated SCell measurements with NCSG - Background: < Agreement >: Agreement from RAN4 meeting#111: - For UE configured with one NCSG and one Type 1/2 MG: All deactivated SCells are measured within NCSG, regardless of the reported UE capabilities [and gap association]. - Further details on the processing delay between NCSG and Type 1/2 MG can be further discussed. - For UE configured with 2 NCSG, deactivated SCells are measured with NCSG - If the association is provided, deactivated SCells are measured with NCSG according to gap association. - If the association is not provided, UE is not expected to cause interruption outside the VIL due to measurement on any of the deactivated SCells, and the existing measurement delay requirement does not apply to this case. Agreement from RAN4 meeting#112: - When UE switches measurement between NCSG and Type 2 MG, no additional processing delay is expected.. Issue 1-1-1: When the UE is configured with Concurrent gaps with NCSG, whether all deactivated SCells are measured within NCSG, regardless the gap association? - Further discuss in the next meeting.

It was agreed that when the UE is configured with Concurrent gaps with NCSG, all deactivated SCells are measured within NCSG, regardless of the reported UE capabilities [and gap association]. The open issue is whether need to consider the gap association.

Our preference is not to consider the gap association. The original reason to discuss Rel-18 UE behavior for deactivated SCell measurements with NCSG is whether the deactivated SCell MO not associated with NCSG can be still measured within NCSG. The benifit is to avoid introducing additional interruption compared with the solution that UE measures the deactivated SCell outside of MG. If gap association is considered, the above benefit is lost, in detail, the deactivated SCell MO not associated with NCSG cannot be measured within NCSG, which means can only be measured outside of MG resulting in additional interruption.

Proposal 1: it is proposed that when the UE is configured with Concurrent gaps with NCSG, all deactivated SCells are measured within NCSG, regardless the gap association.

- Conclusion

This contribution provides discussion on concurrent gaps with NCSG. The proposals are:

Proposal 1: it is proposed that when the UE is configured with Concurrent gaps with NCSG, all deactivated SCells are measured within NCSG, regardless the gap association.

- Reference

- R4-2420213, WF on RRM requirements for NR_MG _enh2, MediaTek.