3GPP TSG-RAN WG4 Meeting #114 	   R4-2501113

# Athens, Greece, 17th – 21st , Feburary 2025

Agenda Item:	7.23.3.1

Source: 	Ericsson

Title:	Discussion on RRM requirements for UE-to-UE CLI handling (SBFD)

Document for:	Discussion

# Introduction

The agreements on the SBFD CLI handling and SBFD operation during RAN4#113 meeting are captured in the approved WF [1].

In this paper we provide our view on the listed open issues on RRM core requirement within related WID [2] objectives:

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

# Discussion

# 2.1 UE to UE CLI handling

### L1-CLI RSSI measurement methods

RAN4 #113 Agreements (from Thu AH session) - RAN4 shall specify L1 CLI measurement requirements at least for below methods: - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Additional methods can be considered based on further RAN1 agreements. - Differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions.

There are some progresses in RAN1 for CLI- RSSI measurement and these can be reflected on the latest RAN1 agreements as follow:

RAN1#119: Agreement For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported - Method #3: UE measures RSSI within UL subband - A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig. Conclusion There is no RAN1 consensus on the following: - Subband CLI-RSSI reporting for L1 CLI-RSSI measurement in Rel-19. Conclusion For Method#1 (RSSI measurement within DL subband) with the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands), two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband is not supported.

For Method #1(RSSI measurement within DL subband), the wideband RSSI measurement report will not differentiate the frequency location of the RSSI measurement resource within the DL subband as noticed from RAN1 agreement.

For Method #3(RSSI measurement within UL subband), due to the UE to UE CLI issue most likely will happen for the case where the aggressor UE transmitting in the UL subband interference the UE which is very close to the aggressor DL transmission.

By agreeing to support method #3, the RSSI measurement can give the gNB a better understanding where the CLI issue exist in the frequency domain.

Also, the RSSI measurement within DL subband and UL subband cannot be within the same OFDM symbol which can differentiate the CLI issue from time perspective.

Given this consideration above, we think it is reasonable from RAN4 perspective to support method #3 and specify requirements accordingly.

- Proposal 1: RAN 4 shall specify L1-CLI-RSSI measurement requirements for method #3 RSSI measurement within UL subband .

One issue to notice is the CLI-RSSI measurement timing for method # 3.

Based on existing RAN1#118 bis agreement for method #1 is the measurement will follow the DL reference timing. This agreement fits the purpose to serve measurement of the RSSI within DL subband.

- Observation 1: L1-CLI-RSSI method#3 measurement timing is not clear based on existing RAN1 agreement.

Currently there are two options for define method# 3 measurement timing.

- Option 1: Follow the L1-SRS-RSRP measurement timing (Rel-16 RSSI measurement timing)

- Option 2: Follow the method#1 DL timing.

The SRS is configured by the gNB to the aggressor UE, and the victim UE will listen and measure the CLI-SRS. The measurement timing needs to have certain error being considered due to the aggressor SRS is being transmitted based on DL reference timing together with preconfigured TA (timing advance) for the aggressor UE. The error occurs due to the TA is calculated based on the distance and propagation delay between the gNB and the aggressor UE/ victim UE, while the measurement of SRS-RSRP is based on the propagation delay between the aggressor UE and the victim UE.

This is not the same case as RSSI measurement within the UL subband. The victim UE being disturbed by the CLI interference, then the network configures the victim UE to listen to specific time and frequency location to identify where is the CLI issue. The L1-CLI-RSSI is total averaging of all the received power within the configured resources, there is no specific aggressor being configured. The RSSI measurement is different from SRS-RSRP due to there is no need for demodulation of the SRS sequence, the victim UE shall only measure where it is going to transmit in the DL.

- Proposal 2: RSSI measurement for UL subband measurement timing shall base on DL reference timing.

To specify RAN4 measurement requirement for method# 3, certain consensus is needed for periodical, semi-persisitence and aperiodic measurement resources

- Maximum number of total measurement resource network can configure for CLI-RSSI measurement.

- Maximum number of total measurement resource UE can monitor for CLI-RSSI

- Number of PRBs and start PRB index for subband indication, configured PRBs are contiguous.

- Number of OFDM symbols and first OFDM symbol index in a slot, configured OFDM symbols are

Based on our understanding, the configuration of the total number of resources and PRB index etc should not be different from method#1. The configuration can be guaranteed by the network. We think there are no difference between method #1 and method #1, the existing RAN4 requirements on method# 1 can be applicable for method# 3.

- Proposal 3: For RSSI measurement for UL subband, single shot measurement is sufficient.

### Scheduling restriction and measurement restriction

RAN4#113 meeting: Agreements (from Thu AH session) - For scheduling restriction, - For L1-SRS-RSRP, - For FR1, R16 requirements are re-used as baseline, FFS exact number of restricted symbols considering the impact of timing assumption - For FR2, FFS whether R16 requirements are re-used as baseline, considering the impact of Rx beam determination, and timing assumption - For L1-CLI-RSSI, FFS whether R16 requirements are re-used as baseline, considering the measurement is based on DL timing. - For measurement restriction, - Requirements for L1 CLI measurement do not apply when the L1 CLI measurement resource collides with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. - FFS whether need to consider the exception cases where L1 CLI measurement can be performed with other measurement in parallel.

For L1-SRS-RSRP, the scheduling restriction needs to be specified when the measurement timing have been confirmed as well as how many symbols due to the timing error being agreed for both FR1 and FR2.

For L1-CLI-RSSI, the measurement timing for method#1 is based on DL timing, scheduling restriction shall be only when the measurement resources are not within the same RX beam. This is most likely only applicable for FR2.  In this case the similar Rel-16 requirement can be re-sued.

- Proposal 4: The scheduling restriction due to L1-CLI-RSSI applies in OFDM symbols when the measurement resource is not type-D QCL-ed with the DL reception PDCCH/PDSCH/CSI etc in FR2.

- Proposal 5: The scheduling restriction due to the L1-SRS-RSRP on how many OFDM symbols can be further discussed when the measurement timing and error being agreed.

The legacy measurement restriction is introduced because when measurement gaps is being configured or the SSB/SMTC is overlapping with the RSSI measurement resources, or the CSI-RS is not within the same RX beam as the RSSI measurement. Even the measurement time for the RSSI have been agreed to be DL reference timing. The RSSI measurement still cannot be done parallel together for example with measurement gaps, or in another RX beam.

- Proposal 6:For both L1-SRS-RSRP and the L1-CLI-RSSI measurement, in principle the legacy measurement restriction shall be applied.

### L1-SRS-RSRP measurement requirements

#### Issue 1-1-6: Measurement period #### Agreements (from Wed AH session) - For L1-SRS-RSRP, further discuss the following options based on simulation results: - Option 1: requirement specified with [1] sample measurement assumption - Option 2: requirement specified based on [1] and [3] samples according to NW configuration, e.g. timeRestrictionForChannelMeasurement

With considering the simulation results by comparing single sample and multiple samples for different channels, we observe when the timing error is larger than the CP, the measurement accuracy can be improved by adding more samples. This can be seen from our simulation results contribution.[3]

Meanwhile, we also understand setting requirements on multiple samples need to take consideration of the extended measurement delay. The extended measurement delay will impact the L1 fast mitigation schemes being specified for Rel-19.

Based on the existing CSI-RS based L1-RSRP in TS 38.133 clause 9.5

The value of TL1-RSRP_Measurement_Period_CSI-RS is defined in Table 9.5.4.2-1 for FR1 and in Table 9.5.4.2-2 for FR2, where - For periodic and semi-persistent CSI-RS resources, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise - For aperiodic CSI-RS resources M=1

For periodic and semi-persistent CSI-RS, the measurement will only take 1 sample if this higher layer parameter being configured. Also, to our understanding, this parameter is defined in TS 38.214 clause 5.2.1.1 as the UE shall derive the channel measurement only on the most recent occasion of the NZP-CSI-RS resources set.

If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "notConfigured", the UE shall derive the channel measurements for computing L1-RSRP value reported in uplink slot n based on only the SS/PBCH or NZP CSI-RS, no later than the CSI reference resource, (defined in [4, TS 38.211]) associated with the CSI resource setting. If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the channel measurements for computing L1-RSRP reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of SS/PBCH or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting.

From measurement point of view, when there is only 1 sample during one measurement occasion, there is no averaging across all the samples. The trade-off between the measurement accuracy and measurement delay cannot be only dependent on the UE measurement, this also needs network consideration.

The legacy L1-RSRP measurement period definition method can be reused here. The higher layer parameter timeRestrictionForChannelMeasurement and both 1 and 3 samples are considered can be introduced for L1-SRS-RSRP.

RAN4 may need to discuss separate requirements for L1-SRS-RSRP measurement accuracy based on 1 and 3 samples.

- Proposal 7: For L1-SRS-RSRP measurement, the CLI measurement period can be specified following L1-RSRP measurement and both 1 and 3 samples are considered based on network configuration .

# Summary

The following have been proposed in the current contribution:

	Proposal 1: RAN 4 shall specify L1-CLI-RSSI measurement requirements for method #3 RSSI measurement within UL subband .

	Proposal 2: RSSI measurement for UL subband measurement timing shall base on DL reference timing.

	Proposal 3: For RSSI measurement for UL subband, single shot measurement is sufficient.

	Proposal 4: The scheduling restriction due to L1-CLI-RSSI applies in OFDM symbols when the measurement resource is not type-D QCL-ed with the DL reception PDCCH/PDSCH/CSI etc in FR2.

	Proposal 5: The scheduling restriction due to the L1-SRS-RSRP on how many OFDM symbols can be further discussed when the measurement timing and error being agreed.

	Proposal 6:For both L1-SRS-RSRP and the L1-CLI-RSSI measurement, in principle the legacy measurement restriction shall be applied.

	Proposal 7: For L1-SRS-RSRP measurement, the CLI measurement period can be specified following L1-RSRP measurement and both 1 and 3 samples are considered based on network configuration

# References

- R4-2420097 “WF on SBFD RRM”, Huawei, November ,2024

- RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June ,2024

- R4-2501114 “Simulation results for L1-SRS-RSRP”, February 2025