3GPP TSG RAN WG4 Meeting #114		 R4-2501119

Athens, GR, 17th – 21nd February, 2025

Agenda Item:	7.8.1

Source: 	MediaTek Inc.

Title: 	Discussion on scope clarification for low-band CA switching

Document for:	Approval

1. Introduction

The low band carrier aggregation has been continued operators interest. The benefit on low band is mainly on the propagation distance thus the coverage is always better than higher frequencies. It is always challenge to implement low-band CA on handheld UE especially when there are some combinations has almost no frequency gap between UL transmission and DL reception channels. Considering to better utilize the low band fragmented spectrums owned by operator, a new WI was approved in RANP#106 last December[1] to introduce low-band CA with switching. The contribution provide initial discussion on the low-band CA with the new carrier switching feature.

2. Discussion

Considering the propagation distance and the benefit for mobile communication coverage, the low band CA is always operator’s interest. As stated in [2], it is quite challenge for OEM to implement low-low band CA due to the frequency spanning and the front-end component capability when SDL band n29 involves low bands carrier aggregation from single antenna design perspective. But it would be cost up and addition volume in the form factor if implement the carrier aggregation with multiple low-band antennas. The approved WI[1] introduces low band switching to make the low-band CA such as CA_n5-n29 become feasible to ease UE antenna design challenge that brings more flexibility on the low band spectrum utilization.

The scope of the WI is copied below:

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives:

-	Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1]

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

-	RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary

-	Specify the switching delay and time mask for carrier switching [RAN4]

-	Specify necessary RRM requirements [RAN4]

-	Define the corresponding UE capabilities [RAN4, RAN2, RAN1]

-	Consider the following deployment constraints:

-	The carrier frequency for all cases is <1 GHz

-	Co-located and synchronized network deployment for both carriers

-	Both carriers are in a single TAG

-	SCS 15KHz on both carriers

Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach

Note 2: Strive to minimize the RAN1 impact

Downlink carrier aggregation had been introduced for several releases and the example combo in the WI scope had been introduced by RAN4. From UE RF requirements perspective, if the example combos are completed CA/SUL configurations in the existing specs, there may be no additional RF requirements needed to be specified except switching delay and time mask for carrier switching.

Observation 1: For the combos that are existed in the spec, no additional RF requirements would be needed to be specified except switching delay and time mask for carrier switching WI

The RAN4 work is to specify switching gap between case 1 and case 2 stated in the WI scope. Since the WI scope is target on “specify only a semi-static switching pattern defined by RAN1 based on RRC configuration”, UE would expect the switching is pre-configured switching pattern.

Proposal 1: WI scope clarification: The WI only targets those completed combos in the existing R-19 or earlier release specs.

Proposal 2: It is expected no additional UE RF requirements based on the WI scope clarification except switching delay and the time mask

3. Conclusion

Observation 1: For the combos that are existed in the spec, no additional RF requirements would be needed to be specified except switching delay and time mask for carrier switching WI

Proposal 1: WI scope clarification: The WI only targets those completed combos in the existing R-19 or earlier release specs.

Proposal 2: It is expected no additional UE RF requirements based on the WI scope clarification except switching delay and the time mask

Reference:

- RP-243315 New WID on low band carrier aggregation via switching, RANP#106

- RP-241537 Rx switched nX-n29 low-low carrier aggregation, RANP#104