# 3GPP TSG-RAN WG4 Meeting #114	R4-2501127

Athens, GR, 17th – 21st February, 2025

Title: 	On RRM requirements of SSB adaptation

Source: 	OPPO

Agenda item:	7.25.2.2

Document for:	Discussion

Introduction

In last meeting, a WF was approved for SSB adaptation [1].

- Sub-topic 1-1 SSB adapation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - No RRM impacts for SSB adaptation requirement for IDLE mode. - No RRM impacts for Cell DTX. Issue 1-1-2: Scenario clarification- SSB periodicity Agreement: - Postpone the discussion and wait for RAN1 progress. Issue 1-1-3: Scenario clarification- SSB adaptation triggering Agreement: - RAN4 to focus on NW controlled SSB adaptation firstly. Issue 1-1-4: Scenario clarification- SSB adaptation mechanism Agreement: - Postpone the discussion and wait for RAN1 progress. - Sub-topic 1-2 SSB adaptation – L1 Requirements impacts Issue 1-2-1: L1 requirements at transition Agreement: - L1 evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation. - FFS the applicable L1 requirements. Issue 1-2-2: L1 requirements subsequent to “transition period” Agreement: - Subsequent to transition period, requirements apply based on the SSB periodicity after transition for SSB adaptation of periodicity. - Note: The transition period is discussed in other issues. - Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement: - Wait for LS reply on whether there is impact on SMTC.

In this contribution, we will provide discussion on RRM requirements for SSB adaptation.

Discussion

RAN4 has agreed to discuss L1/L3 measurement requirement impact as starting point [1]. There are different kinds of measurement to be considered by Rel-18, including L1 serving cell measurement, L1 neighbor cell measurement and L3 neighbor cell measurement.

In last meeting, we already provide 3 observations for L1 measurement on serving cells, L3 measurement on neighbor cells and L1 measurement on neighbor cells[2].

- Observation 1: For the impact of SSB adaptation on L1 measurement on serving cells, SSB periodicity can be reused.

- Observation 2: For the impact of SSB adaptation on L3 measurement on neighbor cells, SMTC periodicity can be used as baseline.

- Observation 3: For the impact of SSB adaptation on L1 measurement on neighbor cells, current time windows W can be reused, which already considers the periodicity of the target SSB.

Based on the agreements and observation above, RAN4 sent LS [3] to RAN2 to confirm whether new SMTC impact would be introduced due to SSB adaptation.

- LS on SSB adaptation（R4-2416911） However, L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration in legacy. RAN4 would like to ask RAN2 that whether there is any SMTC impact due to SSB adaptation. ACTION: RAN4 kindly asks RAN2 to answer the SMTC impact question due to SSB adaptation above.

In our view, we don’t pursue SMTC-based measurement before SCell activation, and provide 3 concerns from our side:

- Even though RAN4 has the common understanding that L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration, it is not always true because enhanced requirements based on SSB periodicity were introduced in R18 by replacing the SMTC periodicity.

- SSB adaptation and OD-SSB can be considered together. In the discussion of OD-SSB, most companies also support to use OD-SSB periodicity to speed up the measurement of deactivated SCell before and during SCell activation. But UE does not expect SMTC adaptation due to SSB adaptation or OD-SSB activation/deactivation as they may happen more frequently and dynamically, e.g., DCI or MAC-CE triggering. SMTC based on RRC configuration is used for measurements on both serving and neighbor cell on the same frequency layer, which should be better as persistent or semi-persistent. Thus, RAN4 should restrict the SMTC impact on all related cells in case of SSB adaptation on a certain cell.

- The OD-SSB before SCell activation is mainly for fast SCell activation, it is hard to restrict the L3 measurement to SMTC window considering the SMTC window is more designed for static SSB transmission. But now the OD-SSB can be dynamically (de)activated with various periodicity (FFS other parameters), so it is hard to always restrict the serving cell L3 measurement into a configured SMTC window (with measurement cycle), i.e., UE cannot wait for a coming SMTC window for L3 measurement, which may further delay the SCell activation. On the other hand, after the SCell has been activated, it seems OK to fallback and limit serving cell measurement as intra-frequency layers to the SMTC window as legacy.

Observation 1: Any SMTC impact should be considered for OD-SSB and SSB adaptation together.

Proposal 1: SMTC adaptation is not expected from UE’s perspective due to dynamic SSB adaptation or OD-SSB activation/deactivation.

Anyway, RAN4 should wait for further information in RAN2’s LS reply. The discussion of SSB adaption only case can be hold on in this meeting. But it should not impact the discussion of OD-SSB based SCell activation.

Conclusions

In this paper we provided our views on the SMTC impact on RRM requirements due to SSB adaptation and/or OD-SSB.

Observation 1: Any SMTC impact should be considered for OD-SSB and SSB adaptation together.

Proposal 1: SMTC adaptation is not expected from UE’s perspective due to dynamic SSB adaptation or OD-SSB activation/deactivation.

Reference

- R4-2420242, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei

- R4-2416122, On RRM requirements of SSB adaptation, OPPO

- R4-2416911, LS on SSB adaptation, Apple