# 3GPP TSG-RAN WG4 Meeting #114	R4-2501131

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.29.4.2

Source: 	OPPO

Title: 	On other requirements of NR NTN Phase 3

Document for:	Approval

Introduction

In last RAN4 meeting, a WF on RRM requirements for Rel-19 NR NTN phase3 was approved [1], including downlink coverage enhancements, support of regenerative payload, and support of (e)RedCap UEs with NR FR1-NTN.

In this paper, we will provide our views on DL coverage enhancement.

Discussion

### Issue 1-1: SSB periodicity enhancement Agreement (online): For SSB periodicity up to 160ms: - No further discussion on the impact on initial access from the perspective of RRM requirement - Discuss whether to Modify the default values for SSB and SMTC periodicities for the following if the UE is not provided with SMTC configuration or measurement object on the corresponding frequency by NW - RRC Re-establishment: TSMTC,i assumption - RRC connection release with redirection to NR: Trs assumption - Handover, Conditional handover, RACH-less HO and satellite switch with re-sync: Trs assumption - Further discuss the details on how to update the requirement. - Further discuss the following for SSB periodicity > 20ms: - Cell Reselection: for the SSB-periodicity of 160ms, whether the values for Tmeasure_NR_xxx_enh need to be updated for DRX Cycles equal to 0.32 s and other DRX cycles - Paging interruption in RRC Idle/Inactive: whether/how to adjust the requirements when SSB periods =160 ms in NGSO scenarios - RRC re-establishment: RRC re-establishment requirements for Serving cell SSB Ês/Iot < -8dB - Handover, Conditional handover, RACH-less HO and satellite switch with re-sync - Further discuss whether and how to update the requirement Agreement (online): For SSB periodicity larger than 160ms if supported by other WGs: - Whether RAN4 will define the RRM requirements for SSB periodicity larger than 160ms is depending on whether it is supported by WGs. ### Issue 1-3: RRM impacts due to Satellite Beam Hopping (e.g. Cell DTX/DRX) Agreement: - RAN4 to wait for more progress from RAN1/2 on the detailed solutions to adopt the concept of Rel-18 cell DTX/DRX based satellite beam hoping support before discussing whether and how to modify the relevant RRM requirements.

RAN4 discussed the impact of SSB periodicity extension (e.g., 320ms) in last meeting. IF RAN1/2 concluded on these, the related measurement requirements may need to be updated, e.g., timing, cell (re)-selection, L3 measurement/L1 measurement.

However, only few companies supported 320ms SSB periodicity in RAN4, and no consensus is reached for T/Rx beam sweeping. We think DTX/DRX, Dynamic and flexible power sharing could have limited impacts on RRM requirements of NTN. RAN4 can start the work after RAN1/2 reached more concrete conclusion. RAN4 can continuous to hold on these discussions in this meeting.

Proposal 1: RAN4 to hold on the discussion on SSB periodicity extension and Satellite Tx Beam Hopping until any progress in other WGs.

Conclusions

In this paper we provided our views on other RRM requirements for R19 NTN.

Proposal 1: RAN4 to hold on the discussion on SSB periodicity extension and Satellite Tx Beam Hopping until any progress in other WGs.

Reference

- R4-2420105, WF on RRM requirements for Rel-19 NR NTN phase3, Qualcomm