# 3GPP TSG-RAN WG4 Meeting #114	R4-2501133

# Athens, Greece, 17th – 21st February, 2025

Agenda Item:	7.21.2

Source: 	OPPO

Title: 	Study of impacts on RAN4 requirements for AI mobility

Document for:	Approval

- Introduction

In RAN4#113 meeting, the RRM requirements for AI mobility were discussed. The agreements and open issues were captured in the WF [1] and topic summary [2] respectively.

In this contribution, we will further evaluate the impacts on RRM requirements based on RAN2 progress and current RAN4 discussions, and some background information derived from RAN2 SI is provided in Annex.

- Discussion

Performance Metrics/KPIs for Measurement Prediction use case

In RAN4#112bis meeting, the following agreements were reached:

Issue 1-1-2: Scenario/sub-use-case priorities Agreement: RAN4 to start the discussion for scenario 2, 3, 4. Further clarification can be discussed for the scenarios. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. Agreement: - Scenario 2, 3 and 4, the 3 cases can be discussed: - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed. Issue 2-1-1: Performance Metrics/KPIs for Measurement Prediction use case Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

For the purpose of study, all the three sub-use cases should be considered. Based on the definition of each sub-use case, from the point view of AI/ML model implementation：

- Sub-use case 1: The AI/ML model employs actual L1 beam-level measurements as the input to predict L1 beam-level measurement outputs. Subsequently, the inferred and actual measured L1 beam-level measurement results could be leveraged to generate L3 cell-level measurement outcomes. The entire process could be denoted by L1-L1-L3, with two segments: the L1-L1 AI/ML prediction forms the first segment, while the second involves L1-L3, potentially through legacy methodologies.

Figure 3-1

- Sub-use case 2: The AI/ML model uses actual L3 cell-level measurements as input to predict their L3 output. This entire process, could be represented by L1-L3-L3, also splits into two parts: initially obtaining actual L3 cell-level measurement results from L1 measurements (L1-L3, e.g., in legacy way with filter design or up to company’s implementation), followed by the L3-L3 RRM measurement prediction by AI/ML.

Figure 3-2

- Sub-use case 3: a L1-L3 end-to-end way, involves the AI/ML model using L1 beam-level measurements as input to foretell the L3 cell-level measurement output directly, using L1 as input and L3 as eventual output.

Figure 3-3

Each of these sub-use cases utilizes L1-level measurements as the origin and aims for L3-level measurement predictions as their destinations, the entire process or a portion thereof being replaced by AI/ML modules or solutions. The differences primarily pertain to various implementation methodologies for transitioning from L1 measurement to L3 prediction.

For RAN4 testing, RAN4 can treat this without excessive fragmentation into different AI/ML implementation strategies and the synergy between AI and traditional algorithms. All three sub-use cases should use L3-level measurements as test objects for RAN4 testing (Point C in the figure 4), rather than check L1/L3 beam-level results(e.g. point A1 or E in figure 4). Concerning sub-use case 1 and sub-use case 2, some additional parameters(if needed) may be included as part of the test conditions in the form of side conditions (e.g., L3 filter). Regarding the possible different performance at point C due to the three implementation methodologies, RAN4 can evaluate it and decide whether to define separate requirements or unified requirements during WI stage.

Figure 4 L1/L3 measurement framework

Observation 1: For Sub-use case 1/2/3, differences primarily pertain to various implementation methodologies for transitioning from L1 measurement to L3 prediction, the entire process or a portion thereof being replaced by AI/ML modules or solutions. For RAN4 testing, RAN4 can treat this without excessive fragmentation into different AI/ML implementation strategies and the synergy between AI and traditional algorithms.

Proposal 1: For all three sub-use cases, RAN4 to define prediction accuracy requirements and testing for L3-level measurements at point C.

Proposal 2: RAN4 to evaluate and decide whether to define separate or unified requirements for different sub-use cases during WI stage.

Regarding the definition of prediction accuracy, the following agreements were reached in RAN4#113 meeting:

Issue 2-1-9a: L3 RSRP prediction accuracy definition Agreement: - For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP - FFS Ground truth definition of L3-RSRP for FR1 and FR2

It was agreed that the absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP and the remaining issue is the definition of ground truth of L3-RSRP. In RAN2 simulation, the prediction accuracy is defined as the RSRP difference between predicted L3 cell level results and actual L3 cell level measurement results of the same cell. For RAN4, we understand there are two options for the definition of ground truth, one is to reuse the definition from RAN2, i.e., the ground truth is defined as the actual L3-RSRP measurement results based on legacy procedure at point C, and the other one is to use the expected measurement results calculated based on transmission power and channel modelling. From requirements perspective, we think both options are feasible. But considering the possible impact on model monitoring in LCM procedure, we understand using actual measurements based on legacy procedure may be more reasonable since UE cannot acquire the expected measurement results in the LCM implementation.

And then the next issue to be considered is how to get the actual L3-RSRP measurement. According to the measurement model in figure 4, from inputs at point A, UE needs to experience L1 filter, beam consolidation/selection and L3 filter to get L3 measurement results at point C in which the latter two parts are RRC configured. To define the prediction accuracy, certain channel modelling, beam consolidation configuration and L3 filter configuration can be considered as possible side conditions.

Regarding the relative accuracy, in legacy measurement requirements, it is defined as the RSRP measured from one cell compared to the RSRP measured from another cell on the same frequency, or between any two RSRP levels measured on the same cell. We understand the similar definition can be reused for AI based mobility, i.e., the relative accuracy can be defined as the RSRP difference between any two reported predicted L3-RSRP results on the same frequency.

Based on the agreements/observations reached in RAN2#127bis and RAN2#128 meeting, the prediction accuracy for intra-frequency temporal domain can be impacted by the UE speed, PW length, MRRT and skipping pattern, and the prediction accuracy for inter-frequency prediction can be impacted by the correlation coefficient between two frequency layers and cluster approach. We understand these parameters can be considered as the side conditions when defining the accuracy requirements in RAN4. Regarding whether and how to define different sets of requirements, we can evaluate it during WI stage.

RAN2#126 3 Prediction accuracy metric for RRM measurement cell level prediction is defined as “RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell” for all RRM sub cases

##### RAN2#127bis ##### Agreements on intra-frequency - For intra-frequency temporal domain, higher UE speeds result in larger prediction errors - Initially, increasing the OW length can enhance prediction accuracy in the temporal domain case A, especially when the OW is relatively short. However, once the OW exceeds a certain threshold, further increases do not yield significant benefits. Conversely, for PW, longer durations correlate with decreased prediction accuracy. RAN2 will not define the actual threshold and fast fading assumption. - Majority of companies observe that among sub cases 1, 2, and 3, at least with shorter prediction window sub case 2 demonstrates the highest prediction accuracy - Companies can provide multiple real time RSRP value(s) and/or average RSRP value over the entire window and should indicate in their simulation results what they have used. The companies should at least provide the results of only one value it should be the last value at the end of the PW. We will add two columns in the spreadsheet to capture the last value and the average value. ##### RAN2#128 Agreements/Observations to be captured in TR - The prediction accuracy for intra-frequency temporal domain case B reduces as MRRT increases. - For temporal domain case B, with the same MRRT, different skipping pattern can provide different prediction performance. Companies may report the adopted skipping pattern when providing simulation results. Companies are not required to run new simulations but can clarify in the spreadsheet. We can capture in the TR how the skipping patter affects the performance. - For the inter-frequency prediction, the evaluation results show that the higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy. FFS on observations on low correlations. - For inter-frequency, cluster as input (i.e. measurements from different cells as inputs )can improve the prediction accuracy than single cell as input. For temporal domain, the gains are unclear. RAN2 will focus on frequency domain for cluster based approach. - In cluster approach the model takes measurements from more than one cell as inputs.

Proposal 3: RAN4 to use the following definition for measurement prediction accuracy:

  - Absolute L3 Predicted RSRP Accuracy is defined as the RSRP difference between reported predicted L3-RSRP and the ground truth of L3-RSRP of the same cell at point C, where ground truth is defined as the actual L3-RSRP measurement results based on legacy measurement procedure.

  - Relative L3 Predicted RSRP Accuracy can be defined as the RSRP difference between any two reported predicted L3-RSRP results.

Proposal 4: The above definitions apply to both intra-frequency and inter-frequency measurement prediction.

Proposal 5: RAN4 to consider certain channel model, beam consolidation configuration and L3 filter configuration as possible side conditions when defining the requirements.

Proposal 6: RAN4 to consider the impact of UE speed, PW length, MRRT or skipping patterns on intra-frequency measurement prediction accuracy requirements.

Proposal 7: RAN4 to consider the impact of correlation coefficient between two frequency layers or cluster approach on inter-frequency measurement prediction accuracy requirements.

Impacts of measurement errors on the prediction accuracy

In RAN4# 113 meeting, 4 options have been proposed for discussion on the impacts of measurement errors

Issue 2-1-3: Impacts of measurement error on prediction accuracy - Candidate options: - Option 1: RAN4 should study the impact of measurement error on the prediction accuracy during SI phase (MTK) - Option 2: RAN4 to not study the impact of measurement error on the prediction accuracy during SI phase (vivo, OPPO) - Option 3: RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4) - Option 4: RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event. (Apple, E///, NTT DCM, Nokia, Xiaomi, CATT)

Our understanding is:

- According to the experience of AI/ML for NR Air Interface, the feasibility study in RAN1/2 does not need to consider the impact of measurement error, instead, whether and how to evaluate the impacts on performance requirement should be considered within RAN4 scope in WI stage.

- The current discussion in RAN4 AI/ML BM case already draws attention to the impact of measurement error, including how this evaluation could be conducted and its potential influence on performance requirements. The simulation work is performing in the AI/ML work item, and we understand the evaluation methodologies should be similar for AI mobility. To avoid redundant work of these assessments, we’d better not to introduce the parallel evaluations/works in the research of AI mobility study at this stage.

- The simulation works for AI mobility in RAN2 are near the end and it is time to start the discussion for LCM framework, we think we should not introduce extra evaluation work to RAN2.

- Also, RAN4 needs to consider our own TU budget. It is challenging to complete within 0.5 TU for the measurement error evaluations/simulations. Aside from the discussions centered on measurement error, there are still a lot of unresolved issues in R19. We do not think that initiating evaluations on measurement error is the most critical task within the current 0.5 TU. framework. And it is better to focus on the possible requirements to be defined and the testability discussion for AI mobility.

Observation 2: To avoid redundant work of assessments, no need to introduce measurement error evaluations in the research of RAN4 AI mobility study at this stage as the similar work is performing in the AI/ML WI.

Observation 3: Initiating evaluations on measurement error is not the most critical task within the current 0.5 TU framework in RAN4.

Proposal 8: RAN4 to focus on the possible requirements to be defined and the testability discussion for AI mobility during SI stage.

Proposal 9: Whether and how to evaluate the impacts of measurement error on performance requirement should be considered within RAN4 scope in WI stage.

Impacts on Core requirements

In RAN4#112bis meeting, RAN4 agreed to start the discussion for scenario 2, 3, 4 as listed below:

| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| --- | --- | --- | --- | --- |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

Based on the following conclusions in RAN2, UE measurement behaviour can be different from legacy, e.g., UE can skip some measurement based on different MRRT values or OW/PW values for intra-frequency measurement prediction or UE can skip some measurement on certain frequency for inter-frequency measurement prediction.

Post email discussion after RAN2#127 meeting: In case A, continuous measurement results in prediction window are predicted by continuous historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either one or more sampling periods (with sliding L1/L3 filtering option) or one or more measurement periods (with non-sliding L1/L3 filtering option), where measurement result(s) are actually measured before sliding. For temporal domain case A, Company can report the observation window and prediction window with following limitation: 1, The ratio between observation window and prediction window is at least limited to value range {5,4,3,2,1,1/2,1/3,1/4,1/5} 2, The window length should be multiple times of sampling period or measurement period of corresponding frequency range. In case B, measurement results in prediction window are predicted by historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option) and measurement result(s) in previous prediction window is/are skipped during window sliding To agree following recommended value range for FR1 intra-frequency temporal domain case B: - MRRT: {50%~80%} - OW: {40ms ~ 2000ms} - PW: {40ms~800ms} RAN2#126 meeting: 5 Definition of measurement reduction for intra-frequency scenario is defined as:(20/20) Measurement reduction rate in temporal domain (MRRT) assuming same length of measurement time instances: MRRT= skipped measurement time instances / total measurement time instances Measurement reduction rate in spatial domain (MRRS): MRRS = skipped beams to be measured/ total beams to be measured

From RAN4 perspective, the measurement period requirements can be different according to the skipped measurements, e.g., the skipped SMTC occasions may not cause scheduling restrictions and the skipped frequencies may not need to be counted in the CSSF calculation.  The impact of skipped measurement instances and the corresponding measurement period requirements can be discussed in WI stage.

Proposal 10: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case A considering the impact of OW or PW length.

Proposal 11: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case B considering the impact of skipped measurement instances, e.g., based on MRRT values.

Proposal 12: RAN4 to discuss the measurement period requirements for inter-frequency measurement prediction considering the impact of skipped measurement frequencies or skipped measurement instances which may not need to be considered in the CSSF calculation.

Proposal 13: RAN4 to investigate the possibility of updates on scheduling restrictions, measurement capabilities or definition of known conditions due to measurement prediction.

- Conclusions

In this contribution, we discussed the possible RAN4 requirements for AI mobility and got the following observations and proposals:

Observation 1: For Sub-use case 1/2/3, differences primarily pertain to various implementation methodologies for transitioning from L1 measurement to L3 prediction, the entire process or a portion thereof being replaced by AI/ML modules or solutions. For RAN4 testing, RAN4 can treat this without excessive fragmentation into different AI/ML implementation strategies and the synergy between AI and traditional algorithms.

Proposal 1: For all three sub-use cases, RAN4 to define prediction accuracy requirements and testing for L3-level measurements at point C.

Proposal 2: RAN4 to evaluate and decide whether to define separate or unified requirements for different sub-use cases during WI stage.

Proposal 3: RAN4 to use the following definition for measurement prediction accuracy:

  - Absolute L3 Predicted RSRP Accuracy is defined as the RSRP difference between reported predicted L3-RSRP and the ground truth of L3-RSRP of the same cell at point C, where ground truth is defined as the actual L3-RSRP measurement results based on legacy measurement procedure.

  - Relative L3 Predicted RSRP Accuracy can be defined as the RSRP difference between any two reported predicted L3-RSRP results.

Proposal 4: The above definitions apply to both intra-frequency and inter-frequency measurement prediction.

Proposal 5: RAN4 to consider certain channel model, beam consolidation configuration and L3 filter configuration as possible side conditions when defining the requirements.

Proposal 6: RAN4 to consider the impact of UE speed, PW length, MRRT or skipping patterns on intra-frequency measurement prediction accuracy requirements.

Proposal 7: RAN4 to consider the impact of correlation coefficient between two frequency layers or cluster approach on inter-frequency measurement prediction accuracy requirements.

Observation 2: To avoid redundant work of assessments, no need to introduce measurement error evaluations in the research of RAN4 AI mobility study at this stage as the similar work is performing in the AI/ML WI.

Observation 3: Initiating evaluations on measurement error is not the most critical task within the current 0.5 TU framework in RAN4.

Proposal 8: RAN4 to focus on the possible requirements to be defined and the testability discussion for AI mobility during SI stage.

Proposal 9: Whether and how to evaluate the impacts of measurement error on performance requirement should be considered within RAN4 scope in WI stage.

Proposal 10: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case A considering the impact of OW or PW length.

Proposal 11: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case B considering the impact of skipped measurement instances, e.g., based on MRRT values.

Proposal 12: RAN4 to discuss the measurement period requirements for inter-frequency measurement prediction considering the impact of skipped measurement frequencies or skipped measurement instances which may not need to be considered in the CSSF calculation.

Proposal 13: RAN4 to investigate the possibility of updates on scheduling restrictions, measurement capabilities or definition of known conditions due to measurement prediction.

- References

R4-2420095 WF on RRM requirements for FS_NR_AIML_Mob

R4-2418278 Topic summary for [113][220] FS_NR_AIML_Mob

RP-234055 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

RP-240082 Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

R2-2407849 summary of [POST127][030][AI mobility] RRM simulation assumptions - Phase 1

R2-2407885 summary of [POST127][030][AI mobility] RRM simulation assumptions - phase 2

Annex: Background information

In RAN#102 meeting, the SID on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR is approved. In RAN#105 meeting, the related work plan in RAN4 has been further checked and updated as follows:

Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

Some background information derived from RAN2 SI is as follows:

To evaluate potential benefits and gains of AI/ML aided mobility, following aspects are agreed in SID[1] for study:

- Measurement prediction (including UE sided and NW sided model)

- HO failure/RLF prediction (only UE sided model)

- Measurement events prediction (only UE sided model)

And as clarified in RAN2 on TR 38.744, the study of the use cases is driven mainly by two study goals.

- The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements.

- The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption).

Regarding the RRM measurement prediction

- 3 sub-use cases are considered for cell-level RRM measurement prediction

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

- Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

- Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

- Temporal/frequency/spatial domain measurement prediction

- For temporal domain measurement prediction,

In RAN2#125bis meeting, it has been agreed as:

- For temporal domain measurement prediction, we will consider the AI-PHY beam management Case A and Case B from the RAN1 AI/ML PHY TR and it applies to both beam level and cell level. As baseline we will focus on pure temporal predication.

In addition, based on the post email discussion after RAN2#127 meeting, temporal domain case A and case B have been further clarified as:

Intra-frequency temporal domain case A: continuous measurement results in prediction window are predicted by continuous historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either one or more sampling periods (with sliding L1/L3 filtering option) or one or more measurement periods (with non-sliding L1/L3 filtering option), where measurement result(s) are actually measured before sliding.

Figure 1

Intra-frequency temporal domain case B: measurement results in prediction window are predicted by historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option) and measurement result(s) in previous prediction window is/are skipped during window sliding

Figure 2-1 temporal domain case example 1

Figure 2-2 temporal domain case example 2

- For frequency domain measurement prediction, typical feature is to use the measurement result from one frequency to predict the measurement result at another frequency, which can be applicable to FR1 to FR1 cases in R19 evaluation.

- For spatial domain measurement prediction, typical feature is to predict cell-level measurements through measurements of partial beams, which can be applicable to sub-use case 1 and 3 in R19.