# 3GPP TSG-RAN WG4 Meeting #114	R4-2501141

# Athens, Greece, 17th – 21st February, 2025

Agenda item:	5.21.3

Source:	OPPO

Title:	Reply LS on soft satellite switch with re-sync

Document for:	Approval

- Introduction

In this meeting, RAN4 received the LS [1] from RAN2 on soft satellite switch with re-sync asking whether the solution to introduce a temporarily shifted SSB pattern is feasible. In this contribution, we discuss the solution and give our proposal.

- Discussion

In previous meeting, RAN4 has discussed the feasibility of UE to perform the downlink synchronization with the target satellite and keep the communication with the source satellite of the same serving cell simultaneously in soft satellite switch and sent the following LS to RAN2 [2]:

RAN4 would like to thank RAN2 for the LS on RAN2 agreements for satellite switch with resync. RAN4 #110 had discussion on the feasibility of UE to perform the downlink synchronization with the target satellite and keep the communication with the source satellite of the same serving cell simultaneously in soft satellite switch. Following conclusion have been achieved: - It is feasible that a soft satellite switch capable UE can perform downlink synchronization with the target NGSO satellite and keep the connection (DL and UL) with the source NGSO satellite simultaneously under the following conditions. - Only if SSBs from the two satellites are spaced apart from each other at least by 1 OFDM symbol in the time domain at UE Rx side. - If the UE starts synchronization to a different satellite from T-ServiceStart, scheduling restriction is expected, if: - the UE is not capable of parallelMeasurementWithoutRestriction-r17, or - the UE is not capable of simultaneousRxDataSSB-DiffNumerology but the SSB has a different SCS than PDCCH/PDSCH - Note: RAN4 is investigating how to minimize the duration of the scheduling restriction.

To ensure non-colliding SSB with same PCI at the UE side, RAN2 has discussed one possible solution to introduce a temporarily shifted SSB pattern during [t-ServiceStart, t-Service], i.e., the target satellite transmits temporarily shifted SSB during [t-ServiceStart, t-Service] which is not overlapping with SSB from the source satellite. At t-Service, the new serving satellite SSB pattern (e.g., SSB offset) is restored to the same as the previous source satellite. The question from RAN2 about this solution is whether he pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. The illustration is shown as below:

From RAN4 perspective, we think the SSB pattern means the location of SSB symbols in the slot and frame, but does not affect the cell timing. So the SSB pattern change would not change the cell timing and synchronization information of UE, i.e., the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained.

Proposal 1: The pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

- Conclusion

This contribution discussed the solution of introducing temporarily shifted SSB pattern for soft satellite switch with re-sync and gave the following proposal:

Proposal 1: The pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

- References

- R4-2500012 (R2-2410978), LS on soft satellite switch with re-sync

- R4-2403493 Reply LS on RAN2 agreements for satellite switch with resync

Annex

3GPP TSG-RAN WG4 Meeting #114	                                                R4-25XXXX

Athens, Greece, 17th – 21st February, 2025

Title:	Reply LS on soft satellite switch with re-sync

Response to:		R2-2410978

Release:	Rel-18

Work Item:	NR_NTN_enh-Core

Source:	RAN4

To:	RAN2, RAN1

Cc:

Contact Person:

#### Name:	Qiuge Guo

E-mail Address:	guoqiuge@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1. Overall Description:

RAN4 thanks RAN2 for the LS on soft satellite switch with re-sync [1]. RAN4 has discussed the solution of introducing temporarily shifted SSB pattern during [t-ServiceStart, t-Service] and reached the following conclusion:

- The pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

# 2. Actions:

To RAN2, RAN1

ACTION: 	RAN4 kindly asks RAN2 and RAN1 to take the above information into account.

# 3. Date of Next RAN4 Meetings:

3GPP TSG RAN WG4#114bis	7 – 11 Apr, 2025		Wuhan, China

3GPP TSG RAN WG4#115	19 – 23 May, 2025		Malta