# 3GPP TSG-RAN WG4 Meeting #114	R4-2501156

# Athens, GR, 17th – 21st February, 2025

Source:	Sony

Title:	Consideration on the OTA test of ambient IoT device 1

Agenda item:	7.21.3.2

Document for:	Approval

# Background

In RAN#106 [1], the WI of Solutions for Ambient IoT (Internet of Things) in NR has been approved, where the following objects have been defined for RAN4:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

In particular, OTA test methods shall be study and developed for AIoT device 1. In this contribution, we provide our views on the testability issues that need to be addressed for OTA test on AIoT devices.

# Impact of RF energy source and CW

To be fully functional, AIoT devices require a dedicated energy source for harvesting to be provided during the test. Focused on the RF energy source in Rel-19, likely the RF energy source would be the it is still a question of how the energy source can be provided within the OTA test chamber. Taking an RF energy source as example, an external antenna may be needed to provide the CW and also provide RF energy, while a separate testing antenna is also needed, which are illustrated in Fig. 1. There are a few issues that need to be studied if such a test system is feasible:

- the power density of the CW and the power of the energy source that exist within the imagined quiet zone (QZ). To trigger the device during the test, the CW power level that impinged on the device needs to be above certain threshold. Therefore, a power density level of CW signal and the energy source needs to be defined in OTA test.

Proposal 1: the power density of CW signal and energy source needs to be defined for the OTA test of AIoT device 1.

- With a power level of the CW signal defined, the cross talk between the CW/energy source antenna and the testing (receiving) antenna is another issue that needs to be investigated. For the purpose of energy harvesting and also backscattering in the Tx testing, it is expected that the CW/RF energy source needs to be provided in the OTA test. For the Rx test, ideally, the device can perform the energy harvesting and DL reception in a TDD manner, so the cross talk between the CW/energy source antenna and the testing (receiving) antenna may not be critical. However, for the Tx testing since the device 1 needs to backscatter the incoming CW signal towards the testing (receiving) antenna, the cross talk between the CW/energy source antenna and the testing (receiving) antenna may significantly impact the measurement results and thus such an issue needs to be resolved.

Observation 1: For the Rx test, ideally, the device can perform the energy harvesting  and DL reception in a TDD manner, so the cross talk between the CW/energy source antenna and the testing (receiving) antenna may not be critical.

Observation 2: for the Tx testing, for the Tx testing since the device 1 needs to backscattered the incoming CW signal towards the testing (receiving) antenna, the cross talk between the CW/energy source antenna and the testing (receiving) antenna may significantly impact the measurement results.

Proposal 2: RAN4 shall study impact the due to the cross talk between the testing (receiving) antenna, the cross talk between the CW/energy source antenna for AIoT OTA test.

Figure. 1. OTA test chamber setup

# Impact of RF energy source and CW

Another major difference between AIoT devices and legacy UEs are the device behaviour due to the energy harvesting. In [2], it is assumed that the duration of one device's unavailability due to charging by energy harvesting can be assumed to be up to several tens of seconds, which was endorsed in RAN#103.

Observation 3: The duration of one device's unavailability due to charging by energy harvesting can be assumed up to several tens of seconds.

It is worth highlighting that this is an entirely different situation than for normal UEs in a test. Regular IoT devices or normal smartphones can operate over hours in conformance tests when intensive transmission and reception are performed. In addition, it is known that the conformance test can be very time-consuming, especially for OTA tests, as each test may last tens of minutes or even over an hour since the probe needs to scan a full space over the device. If an AIoT device were tested similarly, the test time may explode, considering that the device would be unavailable from time to time.

Observation 4: the transmission and reception behavior of AIoT devices are different from legacy UEs in 3GPP.

Proposal 3: RAN4 needs to study the device behaviour (availability and unavailability) due to energy harvesting and investigate how it would affect the conformance test design.

# Conclusion

In this paper, we provide our further analysis of the AIoT device RF implementation and requirement concept, where the following observations and proposals have been made:

Observation 1: For the Rx test, ideally, the device can perform the energy harvesting  and DL reception in a TDD manner, so the cross talk between the CW/energy source antenna and the testing (receiving) antenna may not be critical.

Observation 2: for the Tx testing, for the Tx testing since the device 1 needs to backscattered the incoming CW signal towards the testing (receiving) antenna, the cross talk between the CW/energy source antenna and the testing (receiving) antenna may significantly impact the measurement results.

Observation 3: The duration of one device's unavailability due to charging by energy harvesting can be assumed up to several tens of seconds.

Observation 4: the transmission and reception behavior of AIoT devices are different from legacy UEs in 3GPP.

Proposal 1: the power density of CW signal and energy source needs to be defined for the OTA test of AIoT device 1.

Proposal 2: RAN4 shall study impact the due to the cross talk between the testing (receiving) antenna, the cross talk between the CW/energy source antenna for AIoT OTA test.

Proposal 3: RAN4 needs to study the device behaviour (availability and unavailability) due to energy harvesting and investigate how it would affect the conformance test design.

# References

- R4-2415234   Topic summary for [112bis][130] FS_Ambient_IoT_solutions_part2  Moderator(Huawei)

- RP-240116 "View on Ambient IoT scope". Qualcomm.