3GPP TSG-RAN WG4 Meeting #114	R4-2501169

Athens, Greece, 17th – 21st February, 2025

Agenda item:	7.8.2

Source:            	Google

Title:                	Discussion on low band carrier aggregation via switching

Document for:	Approval

# Introduction

In RAN#106, a new WID on low band carrier aggregation (LB CA) via switching was approved as one of Release 19 work items [1]. The objectives in the WID is to specify the requirements for the LB CA via switching between Case#1, i.e., Tx/Rx on FDD carrier 1 and no RX on SDL carrier 2, and Case#2, i.e., Rx on SDL carrier 2 and no Tx/RX on FDD carrier 1, which is shown as follows. According to the operator request, the CA_n5-n29A is proposed as the example band combination in this WID. In this contribution, we will focus on the RAN4 requirement part in the following discussion.

### 4.1 Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

# Discussion

In RAN#106, the LB CA via switching was approved as a new Rel-19 work item. The justification is that the LB traffic demand for some operators has been increasing a lot in these years, but some LB CA which is already introduced in the TS 38.101-1 cannot be applied to offload user traffic congestion due to LB antenna limitation for the handheld UE. In the current commercial handheld UE design, the LB CA may be usually implemented by the shared antenna for DL/UL transmission. Hence, the fractional bandwidth (BW/fc) for LB CA case can be much higher than other MHB/UHB CA case and throughput degradation would be expected due to the lower center frequency fc. In the other way, one possible solution to offload the data traffic would be that the network can reconfigure some UEs to anther band for the transmission. However, it is not always feasible for some operators who only has SDL band in addition to the serving band in their deployment area. Therefore, LB CA via switching can be a compromised method to avoid throughput degradation and the example band combination CA_n5A-n29A with SDL band as SCell is proposed as the first priority for further study.

In the WID, for LB CA via switching, two cases {Case#1, Case#2} are captured in the objective, where Case#1 is Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 and Case#2 is Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1. For RAN4 requirements, the switching gap for LB CA is the main issue needed to be detremined. Assuming that the LB CA switching pattern has been configured by the network, the switching delay for Case#1 to Case#2 and for Case#2 to Case#1 can be analysed from antenna and RF hardware perspective in the following.

Since the antenna implementation is usually shared between n5 and n29 for the current handheld UE design, the switching delay can be dominated by the RF hardware transition time. For Case#1 to Case#2, n5 Rx/Tx on FDD carrier 1 should be turned off and n29 Rx on SDL carrier 2 should be turn on. The possible RF hardware transition time may contain LO retuning time from n5 FDD carrier 1 to n29 SDL carrier 2, RFFE SPxT switching time from n5 to n29, LNA switching time from n5 to n29, and n5 PA ON to OFF time. For Case#2 to Case#1, n29 Rx on SDL carrier 2 should be turned off and n5 Rx/Tx on FDD carrier 1 should be turned on. The possible RF hardware transition time may contain LO retuning time from n29 SDL carrier 2 to n5 SDL carrier 2, RFFE SPxT switching time from n29 to n5, LNA switching time from n29 to n5, and n5 PA OFF to ON time. In our understanding, the LO retuning time would be < 200 microseconds (us) and RFFE SPxT switching time would be around several microseconds. The LNA switching time would be around ~10 us and PA ON/OFF time would be around ~10 us. Hence, in our view, considering the tolerance for different implementations, the switching delay for LB CA via switching for the example band CA_n5A-n29A would be at least 200 us from antenna and RF hardware perspective.

Observation 1: Considering the tolerance for different implementations, the switching delay for LB CA via switching for the example band CA_n5A-n29A would be at least 200 us from antenna and RF hardware perspective.

In the following table, there are current CA band combinations including CA_n5-n29 is captured from TS38.101-1 (V19.0.0). It is observed that only n5 and n29 are in the low band category (< 1 GHz) among these band combinations, where n1/n3/n25/n66 are in the mid-high band category and n77 is in the ultra-high band category. In our view, one bit indication may be enough to identify LB CA via switching feature between n5 and n29 in a reported band combination. Since it is a Rel-19 feature in the 5G later stage discussion, minimizing signaling overhead for the new introduced feature would be beneficial to reduce the implementation effort. Hence, it is preferred to use one bit indication per band combination for LB CA via switching feature.

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 4) |
| --- | --- | --- |
| CA_n1-n3-n5 | n1, n3, n5 |  |
| CA_n5-n25-n29 | n5, n25, n29 |  |
| CA_n5-n29-n66 | n5, n29, n66 |  |
| CA_n5-n29-n77 | n5, n29, n77 |  |
| CA_n5-n25-n29-n66 | n5, n25, n29, n66 |  |

Proposal 1: Since it is a Rel-19 feature in the 5G later stage discussion, minimizing signaling overhead for the new introduced feature would be beneficial to reduce the implementation effort. Hence, it is preferred to use one bit indication per band combination for LB CA via switching feature.

# Conclusion

The proposals in this contribution are summarized in the following.

Observation 1: Considering the tolerance for different implementations, the switching delay for LB CA via switching for the example band CA_n5A-n29A would be at least 200 us from antenna and RF hardware perspective.

Proposal 1: Since it is a Rel-19 feature in the 5G later stage discussion, minimizing signaling overhead for the new introduced feature would be beneficial to reduce the implementation effort. Hence, it is preferred to use one bit indication per band combination for LB CA via switching feature.

# Reference

- RP-243317, “New WID on low band carrier aggregation via switching”, RAN4 chair (Huawei), RAN#106

- RP-242532, “Motivation for a new work item on low band carrier aggregation via switching in Rel-19”, AT&T, Anterix, Apple, Bell Mobility, Boost Mobile Network, BT plc, Ericsson, Google, HiSilicon, Huawei, Murata, Nokia, OPPO, Qorvo, Qualcomm, Samsung, Skyworks Solutions Inc., Sony, Southern Linc, TELUS, vivo, ZTE, RAN#106

- RP-242591, “Overview on new RAN4 proposal and thinking on LB-LB CA with switching”, Samsung, RAN#106