3GPP TSG-RAN WG4 Meeting # 114		R4-2501215

# Athens, GR, 17th – 21st February, 2025

Agenda item:	5.27.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on requirements maintenance for R18 NES

Document for:	Discussion

# Introduction

RAN4 discussed the maintenance issues for Rel-18 NES in last RAN4 meeting with agreements captured in [1].

# Discussion

One remaining issue is about the definition of intra-frequency and inter-frequency measurement on the carrier with SSB-less SCell, which is also related to RAN2 parallel discussion. In previous RAN2 discussion, RAN2 achieved following agreements:

RAN2 agreements @ RAN2#127bis Agreements 1 From RAN2 perspective, no benefit is identified for configuring servingCellMO for inter-band SSB-less SCell. 2 No LS is sent to R4. 3 Discuss in the next meeting on whether to capture “For inter-band SSB-less SCell(s), this field is not present” in the conditional presence of servingCellMO, depending on R4 progress. 4 Postpone the discussion on intra-band SSB-less SCell.

In last RAN4 meeting, RAN4 agreed on following way forward on how to proceed and try to avoid coupled discussion among different WGs.

Sub-issue 1: Whether servingCellMO can be present for inter-band SSB-less SCell. Agreement: RAN4 provide analysis on the categorization of intra-frequency/inter-frequency, whether MG is needed and any other aspects (if identified) for following cases: - When servingCellMO is configured - When servingCellMO is not configured RAN4 provide the above analysis to RAN2 (if consensus can be reached) and wait for RAN2 decision on whether servingCellMO is present for carriers of SSB-less SCell.

As discussed in last meeting, the definition and corresponding requirements highly depends on whether servingCellMO can be configured or not. Based on the guidance above, RAN4 will provide analysis/information for two possible cases w.r.t servingCellMO is configured or not. And it is up to RAN2 to decide whether servingCellMO can be configured.

## When servingCellMO is configured

The definition of intra-frequency measurement is defined in 9.2 TS38.133 as follows:

### 9.2.1 Introduction A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the SSB of the serving cell and the centre frequency of the SSB of the neighbour cell indicated for measurement are the same, and the subcarrier spacing of the two SSBs are also the same. If the UE supports ncd-SSB-BWP-Wor-r18, a measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the reference SSB of the serving cell and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same. The reference SSB is the SSB defined in BWP-specific servingCellMO under BWP-DownlinkDedicated of active DL BWP. If the field is absent, the reference SSB is the SSB defined in servingCellMO under ServingCellConfig [2].

Based on the discussion happened during Redcap discussion, the common understanding on the interpretation of “SSB of serving cell” refers to the SSB configured in servingCellMO. Regarding the definition of intra-frequency and inter-frequency measurement, following the existing criteria and understanding, whether the neighbor cell measurement is defined as intra-frequency or inter-frequency depends on whether the centre frequency of the SSB of the neighbour cell are same as that of SSB configured in servingCellMO. If they are the same, then it is defined as intra-frequency; otherwise, inter-frequency. But it should be noted that this may contradict with definition of SSB-less SCell since the ssbfrequency shall be the same as absoluteFrequencySSB, which is not configured for SSB-less SCell.

Observation 1: When the servingCellMO is configured, based on existing intra-frequency definition, the neighbor cell measurement is defined as intra-frequency when centre frequency of the SSB of the neighbour cell is same as that of SSB configured in servingCellMO; otherwise, it is defined as intra-frequency.

Based on observation 1, the following question is whether MG is needed and which requirements shall apply. From our understanding, whether the MG is needed can also follow existing criteria. For instance, if the measurement is defined as intra-frequency, whether MG is needed shall be decided by the conditions in clause 9.2.1. For inter-frequency, the conditions are defined in clause 9.3.1.

Observation 2: When the servingCellMO is configured, whether MG is needed can be determined based on existing conditions as defined for intra-frequency/inter-frequency, and corresponding measurement requirements apply accordingly.

## When servingCellMO is not configured

When servingCellMO is not configured, according to existing definition, since the neighbor cell measurement cannot meet the conditions for intra-frequency as mentioned above, thus it will be always categorized as inter-frequency.

Observation 3: When the servingCellMO is not configured, based on existing intra-frequency definition, the neighbor cell measurement is defined as inter-frequency as there is no “SSB configured in servingCellMO”.

Regarding to whether MG is needed, following the same logic, whether the MG is needed can also follow existing criteria as defined in clause 9.3.1, and the corresponding requirements (without gap/with gap) can apply accordingly.

Observation 4: When the servingCellMO is not configured, whether MG is needed can be determined based on existing conditions as defined for inter-frequency measurement, and corresponding measurement requirements apply accordingly.

Based on analysis above, the following input can be provided to RAN2:

Proposal 1: RAN4 to provide following input to RAN2 regarding the definition of intra-frequency/inter-frequency, whether MG is needed and which requirements apply:

When the servingCellMO is configured:

- The neighbor cell measurement is defined as intra-frequency when centre frequency of the SSB of the neighbour cell is same as that of SSB configured in servingCellMO; otherwise, it is defined as intra-frequency

- Whether MG is needed can be determined based on existing conditions as defined for intra-frequency/inter-frequency, and corresponding measurement requirements apply accordingly.

When the servingCellMO is not configured:

- the neighbor cell measurement is defined as inter-frequency as there is no “SSB configured in servingCellMO

- whether MG is needed can be determined based on existing conditions as defined for inter-frequency measurement, and corresponding measurement requirements apply accordingly.

And it could be observed from above analysis, there is no further RAN4 spec impact which can already be reflected by s existing.

Proposal 2: Based Proposal 1, there is no further RAN4 spec impact.

# Conclusions

Observation 1: When the servingCellMO is configured, based on existing intra-frequency definition, the neighbor cell measurement is defined as intra-frequency when centre frequency of the SSB of the neighbour cell is same as that of SSB configured in servingCellMO; otherwise, it is defined as intra-frequency.

Observation 2: When the servingCellMO is configured, whether MG is needed can be determined based on existing conditions as defined for intra-frequency/inter-frequency, and corresponding measurement requirements apply accordingly.

Observation 3: When the servingCellMO is not configured, based on existing intra-frequency definition, the neighbor cell measurement is defined as inter-frequency as there is no “SSB configured in servingCellMO”.

Observation 4: When the servingCellMO is not configured, whether MG is needed can be determined based on existing conditions as defined for inter-frequency measurement, and corresponding measurement requirements apply accordingly.

Proposal 1: RAN4 to provide following input to RAN2 regarding the definition of intra-frequency/inter-frequency, whether MG is needed and which requirements apply:

When the servingCellMO is configured:

- The neighbor cell measurement is defined as intra-frequency when centre frequency of the SSB of the neighbour cell is same as that of SSB configured in servingCellMO; otherwise, it is defined as intra-frequency

- Whether MG is needed can be determined based on existing conditions as defined for intra-frequency/inter-frequency, and corresponding measurement requirements apply accordingly.

When the servingCellMO is not configured:

- the neighbor cell measurement is defined as inter-frequency as there is no “SSB configured in servingCellMO

- whether MG is needed can be determined based on existing conditions as defined for inter-frequency measurement, and corresponding measurement requirements apply accordingly.

Proposal 2: Based Proposal 1, there is no further RAN4 spec impact.

# References

[1] R4-2420121 WF on RRM requirements for NR network energy saving

# Annex

3GPP TSG RAN WG4 Meeting #114                                   R4-25xxxxx

# Athens, GR, 17th – 21st February, 2025

Title:	LS on neighbor cell measurement for Rel-18 inter-band SSB-less

Response to:

Release:	Rel-18

Work Item:	Netw_Energy_NR

Source:	RAN4

To:	RAN2

Cc:

Contact person:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

Based on following agreements in RAN4#113 meeting, RAN4 discuss the issue in RAN4#114 and achieve following agreements:

Agreement in RAN4#113 Agreement: RAN4 provide analysis on the categorization of intra-frequency/inter-frequency, whether MG is needed and any other aspects (if identified) for following cases: - When servingCellMO is configured - When servingCellMO is not configured RAN4 provide the above analysis to RAN2 (if consensus can be reached) and wait for RAN2 decision on whether servingCellMO is present for carriers of SSB-less SCell.

When the servingCellMO is configured:

- The neighbor cell measurement is defined as intra-frequency when centre frequency of the SSB of the neighbour cell is same as that of SSB configured in servingCellMO; otherwise, it is defined as intra-frequency

- Whether MG is needed can be determined based on existing conditions as defined for intra-frequency/inter-frequency, and corresponding measurement requirements apply accordingly.

When the servingCellMO is not configured:

- the neighbor cell measurement is defined as inter-frequency as there is no “SSB configured in servingCellMO

- whether MG is needed can be determined based on existing conditions as defined for inter-frequency measurement, and corresponding measurement requirements apply accordingly.

With above agreements, there is no further RAN4 spec impact, and it is up to RAN2 decision on whether servingCellMO can be present for SSB-less SCell.

# 2	Actions

To RAN2:

ACTION: 	RAN4 respectfully asks RAN2 to take the above into account in their future work.

# 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #114bis	7th – 11th April 2025			    TBD China

TSG-RAN WG4 Meeting #115	19th – 23th May 2025			Malta, MT