3GPP TSG-RAN WG4 Meeting # 114	R4-2501223

# Athens, GR, 17th – 21st February, 2025

Agenda item:	7.31.5

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impacts for IoT NTN TDD mode

Document for:	Discussion

# Introduction

The new WID of NTN IoT TDD mode was approved in [1] and was revised in [2]. RAN4 studied the impact due to periodic pattern from RAN4#113 meeting with agreements captured in [3]. In this paper, we provide our views on the potential impacts on RRM impacts.

# Discussion

The objective of the WID is shown as follows:

The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands. The work objectives assume the following: - LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763) - Target the 1616-1626.5 MHz MSS allocated band - Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT) - Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO - The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with N=9 for the target MSS allocated band. This work item includes the following objectives: - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

Based on the discussion in last RAN4 meeting, the agreements are captured as follows:

### Issue 2-1: Impact on RRM requirements Agreement: - From the RRM requirement point of view, no impact on the following requirements is expected due to the new periodic pattern(s): - UE uplink timing - PUR Agreement: - For the following RRM requirements, modifications are expected due to the new periodic pattern(s): - RLM (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.) - MSG3-based channel quality report (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.) - Cell re-selection (example modifications include extended cell detection/evaluation/measurement periods, side conditions, etc.) - RRC Re-establishment (example modifications include extended cell detection/measurement periods, side conditions, etc.) - RRC Connection Redirection to Non-anchor Carrier (example modifications include updates to the definition of the symbols, side conditions, etc.) - Cell measurements (example modifications include extended cell detection/measurement periods, side conditions, etc.) Note 1: The exact requirement modifications are subject to further discussion in future RAN4 meetings based on RAN1 conclusion/progress on this feature. Note 2: Different modifications can be made for different TDD patterns, if multiple, DRX configurations, SNR values, etc. Note 3: Not all existing requirements for NB-IoT NTN may be applicable to the IoT_NTN_TDD feature, which is subject to further discussions in future RAN4 meetings. Note 4: It is assumed that UE will be made aware of the periodic pattern of frequency layers to be measured. Note 5: UE power consumption aspect may be considered when modifying the above requirements as needed. Note 6: The requirement applicability rule defined in Rel-17/18 applies to the IoT_NTN_TDD feature as well. For instance, the above requirements may not be applicable for inter-satellite scenarios if the UE is not provided with the target inter-satellite’s information.

For the applicable scenario, it is stated in the WID that LEO @600 km and @1200 km with Set-1 parameters captured in TR 36.763 will be taken as the reference scenario. It could be also observed from RAN1 discussion that the targeting operation CNR is rather high, which is shown as follows.

Agreement The target operating DL CNR of NB-IoT NTN TDD is obtained following the parameters in TR 36.763 and modifying the carrier frequency to 1.6GHz: - For LEO-600, the operating DL CNR is 4.91dB (i.e. with 0dBi UE antenna gain) - For LEO-1200, the operating DL CNR is 5.51dB (i.e. with 0dBi UE antenna gain) - When reporting link level simulation results, companies are encouraged to report the CNR margin with respect to the above operating DL CNRs. - Companies may report values for other scenarios (e.g. UE antenna gain of -5.5 dBi, LEO-800) with the corresponding assumptions.

Considering that the cell detection time for NB-IoT in EC is much longer than that in NC, which may be further extended by the TDD pattern. RAN4 shall discuss whether to consider EC scenario.

Observation 1: The targeting operation CNR condition of the targeting scenario is rather high as agreed in RAN1.

Proposal 1: RAN4 to discuss whether to consider EC scenario when defining the requirements.

For Cell reselection requirements, as analysed in last meeting, no blind detection is assumed at the UE side. It means there should be a SMTC like indication to locate the location of transmitted DL subframe not only for serving cell but also for neighbour cell. For the agreed TDD pattern as N = 9, there is one NPSS occasion available every N frame, and there is on NSSS occasion available every N or 2*N frames, which is up to RAN1 further progress on DL subframes design. Compared with the length of DRX cycles, the periodicity is still shorter. However, based on principles of defining requirements, UE is not required to monitor every occasion within the DRX cycles for power saving considerations.

Proposal 2: For Cell reselection requirements, take the existing requirements as baseline.

- FFS whether to relaxed/scaled the requirements for cell detection for short DRX cycles.

For RRC re-establishment requirements. the requirements are defined in absolute time, e..g 1400ms per frequency layer for NC and 14800ms per frequency layer for EC. It is straight forward that the requirements shall be scaled. For instance, if there is one NPSS/NSSS available every N frame, the requirements shall be scaled by at least N.

Proposal 3: For RRC re-establishment requirements, the search time per frequency layer shall be scaled by N=9.

For RLM requirements, it is the same story when DRX is not configured. It is assumed 400/200 ms are need for OOS/INS evaluation when Rmax is smaller than 64 and 4000/2000 ms when Rmax is larger than 64. However, the RLM measurement is basically the SINR evaluation. For large Rmax, UE is expected to operating in lower SINR conditions, thus longer evaluation time is allowed. However, due to the TDD pattern, the available DL subframes is not the same when configured with same Rmax. Thus, RAN4 shall discuss whether the change the limit on NPDCCH Rmax.

Proposal 4: For RLM, the evaluation period when DRX is not configured shall be scaled by N=9. And RAN4 to discuss whether to the change the limit on NPDCCH Rmax.

For measurement requirements in connected mode, which are further divided into serving cell measurement and neighbour cell measurement. For serving cell measurement and intra-frequency measurement, same situation as RLM, when DRX is not configured, the delay requirement shall be scaled.

However, for inter-frequency measurement, the delay requirements are defined in number of available samples. RAN4 defined minimum length of occasion for detection and measurement which is 200/50ms. For TDD with periodic pattern, obviously that the minimum length shall be reconsidered. For instance, if the NSSS periodicity is 90 ms, the minimum length of one measurement occasion should be 90ms.

Proposal 5: For connected mode serving cell measurement and intra-frequency measurement, the delay requirements shall be scaled by N=9. For inter-frequency measurement, the minimum length of measurement occasions shall be defined as the max {existing value, periodicity of corresponding RS}.

For channel quality report requirements, there is requirements for MSG-3 based measurement reporting and connected mode channel quality report. For core requirements, it could be observed that RAN4 only defines the expected measurement period to derive the channel quality (e.g. the measurement period over the NPDCCH carries UL grant). However, for mapping table and performance requirements, as could be observed, it is related to the repetition levels. As mentioned above, the available DL frame maybe different due to periodic TDD patterns. Thus, the impact on mapping table and performance requirements could be FFS in performance part.

Proposal 6: For channel quality report, the existing core requirements can be reused as baseline. The impact on mapping table and performance requirements could be FFS in performance part.

# Conclusions

Observation 1: The targeting operation CNR condition of the targeting scenario is rather high as agreed in RAN1.

Proposal 1: RAN4 to discuss whether to consider EC scenario when defining the requirements.

Proposal 2: For Cell reselection requirements, take the existing requirements as baseline.

- FFS whether to relaxed/scaled the requirements for cell detection for short DRX cycles.

Proposal 3: For RRC re-establishment requirements, the search time per frequency layer shall be scaled by N=9.

Proposal 4: For RLM, the evaluation period when DRX is not configured shall be scaled by N=9. And RAN4 to discuss whether to the change the limit on NPDCCH Rmax.

Proposal 5: For connected mode serving cell measurement and intra-frequency measurement, the delay requirements shall be scaled by N=9. For inter-frequency measurement, the minimum length of measurement occasions shall be defined as the max {existing value, periodicity of corresponding RS}.

Proposal 6: For channel quality report, the existing core requirements can be reused as baseline. The impact on mapping table and performance requirements could be FFS in performance part.

# References

[1] RP-242415 New WID on introduction of IoT-NTN TDD mode

[2] RP-243293 Revised WID on introduction of IoT-NTN TDD mode

[3] R4-2419795 Way Forward for [113][317] IoT_NTN_TDD

[2] R4-164550 WF on NB-IoT cell identification