3GPP TSG-RAN WG4 Meeting #114	R4-2501262

Athens, Greece, 17 – 21 February, 2025

Title: 	Discussion on RRM requirements for less than 5MHz for NTN

Source: 	Huawei, HiSilicon

Agenda item:	7.10.3.4

Document for:	Discussion

Introduction

RRM requirements for less than 5MHz in NTN are discussed in RAN4#113, and the outcomes are captured in WF [1]. Based on the discussion, there is one remaining issue which needs to be further discussed.

- Measurement (SSB index detection requirements)

In this paper, we will provide our views on RRM requirements for less than 5MHz in NTN.

Discussion

### Issue 1-1-2: Measurement(SSB index detection requirements) Wayforward: - Option 1: - The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively. - Option 2: Two sets of requirements - Set 1: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively. - Set 2: smaller delay with higher SNR side condition. - FFS on the exact number of delay to be extended base on the simulation results under the same simulation assumptions for TN less than 5MHz [R4-2306400] - FFS on the separate UE capability to support set 2 requirements

Our first preference is option 1, i.e. to define a single set of requirements based on same extension as in TN. Technically, the performance impact of PBCH puncturing is same for TN and NTN. As the SINR condition is same for TN and NTN, the extension in NTN should be same as TN.

On the other hand, we can understand the concern from some companies that the total delay with extension may be very long especially when SSB period 160ms is used. If there is strong view to define another set of requirements, we can compromise to define it based on -2dB SINR condition. It is the same condition as in HO, for which RAN4 has defined requirements by adding 2 additional SSB samples. In this way, we can avoid additional simulation efforts and new requirements to UE at another SINR. We do not have strong view to define new UE capability for the second set of requirements.

Proposal 1: Define a single set of requirements for SSB index detection by adding [4] and [3] SSB samples for intra-frequency and inter-frequency cases, respectively.

Proposal 2: If Proposal 1 is not agreeable, consider to define a second set of requirements for SSB index detection by adding 2 SSB samples at Es/Iot of -2dB.

Conclusions

In this paper we provided provide our views on RRM requirements for less than 5MHz in NTN.

Proposal 1: Define a single set of requirements for SSB index detection by adding [4] and [3] SSB samples for intra-frequency and inter-frequency cases, respectively.

Proposal 2: If Proposal 1 is not agreeable, consider to define a second set of requirements for SSB index detection by adding 2 SSB samples at Es/Iot of -2dB.

Reference

- R4-2420091, WF on RRM requirements for NR_IoT_NTN_req_test_enh, Moderator (Xiaomi)