3GPP TSG-RAN WG4 Meeting #114	R4-2501268

Athens, Greece, 17 – 21 February, 2025

Title: 	Discussion on RRM impacts for SBFD operation

Source: 	Huawei, HiSilicon

Agenda item:	7.23.3.2

Document for:	Approval

Introduction

RRM requirements for SBFD are discussed in RAN4#113, and the outcomes are captured in WF [1]. Based on the discussion, following issues needs to be further discussed for RRM impacts for SBFD operation.

- CSI-RS based L1 measurement

- CSI-RS based L3 measurement

- Scheduling restriction

- RACH requirements

- MG and BWP switch

In this paper, we will provide our views on RRM impacts for SBFD operation.

Discussion

CSI-RS based L1 measurement

For CSI-RS based L1 measurement, RAN4 discussed the impacts of 3 aspects, namely CSI-RS colliding with UL transmission, CSI-RS puncturing in frequency domain in SBFD symbols, and CSI-RS distributed over SBFD and non-SBFD symbols. For the first and third aspects, RAN4 has sent LS to ask for clarification with RAN1 on UE behaviour, so RAN4 can wait for RAN1 reply before further discussing the impacts on the requirements. For the second aspects, UE behaviour and requirements applicability for DU (or UD) case is clear, and what remains open is DUD case.

The first issue we want to address is single shot measurement v.s. sequential measurement.

First of all, the L1 measurement should be wideband, i.e. the measurement should reflect the channel status (RSRP or SINR) across 2 DL subbands on the OFDM symbol carrying the CSI-RS resource occasion. RAN1 has not defined any per DL subband reporting for L1 measurement.

In last meeting, some companies mentioned sequential measurement. If we understand correctly, it means UE takes measurement of the first DL subband in the first CSI-RS resource occasion, and the second DL subband in the next CSI-RS resource occasion. In this way, the measurement period would be doubled compared to existing requirements because each measurement occasion consists 2 but not 1 CSI-RS resource occasions.

We do not support to assume the measurement as sequential. If UE takes separate measurements for different DL subbands on different CSI-RS resource occasions, the combined measurement will not have a clear reference time point, and the usefulness of the measurement is questionable. Also, the processing efforts for punctured CSI-RS measurement is no larger than normal CSI-RS measurement. CSI-RS resource puncturing already occurs with SSB and CSI-RS on the same symbols, and no sequential measurement was assumed. Last, RAN1 has not defined new time line for sequential measurement e.g. for aperiodic CSI-RS resource.

Proposal 1: For DUD case, UE should be able to take wideband measurement across 2 DL subbands based on each CSI-RS resource occasion, i.e. measurement period for CSI-RS based L1 measurement is not changed.

The next issue we want to address is the applicability of existing accuracy requirements.

Existing requirements for CSI-RS based L1 measurement are defined based on a minimum number of PRBs for CSI-RS resource. For L1-RSRP and L1-SINR, this minimum number is 48, and for RLM/BFD/CBD, this minimum number is 24.

There are 3 scenarios to be discussed:

- Scenario 1: CSI-RS BW is >= the minimum number in each DL subband

- Scenario 2: CSI-RS BW is >= the minimum number in DL subband#1 but < the minimum number in DL subband#2

- Scenario 3: CSI-RS BW is < the minimum number in each DL subband but the total BW across 2 DL subbands is >= the minimum number

It is clear that existing accuracy requirement apply in Scenario 1.

For Scenario 2, we understand the existing accuracy requirements apply provided that the CSI-RS BW in DL subband #2 is not too small, e.g. for 48+8 it may be difficult for UE to account for the channel status on DL subband #2. We suggest that the CSI-RS BW in DL subband #2 is >= half of the minimum number of PRBs, i.e. 24 for L1-RSRP/L1-SINR and 12 for RLM/CBD/BFD. It is noted that RF has made following agreement in RAN4#113, and from this agreement we understand configuration of very asymmetric CSI-RS BW in the 2 DL subbands is unlikely to happen in real deployment.

Issue 1-3-2: DL/UL subband configurations support for SBFD Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB

For Scenario 3, we may need to discuss L1-RSRP/L1-SINR and RLM/CBD/BFD separately due to different Es/Iot conditions and different minimum number of PRBs. First, similar to Scenario 2, we suggest the CSI-RS BW in each DL subband should be >= half of the minimum number of PRBs. Under this condition, we believe existing accuracy requirements can apply for L1-RSRP/L1-SINR. For RLM/BFD/CBD, however, as the CSI-RS BW is limited (12 PRBs) in each DL subband, and the measurement is supposed to work at Es/Iot as low as -15dB, we understand the accuracy degradation is not negligible especially with fading channel with large delay spread (in existing test cases the channel is TDL-C 300ns). We suggest to either relax the accuracy requirements (which is reflected by the SNR levels compared to Qin/Qout levels), or to improve the channel condition e.g. reducing the delay spread.

Based on above analysis, we have the following proposals.

Proposal 2: For DUD case, for L1-RSRP/L1-SINR existing accuracy requirements apply provided that the number of consecutive CSI-RS PRBs is no less than 24 in each DL subband.

Proposal 3a: For DUD case, for RLM/BFD/CBD existing accuracy requirements apply provided that the number of consecutive CSI-RS PRBs is no less than 24 PRBs in one DL subband and no less than 12 in the other DL subband.

Proposal 3b: For DUD case, for RLM/BFD/CBD RAN4 to define relaxed accuracy requirements or update the propagation condition for the case where the number of consecutive CSI-RS PRBs is no less than 12 (but less than 24) in each DL subband.

CSI-RS based L3 measurement

For L3 CSI-RS measurement, RAN2#128 reached the following agreement.

Prioritization of SBFD cells / frequencies during cell reselection is not considered.

RAN2 wait for input from the other WGs regarding whether for inter-cell CSI-RS measurements, UE needs to be provided with information of the SBFD configuration of neighbouring cells.

In our understanding, there are 3-way forwards for L3 CSI-RS measurement.

- Option 1: provide UE information of the SBFD configuration of neighbouring cells, so that UE can adapt the measurement behavior based on this information in a similar manner as L1 measurement.

- Option 2: do not provide UE information of the SBFD configuration of neighbouring cells but UE assumes SBFD configuration of neighbouring cells is same as serving cell, so that UE can adapt the measurement behavior in a similar manner as L1 measurement.

- Option 3: do not provide UE information of the SBFD configuration of neighbouring cells, and UE performs the measurement in legacy way. It means if SBFD is used in neighbor cells, it should be made transparent to the measuring UE. In other words, NW implementation needs to make sure that all CSI-RS resources should be available according to the configuration, e.g. no CSI-RS RE is dropped due to SBFD operation in the neighbor cells compared to the configuration.

Our preference is option 3. On one hand, there will be non-SBFD aware UE also performing CSI-RS L3 measurement, and option 3 is the only choice for such UEs. On the other hand, there will be additional spec and UE implementation efforts following option 1 or option 2, e.g. UE needs to handle CSI-RS puncturing in frequency domain due to UL subband in neighbor cells. Considering the timeline of the WI and significance of this optimization, we prefer a simple solution with less impacts.

Following option 3, we understand existing requirements would apply with the condition that all CSI-RS resources configured for measurement are available according to the configuration. For collision between L3 CSI-RS measurement and UL transmission on SBFD symbols, we suggest to also follow existing scheduling restriction as in clause 9.10.2.6 for intra-frequency and use of MG for inter-frequency.

Proposal 4: For CSI-RS based L3 measurement, existing requirements apply provided that all CSI-RS resources configured for measurement are available according to the configuration.

If option 3 (or even option 2) is agreeable, RAN4 should send LS to inform RAN2 that information of the SBFD configuration of neighbouring cells is not needed. A draft LS is provided in the Annex.

Proposal 5: For CSI-RS based L3 measurement, RAN4 to inform RAN2 that information of the SBFD configuration of neighbouring cells is not needed.

Scheduling restriction

One issue in scheduling restriction is whether to update the existing requirements to take DL/UL transition time into account. For example, RAN4 has defined scheduling restriction for TDD for L3 measurement but not for L1 measurement. The reason is that there is always guard period for the DL-to-UL transition, and UL transmission on UL symbols would never overlap with SSB reception for the serving cell in a reasonable configuration. However, there is no guard period between SBFD symbol and SSB symbol. UL transmission on SBFD symbol immediately after SSB symbol may collide with SSB reception due to TA.

Based on discussions in last meeting, we understand such a collision is already addressed by the following RAN1#116 agreement. Therefore, we suggest RAN4 not to update the existing scheduling restriction requirements to account for DL/UL transition time.

Agreement For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

Taking L1 SSB measurement on SBFD symbols as example, not only the UL transmission on SSB symbols, but also the UL transmission on the symbol immediately after the SSB symbols would be dropped. Based on the note in the RAN1 agreement, we understand that this will be captured in RAN1 spec. In fact, dropping of additional UL transmission due to lack of sufficient transition time is not a specific issue for measurement, but it may also happen for other DL reception, e.g. UL transmission on the symbol immediately after an SBFD symbol where UE receives PDSCH will be dropped.

Considering that the impact of DL/UL transition will be addressed by RAN1 spec and it’s not limited to measurement, we suggest RAN4 not to update existing scheduling restriction or define new scheduling restriction to account for DL/UL transition time.

Proposal 6: RAN4 not to update existing scheduling restriction or define new scheduling restriction to account for DL/UL transition time.

Another issue raised up by some companies last meeting is the case when SSB or CSI-RS for neighbor cell is not within DL subband of UE’s serving cell.

- For SSB, such measurement is inter-frequency because serving cell SSB is always within DL subband. Inter-frequency SSB may be measured outside gaps if UE supports R16 inter-frequency measurement without gap and the SSB is within UE active BWP. In this case, UE would measure SSB outside DL subband, and the question is whether UE can receive data within the DL subband at the same time. Some companies mentioned that UE may not be able to do so and we are open to discuss.

- For CSI-RS, the issue is only for intra-frequency measurement because inter-frequency measurement is always performed in MGs and there is no need to define scheduling restriction. If UE performs the measurement in legacy way (option 3 in section 2.2), the neighbor cell CSI-RS is most likely within a DL subband of serving cell because neighbor cell CSI-RS has to be on the same center frequency as serving cell CSI-RS, and serving cell CSI-RS has to be within serving cell DL subband (otherwise the actually transmitted BW would not align with the configured BW for serving cell CSI-RS. If UE performs the measurement based on serving cell SBFD configuration (option 1 or 2 in section 2.2), UE would only measure CSI-RS resource REs within serving cell DL subband. In both cases, UE is not expected to measure intra-frequency CSI-RS outside serving cell DL subband. Based on the analysis, the issue may not be valid for CSI-RS measurement.

Proposal 7: RAN4 to discuss scheduling restriction when SSB for L3 measurement is within DL active BWP but not completely contained in DL subbands on SBFD symbols/slots.

RACH requirements

In last meeting, some companies suggested that RACH requirements may be impacted by the RAN2 agreement about RO switching, i.e. UE is allowed to switch from SBFD RACH occasions to legacy RACH occasions after certain (configured) number of times of RACH attempt.

In our view, however, the RACH requirements in RAN4 are quite generic, e.g. the details of RO selection for RACH re-attempt are not specified in 38.133 but a reference to RAN2 spec is used. In this sense, we do not see there is any technical impact due to SBFD operation on the RACH requirements.

6.2.2.2.1.2 Correct behaviour when receiving Random Access Response The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble. The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble. ###### 6.2.2.2.1.3 Correct behaviour when not receiving Random Access Response The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window defined in clause 5.1.4 in TS 38.321 [7].

On the other hand, the RACH requirements in RAN4 might need some editorial updates, e.g. if the RA resource selection for SBFD is defined in a new clause in 38.321 than clause 5.1.2, then we may need to add reference to the new clause. Besides, some wording in the RACH requirements may need to be adapted based on later RAN2 agreements, e.g. the applicable RO for RSRP based RO selection. Such updates can be discussed after RAN2 spec is stable.

Proposal 8: RAN4 to wait for RAN2 agreements/spec to decide whether RACH requirements needs to be updated due to SBFD operation.

MG and BWP switch

We do not see impacts of SBFD operation to BWP or CBW switch, as those procedures do not rely on measurement. However, some clarification may be needed because the requirements for DL (and UL) BWP switch are currently defined based on DL (and UL) transmission only on DL (and UL) slot. DL (and UL) transmission on SBFD symbols needs to be accounted.

For DCI-based BWP switch, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay + Y which starts from the beginning of DL slot n. Where, …

Similarly, we do not see impacts of SBFD operation to the MG related requirements, but one clarification may be needed for the UL transmission immediately after MG. Currently it is only defined for UL slot, while with SBFD operation, the UL transmission can be on SBFD symbol.

It is up to UE implementation whether or not the UE is able to conduct transmission in the following slot(s), - when MGTA is not applied, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after measurement gap - when MGTA is applied and the SCS of the UL carrier is other than 15kHz, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after measurement gap - when MGTA is applied and the SCS of the UL carrier is 15kHz, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after the slot partially overlapped with measurement gap where UL slot denotes that all the symbols in the slot are uplink symbols, and L=1 if for the UL transmission is less than the length of one slot; L=2 otherwise.

Proposal 9: RAN4 to update the term “UL slot” to “UL slot and SBFD slot” in MG and BWP switch requirements when SBFD operation is enabled.

Conclusions

In this paper we provided provide our views on RRM impacts for SBFD operation.

Proposal 1: For DUD case, UE should be able to take wideband measurement across 2 DL subbands based on each CSI-RS resource occasion, i.e. measurement period for CSI-RS based L1 measurement is not changed.

Proposal 2: For DUD case, for L1-RSRP/L1-SINR existing accuracy requirements apply provided that the number of consecutive CSI-RS PRBs is no less than 24 in each DL subband.

Proposal 3a: For DUD case, for RLM/BFD/CBD existing accuracy requirements apply provided that the number of consecutive CSI-RS PRBs is no less than 24 PRBs in one DL subband and no less than 12 in the other DL subband.

Proposal 3b: For DUD case, for RLM/BFD/CBD RAN4 to define relaxed accuracy requirements or update the propagation condition for the case where the number of consecutive CSI-RS PRBs is no less than 12 (but less than 24) in each DL subband.

Proposal 4: For CSI-RS based L3 measurement, existing requirements apply provided that all CSI-RS resources configured for measurement are available according to the configuration.

Proposal 5: For CSI-RS based L3 measurement, RAN4 to inform RAN2 that information of the SBFD configuration of neighbouring cells is not needed.

Proposal 6: RAN4 not to update existing scheduling restriction or define new scheduling restriction to account for DL/UL transition time.

Proposal 7: RAN4 to discuss scheduling restriction when SSB for L3 measurement is within DL active BWP but not completely contained in DL subbands on SBFD symbols/slots.

Proposal 8: RAN4 to wait for RAN2 agreements/spec to decide whether RACH requirements needs to be updated due to SBFD operation.

Proposal 9: RAN4 to update the term “UL slot” to “UL slot and SBFD slot” in MG and BWP switch requirements when SBFD operation is enabled.

Reference

- R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

Annex: draft LS on L3 CSI-RS measurement with SBFD

3GPP TSG-RAN WG4 Meeting #114 R4-250xxxx Athens, Greece, 17 – 21 February, 2025 Title: LS on L3 CSI-RS measurement with SBFD Response to: Release: Release 19 Work Item: NR_duplex_evo-Core Source: RAN4 To: RAN2 Cc: RAN1 Contact Person: #### Name: Li Zhang - E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: RAN4 has discussed the requirements for L3 CSI-RS measurement when SBFD is enabled in any of the serving and neighbour cells. RAN4 concludes that the existing measurement requirements apply under the condition that all CSI-RS resources configured for measurement are available according to the configuration. Based on the conclusion, RAN4 understands that information of the SBFD configuration of neighbouring cells is not needed in the configuration of L3 CSI-RS measurement. The applicability condition for the requirements mentioned above is to be guaranteed by network implementation. RAN4 respectfully asks RAN2 to take the above information into account for their future work. 2. Actions: To RAN2: RAN4 respectfully asks RAN2 to take the above information into account for their future work. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #114-bis April 7 – April 11, 2025 TBD, China RAN WG4 Meeting #115 May 19 – May 23, 2025 Malta