3GPP TSG-RAN WG4 Meeting #114	R4-2501287

Athens, Greece, 17th – 21st, February 2025

Source: 	Ericsson

Title:  	Discussion on RedCap RRM requirements for NTN for NR Phase 3

Agenda Item:	7.29.4.1

Document for:	Discussion

# Introduction

In the light of the WI NR Non-Terrestrial Networks (NTN) for NR Phase 3, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on on the underlying impacts to RedCap RRM requirements mandated by the issues listed in the WF.

# Discussion

## Topic #1: General consideration on RRM requirements

#### Issue 1-1-1: Whether to define measurement requirement for inter-RAT (NR NTN to E-UTRAN TN) in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN?

The related agreement is copied as follows:

RAN4#112: - For the requirements that have only been defined for (e)Redcap and not been defined for FR1-NTN, RAN4 NOT to define the following requirements for NTN (e)RedCap in Rel-19: - Inter-RAT measurements in idle/inactive/connected mode RAN4#112-bis: Issue 2-2-1: Further discussion on cell reselection requirement for (e)RedCap UE with FR1-NTN Online agreement <Agreement>: - The existing cell reselection requirements in RRC_IDLE/RRC_INACTIVE state defined for Rel-17 and Rel-18 (e)NTN can be the baseline for (e)RedCap 2Rx UE with FR1-NTN bands. - Both time-based and location-based cell reselection measurement criteria will be defined. - For both earth fixed and earth moving scenario - Both NTN-NTN and NTN-TN (from NTN to TN and from TN to NTN) cell reselection requirements will be defined. - FFS for the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

Since companies had different understandings for the agreement reached in RAN4#112 meeting, and some companies think that inter-RAT (NTN to E-UTRA TN) measurement in idle/inactive are not only defined for (e)RedCap with TN but also defined for normal UE with NTN (TS 38.133 clause 4.2C.2.11), so this issue need to be further revisited.

This issue only about whether to define the related requirements and how to define them for 1Rx/2Rx will be discussed in issue 2-1-1.

<Way Forward>: - Proposal 1 (CATT, Xiaomi, LG, Samsung CMCC, Apple, Ericsson, HW, ZTE): - RAN4 to define measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN. - Proposal 2 (vivo): - RAN4 not to define the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

Since inter-RAT (NTN to E-UTRAN TN) has been supported by NR FR1-NTN, for the same reason, inter-RAT may be supported by RedCap NTN. Another reason is RedCap TN already has supported inter-RAT measurement, it proves the validity of the inter-RAT cell change use case for RedCap.

Proposal 1: RAN4 to define measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN.

#### Issue 1-1-2: The Requirements to be further determined

<Way Forward>: - For SDT requirements in RRC_INACTIVE state, - Proposal 1 (CATT, QC, OPPO, HW): RAN4 to remove the RSRP-based TA validation in CG-SDT RRM requirements. - Proposal 1a (CATT): TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN. - Proposal 1b (LG): TA validation procedure can be skipped for SDT requirements if NTN (e)RedCap UE supports RACH-less HO or UE updates the NTA,adjcommon and NTA,adjUE before transmission. - Proposal 1c (CMCC): For CG-SDT TA validation, define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN. - Proposal 1d (vivo): For SDT requirement in RRC_INACTIVE state for (e)RedCap NTN, RAN4 to follow the same requirements on TA validation for the PUR requirements for IoT NTN (when stable) - Proposal 2 (ZTE): CG-SDT shall be used for RedCap UEs in NTN whatever it’s the GSO or NGSO for 1Rx and 2Rx RedCap UEs, and the requirements can be: Measurement; FR1. RSRP1; (T1 – min(800ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(800ms, M1*TDRX)). RSRP2; (T2 – min(800ms, M1*TDRX)) ≤ T2’ ≤ T2. Note: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment.. - For MDT requirements in RRC_IDLE and RRC_INACTIVE state - Proposal 1 (CATT, Samsung, QC, OPPO, Ericsson, vivo, HW, MTK): - Do not specify Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE with FR1-NTN as the same situation for (e)RedCap UE in TN system. - Proposal 1a (CATT): Deprioritize to define. - Proposal 2 (CMCC): - Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for RedCap over NTN, if the related features are supported in RAN1/2 spec. - Proposal 3 (ZTE): - RAN4 shall consider NTN cell size and RedCap UE capability and then decide whether to define relevant requirements.

Regarding SDT requirements in RRC_INACTIVE state, we agree on Option 1. Wherein, Proposal 1a, c, d are almost same without controversy, the essential intention is reusing/referring to PUR requirements for IoT NTN. Besides, Proposal 1b may be valid but we may stay keep same content as what we have done for IoT NTN and don’t need to pursue further refinements on the TA validation conditions.

Proposal 2: Regarding SDT requirements, agree on Proposal 1a, c, d in Option 1, and no need to pursue Proposal 1b.

As we presented in the last meeting, since MDT isn’t supported in RedCap for TN, we don’t see the necessity to introduce the feature for RedCap for NTN, which conflict with the lower-cost, lower-complexity and energy-efficient design target of RedCap.

Proposal 3: To our opinion, MDT for RedCap NTN isn’t a fundamental feature, we lean toward preclude it.

#### Issue 1-2-1: Relaxed measurement

Online agreement <Agreement>: - FFS: R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for normal UE are applicable for (e)RedCap UE with FR1-NTN bands. - FFS: RAN4 to define Rel-17 RedCap relaxed measurement requirements for cell reselection for (e)RedCap UE with FR1-NTN bands, and the corresponding requirements defined for RedCap UE can be the baseline. - If defined, the requirements apply provided that target cell is GSO

The conditions for TN RedCap relaxed measurement requirements in TS38.133 are duplicated as follows.

4.2B.2.9 Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion for RedCap 4.2B.2.9.1 Introduction This clause contains the requirements for measurements on intra-frequency NR cells when Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ and when the UE is configured any of the following relaxed measurement critera: - Relaxed measurement criterion for a stationary UE defined in clause 5.2.4.9.3 in [1], - Relaxed measurement criteria for a stationary UE and not-at-cell edge defined in clause 5.2.4.9.4 in [1], - Both low mobility criterion and stationary criterion as defined in clause 5.2.4.9.1 and 5.2.4.9.3 or 5.2.4.9.4 in [1] respectively. - Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in [1], - Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4.9.2 in [1], - Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4.9.1 and 5.2.4.9.2 in [1] respectively.

The conditions for R17 NR NTN relaxation requirements in GSO in TS38.133 are duplicated as follows.

4.2C.2.7 Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion The requirements in clause 4.2.2.9 apply provided that target cell’s satellite is GEO.

4.2.2.9 Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion 4.2.2.9.1 Introduction This clause contains the requirements for measurements on intra-frequency NR cells when Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ and when the UE is configured any of the following relaxed measurement critera: - Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in [1], - Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4.9.2 in [1], - Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4.9.1 and 5.2.4.9.2 in [1] respectively.

Refer to R17 NR NTN relaxation requirements in GSO, the UE shall meet the conditions in Clause 4.2.2.9.1 for relaxed measurement even the UE is in a GSO scenario.

For the question concerning the thresholds listed in WF, we understand we don’t need introduce new thresholds besides of the existing the criteria R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation).

Proposal 4:  No need to need introduce new thresholds besides of the existing the criteria R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation).

Regarding relaxation factor, given that TN R16 relaxation is adopted only, it is straightforward to only apply K1=3 for the relaxation as duplicated in the below table.

Proposal 5:  Same as TN R16 relaxation, K1=3 for the relaxation of RedCap NTN.

#### Issue 1-2-2: eDRX configuration

Online agreement < Agreement >: - For earth-moving LEO scenario: - All eDRX cycle cannot be applied for RedCap/eRedCap UEs in NTN and not to define requirements for eDRX in earth-moving beam LEO scenario. - For quasi-earth fixed LEO scenario: - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN. - Requirements are applicable for 2.56s and 5.12s eDRX cycle - FFS for 10.24s eDRX cycle and larger than 10.24s eDRX cycle

We understand the concern of FFS for 10.24s and longer eDRX cycle is the configuration may be not suitable for a relatively short duration of visibility of quasi-Earth LEO. However, for the duration of visibility which may be several minutes, some eDRX cycles longer than 10.24s are still applicable. We don’t suggest to preclude long eDRX cycles but skipping some eDRX cycles is acceptable from deployment perspective.

Proposal 6: We don’t suggest precluding long eDRX cycles from 10.24s. 40.96s or longer cycles may be excluded, but it can be up to NW implementation instead.

#### Issue 1-2-4: Multiple SMTCs

- Background R17 NTN UE capabilities related to 'in parallel' are copied as follows: Features; Index; Feature group; Components; Prerequisite feature groups; Field name in TS 38.331 [2]; Parent IE in TS 38.331 [2]; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 25. NR_NTN_solutions; 25-1; Parallel measurements on multiple SMTC-s for a single frequency carrier; Support of measurements on target cells belonging to 4 SMTC-s on a single frequency carrier; parallelSMTC-r17; MeasAndMobParametersCommon; FDD only; FR1 only; UE is mandatory to support 2 and can optionally support 4 if the feature is supported; Optional with capability signaling. 25. NR_NTN_solutions; 25-2; Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell; Support of measurements on cells belonging to different satellite as the serving cell in parallel with normal operation (i.e. data/control transmission and/or reception, and L1 measurements) of serving cell without scheduling restrictions. The feature is applicable only when the serving satellite is NGSO. If the serving cell belongs to GSO satellite, the scheduling restriction is not applied on the premise that a mixed type of satellites on the same frequency layer is not supported in this release (Rel-17).; parallelMeasurementWithoutRestriction-r17; BandNR; FDD only; FR1 only; For UEs not able to perform measurements in parallel with normal operation of serving cell scheduling restrictions shall apply.; Optional with capability signaling. 25. NR_NTN_solutions; 25-3; Parallel measurements with multiple measurement gaps; Support of 2 measurement gaps; parallelMeasurementGap-r17; MeasAndMobParametersCommon; FDD only; FR1 only; UE is mandatory to support 1 measurement gaps; Optional with capability signaling. 25. NR_NTN_solutions; 25-5; Parallel measurements on multiple NGSO satellites within a SMTC; Support of simultaneously measurements on target cells belonging to different NGSO satellites within a SMTC; maxNumber-NGSO-SatellitesWithinOneSMTC-r17; BandNR; FDD only; FR1 only; Candidate values for the number of NGSO satellites are 1,2,3, or 4. Online agreement <Agreements>: - RAN4 to define RRM requirements for multiple SMTC-s for a single frequency carrier for (e)RedCap UEs. - For whether to support parallel measurements on multiple NGSO satellites within a SMTC (Feature 25-5) for (e)RedCap UEs? - Option 1 (MTK, Xiaomi, Samsung, ZTE, Ericsson, LG, QC): Not support - Option 2 (HW, Nokia): FFS - Further discuss the supported number of SMTCs, the number of cells and SSB, the number of supported satellites, the proximity distance between two SMTCs. - Further discuss the specific measurement requirements, i.e, whether and how to extend measurement period. - Further discuss other R17 NTN UE capabilities (i.e, Feature 25-2/25-3) related to 'in parallel'.

Regarding measuring multiple SSBs in one SMTC, the issue is, if there are more than one SSBs, whether the RedCap UE is able to measure the SSBs simultaneously or one-by-one, i.e., support 25-5 or not, which is represented by NLEO,simul (the number of LEO satellites that UE can measure in parallel within an SMTC ) in RRM requirements.

In our opinion, the RedCap NTN UE may measure them not simultaneously.

Proposal 7: RedCap NTN UE may only support maxNumber-NGSO-SatellitesWithinOneSMTC-r17 =2. But we don’t opposite any enhancement based on that.

Regarding measuring multiple SMTC, the issue is how many SMTCs is supported by the RedCap UE.

Proposal 8: RedCap NTN UE may at the least support 2 SMTCs, i.e.,  parallelSMTC-r17 =2.

Furthermore, since collision between SMTCs has be defined for NR NTN, the RedCap NTN UE can reuse the collision condition and the corresponding measurement delay influence.

Proposal 9: RedCap NTN UE reuses the collision condition and the corresponding measurement delay influence for NR NTN when more than one SMTC is configured.

Taking the number of cells and number of SSB for an intra-frequency measurement layer as example, the ones for RedCap for TN, NR NTN and NR TN as follow.

The number of cells and number of SSB supported by RedCap for TN is duplicated as follows.

9.2B.3 Number of cells and number of SSB 9.2B.3.1 Requirements for FR1 For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 8 identified cells, and - 14 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell is not smaller than the number of configured RLM-RS SSB resources.

The number of cells and number of SSB supported NR TN is duplicated as follows.

9.2.3 Number of cells and number of SSB 9.2.3.1 Requirements for FR1 For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 8 identified cells, and - 14 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell (except for the SCell) is not smaller than the number of configured RLM-RS SSB resources.

The number of cells and number of SSB supported by NR NTN is duplicated as follows.

9.2C.3 Number of cells and number of SSB 9.2C.3.1 Requirements for FR1 For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 8 identified cells, and - 8 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell (except for the SCell) is not smaller than the number of configured RLM-RS SSB resources. - 4 SSBs with different SSB index and/or PCI from neighbour cells in GEO deployment. - In NGSO deployments: cells from 2 satellites including the satellite serving the PCell if UE does not support the capability maxNumber-NGSO-SatellitesPerCarrier-r17; or cells from 3 or 4 satellites satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-NGSO-SatellitesPerCarrier-r17.

It can be observed that, compared with RedCap for TN, the number of SSB supported by NR NTN is reduced because of taking beams from satellites into account. It looks that RedCap NTN might do the same.

Proposal 10: RedCap NTN reuses the number of cells and SSB for NR NTN, i.e.,

For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	8 identified cells, and

-	8 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell is not smaller than the number of configured RLM-RS SSB resources.

-	4 SSBs with different SSB index and/or PCI from neighbour cells in GEO deployment.

-	In NGSO deployments: cells from 2 satellites including the satellite serving the PCell if UE does not support the capability maxNumber-NGSO-SatellitesPerCarrier-r17; or cells from 3 or 4 satellites satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-NGSO-SatellitesPerCarrier-r17.

#### Issue 1-2-7: NCD-SSB

#### Issue 1-2-7: NCD-SSB <Way Forward>: - For the terminology of SSB used in the spec, - Proposal 1 (CATT, Ericsson, HW, CMCC): Use the terminology of SSB without differentiating CD-SSB/NCD-SSB. - For whether to add additional requirement applicability about CD-SSB/NCD-SSB, - Option 1 (CATT, Ericsson, HW): Add a clarification in a separate clause on this aspect (such as clause 3 in TS 38.133), and the wording can be: - The requirements defined for (e)RedCap UEs with FR1-NTN are only applicable to CD-SSB. - Option 2 (CMCC): No need to add additional requirement applicability about CD-SSB/NCD-SSB.

Proposal 11: Support Proposal 1, use the terminology of SSB without differentiating CD-SSB/NCD-SSB in requirement chapters. And support Option 1. Option 2 also is fine if no vague results are resulted in.

#### Issue 1-2-8: Other features

<Way Forward>: - Proposal 1 (CMCC): For the optional NTN features which have impact on RAN4 requirements and need enhanced UE capability, whether Redcap UE could optionally support it and whether RAN4 need to define requirements should be further discussed. - Set 1: propose to be optionally supported by Redcap over NTN - SMTC adjustment in idle/inactive - Location-based measurement report trigger - Enhanced RRM requirements for measurements in IDLE and INACTIVE modes - The number of target LEO satellites the UE can monitor per carrier including serving satellite - Further discuss the max number - Set 2: FFS - Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell - Parallel measurements on multiple NGSO satellites within a SMTC

We express the SMTC adjustment in idle/inactive mode, we understand it is up to UE to perform the operation, actually, RRM requirements doesn’t specify how the UE tracks/adjusts SMTCs.

Observation 1: RRM requirements doesn’t specify how the UE tracks/adjusts SMTCs.

The capability of parallelMeasurementWithoutRestriction plays a crucial role for NTN, we intend to support the capability for RedCap NTN UE.

Proposal 12: The capability parallelMeasurementWithoutRestriction is valid for RedCap UE.

## Topic #2: Reduction in the number of UE Rx branches

#### Issue 2-4-1: L1/L3 Measurement requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1 (CATT, Samsung, CMCC, HW): RAN4 to reuse the legacy FR1-NTN measurement requirements defined for normal UEs in high level. - Depend on the supported UE capabilities (e.g., multiple SMTC),, the existing NR NTN requirements may need be revised. - For 1Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1 (CATT, Samsung, CMCC, ZTE, HW): The time period for PSS/SSS detection defined for intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter. - Not to extend the lower boundary. - Proposal 1a (QC): Extended delays for PSS/SSS detection. - Proposal 1b (OPPO): Support to relax measurement requirements including extending the measurement delays with more measurement samples. - Proposal 2 (CATT, [Samsung?], CMCC, HW): The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter. - Extend the lower boundary from ‘120 ms’ to ‘160ms’. - Proposal 2a (QC): Extended delays for SSB index identification (PBCH decoding). - Proposal 2b (OPPO): Support to relax measurement requirements including extending the measurement delays with more measurement samples. - Proposal 3 (CATT, CMCC, HW): The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Extend the lower boundary from ‘200 ms’ to ‘400ms’ - Keep the same number of samples for T SSB_measurement_period_inter. - Proposal 4 (CATT, CMCC, ZTE, HW): The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same. - Proposal 5 (CATT, CMCC, HW): The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

Proposal 13: Support Proposal 1: The time period for PSS/SSS detection defined for intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter.

- Not to extend the lower boundary.

Proposal 14:  Support Proposal 2: The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

Proposal 15:  Support  Proposal 3: The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

Reuse the same extensions for 1Rx RedCap UE in TN network.

- Extend the lower boundary from ‘200 ms’ to ‘400ms’

- Keep the same number of samples for T SSB_measurement_period_inter.

#### Issue 2-4-2: Network verified UE location for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>: - Proposal 1 (vivo, [QC?], HW): - For 2Rx (e)RedCap UEs with FR1-NTN bands, the measurement period and measurement accuracy requirements for Network verified UE location defined in NTN can be reused. - For 1Rx (e)RedCap UEs with FR1-NTN bands, the measurement accuracy requirements defined for 1Rx RedCap positioning can be referred to, and the same relaxation for accuracy should be reused.

Proposal 16: Support Proposal 1:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, the measurement period and measurement accuracy requirements for Network verified UE location defined in NTN can be reused.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, the measurement accuracy requirements defined for 1Rx RedCap positioning can be referred to, and the same relaxation for accuracy should be reused.

#### Issue 2-5-1: L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1 (CATT, Samsung, CMCC, ZTE): RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN. - For 1Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1 (CATT, Xiaomi, Samsung, [QC?], CMCC, ZTE, [OPPO?], HW): RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs. - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused. - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB. - L1-RSRP: relax by 3dB.

Proposal 17: Support Proposal 1 for 2Rx (e)RedCap and 1Rx (e)RedCap.

## Topic #3: The impact of HD-FDD

#### Issue 3-1-1: General HD-FDD related impact

<Way Forward>: - Proposal 1 (CATT, Xiaomi, Samsung, QC, CMCC, ZTE, Ericsson, HW): - The legacy requirements and applicable conditions can be as baseline for defining requirements of RedCap UE in NTN scenario. - RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reaching further conclusions. - Proposal 1a (CATT, HW, MTK): - To following the TN, at least the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - Minimum requirement for L1 indication for RLM and LR - MAC-CE/DCI/RRC based uplink spatial relation switch delay - Scheduling availability of UE performing intra/inter measurements - Scheduling availability of UE during L1-RSRP measurement - Proposal 1b (QC): RAN4 to wait for further RAN1 process on the enhancements for HD RedCap and eRedCap UEs which may potentially impact the following aspects: - Additional latencies, Scheduling/Measurement restrictions, etc. due to DL/UL collisions. - Proposal 1c (CMCC): - Same principle for HD-FDD (e)RedCap UE in TN network will be followed as baseline. - Wait for further RAN1 progress which may impact Scheduling availability requirement.

Our position is that there is no issue to reuse TN HD-FDD requirements. However, even if RAN1 has settled collision case 3, there is still ongoing work regarding collision case 4. In RAN1 there is also deliberations regarding TA report enhancement. The RAN1 agreements and open issues after November meeting looks like this:

… Agreement For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3, - Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all the other use cases above - This is signaled by RRC configuration Note: UE shall comply to the following existing procedure in 38.331: UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13 … Agreement The collision case 4 (Dynamically scheduled DL reception collides with dynamic scheduled UL transmission) is not considered as an error case for Rel-19 HD-FDD (e)Redcap UE in RRC connected mode. - FFS whether to prioritize UL or prioritize DL …

We therefore support Proposal 1 above:

- The legacy requirements and applicable conditions can be as baseline for defining requirements of RedCap UE in NTN scenario.

- RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reaching further conclusions.

Proposal 18:

- The legacy requirements and applicable conditions can be as baseline for defining requirements of RedCap UE in NTN scenario.

- RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reaching further conclusions.

#### Issue 3-2-1: The impact on NR Handover due to HD-FDD

<Way Forward>: - Proposal 1 (CMCC): - For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added: - For RedCap NTN UE with HD-FDD, the requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU. - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added: - For RedCap NTN UE with HD-FDD, the requirements are met provided that - One SSB is available during T∆ - One SSB is available during TIU - Proposal 2 (Ericsson): - RAN4 to reuse NR SAN FR1 – NR SAN FR1 Handover and NR Satellite switching with re-synchronization requirements for Red-cap NTN and additionally add the following applicability rule for HD-FDD case, - For RedCap NTN UE with HD-FDD, the handover requirements and Satellite switching with re-synchronization requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU. - Proposal 3 (HW): - For HD-FDD (e)RedCap UE with FR1-NTN bands, the CHO and satellite switching with re-sync requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU. - Proposal 4 (Nokia): - Only support hard satellite switching for RedCap UEs

Proposal 19: No difference between Proposal 1, 2, 3. For supported HO, CHO and satellite switching with re-sync requirements are met provided that

- SSB is available at the UE once every SMTC period during Tsearch

- One SSB is available during T∆

- One SSB is available during TIU.

In our opinion, soft satellite switch doesn’t bring more complexities to UE implementation, the difference from hard switch is the time when starting DL sync. to the target satellite is determined by the UE.

Proposal 20: Slightly not agree Proposal 4, soft satellite switch  doesn’t bring more complexities to RedCap UE.

## Topic #4: Others

#### Issue 4-1-1: Specification structure

<Way Forward>: - Proposal 1 (CATT, Ericsson, HW): - Use new sections and section number with new suffix X in the specification to capture requirements for (e)RedCap UE with FR1-NTN bands. - Utilize the reference method for the requirements that can be reused. - Proposal 2 (Nokia): - Adopt RedCap subclauses under NTN sections (under Suffix C).

Proposal 21: We lean toward Proposal 1, in order to avoid further complexing the clauses for NTN.

#### Issue 4-2-1: How to declare the capabilities for (e)RedCap UE with FR1-NTN bands?

<Way Forward>: - Proposal 1 (Nokia): RAN4 to discuss whether a new capability is needed to declare RedCap support of NTN or whether this support is implicit from declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) AND declare support of NTN (nonTerrestrialNetwork-r17) - Proposal 1a (CMCC): Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). - Proposal 1b (Ericsson): RAN4 to define a new capability declaring (e) RedCap for NTN.

In the last meeting, we proposed Proposal 1b, but combination of supporting current RedCap and NTN may be feasible also.

Proposal 22: Both proposals, i.e., a new capability declaring (e) RedCap for NTN or combining (e)RedCap and NTN, are fine with us.  It may be determined later.

# Conclusion

Observation 1: RRM requirements doesn’t specify how the UE tracks/adjusts SMTCs.

Proposal 1: RAN4 to define measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN.

Proposal 2: Regarding SDT requirements, agree on Proposal 1a, c, d in Option 1, and no need to pursue Proposal 1b.

Proposal 3: To our opinion, MDT for RedCap NTN isn’t a fundamental feature, we lean toward preclude it.

Proposal 4:  No need to need introduce new thresholds besides of the existing the criteria R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation).

Proposal 5:  Same as TN R16 relaxation, K1=3 for the relaxation of RedCap NTN.

Proposal 6: We don’t suggest precluding long eDRX cycles from 10.24s. 40.96s or longer cycles may be excluded, but it can be up to NW implementation instead.

Proposal 7: RedCap NTN UE may only support maxNumber-NGSO-SatellitesWithinOneSMTC-r17 =2. But we don’t opposite any enhancement based on that.

Proposal 8: RedCap NTN UE may at the least support 2 SMTCs, i.e.,  parallelSMTC-r17 =2.

Proposal 9: RedCap NTN UE reuses the collision condition and the corresponding measurement delay influence for NR NTN when more than one SMTC is configured.

Proposal 10: RedCap NTN reuses the number of cells and SSB for NR NTN, i.e.,

For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	8 identified cells, and

-	8 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell is not smaller than the number of configured RLM-RS SSB resources.

-	4 SSBs with different SSB index and/or PCI from neighbour cells in GEO deployment.

-	In NGSO deployments: cells from 2 satellites including the satellite serving the PCell if UE does not support the capability maxNumber-NGSO-SatellitesPerCarrier-r17; or cells from 3 or 4 satellites satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-NGSO-SatellitesPerCarrier-r17.

Proposal 11: Support Proposal 1, use the terminology of SSB without differentiating CD-SSB/NCD-SSB in requirement chapters. And support Option 1. Option 2 also is fine if no vague results are resulted in.

Proposal 12: The capability parallelMeasurementWithoutRestriction is valid for RedCap UE.

Proposal 13: Support Proposal 1: The time period for PSS/SSS detection defined for intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter.

- Not to extend the lower boundary.

Proposal 14:  Support Proposal 2: The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

Proposal 15:  Support Proposal 3: The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

Reuse the same extensions for 1Rx RedCap UE in TN network.

- Extend the lower boundary from ‘200 ms’ to ‘400ms’

- Keep the same number of samples for T SSB_measurement_period_inter.

Proposal 16: Support Proposal 1:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, the measurement period and measurement accuracy requirements for Network verified UE location defined in NTN can be reused.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, the measurement accuracy requirements defined for 1Rx RedCap positioning can be referred to, and the same relaxation for accuracy should be reused.

Proposal 17: Support Proposal 1 for 2Rx (e)RedCap and 1Rx (e)RedCap.

Proposal 18:

- The legacy requirements and applicable conditions can be as baseline for defining requirements of RedCap UE in NTN scenario.

- RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reaching further conclusions.

Proposal 19: No difference between Proposal 1, 2, 3. For supported HO, CHO and satellite switching with re-sync requirements are met provided that

- SSB is available at the UE once every SMTC period during Tsearch

- One SSB is available during T∆

- One SSB is available during TIU.

Proposal 20: Slightly not agree Proposal 4, soft satellite switch  doesn’t bring more complexities to RedCap UE.

Proposal 21: We lean toward Proposal 1, in order to avoid further complexing the clauses for NTN.

Proposal 22: Both proposals, i.e., a new capability declaring (e) RedCap for NTN or combining (e)RedCap and NTN, are fine with us.  It may be determined later.

# References

- R4-2414037, The WF on RRM requirements for Rel-19 NR NTN phase3, CATT