3GPP TSG-RAN WG4 Meeting #114	R4-2501289

Athens, Greece, 17th – 21st, February 2025

Source: 	Ericsson

Title:  	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Agenda Item:	7.17.2.1

Document for:	Discussion

# Introduction

In the light of the WI of NR Radio Resource Management (RRM) enhancements (WID in [1]) for Rel-19, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [2].

This contribution outlines our point of view on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor as follows.

# Discussion

## Topic #1: FR2-1 SSB based L3 measurement delay reduction for connected mode

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF (including activation condition and de-activation conditions):

Agreements in RAN4 #112bis meeting:

Agreement in online session: For scenario 1/2/3/4: - RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions. - (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions. - The detailed criteria/condition(s) is FFS. - Meanwhile, RAN4 agree that the criteria/threshold(s) are configured by RRC signaling. FFS the further details. For Scenario 5/7: - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch). - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment.

Agreements in RAN4 #113 meeting:

Agreement: - RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement. Agreement: - Not support UE mobility status based triggering condition in Rel-19.

In the existing TS38.331, there are below RRC IEs concerning the serving cell’s signal quality which could be considered as the reference.

In SIB2, the threshold for cell edge examination is defined.

And the formula of cell edge threshold from TS38.304 is repeated in the below.

##### 5.2.4.9.2 Relaxed measurement criterion for UE not at cell edge The relaxed measurement criterion for UE not at cell edge is fulfilled when: - Srxlev > SSearchThresholdP, and, - Squal > SSearchThresholdQ, if SSearchThresholdQ is configured, Where: - Srxlev = current Srxlev value of the serving cell (dB). - Squal = current Squal value of the serving cell (dB).

In ReportConfigNR, Event A2 is defined for evaluating the (L3) measurement results,

And the parameters of A2 are listed in the below.

| Parameter | Range |  | Value |  |
| --- | --- | --- | --- | --- |
| RSRP threshold | 0 | 127 | -156 dBm | -31 dBm |
| RSRQ threshold | 0 | 127 | -40 dB | 20dB |
| SINR threshold | 0 | 127 | -23 dB | 40 dB |
| Hysteresis | 0 | 30 | 0 dB | 15 dB |

The two approaches - cell edge and A2 - essentially can serve as a guide for designing the multi-Rx triggering condition for L3 measurement.

The cell edge approach, by creating a relative threshold to S-criteria, isn’t intuitive and it doesn’t make sense or at least isn’t suggested to mix up the parameters for RRC_IDLE state and the parameters for RRC_CONNECTED state together.

On the contrary, it appears that the Event A2 approach is more straightforward, by defining an absolute threshold of RSRP/RSRQ. It’s a common understanding that the main scenario of FBS is the rapid mobility assessment, particularly when a UE experiences a low level or poor signal from the serving cell, necessitating a transition to an alternate serving cell. By configuring the parameters in the FBS triggering condition and Event A2 with different values respectively, the UE may enable FBS prior to meeting Event A2 for different purposes and doesn’t cause overlapping between the two conditions.

Proposal 1: Adopt Event A2 as the reference of FBS triggering condition, including RSRP, RSRQ, SINR threshold. Furthermore,

- The hysteresis parameter may be kept.

- The timeToTrigger indicates the validity time of the triggering condition, the use case for multi-Rx L3 measurement shall be checked further.

Observation 1: By various configurations, UE may enable FBS prior to meeting Event A2 for different purposes and doesn’t cause overlapping between the two conditions.

Yet another issue to be dealt with is whether the UE shall report the FBS validity after activating FBS with respect to the NW RRC signaling. It’s known that the purpose of the RRC signaling is to mandate the UE to activate FBS once the condition is met, the NW shall be aware of whether the UE is doing FBS or not, which is helpful for the NW managing the mobility or scheduling work more efficiently. The reporting is crucial in the case that the UE doesn’t report any measurement result if events-based measurement is configured, which is a common configuration.

Proposal 2: UE shall report FBS validity to NW after meeting activating FBS condition.

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”:

[Moderator]: discuss the following options(wording can be revised/polished) and companies can add more details if needed. For conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”: - Option 2 (LGE, OPPO, Ericsson, MTK, CTC): UE indication to the NW when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time). - Option 2a (LGE, OPPO, NTT DCM(scenario 1/2/3/4))Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. - Option 3 (HW, Samsung, Ericsson): Apart from the threshold conditions, UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results, since NW already has the candidate target cells for potential HO or SCell modification. - Option 7 (Samsung, Ericsson): Introduce validity timer configuration with certain ranges via dedicated RRC signalling

First of all, as far as we can recall, the majority view is that the UE shall report NW of deactivating FBS.

Proposal 3: UE shall report deactivating FBS to NW upon deactivation condition being met.

The subsequent issue is what is the condition for the UE determine deactivating FBS, which is controversial as per proposals raised in the last meeting.

In our opinion, the condition shall be specified in requirements, otherwise, UE may deactivate FBS arbitrarily without any clear restriction, which isn’t expected.

Proposal 4: RAN4 to study the below FBS deactivation conditions:

- After UE activates FBS:

  - The condition of activating FBS is invalid again.

    - To avoid too frequent ping-pong between FBS and non-FBS, a minimal during time in FBS/ non-FBS shall be defined.

  - The condition of activating FBS still is valid,

    - L3 measurement with FBS has conducted for a time period, or

      - As an example, the condition of activating FBS still is valid, but the UE is keeping a stationary at cell edge or there’s no qualified neighbour cell to switch. In this case, FBS may be disabled.

    - ﻿Internal overheating is detected.

- Before UE activates FBS, i.e., before meeting FBS activation condition:

  - Internal overheating is detected.

Yet a further question is, if the UE has deactivated FBS from activating FBS state, whether the UE shall apply the provided FBS RRC signaling and the FBS triggering condition again to re-active FBS without any further FBS RRC signaling?

We understand the issue can be resolved in more than one way. Among them, a simple solution is that UE shall re-evaluate the condition provided by NW. But, as described in the last proposal, a minimal during time in FBS/ non-FBS, to avoid too frequent ping-pong between FBS and non-FBS.

Proposal 5: After UE deactivates FBS, the UE may re-evaluate the condition of enabling FBS after a minimal time interval.

An achieved agreement about the handover procedure, which is repeated in the table below, specifies that FBS must always be used during Tsearch. We understand the purpose is that handover command is able to activate FBS for fast measurement and speeding up mobility.

For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch).

For the CHO delay requirements, Tmeasure is the measurement time stated in clause 6.1.4.4.2 of TS38.133. Regarding the description, ‘The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.’, it implies that Tmeasure is later than the time instant when the event is met. In this case, it’s reasonable to adopt FBS as well to speed up the CHO delay.

Proposal 6: For CHO, UE always applies FBS during Tmeasure.

We also observe that FBS for serving the mobility purpose may be triggered in an implicit way. For example, relaxed radio Link Monitoring (RLM) is designed for a UE to be able to operate these relaxation features when it is under good radio conditions and/or at a low mobility state. In other words, it means the radio conditions are stable, and no need to request the UE to perform fast beam sweeping or to shorten the measurement delay time on the neighbor cells. In this sense, there could be some relationships between BFD and/or RLM and FBS.

On the other hand, if beam failure or radio link failure is detected, FBS may be applied to speed up potential procedures. Since we have agreed that RRC re-establishment shall apply BFS, it actually reflects that radio link failure shall trigger BFS.

Proposal 7: BFD and/or RLM status may trigger UE to enable FBS.

- If the UE meets the criteria of relaxed BFD and/or RLM, it indeed implicitly indicates the UE hasn’t the demand to conduct FBS. On the contrary, if the UE doesn’t meet the criteria of relaxed BFD and/or RLM, it indeed implicitly indicates the UE may have the demand to conduct FBS.

- If beam failure or radio link failure is detected, FBS may be applied.

Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC

Agreement: - For scenario 1/2/3/4 - additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement. - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms - where TSSB_processing = 2 ms Agreement: - For scenario 7 - to consider an additional processing delay (2ms) after the last SMTC periodicity [Wayforward] - For scenario 5 - FFS: - Option 1: to consider an additional processing delay (2ms) in the handover delay equation - Option 2: In the handover delay equation, SSB processing time (2ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - Option 3: no need to add additional processing time in the delay requirement.

For scenario 5: handover, a concern was raised by companies in the last meeting: either the additional processing delay is added after the measurements (Tsearch) immediately or not, the next SMTC/SSB occasion could always be used for time tracking (T∆). In other words, the consequent ‘TSSB_processing’ can be embedded in the time duration from the time instant when measurement is completed to the time instant of the next SMTC/SSB occasion for time tracking, which is long enough for the additional processing delay.

Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms

Observation 2: For Scenario5, additional processing time is not the last operation in handover delay. Such that it can be embedded in the time duration from the time instant when measurement is completed to the time instant of the next SMTC/SSB occasion for time tracking.

Proposal 8: Our preference is Option 3: ‘no need to add additional processing time in the delay requirement’, but also we’re open to Option 1.

Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

Agreement: For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability: - FFS: details of the capability

Proposal 9: For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF may be to 2, 4 or 6 according to the UE capability.

Issue 1-2-2: Delay requirements of L3 measurement delay reduction by optimizing Rx BSF

- Option 1 (CATT): - The reduced Rx BSF can be applied to the following formula according to the trigger conditions to be specified: - TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra for SSB based intra-frequency measurement with and without gap; - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for SSB based inter-frequency measurement with and without gap; - Tsearch for FR1-FR2 HO and FR2-FR2 HO; - Tidentify_intra_NR and Tidentify_inter_NR, i for RRC Re-establishment delay; - Tidentify-NR for RRC connection release with redirection to NR. - The reduced Rx BSF can be applied to the associated SSB in CSI-RS based measurement, CHO and DAPS HO with no specification impact. - Option 2 (NTT DCM): For each scenario, the highlighted factors should be replaced with capability value: Scenarios; Factors. 1; Mpss/sss_sync_w/o_gaps =24 = 8*3 Mmeas_period_w/o_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6. 2; Mpss/sss_sync_with_gaps =24 = 8*3 Mmeas_period_with_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6. 3; Mpss/sss_sync_inter = 40 = 8*5 Mmeas_period_inter =40 = 8*5 MSSB_index_inter = 24 = 8*3. 4; Same as scenario 3. 5; If the target cell is unknown, and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms. Where N = 8 when the target cell is in FR2-1.. 7; RRC Re-establishment 80, 3520, 4000 come from 8*10, 8*22*20, 8*25*20 respectively Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); MAX (800 ms, 10 x TSMTC). ≥ -8; FR2-1; N/A; MAX (1000 ms, 80 x TSMTC)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 120 x TSMTC)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 3520Note1. < -8; FR2-2; N/A; 5280Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); MAX (800 ms, 13 x TSMTC, i). ≥ -8; FR2-1; N/A; MAX (1000 ms, 104 x TSMTC, i)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 156 x TSMTC, i)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 4000Note1. < -8; FR2-2; N/A; 6000 Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB.. RRC Connection Release with Redirection Table 6.2.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; MAX (680 ms, 11 x Trs). FR2-1; MAX (880 ms, 8x11 x Trs). FR2-2; MAX (880 ms, 12x11 x Trs). Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell... - Option 3(Nokia): To apply the scaling factor Nreduced to the fixed value of Tidentify_intra/inter_NR e.g., MAX (Nreduced x 1000 ms, Nreduced x 104 x TSMTC, i). - Recommended WF - [Moderator]: can use option 2 as starting point for discussion.

Proposal 10: Option 2 indicates the number to be updated, agree to start from it.

Issue 1-2-3: FFS: requirement at transition between the normal L3 measurement and FBS L3 measurement

In multi-Rx WI, it was decided that no activation delay from a single antenna panel to multi-antenna panels is defined; in the same way, it would appear that defining the activation delay of FBS isn’t critical.

We consider the following case: if a UE is able to complete the switch between two measurement occasions (i.e., SSB occasions), the switching actually doesn’t impact measurement accuracy even if the entire L3 measurement result is averaged between measurement occasions with non-FSB measurement occasions with FSB together. The only change caused by such switch is that the UE is able to complete an entire L3 measurement more quickly, since it only saves a few SMTC periodicities, capturing it in specification isn’t significantly necessary.

As per the deactivation delay of FBS, it’s not critical as well for same reason.

Proposal 11: Prefer not to define activation/deactivation delay of FBS.

# Conclusion

Observation 1: By various configurations, UE may enable FBS prior to meeting Event A2 for different purposes and doesn’t cause overlapping between the two conditions.

Observation 2: For Scenario5, additional processing time is not the last operation in handover delay. Such that it can be embedded in the time duration from the time instant when measurement is completed to the time instant of the next SMTC/SSB occasion for time tracking.

Proposal 1: Adopt Event A2 as the reference of FBS triggering condition, including RSRP, RSRQ, SINR threshold. Furthermore,

- The hysteresis parameter may be kept.

- The timeToTrigger indicates the validity time of the triggering condition, the use case for multi-Rx L3 measurement shall be checked further.

Proposal 2: UE shall report FBS validity to NW after meeting activating FBS condition.

Proposal 3: UE shall report deactivating FBS to NW upon deactivation condition being met.

Proposal 4: RAN4 to study the below FBS deactivation conditions:

- After UE activates FBS:

  - The condition of activating FBS is invalid again.

    - To avoid too frequent ping-pong between FBS and non-FBS, a minimal during time in FBS/ non-FBS shall be defined.

  - The condition of activating FBS still is valid,

    - L3 measurement with FBS has conducted for a time period, or

      - As an example, the condition of activating FBS still is valid, but the UE is keeping a stationary at cell edge or there’s no qualified neighbour cell to switch. In this case, FBS may be disabled.

    - ﻿Internal overheating is detected.

- Before UE activates FBS, i.e., before meeting FBS activation condition:

  - Internal overheating is detected.

Proposal 5: After UE deactivates FBS, the UE may re-evaluate the condition of enabling FBS after a minimal time interval.

Proposal 6: For CHO, UE always applies FBS during Tmeasure.

Proposal 7: BFD and/or RLM status may trigger UE to enable FBS.

- If the UE meets the criteria of relaxed BFD and/or RLM, it indeed implicitly indicates the UE hasn’t the demand to conduct FBS. On the contrary, if the UE doesn’t meet the criteria of relaxed BFD and/or RLM, it indeed implicitly indicates the UE may have the demand to conduct FBS.

- If beam failure or radio link failure is detected, FBS may be applied.

Proposal 8: Our preference is Option 3: ‘no need to add additional processing time in the delay requirement’, but also we’re open to Option 1.

Proposal 9: For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF may be to 2, 4 or 6 according to the UE capability.

Proposal 10: Option 2 indicates the number to be updated, agree to start from it.

Proposal 11: Prefer not to define activation/deactivation delay of FBS.

# References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo (Moderator)

- R4-2420093, WF for NR_RRM_Ph5, Apple