# 3GPP TSG-RAN WG4 Meeting #114	R4-2501297

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	4.5.1

Source: 	Ericsson

Title:	Discussion on multiple BWP switching with dormancy SCells

Document for:	Discussion

# Introduction

In RAN2 meetings, RAN2 agreed to introduce a new capability to handle the multiple BWP switching between non-dormant and dormant SCells.

RAN2 discussed the two options and concluded to down-select approach 2 because Approach 1 is a non-backward compatible change from RAN2 perspective. RAN2 introduced a field bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 corresponding to RAN4 feature 38-9, and a field scellDormancyWithinActiveTime-DCI-0-3-And-1-3-r18 corresponding to its prerequisite feature (RAN1 feature 49-9). It is up to RAN4 whether the UE shall report the same value between RAN4 features 6-3 and 38-9.

In last RAN4 meeting, companies discussed whether to define new requirement for the multiple BWP switching between non-dormant and dormant SCells.

- Requirement

The change on the requirements for simultaneous DCI based BWP switch delay on multiple CCs with new capability are as follows.

UE shall finish BWP switch within the time duration TMultipleBWPswitchDelay + Y, which is defined as: TMultipleBWPswitchDelay = TBWPswitchDelay + D*(N-1) Where: - TBWPswitchDelay is the BWP switching delay on single CC defined in Table 8.6.2-1 depending on UE capability bwp-SwitchingDelay [2]. TBWPswitchDelay shall be based on the smallest SCS among SCS of all involved CCs before and after BWP switch. If the BWP switch on multiple CCs results in the change of the SCS on any CC among involved CCs, TBWPswitchDelay shall be based on the smallest SCS among all SCS values of all involved CCs. - D is the incremental delay for each additional CC involved in simultaneous BWP switch and depends on UE capability bwp-SwitchingMultiCCs-r16 [TS 38.306, 14] for switching between non-dormant BWPs, and bwp-SwitchingMultiDormancyCCs-r16 or bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 for switching between non-dormant and dormant BWPs. - For UE which is capable of per-FR gap, and no BWP switch involves SCS change, N is the number of CCs in same FR; For UE which is not capable of per-FR gap, or the BWP switches on any CC involves SCS changing, N is the number of CCs undergoing simultaneous BWP switch. - Y=0, if the serving cell where UE receives DCI for BWP switch is same as the serving cell on which BWP switch occurs for each involved serving cell. Y equals to the length of one slot at smaller SCS of scheduling cell, scheduled cells before and scheduled cells after active BWP change, - if the serving cell where UE receives DCI for BWP switch is different from the serving cell on which BWP switch occurs for any involved serving cell. If both scheduling cell and scheduled cell are in FR2-2, Y shall follow the SCS of 120 KHz.

The overall BWP switch delay is based BWP switching delay on single CC plus incremental delay D for each additional CC. There are two type of BWP switch delay, i.e., switching between non-dormant BWPs and switching between dormant and non-dormant BWP. Companies proposed to update the requirement to consider the hybrid non-dormant and dormant SCells BWP switching.

In RAN4 96 meeting, RAN4 achieved the following agreement to reuse BWP switching requirement for dormancy SCell.

R4-2012278 Agreement: - Delay requirements for switching of multiple SCells between dormancy and non-dormancy shall be based on corresponding delay requirements for switching of multiple SCells between non-dormant BWPs, i.e., TMultipleBWPswitchDelay or TMultipleBWPswitchDelay+Z for simultaneous BWP switching case. - Incremental value D is reused from BWP switching on multiple CCs

Companies mentioned that dormancy SCell switching can use a larger value compared with BWP switching due to RF warm up. Technically, we agree to this observation. It’s fine to introduce different values to hybrid non-dormant and dormant SCells BWP switching, but it should be clarified that non-dormant BWP switching delay should be shorter than dormant BWP switching.

Observation 1: Dormancy SCell switch delay can be longer than BWP switching.

Proposal 1: RAN4 to introduce different incremental value D for hybrid non-dormant and dormant SCells BWP switching with the clarification non-dormant BWP switching delay should be equal or shorter than dormant BWP switching.

- Summary

In this contribution, we discussed the multiple BWP switching with non-dormant and dormant SCells. Based on the discussions, we have made following proposals and observations:

Observation 1: Dormancy SCell switch delay can be longer than BWP switching.

Proposal 1: RAN4 to introduce different incremental value D for hybrid non-dormant and dormant SCells BWP switching with the clarification non-dormant BWP switching delay should be equal or shorter than dormant BWP switching.

- References

[1]. R4-2012278, “Way Forward on SCell Dormancy”, Ericsson