# 3GPP TSG-RAN WG4 Meeting #114 　　　　　　　　　　　　　　　　　	R4-2501307

Athens, GR, 17th  – 21st February, 2025

Source: 	NTT DOCOMO INC.

Title:	Discussion on MPR improvement for FR2

Agenda item:	7.1.1.2.3

Document for:	Discussion

- Introduction

In Rel-19, the WI “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” [1], includes MPR reduction as one of its objectives. RAN4#113 agreed on the conditions for CC activation-based enhancements and decided to continue discussions regarding UE capability and the MPR applicability when the bandwidth basis = 400 MHz or 800 MHz [2]. This paper presents our perspective on these issues.

- Discussion

  - Discussion on UE capability for FR2 CA MPR improvement

Background:

RAN4#113 discussed UE capability for FR2 CA MPR improvement and agreed to define distinct UE capabilities for configuration-based enhancement and activation-based enhancement. This was captured in the WF [2] as follows:

Issue 3-1-2: Optional UE capability Agreement in main session and WF: - For Rel-19 FR2 UE MPR, define distinct enhancement capabilities for configuration-based enhancements and activation-based enhancement. - FFS if the capabilities should be mutually exclusive

The remaining issues are whether these UE capabilities should be mutually exclusive and how UE capabilities for activation-based enhancement should be designed.

Discussion:

In previous meetings, RAN4 has indicated that regarding the bandwidth basis of the MPR table, it is possible to apply configured UL BWchannel_CA instead of CABW with optional UE capability. Please note that it has been agreed in RAN4#112bis that the capability is implementation agnostic.

#### Issue 3-2-1: Optional UE capability Agreement in main session: - An optional per UE capability for FR2 is introduced - The capability is implementation agnostic - Supporting FR2 MPR enhancement is applicable only from Rel-19

Observation 1: Regarding the bandwidth basis of the MPR table, it has been agreed that configured UL BW can be applied with an optional per UE capability indication.

Observation 2: It has been agreed that the UE capabilities does not depend on implementation.

Additionally, for UEs that can meet the MAC-CE timeline, activated UL CCs BW can be applied. At this time, it is conceivable that there may be UEs that can meet the MAC-CA timeline, but cannot apply UL BW as the bandwidth basis due to implementation issues. Therefore, the applicable bandwidth basis for the MPR Table can be considered as one of the following three options:

- Activated CABW

- Configured UL BWchannel_CA

- Activated UL CCs BW

These enhancements are thought to result in different applicable UEs due to implementation differences. For example, considerations such as meeting or not meeting the MAC-CE timeline and independent LO/common LO may apply. Therefore, it is necessary to design UE capabilities that distinguishes between these cases.

Proposal 1: RAN4 should define distinct UE capabilities, considering cases where activated CABW is applied in addition to cases where configured UL BWchannel_CA or activated UL CCs bandwidth are used as the bandwidth basis for FR2 CA MPR table.

  - Bandwidth basis applicability

Background:

RAN4#113 discussed the MPR applicability when bandwidth basis = 400MHz or 800MHz in the WF [2].

### Sub-topic 3-2: Others - Proposals in R4-2418337 (Samsung) - Proposal 1: Highlight 800MHz upper limit for the enhanced MPR feature when applying MPR values in the tables. - Proposal 2: RAN4 to further discuss the MPR applicability when bandwidth basis = 400MHz or 800MHz. - WF - FFS proposal 2 with the identified discrepancy of CA BW class and MPR table - FFS if 800MHz is explicitly referenced as an upper limit

In R4-2418337 (Samsung) [3], it is pointed out that the applicability of MPR becomes ambiguous when the bandwidth basis = 400 MHz or 800 MHz. For example, in the case of PC1, the MPR value for bandwidth basis = 400 MHz is in the 400–800 MHz category, while in the case of PC3, the MPR value for a bandwidth basis = 400 MHz is in the ≤400 MHz category. This part clarifies the background that led to such applicability and discusses the MPR applicability when the bandwidth basis = 400 MHz or 800 MHz.

Discussion:

The proposal to change the FR2 PC3 CA MPR category from < 400 MHz to ≤ 400 MHz is presented in R4-1905688 (Qualcomm, RAN4#91) [4]. The rationale for changing it to ≤400 MHz is based on the fact that the PC3 CA MPR table was created by referencing the MPR value for CP-OFDM of single CC MPR table (BWchannel = 400 MHz) [5].

Table 6.2A.2.4-1: Maximum power reduction (MPRC_CA) for UE power class 3 in FR2-1 Cumulative aggregated channel bandwidth (CABW). ≤ 400 MHz; > 400 MHz and < 800 MHz; ≥ 800 MHz and ≤ 1400 MHz; > 1400 MHz and ≤ 2400 MHz. DFT-s-OFDM; Pi/2 BPSK; ≤ 5.01; ≤ 7.7; ≤ 8.2; ≤ 8.7. QPSK; ≤ 5.01; ≤ 7.7; ≤ 8.2; ≤ 9.7. 16 QAM; ≤ 6.5; ≤ 8.7; ≤ 9.3; ≤ 9.7. 64 QAM; ≤ 9.0; ≤ 10.7; ≤ 11.2; ≤ 11.7. CP-OFDM; QPSK; ≤ 5.0; ≤ 7.5; ≤ 8.0; ≤ 9.7. 16 QAM; ≤ 6.5; ≤ 8.7; ≤ 9.2; ≤ 9.7. 64 QAM; ≤ 9.0; ≤ 10.7; ≤ 11.2; ≤ 11.7. NOTE 1: (Void)..

Table 6.2.2.3-2 MPRWT for power class 3, BWchannel = 400 MHz, FR2-1 Modulation; MPRWT, BWchannel = 400 MHz. Inner RB allocations, Region 1; Edge RB allocations. DFT-s-OFDM; Pi/2 BPSK; 0.0; ≤ 3.0. QPSK; 0.0; ≤ 3.0. 16 QAM; ≤ 4.5; ≤ 4.5. 64 QAM; ≤ 6.5; ≤ 6.5. CP-OFDM; QPSK; ≤ 5.0; ≤ 5.0. 16 QAM; ≤ 6.5; ≤ 6.5. 64 QAM; ≤ 9.0; ≤ 9.0.

A technical discussion of this proposal is outlined in R4-1908253 (Intel, RAN4#92) [6]. In this paper, it is indicated that the dominant factor influencing MPR is PAPR, which increases when the DMRS sequences are the same between different CCs. It concludes that if the CCs have different DMRS sequences, the single CC CP-OFDM MPR can be reused for CA MPR. Following this examination, a draft CR R4-1912932 (Qualcomm, RAN4#92) [7] was agreed upon to include wording that assumes different DMRS sequences between CCs in UL CA scenarios, along with a draft CR R4-1911704 (DOCOMO, RAN4#92) [8] to change the FR2 PC3 CA MPR from <400 MHz to ≤400 MHz.

Observation 3: The PC3 CA MPR table category was intentionally changed to ≤400 MHz because it was created by referencing the MPR value for CP-OFDM of single CC MPR table (BWchannel = 400 MHz).

Proposal 2: Since the PC3 CA MPR table was intentionally changed to ≤400 MHz based on technical rationale, the boundary should not be altered.

Furthermore, in R4-1905688 (Qualcomm), it is mentioned that while this change does not impede modifications in other PC configurations, it is limited to PC3, considering operators' demands in order to minimize confusion.

Observation 4: The change of the PC3 CA MPR table category was made only for PC3, taking into account the operators' demands, and does not impede changes in other PC configurations.

On the other hand, the PC1 CA MPR table does not directly reference the values of the single CC CP-OFDM MPR [5].

Table 6.2A.2.2-1: Maximum power reduction (MPRWT_C_CA) for UE power class 1 in FR2-1 Waveform Type; Cumulative aggregated channel bandwidth. < 400 MHz; ≥ 400 MHz and < 800 MHz; ≥ 800 MHz and ≤ 1400 MHz; > 1400 MHz and ≤ 2400 MHz. DFT-s-OFDM; Pi/2 BPSK; ≤ 5.51; ≤ 7.7; ≤ 8.2; ≤ 8.7. QPSK; ≤ 6.51; ≤ 8.7; ≤ 9.7; ≤ 9.7. 16 QAM; ≤ 6.5; ≤ 8.7; ≤ 9.2; ≤ 9.7. 64 QAM; ≤ 9.0; ≤ 10.7; ≤ 11.2; ≤ 11.7. 256 QAM2; ≤ 12.5; ≤ 14.2; ≤ 14.7; ≤ 15.7. CP-OFDM; QPSK; ≤ 6.5; ≤ 8.7; ≤ 8.7; ≤ 9.7. 16 QAM; ≤ 6.5; ≤ 8.7; ≤ 8.7; ≤ 9.7. 64 QAM; ≤ 9.0; ≤ 10.7; ≤ 11.2; ≤ 11.7. 256 QAM2; ≤ 12.5; ≤ 14.2; ≤ 14.7; ≤ 15.7. NOTE 1: (Void) NOTE 2: Refer to clause 6.1 for 256 QAM applicability..

Table 6.2.2.1-2 MPRWT for power class 1, BWchannel = 400 MHz in FR2-1 Modulation; MPRWT (dB), BWchannel = 400 MHz. Outer RB allocations; Inner RB allocations. Region 1; Region 2. DFT-s-OFDM; Pi/2 BPSK; ≤ 5.5; 0.0; ≤ 3.0. QPSK; ≤ 6.5; 0.0; ≤ 3.5. 16 QAM; ≤ 6.5; ≤ 4.5; ≤ 4.5. 64 QAM; ≤ 6.5; ≤ 6.5; ≤ 6.5. 256 QAM1; ≤ 9.5; ≤ 9.5; ≤ 9.5. CP-OFDM; QPSK; ≤ 7.0; ≤ 5.0; ≤ 5.0. 16 QAM; ≤ 7.0; ≤ 6.5; ≤ 6.5. 64 QAM; ≤ 9.0; ≤ 9.0; ≤ 9.0. 256 QAM1; ≤ 12; ≤ 12; ≤ 12. NOTE 1: Refer to clause 6.1 for 256 QAM applicability..

Therefore, if RAN4 decides to change the bandwidth basis of the PC1 CA MPR, further technical discussion will be needed.

Observation 5: PC1 CA MPR Table does not directly reference the single CC CP-OFDM MPR.

Observation 6: If the category of the PC1 CA MPR table when bandwidth basis = 400MHz change, RAN4 needs technical evaluation.

- Conclusion

Here we summarize our proposals:

Observation 1: Regarding the bandwidth basis of the MPR table, configured UL BW can be applied instead of CABW with a specific LO implementation (e.g., independent LO).

Observation 2: It has been agreed that the UE capabilities does not depend on implementation.

Proposal 1: RAN4 should define distinct UE capabilities, considering cases where activated CABW is applied in addition to cases where configured UL BWchannel_CA or activated UL CCs bandwidth are used as the bandwidth basis for FR2 CA MPR table.

Observation 3: The PC3 CA MPR table category was intentionally changed to ≤400 MHz because it was created by referencing the MPR value for CP-OFDM of single CC MPR table (BWchannel = 400 MHz).

Proposal 2: Since the PC3 CA MPR Table was intentionally changed to ≤400 MHz based on technical rationale, the boundary should not be altered.

Observation 4: This change was made only for PC3, taking into account the operators' demands, and does not impede changes in other PC configurations.

Observation 5: On the other hand, the PC1 CA MPR Table does not directly reference the single CC CP-OFDM MPR.

Observation 6: If the category of the PC1 CA MPR table when bandwidth basis = 400MHz change, RAN4 needs technical evaluation.

4	References

- RP-241656, “Revised WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 chair (Huawei)

- R4-2420527, “WF on Power domain enhancements”, Huawei, HiSilicon

- R4-2418337, “Discussion on MPR reduction for FR2 CA”, Samsung

- R4-1905688, “On FR2 MPR for CA”, Qualcomm Incorporated

- TS 38.101-2 V18.8.0 (2024-12)

- R4-1908253, “Views on FR2 CA MPR enhancement”, Intel Corporation

- R4-1912932, “dCR to 38.101-2: DMRS exceptions”, Qualcomm Incorporated

- R4-1911704, “Draft CR for FR2 MPR improvement for CA”, NTT DOCOMO, INC.