3GPP TSG-RAN WG4 Meeting #114	R4-2501308

Athens, Greece 17th – 21st February 2025

Source: 	Nokia

Title: 					NTN n254 3 MHz A-MPR simulations

Agenda Item: 			7.10.3.2

Document for:	Discussion

# 1	Introduction

We simulate the A-MPR for n254 and 3 MHz CBW considering the additional emission requirements signaled through NS_03N, NS_04N and NS_05N. In particular, we study the effect of the averaging time limitation in the emission requirements.

# 2	Simulation assumptions and scenario

### 2.1	Simulation parameters

- Operating band n254 (UL 1610 MHz – 1626.5 MHz)

- Power class PC3

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc, CIM5 -70 dBc

- Only additional emission requirements from NS_03N, NS_04N and NS_05N are simulated. Other gating factors (ACLR, SEM, etc.) are excluded because they are handled by MPR.

### 2.2	Emission requirements for NS_03N, NS_04N and NS_05N

Table 6.5.3.3.3-1: Additional out-of-band requirements for "NS_03N"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |  |
| 1559 ≤ f < 1605 | -50 | 700 Hz | Discreet emissions averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -50 + 60/5 (f-1605) | 700 Hz |  |
| 1559 ≤ f < 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

Table 6.5.3.3.4-1: Additional out-of-band requirements for "NS_04N" and "NS_05N"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |  |
| 1559 ≤ f < 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| 1628.5 ≤ f < 1631.5 | -30 | 30kHz |  |
| 1631.5 ≤ f < 1636.5 | -30 | 100kHz |  |
| 1636.5 ≤ f < 1646.5 | -30 | 300kHz |  |
| 1646.5 ≤ f < 1666.5 | -30 | 1MHz |  |
| 1666.5 ≤ f ≤ 2200 | -30 | 3MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

Table 6.5.2.3.1-1: Additional requirements for "NS_04N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0-160 | -2 | 30kHz |
| ± 160-2300 | -2 to -26 |  |
| ± 2300-18500 | -26 |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |

Table 6.5.2.3.2-1: Additional requirements for "NS_05N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0-160 | -5 | 30kHz |
| ± 160-225 | -5 to -8.5 |  |
| ± 225-650 | -8.5 to -15 |  |
| ± 650-1365 | -15 |  |
| ± 1365-1800 | -23 to -26 |  |
| ± 1800-16500 | -26 |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |

Table 6.2.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum power density |
| --- | --- | --- | --- | --- |
| n254 | NS_04N | 5 | 1610 - 1618.25 | 27dBm/4kHz (mean) 15dBm/4kHz (peak limit) |
|  | NS_05N | 5 | 1618.25 - 1626.5 |  |
|  |  | 10, 15 | 1610 – 1626.5 |  |

The power density limits in Table 6.2.1-2 are so relaxed that they never affect the transmission power in PC3. At maximum, a single-RB allocation at 15 kHz SCS would have a 6.5 dBm/4kHz power density.

# 3	Simulation methodology for time-limited emission masks

The additional spurious emission requirements for NS_03N, NS_04N, and NS_05N include a time limitation for spectral analysis. In some mask segments, the emission power averaged over any 2 ms time window must not exceed the specified limit. In [1], we pointed out that this time limit may increase the measured power significantly.

In simulations, at each frequency, the highest 2 ms average power is taken as the result. Since the mask may not be violated in any 2 ms time window, the simulation must cover a sufficient number of such (partly overlapping) averaging windows to capture the highest value at each frequency. For a single frequency, this is depicted in Fig. 1. Fig. 2 (from [1]) presents an example cumulative probability function of such 2 ms time averages, measured at a single measurement center frequency. The difference is also clearly visible in the spectra shown in Fig. 3.

Figure 1. Time-limited spectrum calculation from viewpoint of a single measurement center frequency.

Figure 2: Example cumulative probability density function of 2 ms power average

Fig. 3: Spectra of a fully allocated 3 MHz channel at lower edge of n254

For the 700 Hz measurement bandwidth, the 2 ms time window is very short, close to the fundamental limits of time-frequency analysis, as the minimum FFT duration for 700 Hz frequency resolution is 1 / 700 Hz  1.4 ms. The window function used in spectral analysis should be as short as possible and carefully balanced between time and frequency resolution.

In Table 6.5.3.3.3-1, a note mentions discreet emissions, but the specification does not define what the term means. This needs to be clarified.

# 4	Simulation results

For comparison, we present simulation results both without and with the averaging time limitation. In the former case, the spectrum is averaged across the entire signal.

### 4.1	NS_03N

Figures 4 and 5 show the A-MPR simulation results without and with the 2 ms averaging time limit, respectively, with the channel at the lower edge of n254.

Observation 1: The averaging time limit clearly affects the required A-MPR.

Observation 2: The wider channel bandwidths should be resimulated applying the 2 ms time limit.

Figure 4: Example simulation results for 3 MHz channel at lower edge of n254 (fC = 1611.5 MHz), 
simulated without averaging time limitation

Figure 4: Example simulation results for 3 MHz channel at lower edge of n254 (fC = 1611.5 MHz), 
simulated with the 2 ms averaging time limitation for respective mask segments

In NS_03N, the A-MPR is entirely determined by the mask segments with 1 MHz MBW.

At channel center frequencies  1614.5 MHz, NS_03N needs no A-MPR.

### 4.2	NS_04N

The NS_04N additional SEM does not require A-MPR. Also, the NS_04N additional spurious emission mask segments above n254 do not require A-MPR, particularly because the channel is confined to the lower part of the operating band, 1610-1618.25MHz. Thus, the NS_04N A-MPR is entirely determined by the mask segments below n254 with 1 MHz MBW. These mask segments are common with NS_03N, therefore NS_03N and NS_04N should share the same A-MPR.

### 4.3	NS_05N

NS_05N does not require A-MPR for channels confined within its applicable frequency region 1618.25-1626.5MHz (TS 38.101-5, subclause 6.5.2.3.2).

# 5	Conclusions

Observation 1: The averaging time limit clearly affects the required A-MPR.

Observation 2: The wider channel bandwidths should be resimulated applying the 2 ms time limit.

NS_03N and NS_04N should share the same A-MPR, which is needed only at the lower edge of n254.

NS_05N does not need A-MPR.

# 6	References

[1]	R4-2419282, NTN 3 MHz A-MPR simulations, Nokia, RAN4#113

# 7	Appendix: A-MPR triangles

### 7.1	NS_03N

Time-limited			Unlimited

### 7.2	NS_04N

Time-limited			Unlimited