3GPP TSG-RAN WG4 Meeting #114	R4-2501309

# Athens, GR, 17th – 21st Feburary, 2025

Agenda Item:	6.20.1

Source:	MediaTek Inc.

Title:	Discussion on UE RF requirement for NR-NTN bands to support the Extended L-band and the combined MSS L-band and Extended L-band ranges

Document for:	Discussion

- Introduction

In RAN#104 plenary meeting, the spectrum WID on introduction of extended L-band and combined L-band was approved in [1]. One of the main objectives is to specify UE RF core requirements for fulfilling the urgent deployment demand.

In the contribution, the consideration of UE RF requirements for the new NR-NTN bands are provided for discussion.

- Discussion

2.1		Spectrum WI scope for the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and the combined MSS L-band and Extended L-band ranges (DL 1518-1559 MHz, UL 1626.5-1660.5 MHz and 1668-1675 MHz)

First, regarding UE RF requirements, the objective of the WI RF core part is shown below for reference.

- Specify a new NR NTN FDD band with a UE transmitting at 1668 – 1675 MHz and SAN transmitting at 1518 – 1525 MHz; - Support 5MHz channel bandwidth with 15kHz SCS - Specify new NR NTN FDD band(s) with SAN transmitting at 1518 – 1559 MHz and UE transmitting at 1626.5 – 1660.5 MHz or 1668 – 1675 MHz; - Support channel bandwidths and SCS as presented in Table 4.1-1 below: Table 4.1-1: DL Channel bandwidth and SCS system parameters NTN satellite band; SCS kHz; DL Channel bandwidth (MHz). 5; 10; 15; 20; 25; 30; 35; 40. 15; 5; 10; 15; 20; 25; 30; 35; 40. [n25x]; 30; 10; 15; 20; 25; 30; 35; 40. 60; 10; 15; 20; 25; 30; 35; 40. - Support Asymmetric UE TX-RX Channel BW configurations to allow leveraging the 41 MHz contiguous DL allocation with a smaller UL allocation. NOTE: UL channel allocations spanning across the 1660.5-1668 MHz gap shall not be allowed - For these bands, the following shall be specified: - Support Flexible TX-RX Separation - Mandatory support for Enhanced Channel Raster with raster points at step of 10kHz - Support UE Power class 3 (+23 dBm) as a starting point - Introduce the corresponding SAN and UE RF core requirements. - Introduce the corresponding RRM requirements. - Requirements for protection of the Radio Astronomy in the 1660-1670 MHz range shall be captured - Any relevant additional regional requirements and restrictions will also be captured. - NOTE1: The introduction of ETSI-specific requirements is expected to be addressed at a later stage, due to ongoing discussions within ETSI in respect to NTN - NOTE2: The 1668-1675 MHz range is presently not available for MSS in the continental United States. NOTE: The specification work in this WI shall leverage the studies and requirements for NR NTN n255, and the corresponding IoT NTN Work Item (IoT_NTN_extLband) for Band 253, where applicable. All UE requirements specified as part of this WI shall be Release-independent for the UE starting from Release-17.

2.2		Previous Meeting Agreements

In RAN4#112bis and RAN4#113 meetings, the approved WF [2][3] provided the guidance for following works. The partial contents are listed below for reference.

RAN4#113 Issue 3-1-4: Protection of Radio Astronomy stations – Duplexer dependency Agreement: - The requirements to protect Radio Astronomy service operation are independent with the duplexer implementation. Issue 3-1-5: Requirements and Mechanisms on Protection of Radio Astronomy stations - Tentative Agreement: - Further study mechanisms to implement protection of Radio Astronomy stations, at least the following options could be considered: - TX Disable, considering possible impact on backwards compatibility - RB restrictions - NS RAN4#112bis Issue 3-2-1: UE Duplexer - Agreement: - Further discuss whether implementation with one duplexer is feasible, considering the requirements to protect Radio Astronomy in the 1660-1668MHz range between the two UL.

2.3		UE RF requirement

2.3.1	Requirements and Mechanisms on Protection of Radio Astronomy stations

Some experience leveraged for UE-UE coexistence protection can be considered. The existing NS signalling with controlling A-MPR value can be used for protection of radio astronomy stations. The NS signalling with controlling TX RB allocations or TX RBs disable could also be considered for protection of radio astronomy stations.

Observation 1: The mechanism of existing NS signalling with controlling A-MPR value could be used for protection of radio astronomy stations. As for other solutions, including TX RB allocations or TX RBs disabling to protect radio astronomy stations, it is unclear whether new NS/RRC/DCI would be required.

Proposal 1: As for possible solutions below for protection of radio astronomy stations, discuss whether new NS/RRC/DCI to UE would be required or solutions below are up to SAN implementation.

- Option 1: TX disabling

- Option 2: TX RBs restriction/disabling/allocation

- Option 3: Tx power reduction (e.g., A-MPR/MPR)

Proposal 2: Because TX power reduction cannot provide good connection performance between UE and SAN, consider the TX disabling or TX RBs allocation for protection of radio astronomy stations.

3 Conclusion

In this paper, our views on UE RF requirement for the new NR-NTN bands are provided.

Observation 1: The mechanism of existing NS signalling with controlling A-MPR value could be used for protection of radio astronomy stations. As for other solutions, including TX RB allocations or TX RBs disabling to protect radio astronomy stations, it is unclear whether new NS/RRC/DCI would be required.

Proposal 1: As for possible solutions below for protection of radio astronomy stations, discuss whether new NS/RRC/DCI to UE would be required or solutions below are up to SAN implementation.

- Option 1: TX disabling

- Option 2: TX RBs restriction/disabling/allocation

- Option 3: Tx power reduction (e.g., A-MPR/MPR)

Proposal 2: Because TX power reduction cannot provide good connection performance between UE and SAN, consider the TX disabling or TX RBs allocation for protection of radio astronomy stations.

5 Reference

- RP-241666, “New WID proposal: Introduction of new NR NTN bands to support the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and the combined MSS L-band and Extended L-band ranges (DL 1518-1559 MHz, UL 1626.5-1660.5 MHz and 1668-1675 MHz)”, Inmarsat, Viasat, Thuraya

- R4-2417206, “WF on NTN and IoT-NTN bands”, Inmarsat

- R4-2420465, “WF on requirements for NR IoT NTN bands”, Inmarsat