# 3GPP TSG-RAN WG4 Meeting #114	R4-2501332

# Athens, GR, 17th – 21st February, 2025

Agenda item:	7.17.2.2

Source:	China Telecom

Title:	Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

- Introduction

In RAN4 #113 meeting, FR2-1 SSB based L3 measurement delay reduction for connected mode was discussed and the WF was approved [1].

In this paper, we present our views on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC issues captured in the WF [1] and summary [2].

- Discussion

UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF

Status in the WF [1] and summary [2]:

### Issue 2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2418276. - Option 1 (CATT, Apple, HW, Ericsson): The enhanced CSSF apply to all the 3 aspects and do not define the priority. - Option 2 (Xiaomi, MTK, ZTE): the following aspect to use L3 measurement delay reduction by optimizing CSSF shall be prioritized: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Option 2a(ZTE): To be more concentrated, prioritize the intra-frequency measurement without gap could make the discussion more efficient. But when re-design the CSSF factor, both inter-frequency and inter-RAT measurement without gap should also be considered. - Option 3 (Nokia, Samsung): To start from SSB-based intra-frequency and inter-frequency measurements without MG(aspect 1 and 2) to simplify the CSSF optimization discussion. - Recommended WF [Moderator]: As a compromise, is option 3 agreeable?

It was agreed in previous meeting that 3 aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting. In our views, in order to simplify the discussion, Aspect 1 SSB based Intra-frequency measurement without MG and Aspect 2 SSB based Inter-frequency measurement without MG can be discussed as a starting point. After the baseline is completed, Aspect 3 Inter-RAT SSB measurement without MG can be further discussed.

Proposal 1: To start from SSB-based intra-frequency and inter-frequency measurements without MG (aspect 1 and 2) to simplify the CSSF optimization discussion.

Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Status in the WF [1]:

Issue 2-4-1: Tentative solution 1: measure one serving CC per band [Wayforwad]: FFS the following solution: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured, - Reuse the legacy principle for neighbour cell measurement - Measure PCC if PCC in band or - Measure PSCC if PSCC in band or - Measure SCC with neighbor cell MO or - UE implementation - FFS whether indication is per-UE or per-band - Other options are not precluded. - otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

For tentative solution 1, network indication is preferred, if network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured, and the legacy principle for neighbor cell measurement can be reused. The network indication can be per-band since the solution is applied to the same band, the per-band indication is workable.

Proposal 2: Network indication is preferred, if network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured, and the legacy principle for neighbor cell measurement can be reused.

Proposal 3: The network indication can be per-band.

Status in the WF [1]:

Issue 2-4-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs [Wayforward]: FFS: whether solution 2 is needed if solution 1 is agreed in issue 2-4-1: - UE may change the searcher occupancy ratio of SCC measurement to speed up measurement. - FFS whether network indication is needed - Details is FFS - FFS on whether and how PCC and PSCC resource sharing is supported

In our views, tentative solution 2 is also needed regardless of whether tentative solution 1 is agreed. Our first preference is that UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions, and NW indicates the ratio between PCC, PSCC and SCC. However, in order to make progress, we can comprise that UE can change the searcher occupancy ratio of measurement among the measured SCCs based on network indication, e.g. the priority of the SCCs on searcher.

Proposal 4: Tentative solution 2 is also needed regardless of whether tentative solution 1 is agreed.

Proposal 5: UE can change the searcher occupancy ratio of measurement among the measured SCCs based on network indication, e.g. the priority of the SCCs on searcher.

- Conclusion

The contribution presents our views on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC, with the following proposals:

Proposal 1: To start from SSB-based intra-frequency and inter-frequency measurements without MG (aspect 1 and 2) to simplify the CSSF optimization discussion.

Proposal 2: Network indication is preferred, if network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured, and the legacy principle for neighbor cell measurement can be reused.

Proposal 3: The network indication can be per-band.

Proposal 4: Tentative solution 2 is also needed regardless of whether tentative solution 1 is agreed.

Proposal 5: UE can change the searcher occupancy ratio of measurement among the measured SCCs based on network indication, e.g. the priority of the SCCs on searcher.

- References

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN WG4 Meeting #113, 18 – 22 November, 2024.

- R4-2418276, Topic summary for [113][218] NR_RRM_Ph5_Part1, Apple, RAN WG4 Meeting #113, 18 – 22 November, 2024.