3GPP TSG-RAN WG4 Meeting #114	R4-2501351

# Athens, Greece, February 17th – 21st 2025

Source:	Ericsson

Title:	Simulation results for Rel-19 MMSE-IRC BS demodulation requirements

Agenda item:	7.18.3

Document for:	Information

# Introduction

In RAN4#113 meeting, companies agreed to reuse LTE interference profile for Rel-19 NR MMSE-IRC BS demodulation requirement. But there are still open issues on number of interferers, test parameters and simulation assumptions [1]. In this contribution, some comparison simulation results are delivered.

# Simulations

In this simulation, the DIP values in LTE MMSE-IRC BS interference profile are converted to INR values.

2.1	Simulation assumptions

Table 2.1-1 Mapping from LTE DIP values to INR values

| Scenario | Interferer number | (DIP1, DIP2) [dB] | (INR1, INR2) [dB] |
| --- | --- | --- | --- |
| HomNet | 1 interferer | (-1.11, NA) | (5.36, NA) |
|  | 2 interferers | (-1.11, -10.91) | (7.29, -2.5) |
| HetNet | 1 interferer | (-0.43, NA) | (9.83, NA) |
|  | 2 interferers | (-0.43, -13.78) | (12.38, -0.97) |

Table 2.1-2: Test parameters

| Parameter |  |  |  | Value |
| --- | --- | --- | --- | --- |
| Transform precoding |  |  |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  |  |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ |  | Maximum number of HARQ transmissions |  | 4 |
|  |  | RV sequence |  | 0, 2, 3, 1 |
| DM-RS |  | DM-RS configuration type |  | 1 |
|  |  | DM-RS duration |  | single-symbol DM-RS |
|  |  | Additional DM-RS position |  | pos1 |
|  |  | Number of DM-RS CDM group(s) without data |  | 2 |
|  |  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |
|  |  | DM-RS port(s) |  | {0} |
|  |  | DM-RS sequence generation |  | NID0=0, nSCID =0 |
| Time domain resource assignment |  | PUSCH mapping type |  | A |
|  |  | Start symbol |  | 0 |
|  |  | Allocation length |  | 14 |
| Frequency domain resource assignment |  | RB assignment |  | Full applicable test bandwidth |
|  |  | Frequency hopping |  | Disabled |
| TPMI index for 2Tx two layer or 4Tx four layer spatial multiplexing transmission |  |  |  | 0 |
| Code block group based PUSCH transmission |  |  |  | Disabled |
| Interference (Note 2) | Set 1 (HomNet) | 1 interferer | INR1 (dB) | 5.36 |
|  |  | 2 interferers | INR1 (dB) | 7.29 |
|  |  |  | INR2 (dB) | -2.5 |
|  | Set 2 (HetNet) | 1 interferer | INR1 (dB) | 9.83 |
|  |  | 2 interferers | INR1 (dB) | 12.38 |
|  |  |  | INR2 (dB) | -0.97 |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. |  |  |  |  |

2.2	Simulation results

Table 2.2-1: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 5 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled with 1 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  | IRC | MRC | Gain |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | 2 | -1.17 | 2.57 | 3.74 |
|  |  | TDLB100-400 |  | 5.36 | 2 | -1.52 | 0.28 | 1.8 |
|  | 4 | TDLA30-10 | TDLC300-100 | 9.83 | 16 | 5.73 | 11.13 | 5.4 |
|  |  | TDLB100-400 |  | 5.36 | 16 | 7.61 | 10.22 | 2.61 |
|  | 8 | TDLA30-10 | TDLC300-100 | 9.83 | 20 | 5.55 | 10.60 | 5.05 |
|  |  | TDLB100-400 |  | 5.36 | 20 | 7.32 | 9.65 | 2.33 |

Table 2.2-2: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 5 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled with 2 Interferers

| TX | RX | Propagation conditions and correlation matrix |  | (INR1, INR2) [dB] | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1&2 |  |  | IRC | MRC | Gain |
| 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1.20 | 5.17 | 3.97 |
|  |  | TDLB100-400 |  | (7.29, -2.5) | 2 | 0.31 | 2.19 | 1.88 |
|  | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 16 | 7.55 | 13.65 | 6.1 |
|  |  | TDLB100-400 |  | (7.29, -2.5) | 16 | 9.08 | 12.15 | 3.07 |
|  | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 20 | 6.66 | 13.15 | 6.49 |
|  |  | TDLB100-400 |  | (7.29, -2.5) | 20 | 8.30 | 11.57 | 3.27 |

Table 2.2-3: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 10 MHz channel bandwidth, 30 kHz SCS with Transform Precoding Disabled with 1 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  | IRC | MRC | Gain |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | 2 | -0.80 | 2.60 | 3.4 |
|  |  | TDLB100-400 |  | 5.36 | 2 | -1.23 | 0.40 | 1.63 |
|  | 4 | TDLA30-10 | TDLC300-100 | 9.83 | 16 | 6.24 | 11.39 | 5.15 |
|  |  | TDLB100-400 |  | 5.36 | 16 | 6.99 | 9.54 | 2.55 |
|  | 8 | TDLA30-10 | TDLC300-100 | 9.83 | 20 | 5.75 | 10.59 | 4.84 |
|  |  | TDLB100-400 |  | 5.36 | 20 | 6.46 | 8.87 | 2.41 |

Table 2.2-4: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 10 MHz channel bandwidth, 30 kHz SCS with Transform Precoding Disabled with 2 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | (INR1, INR2) [dB] | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1&2 |  |  | IRC | MRC | Gain |
| 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1.61 | 5.16 | 3.55 |
|  |  | TDLB100-400 |  | (7.29, -2.5) | 2 | 0.59 | 2.36 | 1.77 |
|  | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 16 | 8.13 | 13.94 | 5.81 |
|  |  | TDLB100-400 |  | (7.29, -2.5) | 16 | 8.45 | 11.46 | 3.01 |
|  | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 20 | 6.98 | 13.13 | 6.15 |
|  |  | TDLB100-400 |  | (7.29, -2.5) | 20 | 7.45 | 10.77 | 3.32 |

Table 2.2-5: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 5 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled with 1 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  | IRC | MRC | Gain |
| 1 | 2 | TDLB100-400 | TDLC300-100 | 9.83 | 2 | 0.007 | 3.47 | 3.47 |
|  |  |  |  | 5.36 | 2 | -1.52 | 0.28 | 1.8 |
|  | 4 | TDLC300-100 | TDLC300-100 | 9.83 | 16 | 7.65 | 12.53 | 4.88 |
|  |  |  |  | 5.36 | 16 | 6.70 | 9.39 | 2.69 |
|  | 8 | TDLA30-10 | TDLC300-100 | 9.83 | 20 | 5.56 | 10.60 | 5.04 |
|  |  |  |  | 5.36 | 20 | 5.14 | 7.70 | 2.56 |

Table 2.2-6: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 5 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled with 2 Interferers

| TX | RX | Propagation conditions and correlation matrix |  | (INR1, INR2) [dB] | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1&2 |  |  | IRC | MRC | Gain |
| 1 | 2 | TDLB100-400 | TDLC300-100 | (12.38, -0.97) | 2 | 2.45 | 6.02 | 3.57 |
|  |  |  |  | (7.29, -2.5) | 2 | 0.31 | 2.19 | 1.88 |
|  | 4 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 16 | 9.69 | 15.08 | 5.39 |
|  |  |  |  | (7.29, -2.5) | 16 | 8.18 | 11.31 | 3.13 |
|  | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 20 | 6.66 | 13.15 | 6.49 |
|  |  |  |  | (7.29, -2.5) | 20 | 5.95 | 9.58 | 3.63 |

Table 2.2-7: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 10 MHz channel bandwidth, 30 kHz SCS with Transform Precoding Disabled with 1 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  | IRC | MRC | Gain |
| 1 | 2 | TDLB100-400 | TDLC300-100 | 9.83 | 2 | 0.60 | 3.64 | 3.04 |
|  |  |  |  | 5.36 | 2 | -1.23 | 0.40 | 1.63 |
|  | 4 | TDLC300-100 | TDLC300-100 | 9.83 | 16 | 7.85 | 12.70 | 4.85 |
|  |  |  |  | 5.36 | 16 | 6.82 | 9.54 | 2.72 |
|  | 8 | TDLA30-10 | TDLC300-100 | 9.83 | 20 | 5.76 | 10.59 | 4.83 |
|  |  |  |  | 5.36 | 20 | 5.26 | 7.69 | 2.43 |

Table 2.2-8: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 10 MHz channel bandwidth, 30 kHz SCS with Transform Precoding Disabled with 2 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | (INR1, INR2) [dB] | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1&2 |  |  | IRC | MRC | Gain |
| 1 | 2 | TDLB100-400 | TDLC300-100 | (12.38, -0.97) | 2 | 3.07 | 6.18 | 3.11 |
|  |  |  |  | (7.29, -2.5) | 2 | 0.59 | 2.36 | 1.77 |
|  | 4 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 16 | 9.94 | 15.27 | 5.33 |
|  |  |  |  | (7.29, -2.5) | 16 | 8.29 | 11.47 | 3.18 |
|  | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 20 | 6.98 | 13.13 | 6.15 |
|  |  |  |  | (7.29, -2.5) | 20 | 6.13 | 9.56 | 3.43 |

# Conclusions

The simulation results for NR MMSE-IRC PUSCH are delivered.

# References

[1] R4-2419780, “Way Forward for [113] [321] NR_demod_Ph5_Part1_General_BS”, China Telecom, MediaTek, RAN4#113, Orlando, USA.